| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels,BE, 7–17January 2020 | Document: JVET-Q0367 |
| --- | --- |

| Title: | Cleanup on the adaptive colour transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenting Cai, Jianqing Zhu, Kimihiko Kazui | Tel: Email: | +86-21-63350606 caiwenting@cn.fujitsu.com zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution proposes two fixes to make the specification text related to ACT more logical: a) change the inferred value of intra_chroma_pred_mode from 0 to 4 when it is not present, thus the derived chroma mode is inferred to be DM which is aligned with that of an intra CU with ACT applied; b) disable ACT when the three colour components of the 4:4:4 chroma format are coded separately as monochrome sampled pictures.

Fix 1 on the inferred value of intra_chroma_pred_mode

Problem

For intra CUs, the ACT is only enabled when the intra prediction mode of chroma component is DM mode. Therefore, after fixing the bug in the Draft 7[1] as reported in Spec Ticket 780 (https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/780), intra_bdpcm_chroma_flag, cclm_mode_flag, intra_chroma_pred_mode and so on will be not signaled if ACT is used as shown in the bellow Table 1. In this case, cclm_mode_flag and intra_chroma_pred_mode are both inferred to be equal to 0 according to the slice data semantics in spec illustrated in Table 2.

As a result, when cclm_mode_flag and intra_chroma_pred_mode are not present, the mapping chroma prediction mode is planar which could be derived from Specification of IntraPredModeC[ xCb ][ yCb ] depending on cclm_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode in Table 3. However, it is not cosistent with the derivation process for chroma intra prediciton mode when ACT is applied.

Table 1 Syntax related to ACT

7.3.9.5 Coding unit syntax ……. if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) {. …… {. if( !cu_act_enabled_flag ) {. if( cbWidth <= MaxTsSize && cbHeight <= MaxTsSize && sps_bdpcm_chroma_enabled_flag ). intra_bdpcm_chroma_flag; ae(v). if( intra_bdpcm_chroma_flag ). intra_bdpcm_chroma_dir_flag; ae(v). else {. if( CclmEnabled ). cclm_mode_flag; ae(v). if( cclm_mode_flag ). cclm_mode_idx; ae(v). else. intra_chroma_pred_mode; ae(v). }. }. }. }. } ……. …….

Table 2 Semantics of cclm_mode_flag and intra_chroma_pred_mode

7.4.10.5 Coding unit semantics cclm_mode_flag equal to 1 specifies that one of the INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra chroma prediction modes is applied. cclm_mode_flag equal to 0 specifies that none of the INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra chroma prediction modes is applied. When cclm_mode_flag is not present, it is inferred to be equal to 0. intra_chroma_pred_mode specifies the intra prediction mode for chroma samples. When intra_chroma_pred_mode is not present, it is inferred to be equal to 0.

Table 3 Specification of IntraPredModeC[ xCb ][ yCb ] depending on cclm_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode

| cclm_mode_flag | cclm_mode_idx | intra_chroma_pred_mode | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | - | 0 | 66 | 0 | 0 | 0 | 0 |
| 0 | - | 1 | 50 | 66 | 50 | 50 | 50 |
| 0 | - | 2 | 18 | 18 | 66 | 18 | 18 |
| 0 | - | 3 | 1 | 1 | 1 | 66 | 1 |
| 0 | - | 4 | 0 | 50 | 18 | 1 | X |
| 1 | 0 | - | 81 | 81 | 81 | 81 | 81 |
| 1 | 1 | - | 82 | 82 | 82 | 82 | 82 |
| 1 | 2 | - | 83 | 83 | 83 | 83 | 83 |

Proposed method

It is proposed to modify the semantics in Table2

“When intra_chroma_pred_mode is not present, it is inferred to be equal to 0” as “When intra_chroma_pred_mode is not present, it is inferred to be equal to 4” to make the inferred chroma prediction mode into DM accordingly.

Fix 2 on the enabled condition

Problem

In the Draft 7, the SPS flag of ACT is signaled for sequence in the chroma format of 4:4:4 and the ACT flag named cu_act_enabled_flag is signaled for each intra CU in the single tree to indicate whether the color space is selected for coding its residuals in Table 4.

However, when separate_colour_plane_flag is equal to 1 and sps_act_enabled_flag is equal to 1, cu_act_enabled_flag will be signaled of which value should be always equal to 0 for each CU.

Table 4 Syntax of ACT

7.3.2.3 Sequence parameter set RBSP syntax ……. chroma_format_idc; u(2). if( chroma_format_idc = = 3 ). separate_colour_plane_flag; u(1). ……. if( chroma_format_idc = = 3 ) {. sps_act_enabled_flag; u(1). sps_palette_enabled_flag; u(1). }. ……. 7.3.9.5 Coding unit syntax coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) {; Descriptor. chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0. ……. if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_act_enabled_flag && treeType = = SINGLE_TREE ). cu_act_enabled_flag; ae(v). …….

Proposed method

It is proposed to add the check of separate_colour_plane_flag highlighted in yellow to avoid enabling ACT for 4:4:4 sequences coded as monochrome sampled pictures.

7.3.2.3	Sequence parameter set RBSP syntax

| …… |  |
| --- | --- |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| …… |  |
| if( chroma_format_idc = = 3 ) { |  |
| if(separate_colour_plane_flag == 0) |  |
| sps_act_enabled_flag | u(1) |
| sps_palette_enabled_flag | u(1) |
| } |  |
| …… |  |

Conclusions

This proposal proposes two fixes related to ACT to make the specification text more logical and consistent. It is recommended to adopt the proposed fixes to be included into VVC.

References

- B. Bross, J. Chen, and etc, “Versatile Video Coding (Draft 7),” JVET-P2001, Nov 2019.

Patent rights declaration(s)

Fujitsu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).