| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0375-v1 |
| --- | --- |

| Title: | AHG9: A cleanup on the signalling of subpicture IDs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim, Geonjung Ko, Jaehong Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 dongcheol.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a cleanup for the signalling of subpicture identifier. In the 16th JVET meeting, the signalling method of subpicture IDs was adopted. However, even when sps_subpic_id_present_flag is equal to 1 in SPS, it is not guaranteed that subpicture IDs are signaled in at least one of SPS, PPS, and PH. This contribution proposes adding a condition to the semantic of pps_subpic_id_signalling_present_flag or sps_subpic_id_present_flag to signal the subpicture IDs in at least one of SPS, PPS, and PH when sps_subpic_id_present_flag in SPS is equal to 1.

Introduction

In the 16th JVET meeting, the signalling method of the subpicture IDs was adopted. In the current VVC draft [1], the signalling of subpicture IDs is specified in the SPS, PPS, and PH.

Table 1. SPS RBSP syntax

| sps_subpic_id_present_flag | u(1) |
| --- | --- |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

When the value of sps_subpics_id_present_flag is equal to 1 and the value of sps_subpics_id_signalling_present_flag is equal to 1, sps_subpic_id[ i ] is signalled in the SPS.

Table 2. PPS RBSP syntax

| pps_subpic_id_signalling_present_flag | u(1) |
| --- | --- |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |

When the value of pps_subpics_id_signalling_present_flag is equal to 1, pps_subpic_id[ i ] is signalled in the PPS.

Table 3. PH RBSP syntax

| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| --- | --- |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

When the value of sps_subpic_id_present_flag is equal to 1 , the value of sps_subpic_id_signalling_flag is equal to 0 and the value of ph_subpics_id_signalling_present_flag is equal to 1, ph_subpic_id[ i ] is signalled in the PH.

The list SubpicIdList[ i ] is used as the value of slice_subpic_id in the Slice Header RBSP syntax.

7.4.3.6 Picture header RBSP semantics … The list SubpicIdList[ i ] is derived as follows: for( i = 0; i <= sps_num_subpics_minus1; i++ ) SubpicIdList[ i ] = sps_subpic_id_present_flag ? ( sps_subpic_id_signalling_present_flag ? sps_subpic_id[ i ] : ( ph_subpic_id_signalling_present_flag ? ph_subpic_id[ i ] : pps_subpic_id[ i ] ) ) : i

ph_subpic_id[ i ] in the PH overrides pps_subpic_id[ i ] in the PPS if both are present.

However, when sps_subpic_id_present_flag is equal to 1 and sps_subpic_id_signalling_present_flag is equal to 0, sps_subpic_id[ i ] is not signalled in the SPS. With those signalling conditions in the SPS, when pps_subpics_id_signalling_present_flag is equal to 0 and ph_subpics_id_signalling_present_flag is equal to 0, neither pps_subpic_id[i] nor ph_subpic_id[i] is signalled in the PPS or PH.

Proposed Method

In this contribution, it is asserted that the subpicture identifiers shall be signalled when sps_subpic_id_present_flag in the SPS is equal to 1.

For the solution, two options with the green highlights in the semantic below are proposed as follows:

Option 1:

pps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PPS. pps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PPS. When sps_subpic_id_present_flag is 0 or sps_subpic_id_signalling_present_flag is equal to 1, pps_subpic_id_signalling_present_flag shall be equal to 0. When sps_subpic_id_present_flag is equal to 1 and sps_subpic_id_signalling_present_flag is equal to 0, pps_subpic_id_signalling_present_flag shall be equal to 1.

Option 2:

sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is present in the SPS. sps_subpic_id_present_flag equal to 0 specifies that subpicture ID mapping is not present in the SPS.

It is a requirement of bitstream conformance that at least one of sps_subpic_id_signalling_present_flag, pps_subpic_id_signalling_present_flag, and ph_subpic_id_signalling_present_flag shall be equal to 1 when sps_subpic_id_present_flag is equal to 1.

Conclusions

A cleanup of signalling the subpicture IDs is proposed.

- The subpicture identifiers shall be signalled in at least one of the SPS, PPS, and PH when sps_subpic_id_present_flag in the SPS is equal to 1.

References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).