| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0432 |
| --- | --- |

| Title: | On TT or BT split modes disabling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec T. Poirier F. Galpin F. Urban | Tel: Email: | fabrice.leleanne@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution deals with deactivating the use of the TT or BT split modes. The simplest way to deactivate the use of the TT split mode in VVC 7 is to set the maximum TT size parameter to its minimum values, i.e. 0. Then, TT is automatically set to off except for coding units that correspond to the quad-tree leaves with a size equal to the minimum QT size.

This contribution proposes to change the specification of the maximum BT and TT size, in a way that allows selectively deactivating the use of the TT or BT split modes for any coding tree node, allowing to infer some split mode (e.g. TT) to off whatever the quad-tree depth of the considered node.

# Introduction

In VVC 7, the simplest way to deactivate the use of the TT split mode is to set the maximum TT size parameter 0. However, even if TT is automatically set to off, it is not the case for coding units that correspond to the quad-tree leaves with a size equal to the minimum QT size.

Indeed, in VVC draft 7, the maximum TT size is defined relatively to the minimum QT size, under the form of the sps_log2_diff_max_tt_min_qt_intra_slice_luma, sps_log2_diff_max_tt_min_qt_inter_slice, or sps_log2_diff_max_tt_min_qt_intra_slice_chroma  syntax elements.

Thus, for coding tree nodes corresponding to a quad-tree leaf, the deactivation of TT has to be explicitly signaled on the coding tree node level, because it cannot be deactivated at a higher level, hence inferred to off.

# Proposal

The proposed solution do allow the deactivating the use of TT or BT in a simpler way than in VVC 7 is to change the sps_log2_diff_max_tt_min_qt_intra_slice_luma syntax element into the sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma syntax element, the value 0 indicating that the TT split mode is disabled. Indeed, in this case the maximum TT size in strictly inferior to the minimum QT size, so it is never used in intra slice, luma component.

Similarly, the syntax elements sps_log2_diff_max_tt_min_qt_inter_slice and is changed to sps_log2_diff_max_tt_min_qt_plus_one_inter_slice.

Similarly, the syntax elements sps_log2_diff_max_tt_min_qt_intra_slice_chroma and is changed to sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma.

The max BT size related SPS syntax elements are changed exactly in the same way as for the TT split mode. This leads to the new sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma syntax element, sps_log2_diff_max_bt_min_qt_plus_one_inter_slice syntax element, and finally the syntax element  sps_log2_diff_max_bt_min_qt_plus_one_intar_slice_chroma.

# Modified VVC specification: proposal 1

The proposed modified SPS syntax table of JVET-P2001 is shown by the following table Table 1. The proposed modified picture header syntax of JVET-P2001 is shown by the next Table 2.

Table 1: proposal 1 for modified Sequence Parameter Set syntax table on top of JVET-P2001.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_bt_min_qt_plus_one_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_plus_one_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| …… |  |
| } |  |

Table 2: proposal 1 for modified Picture Header syntax table on top of JVET-P2001

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_bt_min_qt_plus_one_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_plus_one_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Additionally, the semantics of the maximum TT size for the case of intra slice luma is modified according to the following:

sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma specifies the default difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU plus one in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by pic_log2_diff_max_tt_min_qt_plus_one_luma present in PHs referring to the SPS. The value of sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY+1, inclusive. When sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma is not present, the value of sps_log2_diff_max_tt_min_qt_ plus_one_intra_slice_luma is inferred to be equal to 0.

Additionally, the semantics of the maximum TT size on the picture header level for the case of intra slice luma is modified according to the following:

pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU plus one in slices with slice_type equal to 2 (I) associated with the PH. The value of pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY + 1, inclusive. When not present, the value of pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma is inferred to be equal to sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma.

Similar modifications are brought to define the semantics, the following syntax elements:

- sps_log2_diff_max_tt_min_qt_plus_one_inter_slice

- sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma

- sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma

- sps_log2_diff_max_bt_min_qt_plus_one_inter_slice

- sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma

- pic_log2_diff_max_tt_min_qt_plus_one_inter_slice

- pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma

- pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma

- pic_log2_diff_max_bt_min_qt_plus_one_inter_slice

- pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma

Finally, the maximum BT and TT sizes are derived according to the following modified equations of JVET-P2001:

MaxBtSizeY =  1  <<  ( MinQtLog2SizeY + pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma -1 )	(123)

MaxBtSizeC = 1  <<  ( MinQtLog2SizeC + pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma -1 )	(124)

MaxTtSizeY = 1  <<  ( MinQtLog2SizeY + pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma -1 )	(125)

MaxTtSizeC = 1  <<  ( MinQtLog2SizeC + pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma -1 )	(126)

# Modified VVC specification: proposal 2

The second proposal for modified SPS and picture header syntax tables of JVET-P2001 is shown by the following Table 3 and Table 4. It consists in removing the redundancy between the maximum MTT depth and the introduced diff_max_{tt,bt}_min_qt_plus_one syntax elements.

As for the intra slice luma case, the maximum MTT hierarchy depth sps_max_mtt_hierarchy_depth_intra_slice_luma is signaled conditionally to the value of  syntax elements sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma and sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma. It is signaled if at least one of these fields is non-zero.

Similar modifications are brought to the signaling of all MTT hierarchy depth syntax elements of the SPS and picture header, see the 2 tables below.

Table 3: proposal 1 for modified Sequence Parameter Set syntax table on top of JVET-P2001.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma | ue(v) |
| if(sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma !=0 ¦¦ sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma !=0 ) { |  |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_bt_min_qt_plus_one_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_plus_one_inter_slice | ue(v) |
| if(sps_log2_diff_max_bt_min_qt_plus_one_inter_slice !=0 ¦¦ sps_log2_diff_max_tt_min_qt_plus_one_inter_slice !=0 ) { |  |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| if(sps_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma !=0 ¦¦ sps_log2_diff_max_tt_min_qt_plus_one_intra_slice_croma !=0 ) { |  |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| …… |  |
| } |  |

Table 4: proposal 1 for modified Picture Header syntax table on top of JVET-P2001

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_luma | ue(v) |
| if( pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma != 0 ¦¦ pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_luma != 0) { |  |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_bt_min_qt_plus_one_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_plus_one_inter_slice | ue(v) |
| if( pic_log2_diff_max_bt_min_qt_plus_one_inter_slice != 0 ¦¦ pic_log2_diff_max_bt_min_qt_plus_one_inter_slice != 0) { |  |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_plus_one_intra_slice_chroma | ue(v) |
| if( pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma != 0 ¦¦ pic_log2_diff_max_bt_min_qt_plus_one_intra_slice_chroma != 0) { |  |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Conclusion

This contribution proposes a slightly modified signaling of the maximum BT and TT sizes compared to JVET-P2001. It leads to simpler way to deactivate the use of TT or BT compared to VVC draft 7.

In particular, it allows inferring the deactivated TT or BT split mode to off for any coding tree node or leaf, whatever the quad-tree or multi-type tree depth of the considered coding tree node/leaf. This is not possible with the VVC 7 specification, where explicitly coding the concerned TT or BT split mode to 0 at the CU level is needed to fully deactivate the TT or BT split mode.

It is proposed to adopt the proposed modified maximum BT/TT size signaling into the next version of the VTM and VVC specification.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).