| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0441 |
| --- | --- |

| Title: | SAO Modification for 12-bit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Karl Sharman Steve Keating | Email: | adrian.browne@sony.com karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document proposes a change to the formula for the offset for SAO.  This offset ensures SAO is applied with the same relative magnitude for bit depths greater than 10 as it is at bit depths of 8 and 10.

# Introduction

Section 7.4.10.3 of the specification[1] states:

“SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = ( 1 − 2 * sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] ) *	(155)
		sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]”

Where sao_offset_abs is defined in Table 123 in Section 9.3.3.1 as:

“cMax = ( 1  <<  ( Min( BitDepth, 10 ) − 5 ) ) – 1”

This ensures that the relative offset and thus effect of Sample Adaptive Offset (SAO) is the same for both 8 and 10 bit processing.  However for 12 bit processing the maximum offset is 4 times smaller in relative terms.  This means processing the same content, but at different bit depths, the offset is applied differently, and SAO is chosen much less frequently.  Given that SAO is a primarily a subjective tool, this document proposes to modify the formula for the offset so that bit depths greater than 10 can behave in a similar way to those of 8 or 10 bit.  This allows the number of times SAO is used to approach the number of times it is used for 8 and 10 bit processing  (Table 1), more-or-less, continuing the trend lines, as shown in Figure 1.

The modification has no impact for 10 bit profiles; however, with the prospect of 12 bit profiles being added, for example, for HDR, this will help to allow SAO to function for such higher bit depth spaces.

|  | SAO usage count at given bit depth (K) |  |  |  | As a fraction of the 10 bit usage count |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 10 | 12 | 12 with modification | 12 | 12 with modification |
| AI | 1512 | 1208 | 771 | 1068 | 64% | 88% |
| RA | 2289 | 1956 | 1149 | 1577 | 59% | 81% |
| LDB | 1167 | 926 | 520 | 780 | 56% | 84% |

Table 1. Number of uses of SAO for blocks within CTC tests (AI, RA, LDB) at different bit depths

Figure 1 -Usage count of SAO for the CTCs, showing VTM vs the modification.

# Proposal

It is proposed that equation 155 above becomes:

“SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = ( 1 − 2 * sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] ) *	(155)
		(sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] << (bitDepth – Min(10, bitDepth)) ”

This introduces a shift left in bit depths above 10 equal to the bit depth minus 10.  This ensures the maximum SAO offset relative to the bit depth remains constant.

# Results

Using CTC conditions[2] excepting setting InternalBitDepth and MaxBitDepthConstraint to 12 for both the anchor and the test, the following results were obtained:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM7.0_12bit |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | -0.13% | -0.17% | -0.01% | 101% | 101% |
| Class A2 | 0.02% | -0.27% | -0.16% | -0.01% | 100% | 100% |
| Class B | -0.01% | -0.07% | -0.15% | -0.02% | 100% | 100% |
| Class C | 0.03% | -0.24% | -0.34% | -0.02% | 101% | 102% |
| Class E | -0.03% | 0.00% | -0.02% | -0.03% | 101% | 102% |
| Overall | 0.01% | -0.14% | -0.17% | -0.02% | 100% | 101% |
| Class D | -0.03% | -0.08% | -0.11% | -0.04% | 98% | 97% |
| Class F | -0.15% | -0.30% | -0.45% | -0.16% | 100% | 101% |
| Class TGM | -0.66% | -0.35% | -0.39% | -0.54% | 101% | 101% |

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM7.0_12bit |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | 0.04% | -0.05% | -0.03% | 100% | 100% |
| Class A2 | -0.05% | -0.21% | -0.20% | -0.08% | 100% | 100% |
| Class B | -0.05% | -0.02% | -0.15% | -0.05% | 100% | 100% |
| Class C | -0.03% | -0.16% | -0.17% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.04% | -0.08% | -0.15% | -0.05% | 100% | 100% |
| Class D | 0.11% | -0.05% | 0.07% | 0.10% | 99% | 100% |
| Class F | -0.20% | -0.17% | -0.29% | -0.17% | 100% | 99% |
| Class TGM | -0.73% | -0.26% | -0.30% | -0.53% | 100% | 100% |

# Discussion

It is understood that SAO is primarily a subjective-based tool, and not objective. This is shown in previous AHG13 studies, such as presented in JVET-P0013, where SAO showed only a 0.08%, 0.28%, 0.35% in BD-rate for RA (Y, U, V, respectively) for the tool off test of VTM 6.

The following two tables show the AI and RA AHG13 tool off results for VTM7.0, as provided at https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-7.0/Results/.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SAO tool off Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.22% | 0.12% | 100% | 96% |
| Class A2 | 0.02% | 0.30% | 0.17% | 100% | 94% |
| Class B | 0.02% | 0.05% | 0.17% | 100% | 96% |
| Class C | -0.02% | 0.28% | 0.38% | 100% | 96% |
| Class E | 0.03% | -0.08% | -0.05% | 100% | 96% |
| Overall | 0.00% | 0.15% | 0.17% | 100% | 96% |
| Class D | -0.01% | 0.18% | 0.21% | 100% | 96% |
| Class F | 0.36% | 0.35% | 0.50% | 103% | 96% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SAO tool off Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.13% | 0.22% | 101% | 96% |
| Class A2 | 0.18% | 0.35% | 0.29% | 100% | 99% |
| Class B | 0.11% | 0.21% | 0.52% | 100% | 99% |
| Class C | -0.01% | 0.12% | 0.19% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.20% | 0.33% | 100% | 98% |
| Class D | -0.15% | -0.06% | -0.24% | 100% | 99% |
| Class F | 0.53% | 0.39% | 0.53% | 101% | 98% |

These tables for 10 bit can be compared with the 12 bit tool off results, without the proposed change, given below.  These show that the chroma AI gains achieved by SAO for 10 bit processing are replaced by small losses.

|  |  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 12bit (without SAO modification) |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.01% | 0.03% | -0.24% | -0.02% | 99% | 98% |
| Class A2 | 0.04% | -0.02% | -0.02% | 0.01% | 100% | 98% |
| Class B | 0.02% | -0.04% | -0.03% | 0.01% | 99% | 97% |
| Class C | 0.00% | 0.02% | 0.00% | 0.01% | 101% | 101% |
| Class E | 0.00% | -0.08% | -0.08% | -0.01% | 99% | 98% |
| Overall | 0.01% | -0.02% | -0.07% | 0.00% | 100% | 98% |
| Class D | -0.03% | 0.04% | 0.04% | -0.01% | 101% | 99% |
| Class F | 0.21% | -0.02% | 0.01% | 0.13% | 100% | 99% |
| Class TGM | 1.28% | 0.04% | 0.08% | 0.81% | 100% | 97% |

|  |  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 12bit SAO (without SAO modification) |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | 0.05% | -0.06% | 0.00% | 100% | 100% |
| Class A2 | 0.10% | 0.06% | -0.01% | 0.07% | 100% | 99% |
| Class B | 0.04% | 0.17% | 0.22% | 0.05% | 100% | 99% |
| Class C | -0.04% | 0.06% | 0.10% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.03% | 0.10% | 0.08% | 0.03% | 100% | 99% |
| Class D | -0.01% | -0.13% | 0.14% | -0.01% | 100% | 100% |
| Class F | 0.26% | -0.05% | 0.01% | 0.19% | 100% | 100% |
| Class TGM | 1.32% | 0.24% | 0.22% | 0.89% | 101% | 101% |

The final set of tables given below are 12 bit tool off tests with the proposed change applied.  These results correspond much more closely with the 10 bit results than the original 12 bit results.  They also show that the overall chroma losses noted for the original 12 bit results have been replaced with gains.

|  |  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 12 bit (with SAO modification) |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | 0.16% | -0.07% | 0.00% | 100% | 98% |
| Class A2 | 0.01% | 0.25% | 0.14% | 0.02% | 101% | 99% |
| Class B | 0.02% | 0.03% | 0.12% | 0.04% | 98% | 96% |
| Class C | -0.03% | 0.27% | 0.34% | 0.03% | 102% | 100% |
| Class E | 0.03% | -0.08% | -0.07% | 0.02% | 99% | 97% |
| Overall | 0.00% | 0.12% | 0.11% | 0.02% | 100% | 98% |
| Class D | 0.00% | 0.12% | 0.15% | 0.03% | 101% | 100% |
| Class F | 0.36% | 0.29% | 0.47% | 0.29% | 99% | 97% |
| Class TGM | 2.00% | 0.39% | 0.48% | 1.37% | 101% | 98% |

|  |  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 12 bit (with SAO modification) |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.05% | 0.01% | -0.01% | 0.04% | 101% | 100% |
| Class A2 | 0.15% | 0.27% | 0.20% | 0.15% | 99% | 98% |
| Class B | 0.10% | 0.19% | 0.37% | 0.11% | 100% | 99% |
| Class C | -0.01% | 0.23% | 0.27% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.07% | 0.18% | 0.23% | 0.08% | 100% | 99% |
| Class D | -0.12% | -0.08% | 0.07% | -0.12% | 101% | 100% |
| Class F | 0.47% | 0.12% | 0.30% | 0.35% | 101% | 100% |
| Class TGM | 2.09% | 0.50% | 0.52% | 1.43% | 101% | 101% |

# Conclusion

A modification is proposed to the formula for the offset for SAO.  This modification has no effect for bits depths of 10 or less, but ensures the relative amplitude of the offset at bit depths greater than 10 is the same as for a bit depth of 10. Tool off tests and an analysis of the number of uses of SAO indicate that when compared to SAO without the modification, SAO with the modification is much more closely aligned to the accepted 10 bit version of SAO.

# Further considerations

Further work is focussed on the drop of usage of SAO between 8 bit processing and 10 bit processing.  Amongst the techniques being investigated is limiting sao_offset_abs to 7 for 10 bit.  This is a variation of the technique described earlier but uses three rather than 5 bits for the offset, with a similar left shift of two bits to ensure the relative magnitude of the offsets are the same for 8 and 10 bit processing.

A further technique being investigated is replacing the current unary coding of the offset with an exponential Golomb coding. This scheme could also use the maximum bit length of the offset to remove the need to code an end to the exponential component for the largest values.

Tentative simulation results do not show any obvious advantage of these alternative techniques.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE the 16th JVET meeting, October 2019, Geneva, CH

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).