| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0447-v2 |
| --- | --- |

| Title: | Encoder estimation of Weighted-Prediction parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, T. Poirier, F. Le Léannec | Email: | philippe.bordes@interdigital.com tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

The current VTM-7.0 encoder software implements 5 methods for estimating WP parameters.

The methods 0,1 use AC and DC of the original and reference pictures to derive WP parameters. The methods 2,3,4 use histograms of the original and reference pictures to refine the WP parameters computed with AC, DC.

All these methods use sum of absolute difference (SAD) between the original and the corrected reference samples to finally select the best WP parameters.

To be more consistent, it is proposed one alternative histogram-based method replacing SAD with histogram distortion for selecting the best WP parameters.

It is reported that compared to the VTM-7.0 method-2, the proposed method provides BD-rate gains of x.xx%(Y), x.xx%(U), x.xx%(V) / x.xx%(Y), x.xx%(U), x.xx%(V) in RA / LDB configurations with fade-to-black sequences, and x.xx%(Y), x.xx%(U), x.xx%(V) / x.xx%(Y), x.xx%(U), x.xx%(V) in RA / LDB configurations with fade-to-white sequences.

The encoder and decoder complexity are unchanged compared to anchors.

# Introduction / Problem Statement

The current VTM-7.0 encoder software implements 5 methods for estimating WP parameters.

Two methods use AC and DC of the original and reference pictures to derive WP parameters:

- Method-0: scale w is estimated as ACorig / ACref and offset is estimated as DCorig – w.DCref . The sum of absolute difference (SAD) between the original and the corrected reference samples of all components is used to finally decide whether enabling WP or not (WP default weights candidate) for all component.

- Method-1: same as method-0 but using the sum of absolute difference (SAD) between the original and the corrected reference samples of each components to finally decide whether enabling WP or not (WP default weights candidate) for each component.

The three methods 2,3,4 use histogram-distortion between histograms of the original and reference pictures to refine the WP parameters estimated with AC/DC method and create another WP parameter candidate:

- Method-2: The sum of absolute difference (SAD) between the original and the corrected reference samples of each component is used to finally select best WP parameter candidate for each component.

- Method-3: same as method-2, but SAD is computed with clipping of the out-of-range corrected samples.

- Method-4: same as method-3, plus test refining from default WP values too (additional candidate).

# Proposal

It turns out that the use of SAD to select the best WP parameter candidates is very sensitive to motion in the video sequence since it measures the differences between the original samples and the co-located corrected reference samples. The histogram-based methods (2,3,4) are supposed to get ride of this by using histogram distortion rather SAD to refine the WP parameters. However, the methods 2,3,4 still use SAD for selecting best WP parameter candidates finally.

To be more consistent with histogram-based intent, it is proposed to also use histogram distortion for selecting the best WP parameters. It is also proposed to replace histograms with cumulated histograms and possibly use histogram-based method for luma and SAD method for chroma (method-5B).

Figure 1 – synoptic of the current encoder algorithm for estimating WP parameters (black) and the added modes in blue.

# Experimental results

Since weighted prediction is not used on CTC, the experiments by using weighted prediction and fade-sequences have been conducted in this contribution.

The fade sequences are generated with tool provided in [1]. In our experiments, a linear fade is applied to the about first 2 seconds of the regular sequence set, and the experiments are conducted with them. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs.

The anchors are VTM-7.0 with WP enabled, using method-2. Note that the methods 3,4 are variant to method-2 (clipping + additional refined WP parameters from defaults) and have results very close to the method-2. The method-5B is same as method-5A but using histogram-based method for luma and SAD method for chroma.

Table 1: summary of the proposed method for estimating WP (fade-to-black and fade-to-white sequences)(method-5A).

|  | Proposal difference with VTM-7.0 (Fade to black) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.03% | -1.91% | -1.02% | 101% | 97% |  |  |  |  |  |
| Class A2 | -0.67% | 2.77% | 4.42% | 99% | 97% |  |  |  |  |  |
| Class B | -1.44% | 0.71% | 0.42% | 103% | 125% | -0.45% | 2.81% | 0.18% | 101% | 110% |
| Class C | -3.31% | -4.15% | -5.59% | 98% | 102% | -1.03% | -1.92% | -2.16% | 96% | 95% |
| Class E |  |  |  |  |  | -1.15% | -2.85% | -7.14% | 95% | 92% |
| Overall | -1.70% | -0.70% | -0.67% | 100% | 107% | -0.82% | -0.18% | -2.43% | 98% | 100% |
| Class D | -2.42% | -2.67% | -4.23% | 98% | 100% | -0.56% | 2.20% | 0.61% | 99% | 94% |
| Class F | -2.70% | -1.72% | -2.50% | 97% | 96% | -0.54% | -1.21% | -1.03% | 102% | 102% |

|  | Proposal difference with VTM-7.0 (Fade to white) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -1.59% | -1.18% | 98% | 96% |  |  |  |  |  |
| Class A2 | -0.36% | 2.62% | 4.64% | 96% | 97% |  |  |  |  |  |
| Class B | -1.23% | 0.86% | 0.03% | 102% | 115% | -0.79% | 3.46% | -0.14% | 100% | 114% |
| Class C | -2.63% | -4.39% | -5.11% | 100% | 105% | -1.00% | -1.52% | -2.36% | 100% | 100% |
| Class E |  |  |  |  |  | -1.77% | -2.39% | -4.10% | 93% | 90% |
| Overall | -1.28% | -0.68% | -0.66% | 100% | 105% | -1.10% | 0.34% | -1.87% | 98% | 103% |
| Class D | -1.79% | -2.26% | -3.99% | 98% | 98% | -0.52% | 0.58% | 0.71% | 100% | 97% |
| Class F | -3.75% | -2.25% | -3.28% | 95% | 92% | -0.16% | -1.08% | -1.41% | 101% | 98% |

Table 2: summary of the proposed method for estimating WP (fade-to-black and fade-to-white sequences) with variant-B.

|  | Proposal difference with VTM-7.0 (Fade to black) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.59% | -0.25% | 0.28% | 100% | 93% |  |  |  |  |  |
| Class A2 | -0.70% | -0.28% | -0.34% | 99% | 88% |  |  |  |  |  |
| Class B | -1.16% | -1.09% | -0.81% | 105% | 108% | -0.69% | 1.59% | 1.74% | 101% | 100% |
| Class C | -3.01% | -2.69% | -2.58% | 100% | 115% | -0.60% | -0.54% | 1.42% | 99% | 106% |
| Class E |  |  |  |  |  | -0.29% | 0.42% | -0.27% | 97% | 94% |
| Overall | -1.45% | -1.19% | -0.97% | 101% | 103% | -0.56% | 0.59% | 1.13% | 99% | 100% |
| Class D | -2.52% | -2.57% | -2.46% | 97% | 97% | -1.05% | 1.01% | 1.48% | 98% | 94% |
| Class F | -2.40% | -0.79% | 0.21% | 94% | 84% | -0.46% | 0.53% | 1.73% | 101% | 95% |

|  | Proposal difference with VTM-7.0 (Fade to white) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | 0.49% | 0.19% | 98% | 91% |  |  |  |  |  |
| Class A2 | -0.27% | 0.00% | 0.02% | 95% | 85% |  |  |  |  |  |
| Class B | -0.98% | -1.20% | -0.91% | 102% | 110% | -0.59% | 1.94% | 1.21% | 100% | 100% |
| Class C | -2.48% | -2.41% | -2.44% | 103% | 118% | -0.66% | -0.36% | 0.37% | 102% | 104% |
| Class E |  |  |  |  |  | -1.53% | -0.29% | 2.16% | 94% | 90% |
| Overall | -1.10% | -0.95% | -0.91% | 100% | 103% | -0.85% | 0.61% | 1.17% | 99% | 99% |
| Class D | -2.08% | -1.65% | -1.94% | 96% | 93% | -1.10% | -0.52% | 1.23% | 99% | 93% |
| Class F | -3.64% | -1.45% | -1.18% | 92% | 82% | -0.23% | 0.14% | 0.78% | 101% | 95% |

# Conclusion

It is proposed another encoder method for estimating WP parameters. It is a variant to the existing histogram-based methods where the selection of the best WP parameter candidates is made using histogram-distortion rather than using SAD.

# References

- P. Bordes, E. François and F. L. Léannec, AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences, Joint Video Exploration Team Document, JVET-L0201, Macao, Oct. 2018.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).