| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0450-v2 |
| --- | --- |

| Title: | On fixed sW and fO in MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Haixin Wang, Yu Sun, Yanzhuo Ma, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | jyhuo@mail.xidian.edu.cn hxwang_2@stu.xidian.edu.cn ysun_xd@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd. |  |  |

_____________________________

- Abstract

In this document, it is proposed 3 MIP modifications including sign change, fixed fO and fixed sW. Specifically, sign change is to change the sign of input value p[0]; fixed fO is to remove the table of fO in VVC and use a fixed value; fixed sW is to remove the table of sW in VVC and use a fixed shift value.

Three Tests are conducted.

For test1, sign change (the input value p[0] modification), the prediction is identical with the prediction of VVC7 and the coding performance is identical with that of VTM7.0. The experimental results are as below:

For AI configuration: 0.00%, 0.00%, and 0.00%, with 100% EncT, 100% DecT;

For RA configuration: 0.00 %, 0.00%, and 0.00%, with 100% EncT, 100% DecT.

For test2, sign change and fixed fO are included, the experimental results are as below:

For AI configuration: 0.00%, 0.00%, and 0.01%, with 100% EncT, 101% DecT;

For RA configuration: 0.01 %, 0.09%, and 0.07%, with 100% EncT, 100% DecT.

For test3, sign change, fixed fO and fixed sW are all included, the experimental results are as below:

For AI configuration: 0.04%, -0.01%, and -0.01%, with 100% EncT, 100% DecT;

For RA configuration: 0.02 %, -0.01%, and 0.06%, with 100% EncT, 100% DecT.

Introduction

Input value p[i] is calculated by:

- If mipSizeId is equal to 2, the following applies:

p[ i ] = pTemp[ i + 1 ] − pTemp[ 0 ]	for i = 0..inSize – 1

- Otherwise (mipSizeId is less than 2), the following applies:

p[ 0 ] = pTemp[ 0 ] − ( 1  <<  ( BitDepth − 1 ) )
p[ i ] = pTemp[ i ] − pTemp[ 0 ]	for i = 1..inSize – 1

In VTM7.0, matrix-based intra prediction (MIP) is applied to derive the intra predicted samples of luma component as：

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*predSize+x *p i  ) +            
			( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i ) ) >> sW ) + pTemp[ 0 ]

wherein sW and fO is determined using Table 1 and Table 2 as below:

Table 1 – table of sW in VVC draft [1]

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 6 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 5 | 6 | 5 | 5 | 5 |
| 1 | 7 | 7 | 6 | 6 | 6 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |
| 2 | 6 | 7 | 5 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |  |  |

Table 2 – table of fO in VVC draft [1]

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 1 | 28 | 28 | 42 | 56 | 22 | 9 | 6 | 35 | 14 | 50 | 66 | 29 | 50 | 31 | 19 |
| 1 | 15 | 14 | 23 | 45 | 10 | 14 | 10 | 27 |  |  |  |  |  |  |  |  |
| 2 | 15 | 19 | 46 | 16 | 14 | 11 |  |  |  |  |  |  |  |  |  |  |

Proposed Methods

Test1: sign change

As proposed in [2], when MipSizeId is equal to 0 or 1, it is proposed to change the calculation of p[0]:

p[ 0 ] = pTemp[ 0 ] − ( 1  <<  ( BitDepth − 1 ) )

by

p[ 0 ] =  ( 1  <<  ( BitDepth − 1 ) ) − pTemp[ 0 ].

Mathematically, sW, fO, and mWeight are also changed correspondingly to make the calculation of predMip[ x ][ y ] with the new p[0] identical to the one in VVC draft.

wherein sW is same as VVC draft, while fO is determined using Table 3 as below:

Table 3 – table of fO in test1

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 9 | 9 | 16 | 20 | 29 | 22 | 9 | 6 | 16 | 14 | 21 | 26 | 19 | 13 | 28 | 17 |
| 1 | 15 | 14 | 23 | 24 | 10 | 14 | 10 | 27 |  |  |  |  |  |  |  |  |
| 2 | 15 | 19 | 46 | 16 | 14 | 11 |  |  |  |  |  |  |  |  |  |  |

Test2: sign change and fixed fO

Based on test1, by changing three sWs from 7 to 6, it is proposed to remove fO table. In test2, fO is set as 46 [2]. To easy to implement the fO multiplification, [3] proposed to use a offset multiplication with a shift operation. Here, we can also set fO to 48. And the mWeight is changed correspondingly. 48 can be designed as (1<<5) + (1<<4).

wherein sW is determined using Table 4 as below.

Table 4 – table of sW in test2

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 6 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 5 | 6 | 5 | 5 | 5 |
| 1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |
| 2 | 6 | 6 | 5 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |  |  |

Test3: sign change, fixed fO and fixed sW

Based on test1, it is proposed to always set sW as 5 [4]and to always set fO as 46 or 48. Thus, sW and fO tables are removed. Here, mWeight is also changed correspondingly.

Experimental results

The experiments were performed based on VTM7.0 software [5] under JVET common test condition [6]. Table 5 ~ Table 7 present a summary of the results.

Table 5. Simulation results of test1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 6. Simulation results of test2

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.19% | 0.09% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | -0.07% | 100% | 101% |
| Class B | 0.01% | 0.05% | -0.04% | 100% | 100% |
| Class C | -0.01% | 0.11% | 0.03% | 100% | 102% |
| Class E | 0.00% | -0.03% | 0.07% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class D | 0.00% | -0.08% | -0.11% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.19% | 100% | 100% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | 0.05% | 100% | 100% |
| Class A2 | -0.02% | 0.08% | 0.06% | 101% | 100% |
| Class B | 0.00% | 0.22% | 0.15% | 100% | 100% |
| Class C | 0.02% | 0.00% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.07% | 100% | 100% |
| Class D | 0.00% | -0.12% | -0.01% | 100% | 100% |
| Class F | 0.02% | 0.13% | 0.06% | 100% | 100% |

Table 7. Simulation results of test3

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.10% | 0.14% | 100% | 100% |
| Class A2 | 0.03% | 0.00% | -0.05% | 100% | 101% |
| Class B | 0.03% | 0.01% | -0.07% | 100% | 100% |
| Class C | 0.02% | 0.03% | -0.03% | 100% | 101% |
| Class E | 0.05% | -0.01% | 0.00% | 99% | 99% |
| Overall | 0.04% | -0.01% | -0.01% | 100% | 100% |
| Class D | 0.02% | -0.07% | -0.06% | 100% | 100% |
| Class F | 0.01% | 0.02% | 0.04% | 99% | 100% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.08% | 0.03% | 100% | 100% |
| Class A2 | -0.03% | 0.05% | -0.02% | 101% | 100% |
| Class B | 0.02% | 0.09% | 0.25% | 100% | 100% |
| Class C | 0.04% | -0.11% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | 0.06% | 100% | 100% |
| Class D | -0.02% | -0.23% | -0.06% | 100% | 100% |
| Class F | -0.01% | 0.09% | 0.09% | 99% | 99% |

Conclusions

In this contribution, based on sign change, the fO table and/or sW table are removed. Thus, it is proposed to remove the dependency of fO and/or sW on the modeId and the mipSizeId, which introduce extremely small coding loss compared to VTM7.0. We propose to adopt these simiplifications into VVC.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] J. Huo, H. Wang, Y. Sun, Y. Ma, S. Wan, F. Yang, Y. Yu, Y. Liu,  Non-CE 3: MIP simplification, Document of JVET-P0136, Geneva, Switzerland, October 2019

[3] T. Biatek, A.K. Ramasubramonian, G. Van Der Auwera, M. Karczewicz,  Non-CE3: simplified MIP with power-of-two offsets, Document of JVET-P0625, Geneva, Switzerland, October 2019

[4] Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki, Junyan Huo, Yanzhuo Ma, Fuzheng Yang, Shuai Wan and Yuanfang Yu, “CE3-2: On rounding shift of MIP”, Document of JVET-P0056, Geneva, Switzerland, October 2019

[5] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

[6] F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document  JVET-N1010, Geneva, Switzerland, March 2019

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).