| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET- Q0463 |
| --- | --- |

| Title: | CE3-related: Inter BDPCM with RRC for lossless video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gosala Kulupana, Saverio Blasi 201 Wood Lane, The Lighthouse W12 7TQ, London, UK | Email: | gosala.kulupana@bbc.co.uk saverio.blasi@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to apply BDPCM on inter-predicted residuals for lossless coding. The proposed method is an extension to Regular Residual Coding (RRC) based BDPCM described in CE3-2.4 in JVET-Q0089 [1]. Identically to intra BDPCM, the proposed inter BDPCM works on quantized residuals. Therefore, the tool re-uses entirely the residual prediction operations used in the intra BDPCM pipeline. Furthermore, the proposed inter BDPCM operation is applied both for luma and chroma residuals. As with intra BDPCM, when inter BDPCM is used, residuals are not subjected to any transform operation (i.e., Transform Skip is used). When the proposed inter BDPCM is integrated into CE3-2.4, an average BD-rates of -6.97% is reported for RA with 122% encoding time and 99% decoding time with respect to lossless VTM 7.0 anchor.

# Proposed method

The current VVC draft specification makes use of intra BDPCM [2] for screen content video sequences and during the 16th JVET meeting in Geneva, there were few proposals proposing to use the tool for lossless video coding. This contribution proposes to apply BDPCM operation on inter-predicted residuals similar to intra BDPCM in VTM-7.0 [3]. Related algorithms can be described as following.

For a block of size M (rows) × N (cols),  let ri,j, 0≤i≤M-1, 0≤j≤N-1rij, 0≤i≤M-1, 0≤j≤(N-1) be the prediction residual after performing inter prediction. Let Q(ri,j), 0≤i≤M-1, 0≤j≤N-1 denote the quantized version of the residual ri,j, where residual is difference between original block and the predicted block values. Then the block DPCM is applied to the quantized residual samples, resulting in modified M × N array R with elements ri,j. When vertical BDPCM is signalled:

ri,j=Q(ri,j),i=0,  0≤j≤(N-1)Q(ri,j)-Q(r(i-1),j),1≤i≤(M-1),  0≤j≤(N-1).

For horizontal prediction, similar rules apply, and the residual quantized samples are obtained by

ri,j=Q(ri,j), 0≤i≤M-1,  j=0Q(ri,j)-Q(ri,(j-1)),0≤i≤(M-1),  1≤j≤(N-1).

The residual quantized samples ri,j are sent to the decoder.

On the decoder side, the above calculations are reversed to produce Q(ri,j), 0≤i≤M-1, 0≤j≤N-1Q(rij), 0≤i≤M-1, 0≤j≤(N-1). For vertical BDPCM case,

Q(ri,j)=k=0irk,j,0≤i≤(M-1),  0≤j≤(N-1).

For horizontal case,

Q(ri,j)=k=0jri,k,0≤i≤(M-1),  0≤j≤(N-1).

The inverse quantized quantized residuals, Q-1Qri,j, are added to the inter block prediction values to produce the reconstructed sample values.

When using inter BDPCM, no transform is applied to the residuals, namely Transform Skip is used.

Signalling of inter BDPCM requires a flag for identifying whether a block is BDPCM-predicted or not. Moreover, in case inter BDPCM is used in a block, a further flag is needed to identify which direction (vertical or horizontal) inter BDPCM is performed on.

The spec changes are attached:

# Performance results

The performance of the proposed method (combination of inter and intra BDPCM with RRC) is summarised in Table 1. The tests were performed using the JVET lossless test conditions [4] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 7.0 [3], compiled with gcc 4.8.2. All experiments were run using AVX.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | VTM-7.0 with intra BDPCM=0 | inter BDPCM |  | VTM-7.0 with intra BDPCM=0 | inter BDPCM |  | VTM-7.0 with intra BDPCM=0 | inter BDPCM |  |
| Class A1 |  |  |  | 2.2 | 2.4 | -7.41% |  |  |  |
| Class A2 |  |  |  | 1.7 | 1.9 | -9.87% |  |  |  |
| Class B |  |  |  | 2.3 | 2.5 | -5.16% | 2.3 | 2.5 | -5.28% |
| Class C |  |  |  | 2.4 | 2.6 | -6.71% | 2.4 | 2.6 | -6.70% |
| Class D |  |  |  | 2.8 | 3.0 | -7.13% | 2.7 | 3.0 | -7.17% |
| Class E |  |  |  |  |  |  | 3.1 | 3.4 | -9.15% |
| Class F |  |  |  | 33.7 | 36.3 | -7.64% | 50.7 | 53.4 | -7.35% |
| TGM |  |  |  | 107.1 | 110.8 | -3.80% | 124.9 | 126.3 | -2.41% |
| Overall |  |  |  | 2.2 | 2.4 | -6.97% | 2.6 | 2.8 | -6.72% |
| Enc Time[%] |  |  |  | 122% |  |  | 122% |  |  |
| Dec Time[%] |  |  |  | 99% |  |  | 105% |  |  |

Table 1 - Performance of intra and inter BDPCM with RRC compared to VTM-7.0 with intra BDPCM =0.

# References

- A. Nalci, H. Wang, M. Coban, M. Karczewicz (Qualcomm), T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony), Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang (Kwai) “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding”, JVET-P0089, Brussels, Jan. 2020.

- M. Karczewicz, and M. Coban, “CE8-related: Quantized residual BDPCM”, JVET-N0413, Geneva, March 2019.

- VTM-7.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, November 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).