| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0468 |
| --- | --- |

| Title: | Constraint on minimum CU size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

It is proposed to allow minimum CU size setting in VTM software. It is further proposed to set the range of minimum CU size value in the range of {4, 8, 16} in luma samples, according to the investigation of the performance impact of applying minimum coding unit sizes on VTM-7.0. It is reported that compared with VTM-7.0 anchor with minimum CU size of 4, the following minimum CU size settings have performance impact (BD-rate Y/Enc Time/Dec Time) as following.

- Min CU size 8: AI: 2.66% / 54% / 95%; RA: 2.26% / 78% / 104%; LB: 1.96% / 80% / 104; LP 2.61% / 78% / 103%.

- Min CU size 16: AI: 10.36% / 18% / 85%; RA: 12.04% / 54% / 102%; LB 11.79%% / 59% / 103%; LP 13.43% / 55% / 101%;

- Min CU size 32: AI: 23.75% / 7% / 78%; RA: 35.60% / 29% / 95%; LB 40.10% / 33% / 97%; LP 43.59% / 30% / 95%;

# Introduction

In current VVC working draft the minimum CU size may be set to 4 to 128 luma samples, inclusive. But setting minimum CU size to a very large value may be not practical for a video encoder and decoder. Moreover, minimum CU size is fixed to 4 and cannot be changed in VTM-7. This proposal implemented the minimum CU setting and further studied the performance impact of applying different minimum CU sizes. It is proposed to limit the value range of the minimum CU size.

# Tests of Minimum CU Size

Based on the implementation of minimum CU size on top of VTM-7, the range of minimum CU sizeis investigated by evaluating the tradeoff between runtime and coding efficiency.

# Experimental Results

The minimum CU size related software changes were implemented on top of VTM-7.0. Simulations were conducted following JVET common test conditions. VTM-7.0 with default minimum CU size of 4 luma samples was used as anchor.

As shown in Table 3.1, it was observed that with Min CU size 8, performance impact is: AI: 2.66% / 54% / 95%; RA: 2.26% / 78% / 104%; LB: 1.96% / 80% / 104; LP 2.61% / 78% / 103%.

Table 3.1 Simulation Results of Min CU Size 8

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.57% | -0.22% | 0.00% | 64% | 100% |
| Class A2 | 1.14% | 0.61% | 0.55% | 57% | 98% |
| Class B | 2.05% | 1.35% | 1.34% | 51% | 94% |
| Class C | 5.03% | 2.94% | 2.88% | 49% | 90% |
| Class E | 4.14% | 2.28% | 2.41% | 52% | 95% |
| Overall | 2.66% | 1.47% | 1.51% | 54% | 95% |
| Class D | 5.67% | 3.52% | 3.90% | 47% | 88% |
| Class F | 10.47% | 8.43% | 8.70% | 38% | 89% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.51% | 0.22% | 0.65% | 83% | 106% |
| Class A2 | 1.47% | 1.89% | 1.92% | 80% | 105% |
| Class B | 1.82% | 2.18% | 2.35% | 78% | 103% |
| Class C | 4.72% | 4.27% | 4.81% | 75% | 101% |
| Class E |  |  |  |  |  |
| Overall | 2.26% | 2.29% | 2.58% | 78% | 104% |
| Class D | 5.64% | 6.33% | 6.68% | 73% | 101% |
| Class F | 9.75% | 9.39% | 9.54% | 73% | 105% |
|  |  |  | LB |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.17% | 1.96% | 2.42% | 79% | 102% |
| Class C | 3.02% | 3.77% | 4.01% | 75% | 101% |
| Class E | 1.86% | 0.99% | 2.23% | 90% | 109% |
| Overall | 1.96% | 2.32% | 2.90% | 80% | 104% |
| Class D | 3.99% | 6.66% | 6.37% | 72% | 100% |
| Class F | 7.36% | 8.35% | 10.31% | 80% | 111% |
|  |  |  | LP |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.71% | 3.43% | 3.37% | 77% | 101% |
| Class C | 3.92% | 5.38% | 5.40% | 73% | 101% |
| Class E | 2.36% | 0.64% | 2.38% | 88% | 109% |
| Overall | 2.61% | 3.38% | 3.80% | 78% | 103% |
| Class D | 5.22% | 7.96% | 9.07% | 70% | 100% |
| Class F | 7.88% | 8.96% | 10.56% | 78% | 110% |

As shown in Table 3.2, it was observed that with Min CU size 16, performance impact is: AI: 10.36% / 18% / 85%; RA: 12.04% / 54% / 102%; LB 11.79%% / 59% / 103%; LP 13.43% / 55% / 101%.

Table 3.2 Simulation Results of Min CU Size 16

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.40% | 3.23% | 3.48% | 25% | 95% |
| Class A2 | 5.50% | 9.84% | 9.65% | 18% | 90% |
| Class B | 9.34% | 10.65% | 10.98% | 17% | 84% |
| Class C | 16.76% | 17.49% | 18.73% | 14% | 74% |
| Class E | 15.33% | 12.46% | 12.61% | 19% | 88% |
| Overall | 10.36% | 11.10% | 11.50% | 18% | 85% |
| Class D | 16.82% | 18.59% | 19.59% | 14% | 77% |
| Class F | 35.26% | 33.34% | 34.41% | 10% | 80% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.76% | 4.47% | 5.88% | 58% | 106% |
| Class A2 | 8.92% | 14.22% | 13.95% | 57% | 103% |
| Class B | 10.94% | 14.58% | 15.05% | 56% | 102% |
| Class C | 21.98% | 24.28% | 26.37% | 48% | 96% |
| Class E |  |  |  |  |  |
| Overall | 12.04% | 15.07% | 16.01% | 54% | 102% |
| Class D | 23.93% | 31.58% | 32.70% | 49% | 96% |
| Class F | 34.46% | 35.70% | 36.76% | 51% | 105% |
|  |  |  | LB |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 8.87% | 10.93% | 10.76% | 59% | 102% |
| Class C | 17.48% | 20.59% | 21.87% | 50% | 98% |
| Class E | 9.06% | 11.29% | 11.57% | 74% | 111% |
| Overall | 11.79% | 14.24% | 14.67% | 59% | 103% |
| Class D | 20.42% | 28.20% | 28.42% | 51% | 96% |
| Class F | 28.43% | 33.38% | 35.88% | 58% | 111% |
|  |  |  | LP |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 10.09% | 13.38% | 12.72% | 54% | 100% |
| Class C | 19.49% | 23.16% | 23.80% | 46% | 96% |
| Class E | 10.92% | 12.50% | 13.78% | 71% | 112% |
| Overall | 13.43% | 16.42% | 16.68% | 55% | 101% |
| Class D | 22.80% | 30.76% | 32.08% | 48% | 94% |
| Class F | 29.88% | 34.80% | 38.07% | 53% | 110% |

As shown in Table 3.3, it was observed that with Min CU size 32, performance impact is: AI: 23.75% / 7% / 78%; RA: 35.60% / 29% / 95%; LB 40.10% / 33% / 97%; LP 43.59% / 30% / 95%.

Table 3.3 Simulation Results of Min CU Size 32

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 11.72% | 13.57% | 14.71% | 12% | 90% |
| Class A2 | 14.47% | 26.31% | 25.43% | 7% | 82% |
| Class B | 20.90% | 27.79% | 28.77% | 7% | 76% |
| Class C | 32.99% | 39.07% | 42.10% | 5% | 66% |
| Class E | 37.47% | 37.02% | 36.90% | 9% | 81% |
| Overall | 23.75% | 29.22% | 30.19% | 7% | 78% |
| Class D | 35.42% | 41.44% | 44.45% | 7% | 71% |
| Class F | 69.05% | 65.64% | 67.89% | 4% | 73% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 15.76% | 19.86% | 24.34% | 32% | 101% |
| Class A2 | 27.51% | 40.40% | 37.60% | 35% | 98% |
| Class B | 31.18% | 41.58% | 43.23% | 30% | 95% |
| Class C | 62.09% | 68.04% | 75.67% | 23% | 89% |
| Class E |  |  |  |  |  |
| Overall | 35.60% | 44.05% | 46.98% | 29% | 95% |
| Class D | 74.15% | 89.37% | 91.37% | 33% | 95% |
| Class F | 78.79% | 82.66% | 86.27% | 31% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 28.08% | 34.70% | 35.78% | 32% | 95% |
| Class C | 56.70% | 62.20% | 66.68% | 25% | 92% |
| Class E | 38.02% | 41.98% | 42.18% | 50% | 107% |
| Overall | 40.10% | 45.69% | 47.68% | 33% | 97% |
| Class D | 77.67% | 90.76% | 92.90% | 35% | 96% |
| Class F | 73.65% | 83.20% | 89.55% | 36% | 105% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 30.52% | 38.98% | 39.19% | 28% | 94% |
| Class C | 60.78% | 65.90% | 69.60% | 23% | 89% |
| Class E | 42.46% | 45.94% | 47.39% | 48% | 106% |
| Overall | 43.59% | 49.69% | 51.38% | 30% | 95% |
| Class D | 81.21% | 93.73% | 97.60% | 34% | 93% |
| Class F | 77.00% | 85.99% | 93.76% | 33% | 105% |

Table 3.4 Summary of RA Results of different Min CU Size

| Min CU size | BD Rate-Y | BD Rate-U | BD Rate-V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 4x4 | 0% | 0% | 0% | 100% | 100% |
| 8x8 | 2.26% | 2.29% | 2.58% | 78% | 104% |
| 16x16 | 12.04% | 15.07% | 16.01% | 54% | 102% |
| 32x32 | 35.60% | 44.05% | 46.98% | 29% | 95% |

# Conclusion

It was observed that as minimum CU size increases, the BD-rate performance drops significantly. Although encoding time is also reduced prominently, the BD-rate trade off appears unreasonable when Min CU size is larger than 16 luma samples. Based on the analysis, it is proposed to limit value range of minimum CU size to {4, 8, 16} in luma samples to reduce the burden of conforming tests.

It is proposed to adopt the proposed method into VVC.

# Specification Text

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_luma_coding_block_size_minus2 | ue(v)u(2) |
| … |  |
| } |  |

#### Sequence parameter set RBSP semantics

…

log2_min_luma_coding_block_size_minus2 plus 2 specifies the minimum luma coding block size. The value range of log2_min_luma_coding_block_size_minus2 shall be in the range of 0 to 2, inclusive.

The variables MinCbLog2SizeY, MinCbSizeY, IbcBufWidthY, IbcBufWidthC and Vsize are derived as follows:

MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2	(49)

MinCbSizeY = 1  <<  MinCbLog2SizeY		(50)

…

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).