| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0469-v2 |
| --- | --- |

| Title: | On minimum chroma QT size derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a bug fix of deriving the minimum size of the chroma quadtree nodes in VTM software. The proposed method doesn’t have impact on common test condition. It reportedly fixes a signaling issue when the minimum luma coding block size is larger than 4.

# Problem Statement

In current VVC design, when dual tree mode is enabled, the minimum intra chroma quadtree node size is determined by sps_log2_diff_min_qt_min_cb_intra_slice_chroma value and the minimum luma coding block size.

MinQtLog2SizeIntraC = sps_log2_diff_min_qt_min_cb_intra_slice_chroma + MinCbLog2SizeY	   (1)

In the VTM-7.0 software, VTM encoder crashes when the minimum luma coding block size is set to larger than 4 and it’s trying to use intra chroma quadtree node size smaller than the luma coding block size. A fix is desirable to support smaller intra chroma quadtree node size when minimum luma coding block size is larger than 4.

The problem was caused by the software bug when deriving the value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma according to equation (1). In the software, MinCbLog2SizeY is in luma samples, but the value for MinQtLog2SizeIntraC was obtained in chroma samples.

# Proposed Method

The proposed software fix is to consider the chroma down sampling ratio to unify the unit for MinQtLog2SizeIntraC and MinCbLog2SizeY, so that the following calculation is garanteed to be non-negative.

sps_log2_diff_min_qt_min_cb_intra_slice_chroma = MinQtLog2SizeIntraC - MinCbLog2SizeY	(2)

It was previously considered a bug in both the VTM software and the VVC working draft text. But after some further study, it was found that the text is correct.

# Conclusion

The proposed method fixes an issue with signalling and deriving the minimum size of the chroma quadtree nodes in the VTM-7.0 software. With the fix, encoder crash issue is fixed when minimum luma coding block size is set to larger than 4. It is proposed to adopt the proposed fix to VTM.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, ISO/IEC JTC1/SC29/WG11 JVET-P2001, Oct. 2019.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.