| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0475 |
| --- | --- |

| Title: | [AHG9/AHG12] On subpicture signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to signal subpicture boundaries in PPS to utilize slice syntax elements as follows.

- Proposed to remove subpicture layout and ID mapping from SPS

- Propose to indicate subpicture boundaries in PPS using slices and conditioned by rect_slice_flag

- Propose to apply loop_filter_across_subpic_enabled_flag and subpic_treated_as_pic_flag to all subpictures instead of each subpicture

# Introduction

Since a slice may consist of one or multiple tiles, and a tile may consist of one or multiple slices, a subpicture may consist of one or multiple rectangular slices, it is desirable to utilize the dependency between slices and subpicture to simplify the signaling on both subpicture boundary indication and loop filter operation.

# Proposal

Subpicture layout is signaled in SPS as the subpicture layout is not changed within a CLVS. A constraint on subpicture ID mapping in PPS is imposed as below to avoid dependency between SPS and PPS.

“It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to  sps_num_subpics_minus1.”

Due to the dependency among tiles, slices and subpictures, it may be straightforward to signal subpicture in PPS to indicate the boundary and loop filter operation. It is proposed to remove subpicture syntax elements, such as subpics_present_flag,  sps_num_subpics_minus1, subpicture layout syntax struct, from SPS as Table 1.

Table 1 Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| bit_depth_minus8 | ue(v) |
| … |  |
| } |  |

Table 2 shows the proposed subpicture signaling and condition.

Table 2 Proposed PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) { |  |
| subpics_present_flag | u(1) |
| if ( subpics_present_flag ) { |  |
| pps_num_subpics_minus1 | u(8) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| single_slice_per_subpic_flag | u(1) |
| } |  |
| } |  |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| if (subpics_present_flag ) |  |
| pps_slice_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| if (subpics_present_flag ) { |  |
| subpic_treated_as_pic_flag | u(1) |
| loop_filter_across_subpic_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

subpics_present_flag  equal to 1 specifies that subpicture parameters are present in in the PPS RBSP syntax. subpics_present_flag equal to 0 specifies that subpicture parameters are not present in the PPS RBSP syntax.

pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS.

It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to  sps_num_subpics_minus1.

pps_slice_subpic_id[ i ] specifies the subpicture identifier of the subpicture associated with i-th slice. The length of slice_subpic_id, in bits, is derived as follows:

- if pps_subpic_id_signalling_present_flag is equal to 1, the length of pps_slice_subpic_id is equal to pps_subpic_id_len_minus1 + 1.

- Otherwise, the length of pps_slice_subpic_id is equal to Ceil(Log2(pps_num_subpics_minus1 + 1)).

subpic_treated_as_pic_flag equal to 1 specifies that the subpictures of each coded picture in the CLVS are treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag equal to 0 specifies that the subpictures of each coded picture in the CLVS are not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag is inferred to be equal to 0.

loop_filter_across_subpics_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across the subpicture boundaries in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across the subpicture boundaries in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag is inferred to be equal to 1.

Table 3 shows the corresponding update on PH syntax.

Table 3 Proposed PH syntax modification

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| if (subpics_present_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

The semantic of slice_subpic_id in SH is updated accordingly as below.

slice_subpic_id specifies the subpicture identifier of the subpicture that contains the slice. If slice_subpic_id is present, the value of the variable SubPicIdx is derived to be such that SubpicIdList[ SubPicIdx ] is equal to slice_subpic_id. Otherwise (slice_subpic_id is not present), the variable SubPicIdx is derived to be equal to 0. The length of slice_subpic_id, in bits, is derived as follows:

- If sps_subpic_id_signalling_present_flag is equal to 1, the length of slice_subpic_id is equal to sps_subpic_id_len_minus1 + 1.

- Otherwise, if ph_subpic_id_signalling_present_flag is equal to 1, the length of slice_subpic_id is equal to ph_subpic_id_len_minus1 + 1.

- Otherwise, if pps_subpic_id_signalling_present_flag is equal to 1, the length of slice_subpic_id is equal to pps_subpic_id_len_minus1 + 1.

- Otherwise, the length of slice_subpic_id is equal to Ceil( Log2 ( spspps_num_subpics_minus1 + 1 ) ).

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).