| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0496 |
| --- | --- |

| Title: | AHG16: On motion shift derivation of SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

The construction of subblock-based TMVP (SbTMVP) requires the derivation of motion shift from the left neighboring block (A1), which causes latency issue for hardware encoder. In fact, the left neighboring block (A1) is the worst choice among all the immediate neighboring blocks for that purpose.

In this contribution, it is proposed to obtain the motion shift from alternative positions to make the design hardware friendly. In one proposed method, the motion shift is derived from the above neighboring block (B1). In another proposed method, the motion shift is derived from a block outside a predefine regions such as a 64x64 VPDU. The first modification introduces 0.01% (RA) and -0.01%(LDB) BD-rate changes, and the second modification introduces 0.03% (RA) and 0.02%(LDB) BD-rate changes under CTC.

# Introduction

The way the subblock TMVP (SbTMVP) is constructed introduces dependency on the left neighboring block. In hardware encoder implementations, the motion estimation stage of multiple blocks is typically performed in parallel or at least pipelined to increase the throughput. For AMVP, this is not a big issue since the MVP is used to differentially code the MV found by the motion search. The motion estimation stage for SbTMVP, however, would typically just consist of the derivation of motion shift and the motion information for each subblock. Due to the aforementioned dependency on the left neighboring block, SbTMVP of blocks cannot be generated in parallel. Therefore, two methods are proposed to make the derivation of SbTMVP more hardware friendly.

# Proposal

In the proposed first method, the motion shift is derived from the above neighboring block (B1) instead of the left neighboring block (A1) because the MV of the left neighboring CU is usually not reconstructed when current CU is in the pipeline stage of deriving SbTMVP.

In the proposed second method, as shown in Figure 1, all CUs (e.g. CU3, CU4, CU5, CU6 and CU7) within one predefine region such as VPDU (64x64 block) use the same left block immediately adjacent to the VPDU to derive the motion shift of SbTMVP.

Figure 1 The proposed second method: the motion shift is derived from the block A1’ outside the VPDU (64x64) for all the CUs withing the same VPDU.

# Simulation Results

Both proposed methods are implemented based on the VTM-7.0 reference software [1] and tested using common test condition defined in [2].

Table 1 Results of the proposed first method

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | 0.04% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.02% | 102% | 101% |
| Class B | 0.00% | 0.05% | 0.02% | 100% | 99% |
| Class C | 0.01% | 0.02% | 0.06% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.03% | 101% | 100% |
| Class D | -0.02% | -0.03% | -0.04% | 99% | 98% |
| Class F | 0.01% | -0.04% | 0.00% | 100% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.15% | 0.10% | 101% | 100% |
| Class C | -0.03% | -0.14% | -0.06% | 99% | 97% |
| Class E | 0.01% | 0.16% | -0.72% | 98% | 98% |
| Overall | -0.01% | -0.07% | -0.16% | 100% | 98% |
| Class D | -0.07% | 0.78% | 0.25% | 100% | 99% |
| Class F | 0.04% | -0.37% | -0.12% | 100% | 97% |

Table 2 Results of the proposed second method

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | -0.02% | 99% | 98% |
| Class A2 | 0.00% | 0.01% | 0.03% | 101% | 101% |
| Class B | 0.02% | 0.03% | 0.01% | 100% | 100% |
| Class C | 0.05% | 0.04% | 0.06% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.02% | 0.02% | 99% | 99% |
| Class D | 0.03% | -0.03% | 0.02% | 99% | 98% |
| Class F | 0.02% | 0.00% | 0.01% | 99% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.08% | 0.23% | 100% | 101% |
| Class C | 0.00% | 0.11% | 0.03% | 93% | 97% |
| Class E | -0.01% | 0.33% | -0.52% | 102% | 101% |
| Overall | 0.02% | 0.09% | -0.02% | 98% | 100% |
| Class D | 0.02% | -0.35% | -0.10% | 100% | 100% |
| Class F | 0.05% | -0.24% | 0.64% | 103% | 101% |

# References

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “ JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011, 16th meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).