| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0519 |
| --- | --- |

| Title: | Non-CE2: On signaling of maximum palette size and maximum palette predictor size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Hong-Jheng Jhu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, jhuhong-jheng, yiwenchen, tsung-chuanma, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In HEVC standard, maximum palette table size and maximum palette predictor size are signaled in sequence parameter set (SPS) while the palette in the VVC draft 7 uses fixed maximum palette table size of 31 and fixed maximum palette predictor size of 63. In this contribution, it is proposed to bring the syntax of signaling maximum palette table size and maximum palette predictor size in the HEVC back to the palette design in the VVC. The proposed signaling was implemented on top of VTM-7.0 and tested using different maximum palette table size and maximum palette predictor size. Compared to VTM-7.0 anchors, the performance for the sequences in the category YCbCr444 TGM 1080p is summarized as

For maximum palette table size of 64 and maximum palette predictor size of 128, for lossy coding, it provides average {Y, Cb, Cr} BD-rate differences of {-0.66%, -1.23%, -0.96%} and {-0.43%, -0.79%, -0.69%} for AI and RA configurations, respectively. For lossless coding, the average AI and RA bit-rate differences are -0.47% and -0.36%.

For maximum palette table size of 32 and maximum palette predictor size of 64, for lossy coding, it provides average {Y, Cb, Cr} BD-rate differences of {2.00%, 2.95%, 2.59%} and {1.46%, 1.84%, 1.73%} for AI and RA configurations, respectively. For lossless coding, the average AI and RA bit-rate differences are 2.30% and 2.17%.

# Introduction

In HEVC standard [1], maximum palette table size and maximum palette predictor size are signaled in sequence parameter set (SPS). But, in the existing palette design in VVC draft 7 [2], those two parameters are fixed to be 31 and 63, respectively. It is asserted that varying maximum palette table size and maximum palette can give more flexibilities for practical hardware/software codec devices by providing various performance/complexity trade-offs. For instance, using smaller maximum palette size or smaller maximum palette predictor size will lower palette coding efficiency while decreasing the memory used for storing representative colors in palette table and palette predictor. Additionally, decreasing maximum palette table size or maximum palette predictor size can also reduce computational complexity, for instance, due to the reduced checking operations as follows:

- When generating the palette table of one palette CU, the colors in the palette predictor need to checked such that the palette predictor colors that are reused in the current CU are put at the beginning of the palette table.

- When updating the palette predictor, all the colors in the palette predictor need to be checked such that only the palette predictor colors that are not included in the palette table of the current CU are added to the updated palette predictor.

For different combinations of maximum palette table size and maximum palette predictor size, Table 1 illustrates the corresponding memory size that are used to store the representative palette colors and the average number of checking operations that are performed for each palette CU at decoder for the YCbCr444 sequence Console at QP 32 for AI configuration.

Table 1 Memory size and number of checks that are used for different maximum palette table size and maximum palette predictor size

| Memory size (KBits) |  |  | Number of checks (per CU) |  |  |
| --- | --- | --- | --- | --- | --- |
| PLT31 PLTPred63 (CTC) | PLT16 PLTPred32 | PLT64 PLTPred128 | PLT31 PLTPred63 (CTC) | PLT16 PLTPred32 | PLT64 PLTPred128 |
| 2.8 | 1.4 | 5.8 | 187.1 | 94.5 | 375.9 |

As can be seen in Table 1, the corresponding memory size and the number of checking operations are approximately increased by one time when doubling the maximum palette table size and the maximum palette predictor size each time.

# Proposal

In this contribution, it is proposed to bring the syntax of signaling maximum palette table size and maximum palette predictor size in the HEVC to the palette design in the VVC.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| if(sps_palette_enabled_flag) { |  |
| palette_max_size | ue(v) |
| if(palette_max_size) |  |
| delta_palette_max_predictor_size | ue(v) |
| } |  |
| …… |  |
| } |  |

palette_max_size specifies the maximum allowed palette size. When not present, the value of palette_max_size is inferred to be 0.

delta_palette_max_predictor_size specifies the difference between the maximum allowed palette predictor size and the maximum allowed palette size. When not present, the value of delta_palette_max_predictor_size is inferred to be 0.

Additionally, one bitstream conformance constraint is proposed that the maximum palette table size and the maximum palette predictor size should not be larger than 64 and 128.

# Simulation results

The proposed signaling was implemented on top of VTM-7.0 [3] and tested following the test conditions used by the core experiment (CE) 2 [4] for lossy coding and the core experiment (CE) 3 [5] for lossless coding. The simulation results with dual-tree being disabled are provided in the below.

- Maximum palette table size of 64 and maximum palette predictor size of 128

Table 2 Lossy performance of maximum palette table size 64 and maximum palette predictor size 128

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.66% | -1.23% | -0.96% | 96% | 98% |
| TGM 720p | -0.18% | -0.22% | -0.35% | 101% | 104% |
| Animation | -0.16% | -0.28% | -0.29% | 108% | 109% |
| Mixed content | -0.52% | -0.62% | -0.55% | 106% | 104% |
| Camera-Captured content | 0.01% | 0.00% | 0.01% | 98% | 96% |
| Overall | -0.33% | -0.53% | -0.48% | 102% | 102% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.43% | -0.79% | -0.69% | 100% | 100% |
| TGM 720p | -0.09% | -0.13% | -0.15% | 99% | 99% |
| Animation | -0.02% | -0.08% | -0.09% | 98% | 101% |
| Mixed content | -0.54% | -0.42% | -0.32% | 102% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | -0.23% | -0.32% | -0.29% | 100% | 100% |

Table 3 Lossless performance of maximum palette table size 64 and maximum palette predictor size 128

|  | All Intra Main |  |  | Random Access Main |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | VTM7 | test |  | VTM7 | test |  |
| TGM 1080p | 26.8 | 26.9 | -0.47% | 54.1 | 54.3 | -0.36% |
| TGM 720p | 13.4 | 13.4 | -0.34% | 16.7 | 16.7 | -0.13% |
| Animation | 6.0 | 6.0 | 0.00% | 1.7 | 1.7 | 0.00% |
| Mixed content | 7.9 | 7.9 | -0.05% | 6.3 | 6.3 | -0.01% |
| Camera-Captured content | 1.8 | 1.8 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 11.2 | 11.2 | -0.17% | 15.8 | 15.9 | -0.13% |
| Enc Time[%] | 105% |  |  | 98% |  |  |
| Dec Time[%] | 106% |  |  | 96% |  |  |

- Maximum palette table size of 16 and maximum palette predictor size of 32

Table 4 Lossy performance of maximum palette table size 16 and maximum palette predictor size 32

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 2.00% | 2.95% | 2.59% | 98% | 102% |
| TGM 720p | 0.60% | 0.83% | 1.05% | 100% | 100% |
| Animation | 0.28% | 0.49% | 0.47% | 101% | 103% |
| Mixed content | 1.25% | 1.71% | 1.39% | 100% | 103% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Overall | 0.94% | 1.36% | 1.26% | 100% | 102% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 1.46% | 1.84% | 1.73% | 100% | 100% |
| TGM 720p | 0.58% | 0.72% | 1.37% | 100% | 99% |
| Animation | 0.14% | 0.28% | 0.29% | 98% | 100% |
| Mixed content | 0.93% | 1.34% | 1.14% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.71% | 0.94% | 1.04% | 100% | 100% |

Table 5 Lossless performance of maximum palette table size 16 and maximum palette predictor size 32

|  | All Intra Main |  |  | Random Access Main |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | VTM7 | test |  | VTM7 | test |  |
| TGM 1080p | 26.8 | 26.1 | 2.30% | 54.1 | 52.5 | 2.17% |
| TGM 720p | 13.4 | 13.2 | 1.19% | 16.7 | 16.5 | 0.71% |
| Animation | 6.0 | 6.0 | 0.00% | 1.7 | 1.7 | 0.00% |
| Mixed content | 7.9 | 7.9 | 0.05% | 6.3 | 6.3 | 0.01% |
| Camera-Captured content | 1.8 | 1.8 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 11.2 | 11.0 | 0.71% | 15.8 | 15.4 | 0.72% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

# Reference

High Efficiency Video Coding, Rec. ITU-T H.265 and ISO/IEC 23008-2, Feb. 2018.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode”, JVET-P2022, Oct. 2019.

T. Nguyen, T.-C. Ma, A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023, Oct. 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).