| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0694-v2 |
| --- | --- |

| Title: | AHG12: A summary of proposals on subpicture extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Skupin | Tel: Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of the proposals on subpicture extraction from Sections 6.19.10, 6.19.11, 6.19.12, 6.20.1.3, and 6.20.1.4 and a list of design questions.

It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

Revision 1 of this document adds contribution Q0379 as requested by the authors.

# List of proposals on subpicture sub-bitstreams

This category of proposals is listed in the agenda item XXX, including X contributions. These contributions and summaryies of the related proposals are listed below.

JVET-Q0285 AHG9: On adaptation parameter set ID [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

This contribution contains two proposals:

Proposal-1: Method for identification of APSs associated with a subpicture for bitstream extraction and merge, by signaling the list of associated subpicture IDs in each APS.

Proposal-2: Replacement of u(5) with ue(v), to code adaptation_parameter_set_id, for future extensibility.

JVET-Q0281 AHG12: Independently coded regions output window SEI message [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

A new SEI message “Independently coded regions output window SEI message” that carries useful information for sub-picture extraction and merge is proposed. When bitstreams are extracted or merged, the values of syntax elements related to picture size, conformance windows and sub-picture layout in parameter sets of the output bitstream need to be updated. However, the updated values of those parameters for the output bitstream cannot be simply calculated from the original parameters. The proposed SEI message contains the picture size, the location, the conformance (output) window offsets and the associated subpicture IDs of the extracted independent coded region, which is composed of one or more motion constrained sub-pictures. As long as the independent coded region composed of the selected sub-pictures has a rectangular shape and the motion constraint, the extracted sub-bitstream of the independently coded region can be decoded and outputted.

JVET-Q0283 AHG9: On subbitstream extraction [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

A subpictures extraction functionality is proposed. To enable this functionality in the current VVC spec., three following items needs to be addressd:

1.	Identification of adaptation parameter sets referenced by a specific sub-picture.

2.	Rewriting of picture size, conformance window and sub-picture layout information in parameter sets for the extracted sub-bitstream.

3.	Sub-bitstream extraction process for the target sub-pictures, with the solutions to address the items 1 and 2.

JVET-Q0285 (On adaptation parameter set identification) and JVET-Q0281 (Independently coded regions output window SEI message) are proposed in this meeting, to address the items 1 and 2, respectively. This contribution proposes the third item, the sub-picture extraction process from the bitstream, utilizing JVET-Q0285 and JVET-Q0281.

JVET-Q0379 AhG9: On the position of APS IDs in Picture Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

This contribution proposes to move the syntax elements related to the APS ID at an early stage of the Picture header and Slice header. The aim of this proposed modification is to reduce the parsing complexity for some streaming applications that need to track the APS ID in the Picture header and Slice header to remove unused APS.

The proposed modification has no impact on the BDR performance.

JVET-Q0397 AHG12: On subpicture extraction [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

This contribution asserts that the current VVC draft specification misses a normative subpicture extraction process and proposes to add a subpicture specific extraction support to VVC by a twofold proposal:

- Add an extraction process for subpictures with the required parameter set rewriting which would support AU-based HRD operation.

- Extending the scalable-nesting SEI message with subpicture support to support DU-based HRD operation.

JVET-Q0411 AHG12: On Subpictures merging for VVC [N. Ouedraogo, F. Denoual, F. Mazé (Canon)]

This contribution addresses the signaling of subpictures suitable for merging operation. Indeed, VVC provides several possible configurations for specific use cases of subpictures. For simple merging operation it is asserted that defining a flag that constrains a subpicture for simple merging operations is desirable.

In particular it is proposed to constrain the subpicture signaling for merging operations as follows:

- Constrain picture width and height to be a multiple of CTB size. This constraint fixes a technical issue, discussed at JVET#16 meeting in JVET-P0351 (further study was encouraged), encountered with VVC subpictures merging coming from pictures with sizes that are not multiple of CTB size.

- Constrain subpicture boundaries to be treated as picture boundaries

- Constrain loop filter to be disabled at subpicture boundaries

- Constrain subpicture ID signaling to be defined at SPS or PPS but not at PH level. This is to prevent any picture-based changes of the subpicture ID mapping which is author asserted as complex to monitor

- Provide subpicture conformance window SEI message for cropping operation. This was also discussed during JVET#16.

JVET-Q0404 AHG12: On CBR subpicture extraction [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

This contribution proposes several options to specify an extraction process for subpictures that allows a CBR mode after extraction.

JVET-Q0443 Independent Subpictures [K. Sharman, S. Keating, A. Browne (Sony)]

Subpictures allow regions of a picture to be independently decodable, with care added to the DIS (JVET-P2001-v14) to handle edges of subpictures and describe the subpicture requirements at different levels via a dedicated SEI message.

This contribution proposes that constrains are added to subpictures so that CABAC zero words and MinCR picture level constraints are also applied at a subpicture level. The proposal also highlights some potential errors in the current Subpicture SEI message.

JVET-Q0406 AHG12: On CABAC zero words for subpictures [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

This contribution asserts that subpicture extraction could result in violating the bin-to-bit ratio constraint as CABAC zero words are not associated to subpictures. Therefore, it is proposed to add a bitstream constraint for the VCL NAL units of subpictures to fulfil the bin-to-bit ratio constraint individually in addition to the picture unit level constraint when the following conditions apply:

- a subpicture has subpic_treated_as_pic_flag[ ] equal to 1.

- a subpicture level information SEI message is present in the CLVS.

# List of design questions

- Specify a normative subpicutre sub-bitstream extraction process (Q0283, Q0397)

  - if yes, do we want to allow External means to provide correct Parameter Sets in the extraction process? (Q0397)

  - if yes, do we want to specify rewriting of Parameter Sets

    - DPS, VPS, SPS, PPS (Q0283, Q0397)

    - do we want to specify rewriting of level information and picture size (Q0281, Q0283, Q0397)

    - do we want to specify rewriting of conformance window (Q0283, Q0411)

    - should a new SEI message carry additional support information (Q0281, Q0411)

    - APS by adding subpicture association info to be considered in the extraction process (Q0283, Q0285)

  - if yes, do we want to constrain the subpicture extraction process on

    - presence of subpicture level information SEI message? (Q0397, Q0411)

    - subpic_treated_as_pic_flag[ i ] being equal to 1? (Q0397, Q0411)

    - loop_filter_across_subpic_enabled_flag[ i ] being equal to 1? (Q0411)

    - ph_subpic_id_signalling_present_flag being equal 1 (Q0411)

    - pic width and height to be multiples of CtbSizeY (Q0411)

- Do we want to establish subpicture-specific CABAC zero word and and MinCR picture level constraints constraints? (Q0406, Q0443)

  - if yes, should it be express it in the same place than the existing CABAC zero word constraint, i.e. RBSP slice trailing bits semantics? (Q0406, Q0443), or

  - if yes, should it be express it the subpicture level information SEI message semantics? (Q0443)

  - if yes, should the constraints be dependent on the value of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ]? (Q0406)

- Do we want to provide HRD conformance for subpictures?

  - if yes, should we have information (BP/PT/DUI SEI messages) for subpictures in a subpicture-nested form to support DU-based HRD operation? (Q0397)

  - if yes, should extraction of subpicture sub-bitstreams with constant bitrate (CBR) mode be enabled?

      - if no, add removal of filler data (FD_NUTs and filler payload SEI messages) to subpicture extraction process? (Q0404 option 2)

      - if yes, do we want to add a general constraint flag indicating the presence of subpicture specific filler data? (Q0404 options 1a - 1c)

        - if yes, how to identify subpicture specific filler data?

          - bitstream position (Q0404 option 1a) , or

          - add SEI message to associtate filler data and subpicture VCL NAL units by bitstream order (Q0404 option 1b), or

          - added subpicture indicators to FD_NUT and filler payload SEI message (Q0404 option 1c)?

- Do we want to add a shortcut in the syntax to indicate all subpictures have subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are equal to 1? (Q0411)

(already adopted in discussion of JVET-Q0271, aspect 2)

- Do we want to shift position of APS related syntax towards beginning of PH and SH to reduce the parsing complexity in applications that need to keep track of APS data (Q0379).