| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0729 |
| --- | --- |

| Title: | Cross-check of JVET-Q0138 (Separate transform skip checking of Luma and Chroma for LFNST index signalling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | C. Rosewarne, J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@canon.com.au jonathan.gan@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

This contribution contains a cross-check of proposal JVET-Q0138 from LGE entitled ‘Separate transform skip checking of Luma and Chroma for LFNST index signalling’. The source code was studied and found to match the proposal description (two methods described). The coding result matches the results reported by the proponent for each method (for method 2, two results not completed but otherwise matching).

# Cross check procedure

The source code for JVET-Q0138 [1]. was provided by LGE. Canon compiled the source code using GCC 9.1.0 and the simulation was performed on a Linux cluster under conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

The following notes were made based on analysis of the proposal and the provided source code:

- Rather than explicitly check the various TS skip flags to suppress signalling of lfnst_idx, rely on LfnstDcOnly flag, which may be updated each time a transformed residual is parsed.

- At the coding_unit syntax structure level: Initialised to '1', then invoke transform_tree, then needs to be '0' to (de-)code lfnst_idx.

- Within each transform block:

  - LfnstDcOnly flag set to '0' in residual_coding when not transform skip (redundant condition of VTM-7.0, as residual_coding() not invoked when TS=1).  Other condition like last_pos > DC, last sub-block is 0 (i.e. other gating conditions for LFNST application) are also applied.

- Finally, lfnst_idx is only parsed when LfnstDcOnly flag is zero, without additional tests of transform skip flags according to different treeTypes (as these are already handled by iterating over transform_tree).

- Method 2 keeps current VTM-7.0 behaviour for single-tree case. It means single tree LFNST idx signalling only depends on luma TS flag.

The source code changes were found to match the proposal description.

WD text was also provided and found to match the source code.

# Coding performance measurement

Coding performance was measured under JVET SDR common test conditions (CTC), with AI and RA configs run, [2] and is shown in Table 1 and Table 2 for methods 1 and 2, respectively.

Negligible runtime impact was observed.

Table 1. Method 1 results.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class B | 0.00% | 0.00% | -0.06% | 101% | 101% |
| Class C | 0.01% | 0.06% | 0.06% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.04% | 100% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.02% | 0.09% | 101% | 101% |
| Class F | 0.00% | -0.01% | 0.07% | 100% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | -0.04% | 99% | 100% |
| Class A2 | -0.03% | 0.04% | -0.03% | 99% | 100% |
| Class B | 0.00% | -0.03% | 0.17% | 99% | 100% |
| Class C | 0.02% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.04% | 99% | 100% |
| Class D | -0.03% | -0.07% | 0.07% | 99% | 100% |
| Class F | 0.00% | 0.03% | 0.03% | 100% | 100% |

Table 2. Method 2 results.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.06% | 100% | 101% |
| Class C | 0.01% | 0.06% | 0.06% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.04% | 100% | 99% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.02% | 0.09% | 101% | 101% |
| Class F | 0.00% | -0.01% | 0.07% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class A2 | -0.02% | -0.02% | -0.07% | 99% | 100% |
| Class B | 0.00% | -0.04% | 0.15% | 100% | 100% |
| Class C | 0.00% | 0.05% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.06% | 100% | 100% |
| Class D | -0.04% | -0.09% | 0.06% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

Based on coding performance result, coding performance of methods 1 and 2 were found to match those of the proponents (excepting two unfinished results in class F at the time of upload). The proposal was found to match the description and provided WD text.

# References

- M. Koo, M. Salehifar, J. Lim, S. Kim, “Separate transform skip checking of Luma and Chroma for LFNST index signalling”, JVET-Q0138, Brussels, BE, January 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March, 2019.