| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R_Notes_d0 |
| --- | --- |

| Title: | Meeting Report of the 18th Meeting of the Joint Video Experts Team (JVET), by teleconference, 15–24 Apr. 2020 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chairs of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its eighteenth meeting during 15–24 April 2020 as an online-only meeting. It had previously been planned to be held in Alpbach, Austria, at Congress Centrum Alpbach. The conversion of the meeting to be conducted only online was necessitated due to issues associated with the recently declared COVID-19 pandemic. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.13 of this document. It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of a new standard. The name Versatile Video Coding (VVC) was chosen in April 2018 as the informal nickname for the new standard.

The JVET meeting began at approximately 0500 hours UTC on Wednesday 15 April 2020. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours UTC on Friday 24 April 2020. On the first and second day of the meeting, only aspects related to high level syntax were on the agenda. Approximately XXX people attended the JVET meeting, and approximately XXX input documents, and 16 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of developing video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or otherwise gives better support regarding the requirements of future application domains of video coding. As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the seventeenth JVET meeting in producing an eighth draft of the VVC standard and the eighth version of the associated VVC test model (VTM). Further important goals were reviewing technical input on novel aspects of video coding technology, producing the next versions of the VVC draft text and VTM, and plan next steps for further investigation of candidate technology towards the formal standard development.

The JVET produced 11 output documents from the meeting (update):

- JVET-Q2001 Versatile Video Coding specification text (Draft 8)

- JVET-Q2002 Algorithm description for Versatile Video Coding and Test Model 8 (VTM 8)

- JVET-Q2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 10)

- JVET-Q2005 Methodology and reporting template for coding tool testing

- JVET-Q2007 Supplemental enhancement information messages for coded video bitstreams (Draft 3)

- JVET-Q2008 Conformance testing for versatile video coding (Draft 2)

- JVET-Q2009 Preliminary plan for VVC verification testing (Draft 1)

- JVET-Q2013 JVET common test conditions and software reference configurations for non-4:2:0 colour formats

- JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding

- JVET-Q2016 Summary information on BD-rate experiment evaluation practices.

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, no Core Experiments (CE) were defined. The next four JVET meetings were planned for 23 June – 01 July 2020 under ITU-T SG16 auspices in Geneva, CH, during 7–16 October 2020 under WG 11 auspices in Rennes, FR, during 6–15 January 2021 under WG 11 auspices in Capetown, ZA, and during 20–28 April 2021 under ITU-T SG16 auspices in Geneva, CH.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/SC 29/WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its eighteenth meeting during 15–24 April 2020 as an online-only meeting, using Zoom teleconferencing tools. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of a new standard. The name Versatile Video Coding (VVC) was chosen in April 2018 as the informal nickname for the new standard.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2020_01_Q_Brussels/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the sixteenth JVET meeting in producing an eighth draft of the VVC standard and the eighth version of the associated VVC test model (VTM). Further important goals were reviewing technical input on novel aspects of video coding technology, producing the next versions of draft text and VTM, and planning next steps for further investigation of candidate technology towards the formal standard development.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the VTM software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the VTM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of the draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

This meeting report is based primarily on notes taken by the JVET chairs. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 8 April 2020. Any documents uploaded after 1159 hours Paris/Geneva time on Thursday 9 April 2020 were considered “officially late”, giving a grace period of 12 hours to accommodate those living in different time zones of the world. The deadline does not apply to AHG reports, and other such reports which can only be produced after the availability of other input documents.

Prior to the regular JVET meeting, a series of AHG meetings were held during 6-8 April for HLS topics (“category 1”: AHG8/AHG9/AHG12) and on 9, 13 and 14 April for coding tools (“category 2”: AHG2/AHG3/AHG6/AHG7/AHG11/AHG14/AHG16). An earlier upload deadline of 3 April 2020 had been announced for documents to be discussed in those meetings.

All contribution documents with registration numbers higher than JVET-R0XXX were registered after the “officially late” deadline (and therefore were also uploaded late). Likewise, AHG related proposal documents with registration numbers higher than JVET-R0336 were registered late. However, some documents in the “late” range might include break-out activity reports that were generated during the meetings, and are therefore better considered as report documents rather than as late contributions. Also, all cross-check reports were uploaded late.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:qq

- JVET-R0XXX (a proposal on …), uploaded XX-XX.

- …

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline. For example, some of them were proposing combinations or simplifications of other proposals.

The following other document not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-R0XXX (a document on …), uploaded XX-XX.

- …

All cross-verification reports at this meeting (except for JVET-R0XXX) were registered late and all were uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following (X) contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-R0XXX, ….

The following cross verification reports had not been uploaded yet by the end of the meeting, but were provided later (check later, or withdraw): JVET-R0XXX, ….

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then typically be counted as a late contribution). At the current meeting, this situation applied to the initial uploads of documents JVET-R0XXX, … .

Contributions that had significant problems with uploaded versions included the following:

- JVET-R0XXX (…)

- …

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision although participants tend to seldom notice what is recorded there.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-Q2000, the Versatile Video Coding specification text (Draft 8) JVET-Q2001, the Algorithm description for Versatile Video Coding and Test Model 8 (VTM 8) JVET-Q2002, the Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 10) JVET-Q2004, the Methodology and reporting template for coding tool testing JVET-Q2005, the Supplemental enhancement information messages for coded video bitstreams (Draft 3) JVET-Q2007, the Conformance testing for VVC (Draft 2) JVET-Q2008, the Preliminary plan for VVC verification testing (Draft 1) JVET-Q2009, the JVET common test conditions and software reference configurations for non-4:2:0 colour formats JVET-Q2013, the JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding JVET-Q2014, and the Summary information on BD-rate experiment evaluation practices JVET-Q2016, had been completed and were approved. The software implementation of VTM (versions 8.0 and 8.1) was also approved.

The group was initially asked to review the meeting report of the previous meeting for finalization. The meeting report was later approved without modification.

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting on the WG11 host’s website. Access to the teleconference sessions of the main JVET meeting was controlled with a password that is distributed to the registered participants; this should help overloading the teleconferencing tool.

The following rules were initially set up for the Zoom teleconference meeting:

o Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally.

o Stay muted unless you have something to say. (people were muted by default when they join and would need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

o Identify who you are and your affiliation when you begin speaking.

o Use your full name and company/organization affiliation in your joining information. We will use the participation list for attendance records.

o Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

o Avoid overloading people’s internet connections, we do not plan to use video for the teleconferencing calls – only voice and screen sharing. Extensive use of screen sharing is encouraged.

## Agenda

The agenda for the meeting was as follows:

Opening remarks and review of meeting logistics and communication practices

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Reports of ad hoc group (AHG) activities

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of information contributions

Coordination activities

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, meeting planning, other planning issues

Other business as appropriate for consideration

- On the first two days of the meeting (April 15 and 16), only aspects related to high level syntax (including AHG8, AHG9, and AHG12 reports) were on the agenda. In the morning of April 17, the meeting was continued with general status review and administrative matters, and then proceeded with reports of ad hoc group activities, and other matters.

- The plans for the times of meeting sessions were established as follows, in UTC (2 hours behind the time in Geneva, Paris (and Alpbach); 7 hours ahead of the time in Los Angeles, etc.). No session should last longer than 2 hrs.

0500-0700 1st “morning” session [break after 2 hours]

0715-0915 2nd “morning” session

[“lunch” break – nearly 4 hours]

1300-1500 1st “afternoon” session [break after 2 hours]

1515-1715 2nd “afternoon” session

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM software implementation package uses the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the VTM software, and for evaluating proposals for technology to be potentially included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It was reminded to send a notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 1221.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

(check for completeness with JVET-N0013, and draft text)

- ACT: Adaptive colour transform.

- AFF: Affine.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Adaptation parameter set.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BDOF: Bi-directional optical flow (formerly known as BIO).

- BDPCM: Block-wise DPCM.

- BL: Base layer.

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CIIP: Combined inter/intra prediction.

- CL-RAS: Cross-layer random-access skip.

- CPMV: Control-point motion vector.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CST: Chroma separate tree.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DPS: Decoding parameter sets.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- GRA: Gradual random access

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HMVP: History based motion vector prediction.

- HRD: Hypothetical reference decoder.

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation.

- LM: Linear model.

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MCP: Motion compensated prediction.

- MDNSST: Mode dependent non-separable secondary transform.

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode.

- MMVD: Merge with MVD.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MRL: Multiple reference line intra prediction.

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- operation point: A temporal subset of an OLS.

- PDPC: Position dependent (intra) prediction combination.

- PERP: Padded equirectangular projection (a 360° projection format).

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format).

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set (as in AVC and HEVC).

- PTL: Profile/tier/level combination.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- BT: Binary tree.

- TT: Ternary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RDPCM: Residual DPCM

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SCIPU: Smallest chroma intra prediction unit.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal-dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIF: Switchable (motion) interpolation filter.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- STSA: Step-wise temporal sublayer access.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TPM: Triangular partitioning mode

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VTM: VVC Test Model.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: Wide-angle intra prediction

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Opening remarks

Remarks during the opening session of the meeting 0700 Wednesday 15 April (chaired by GJS and JRO) were as follows.

- The first two days were dedicated to high-level syntax (incl. AHGs 8, 9, 12)

- Timing and organization of online meetings, calendar

- Balloting and approval timelines: 
"H.VVC" | ISO/IEC 23090-3 for VVC and H.SEI | ISO/IEC 23002-7

- The meeting logistics, agenda, working practices, policies, and document allocation were reviewed.

  - The meeting host is Zoom

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

  - There were no objections voiced in the opening plenary to the consideration of late contributions.

- The results of the previous meeting and the meeting report were reviewed.

  - See the AHG3 report for the software integration status

  - The relationship between the VVC and SEI texts was noted

    - VUI is in the SEI text, mostly for providing colour interpretation

      - It was noted that VUI is within the SPS, whereas SEI is in the SEI payload syntax structure, although this is not so relevant to the SEI text itself, and is more tied with the bitstream (less likely to be altered or removed).

      - VUI has a clear scope, is more tied to the sequence level

      - Should VUI be in the VVC spec instead of the SEI spec?

      - VUI could contain other info, such as constraint indicators (info that does not affect the decoding process)

      - SEI has a length parameter that enables discarding; VUI does not. SPS extension data follows the VUI. It was remarked that having a size indicator for VUI may be desirable.

    - field_seq_flag was put into the SPS to improve

- AHG pre-meetings

- There was somewhat less of a problem of late non-cross-check documents and no “placeholders” –  (see section 2.4.2).

- The primary goals of the meeting were … .

- Due to the high number of input contributions, parallelization and breakout work were planned to be used at the meeting.

- Visual comparison of VVC vs. HEVC – how to make progress with remote meeting in that?

- Principles of standards development were discussed.

  - It was noted that now is the time for the filing of formal IPR declarations for those who have patent rights that would be necessary for implementation of VVC or the associated SEI standard.

## Scheduling of discussions

- The plans for the times of meeting sessions were established as follows, in UTC (2 hours behind the time in Geneva, Paris (and Alpbach); 7 hours ahead of the time in Los Angeles, etc.). No session should last longer than 2 hrs.

0500-0700 1st “morning” session [break after 2 hours]

0715-0915 2nd “morning” session

[“lunch” break – nearly 4 hours]

1300-1500 1st “afternoon” session [break after 2 hours]

1515-1715 2nd “afternoon” session

0500-0700 1st “morning” session [break after 2 hours]

0715-0915 2nd “morning” session

[“lunch” break – nearly 4 hours]

1300-1500 1st “afternoon” session [break after 2 hours]

1515-1715 2nd “afternoon” session

All sessions were announced via the new calendar in the JVET document site at least 22 hrs. in advance. Particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Wed. 15 Apr., 1st day

  - 0500–0700 Opening remarks, review of practices, agenda, IPR reminder

  - 0715–0915 Reports of AHGs 8, 9, 12

  - 1300-1500 …

  - 1515-1715 …

- Thu. 16 Apr., 2nd day

  - 0500–0700 …

  - 0715–0915 …

  - 1300-1500 …

  - 1515-1715 …

- …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks, and may not be completely accurate):

- AHG reports (17) (section 3) (Plenary)

- Project development (section 4) (Plenary or Track B)

  - General (2)

  - Text and software development (0)

  - Test conditions (1)

  - Performance assessment (0)

  - Coding studies and tools on specific use cases (3)

  - Test Material (0)

  - Conformance (2)

  - Implementation studies (4)

  - Profile/level specification (3)

- Low-level tool technology proposals (section 5) with subtopics (Track B)

  - Inter prediction and MV coding (15) (section 5.1.1)

  - Intra prediction and mode coding (9) (section 5.1.2)

  - Loop filtering (24) (section 5.1.3) (Track B)

  - Transforms and transform signalling (16) (section 5.1.4)

  - Partitioning (5) (section 5.1.5)

  - ACT related (6) (section 5.1.6)

  - Other (1) (section 5.1.7)

  - AHG6: 360° video (1) (section 5.2)

  - AHG11: Screen content coding (10) (section 5.3)

  - AHG14: Lossless and near lossless coding (23) (section 5.4)

  - AHG15: Quantization control (4) (section 5.5)

- High-level syntax (HLS) proposals (section 6) with subtopics (Track A)

  - AHG9: General high-level syntax (173) (section 6.1)

  - AHG12: High-level parallelism and coded picture regions (51) (section 6.2)

  - AHG8: Layered coding and resolution adaptation (29) (section 6.3)

- Complexity analysis (0) (section 7) (Track B)

- Encoder optimization (6) (section 8) (Track B)

- Metrics and evaluation criteria (0) (section 9) (Track B)

- Withdrawn (7) (section 10) (Track none)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 11)

- Project planning (section 12)

- Establishment of AHGs (section 13)

- Output documents (section 14)

- Future meeting plans and concluding remarks (section 15)

The document counts above do not include cross-checks and CE summary reports.

Track A (253) was generally chaired by GJS and Track B (120+) by JRO.

# AHG reports (17)

These reports were discussed XXday X April 2020 during XXX-XXX (chaired by XXX)

JVET-R0001 JVET AHG report: Project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

JVET-R0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye]

JVET-R0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring]

JVET-R0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien, R. Chernyak, A. Norkin]

JVET-R0005 JVET AHG report: Conformance testing (AHG5) [J. Boyce, E. Alshina, K. Kawamura, I. Moccagatta, S. McCarthy, K. Sühring, W. Wan]

JVET-R0006 JVET AHG report: 360° video coding tools, software and test conditions (AHG6) [J. Boyce, J. He, K. Choi, J.-L. Lin, Y. Ye]

JVET-R0007 JVET AHG report: Coding of HDR/WCG material (AHG7) [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

JVET-R0008 JVET AHG report: Layered coding and resolution adaptivity (AHG8) [S. Wenger, A. Segall, M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun, P. Topiwala, M. Zhou]

JVET-R0009 JVET AHG report: High-level syntax (AHG9) [R. Sjöberg, J. Boyce, B. Choi, S. Deshpande, M. M. Hannuksela, R. Skupin, A. Tourapis, Y.-K. Wang, W. Wan, P. Wu]

JVET-R0010 JVET AHG report: Encoding algorithm optimization (AHG10) [A. Duenas, A. Tourapis, S. Ikonin, A. Norkin, R. Sjöberg, J. Le Tanou, J.-M. Thiesse]

JVET-R0011 JVET AHG report: Screen content coding (AHG11) [S. Liu, J. Boyce, A. Filippov, Y.-C. Sun, J. Xu]

JVET-R0012 JVET AHG report: High-level parallelism and coded picture regions (AHG12) [S. Deshpande, B. Choi, M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, B. Wang, Y.-K. Wang]

JVET-R0013 JVET AHG report: Tool reporting procedure and testing (AHG13) [W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu]

JVET-R0014 JVET AHG report: Lossless and near-lossless coding (AHG14) [T. Nguyen, T.-C. Ma, M. Ikeda, H. Jang, X. Zhao]

JVET-R0015 JVET AHG report: Quantization control (AHG15) [R. Chernyak, E. François, C. Helmrich, S. McCarthy, A. Segall]

JVET-R0016 JVET AHG report: Implementation studies (AHG16) [M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu]

JVET-R0017 JVET AHG report: Film grain synthesis (AHG17) [A. Norkin, A. Tourapis, D. Grois, P. de Lagrange, X. Li, S. McCarthy, R. Sjöberg]

JVET-R0339 Agenda and report of the category 1 AHG pre-meeting of the 18th JVET meeting [G. J. Sullivan, Y.-K. Wang]

JVET-R0340 Agenda and report of the category 2 AHG pre-meeting of the 18th JVET meeting [J.-R. Ohm, B. Bross, A. Segall, Y. Ye]

# Project development

## General (2)

JVET-R0365 Proposals on VVC extensions for higher fidelity video [T. Suzuki, M. Ikeda, Y. Yagasaki (Sony), T. Toma, K. Abe (Panasonic), M. Shima (Canon)]

JVET-R0383 MC-IF VVC interoperability survey and sub-profile registration [L. Litwic (Ericsson), J. Boyce (Intel), S. McCarthy (Dolby)]

## Text and software development (0)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

## Test conditions (1)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0321 AHG3: Chroma QP table bug-fix and CTC update for RGB coding in VTM-8.0 [J. Xu, L. Zhang, W. Zhu (Bytedance), X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

JVET-R0442 Crosscheck of JVET-R0321 (AHG3: Chroma QP table bug-fix and CTC update for RGB coding in VTM-8.0) [Y.-H. Chao (Qualcomm)] [late]

## Performance assessment (0)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

## Coding studies and tools on specific use cases (3)

JVET-R0359 AHG 17: Illustration of the film grain characteristics SEI message for VVC [Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen]

Move to SEI?

JVET-R0455 AHG17: Cross-check report of JVET-R0359 on Illustration of the film grain characteristics SEI message for VVC [P. de Lagrange, E. François (InterDigital)] [late]

JVET-R0376 Versatile Video Coding for VPCC [D. Mehlem, C. Rohlfing (RWTH)]

JVET-R0384 Alternative film grain characteristics SEI message [A. Norkin (Netflix)]

Move to SEI?

JVET-R0456 Crosscheck of JVET-R0384 on Alternative film grain characteristics SEI message [A. M. Tourapis (Apple)] [late]

## Test material (0)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

## Conformance (2)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0254 AHG5: Conformance bitstreams with decoder conditions [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn, R. Yu, J. Ström (Ericsson)]

JVET-R0405 Editors input on VVC conformance testing [J. Boyce, E. Alshina, K. Kawamura, I. Moccagatta, S. McCarthy, K. Sühring, W. Wan] [late]

## Implementation studies (AHG16) (4)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0224 AHG16: Realization of RPR based real-time VVC decode and playback on ARM based mobile devices [J. Shingala, A. Natesan, A. Chelawat (Ittiam)]

JVET-R0351 High bit depth coding [A. Browne, S. Keating, K. Sharman (Sony)]

JVET-R0364 Information on cinematic aspect ratios in the context of JVET-Q0065 [Sean McCarthy, Walt Husak, Peng Yin, Taoran Lu, Fangjun Pu, Tao Chen]

JVET-R0390 [AHG16] VVC multi-thread decoder and performance analysis [S. Gudumasu, T. Poirier, F. Urban, F. Hiron, P. de Lagrange (InterDigital)]

## Profile/level specification (3)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0370 Main 10 Still Picture and Main 4:4:4 10 Still Picture profiles for VVC version 1 [J. Chen, M. Karczewicz (Qualcomm), B. Bross (HHI), Y.-K. Wang (Bytedance)]

JVET-R0379 Palette mode support in VVC main profile [Y. Ye, R.-L. Liao, M. Sarwer (Alibaba), Y.-H. Chao, W.-J. Chien, J. Chen, M. Karczewicz (Qualcomm), P. Onno, C. Gisquet, G. Laroche (Canon), X. Wang (Kwai)]

JVET-R0392 VVC Version 1 Profiles [W. Wan (Broadcom), D. LeGall (Ambarella), A. Wells (Ambarella), D. Singer (Apple), A. Tourapis (Apple), S. Pejhan (ATEME), M. Raulet (ATEME), S. Davis (Charter Communications), D. Grois (Comcast Cable), Y. Syed (Comcast Cable), X. Ducloux (Harmonic Inc.), P. Haskell (Harmonic Inc.), T. Suzuki (Sony), E. Chai (Ubilinx)]

# Low-level tool technology proposals (114)

## AHG2/AHG3/AHG16: General coding tools (76)

### Inter prediction and MV coding (15)

Initially discussed in AHG session 1.8 Tuesday 14 April 1530-1720 (chaired by JRO)

JVET-R0137 On mvd_l1_zero_flag and NoBackwardPredFlag [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

Only first aspect (problem 1) on NoBackwardPredFlag and ColPic

In this contribution, some solutions for two problems of current VVC Draft 8 have been proposed. One problem is that there is no specification of the variables ColPic and NoBackwardPredFlag and the other problem is that mvd_l1_zero_flag is specified in only picture header even if reference picture list structure can be changed on slice header. Two solutions for the first problem have been shown Option 1 is that the variable ColPic as the almost same as that of HEVC is defined and a new variable IdenticalDirectionalFlag which is replaced to previous NoBackwadPredFlag is specified by using the decoding process for symmetric motion vector difference reference indices. Option 2 is that the variables ColPic and NoBackwadPredFlag as the almost same as that of HEVC are defined. Also, two solutions for the second problem have been shown. Option 1 is that the change of enabling condition of symmetric motion vector difference and mvd_l1_zero_flag by the variable IdenticalDirectionalFlag and option 2 is that mvd_l1_zero_flag is specified in the picture header or in the slice header by the syntax element rpl_info_in_ph_flag exclusively. Neither proposal changes the results of the CTC.

The proposal is filling an existing hole regarding the definition of collocated picture and NoBackwardPredFlag.

The definition proposed for collocated picture seems appropriate (just transferring the HEVC method which is also matching with software). In terms of the NoBackwardPredFlag, the proposed option 1.1 seems to deviate from the SW implementation, option 1.2 also but with less change.

It is recommended to fill the gap in the spec by transferring the corresponding text from HEVC as much as possible, while matching with the decoding process as implemented in SW. The difference compared to HEVC is e.g. related to processing of long term pictures. To be further discussed with HLS experts what the issues are – revisit.

JVET-R0212 On modes in geometric partitioning [C. Hollmann, D. Liu, R. Yu, J. Ström (Ericsson)]

In this contribution three methods to reduce the number of modes for geometric partitioning are presented. These methods are claimed to reduce the number of modes from 64 to 50, 38 and 32, respectively. It is further claimed that these methods have a minor impact on the compression efficiency. It is also asserted that the number of combinations that are required to be tested during verification testing is reduced by up to 50%.

- Method 1 (50 modes): 0.03% RA, 0.00% LDB, 700 combinations to test (-22%)

- Method 2 (38 modes): 0.05% RA, 0.01% LDB, 532 combinations to test (-40%)

- Method 3 (32 modes): 0.07% RA, 0.06% LDB, 448 combinations to test (-50%)

The main intent is reducing the number of combinations for conformance testing.

The issue of testing a large number of combinations in geo was already discussed in the last meeting, and the adopted solution was agreed to be a good compromise.

No action.

JVET-R0385 Crosscheck of JVET-R0212 (On modes in geometric partitioning) [K. Zhang (Bytedance)] [late]

JVET-R0213 Modifications of motion storage in geometric partition mode [R. Yu, D. Liu, C. Hollmann, J. Ström (Ericsson)]

In the current VVC, for a geometric partition mode (GPM) coded block, the block is split into two partitions with a splitting line defined by an angle and a distance. Each partition is associated with a uni-motion. The prediction sample for each partition is generated using the uni-motion. For the prediction samples near the splitting line, a blending operation is carried out to reduce discontinuity.

The motion storage process for the GPM coded block stores three types of motion. The three types of motion are the two uni-motions associated with each partition and a third motion which is a combination of the two uni-motions. The third motion is also referred to as type2 motion. Each 4x4 subblock within the GPM coded block stores one of the three types of motion. The type2 motion is stored within 4x4 subblocks that are within the blending area. It is asserted that since the blending area is narrow in general the storage of the type2 motion will be unnecessary.

This contribution proposes to remove the storage of type2 motion for GPM coded blocks. In other words, only the two uni-motions are stored. It is claimed that with the modification, one absolute operation and one comparison operation for each 4x4 subblock can be saved if the motion storage map is computed on the fly. It is also claimed that the determination process for the type2 motion can also be removed. It is further claimed that the specification text for the motion storage for GPM can be significantly cleaned up. The modification was implemented in the VTM-8.0 and the BD-rate impact is reported to be -0.01% for RA and 0.01% for LDB. It is proposed to adopt the modification considering that the BD-rate impact is negligible.

There is no real problem to be solved, several experts pointed out that similar methods had been proposed earlier in the context of TPM. Some concern was also expressed with regard to possible impact on subjective quality with regard to blending.

No action.

JVET-R0389 Crosscheck of JVET-R0213 (Modifications of motion storage in geometric partition mode) [Z. Deng (Bytedance)] [late]

JVET-R0223 AHG16: On DMVR and wraparound motion compensation [J. Luo, J. Chen, Y. Ye (Alibaba)]

In VVC draft 8, when wrap around motion compensation is enabled, the bilinear interpolation in DMVR motion search process also uses wrap-around clipping operation. In this contribution, it is proposed to apply regular clipping operation during DMVR motion search, and apply wrap-around clipping operation only for regular interpolation during the final motion compensation process. It is asserted that this simplifies the motion search process in DMVR. Experiment results reportedly show that the BD rate difference is -0.01%, -0.01%, -0.02% for end-to-end WS-PSNR for Y, U and V respectively. Informal subjective viewing was conducted and no visible difference was observed.

Results with 360 video PERP.

It was commented that the implementation seems to become more complicated, as an additional reference area fetch would become necessary for DMVR.

No action on the proposal

There is another aspect in the proposed text that the DMVR text could be simplified, e.g. in terms of that the combination with RPR would never be used. Recommendation: Editorial improvement left to editor.

JVET-R0425 Crosscheck of JVET-R0223 (AHG16: On DMVR and wraparound motion compensation) [Y.-H. Lee, J.-L. Lin (MediaTek)] [late]

JVET-R0282 GEO with MMVD [K. Panusopone, S. Hong, L. Wang (Nokia)]

This contribution proposes to harmonize GEO with MMVD such that MVD can be applied to derive MV of a GEO partition. The proposed method first determines base MV using current GEO MV calculation, then computes MVD for each base MV following method like MVD calculation for MMVD. Simulation results show the proposed method has BD-rate of approximately -0.25%, -0.36%, -0.49% for RA, and -0.51%, -0.66%, -0.97% for LB, respectively, compared to VTM-8.0 anchor.

Encoding time increase is roughly 30% for RA, and over 30% for LB. This is not an attractive tradeoff.

No action.

JVET-R0407 Crosscheck of JVET-R0282: GEO with MMVD [K. Reuzé (??)] [late]

JVET-R0292 Fixes for 4-tap interpolation filtering [K. Andersson, R. Yu, Z. Zhang, J. Ström (Ericsson)]

Only one aspect related to inter:

- 4-tap interpolation filter for chroma motion compensation

- 4-tap interpolation filter for intra angular prediction

It is asserted that the current 4-tap interpolation filters which are used both for intra angular prediction and chroma motion compensation have significant phase misalignments compared to the ideal phases of the interpolation filters. For ten of the interpolation filters the phase misalignments are as large as the expected phase differences between two interpolation filters of adjacent fractional positions. This contribution proposes 4-tap interpolation filters that fixes the phase misalignments of the current 4-tap interpolation filters.

The fixes were reported to be tested under VTM-8.0 CTC. The impact on coding efficiency is reported for Luma/Cb/Cr as follows:

-0.05%/-0.03%-0.06% for AI, -0.02%/-0.05%/-0.15% for RA and -0.11%/-0.24%/-0.30% for LDB.

The impact of only fixing the 4-tap interpolation filters for intra is reported to be:

-0.05%/-0.03%-0.06% for AI, -0.04%/-0.04%/-0.02% for RA and -0.06%/0.01%/-0.09% for LDB.

Other experts commented that the problem of phase mismatch is known to them and expressed that an alignment might be desirable.

It is asked if the energy gain of the filters is close to unity over the different filters from the set?

It was also asked if there could be visual impact? For coded video, proponents did not observe differences, but they did not check the prediction.

It is confirmed that the 16 bit precision is retained.

It is pointed out that the variation of magnitude responses among the different filters of the set may be of concern (this also relates to the energy gain question). Is it better or worse in that compared to the current filters?

Revisit (along with the related contribution R0293)

JVET-R0293 Fixes for 6-tap interpolation filtering for affine motion compensation [K. Andersson, R. Yu, Z. Zhang, J. Ström (Ericsson)]

TBP

JVET-R0311 [AHG2] Fix cu_skip_flag signaling for IBC [H. Jang, J. Nam, N. Park, S. Kim, J. Lim (LGE)]

TBP

JVET-R0357 Geometric prediction mode with motion vector difference [K. Zhang, L. Zhang, Z. Deng, H. Liu, Y. Wang (Bytedance)]

TBP

JVET-R0429 Cross-check of JVET-R0357: Geometric prediction mode with motion vector differences [C. Hollmann (Ericsson)] [late]

JVET-R0366 Simplified disLut for GPM [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, H.-X. Wang, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

TBP

JVET-R0447 Crosscheck of JVET-R0366 (Simplified disLut for GPM) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-R0367 Adjustment of shiftHor calculation in GPM [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, M.-L. Zhang, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

TBP

JVET-R0448 Crosscheck of JVET-R0367 (Adjustment of shiftHor calculation in GPM) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-R0368 GPM merge list construction modification [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

TBP

JVET-R0422 Crosscheck of JVET-R0368 (GPM merge list construction modification) [H. Chen, H. Yang (Huawei)] [late]

JVET-R0369 Combination of JVET-R0367 and JVET-R0368 for GPM [Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu, J.-Y. Huo, F.-Z. Yang (Xidian Univ), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

TBP

JVET-R0423 Crosscheck of JVET-R0369 (Combination of JVET-R0367 and JVET-R0368 for GPM) [H. Chen, H. Yang (Huawei)] [late]

JVET-R0371 AHG2/9: On max num of subblock merge candidates [H. Huang, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

TBP

JVET-R0373 AHG9: On Maximum Number of Subblock Merge Candidates [Y.-C. Yang, C.-Y. Teng (Foxconn)]

TBP

### Intra prediction and mode coding (9)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0280 AHG16: Cleanup of intra reference sample filter selection [J. Heo, H. Jang, J. Choi, J. Nam, M. Koo, J. Lim, S. Kim (LGE)]

JVET-R0432 Crosscheck of JVET-R0280 (AHG16: Cleanup of intra reference sample filter selection) [F. Bossen (Sharp)] [late]

JVET-R0281 AHG16: Cleanup MIP flag signaling [J. Heo, H. Jang, J. Choi, J. Lim, S. Kim (LGE)]

JVET-R0435 Crosscheck of JVET-R0281 (AHG16: Cleanup MIP flag signalling) [J. Pfaff (HHI)] [late]

JVET-R0288 AHG16: Reference samples for ISP [F. Bossen (Sharp)]

JVET-R0399 Crosscheck of JVET-R0288 (AHG16: Reference samples for ISP) [S. De-Luxán-Hernández (HHI)] [late]

JVET-R0314 AHG2/AHG16: Fixes on CCLM [Y. Wang, K. Zhang, L. Zhang, H. Liu (Bytedance)]

JVET-R0350 MIP for all channels in the case of 4:4:4 and single tree [J. Pfaff, B. Stallenberger, P. Merkle, M. Schäfer, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [AHG late]

JVET-R0356 CCLM-related bugfixes for the VVC specification draft [A. Filippov, V. Rufitskiy, E. Alshina (Huawei)] [AHG late]

JVET-R0375 AHG2/AHG16: CCLM bug fix in luma reference down-sampling [L. Pham Van, G. Van Der Auwera, J. Chen, V. Seregin, M. Karczewicz (Qualcomm)] [AHG late]

JVET-R0434 Crosscheck of JVET-R0375 (AHG2/AHG16: CCLM bug fix in luma reference down-sampling) [J. Pfaff (HHI)] [late]

JVET-R0391 Simplification on CCLM [L. Li, X. Li, S. Liu (Tencent)] [AHG late]

JVET-R0449 Crosscheck of JVET-R0391 (Simplification on CCLM) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-R0452 CCLM: common text for spec bugfixes [L. Li, X. Li, S. Liu (Tencent), A. Filippov, V. Rufitskiy, E. Alshina (Huawei)] [late]

### Loop filtering (24)

#### Deblocking filter (10)

Initially reviewed in AHG session 2.2 Thu 9 April 1520-1705 UTC (chaired by JRO)

JVET-R0130 Cleanup of tC value derivation process for deblocking filter [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

TBP

JVET-R0134 AHG2: Mismatch related to deblocking of subblock motion edges [B. Heng, M. Zhou, W. Wan (Broadcom)]

This contribution asserts that there is a mismatch between the VVC draft text and VTM sotware related to deblocking of coding subblock boundaries. Within a subblock motion CU, the length of the deblocking filter used depends on the distance the nearest transform edge. However, when this neighboring transform edge aligns with a virtual boundary, the behavior of the text and software differ.

Specifically, the VTM software treats the neighboring transform edge as a transform edge, regardless of whether it aligns with a virtual boundary or not. While the VVC draft text ignores the neighboring transform edge altogether if it aligns with a virtual boundary. This difference will cause the text and software to use different filter lengths for subblock motion edges, and therefore they will produce different results.

This contribution proposes to modify the text to match the software behavior to resolve this mismatch. Proposed text changes are provided.

Presented Thu 9 April 1733 (chaired by JRO).

There is a ticket #857 which also identifies this issue (as well as other issues). This was partially resolved by submitting a software patch. The new contribution points out that after that we have still a mismatch between text and software. Text appears to be appropriate.

Recommendation (BF text): The proposed text changes should be adopted.

This also would resolve ticket #857 as far as the text is concerned.

JVET-R0168 Issue on bS derivation of deblocking filter [K. Abe, T. Toma (Panasonic)]

This contribution points out the mismatch between VVC text and VTM on bS derivation process of deblocking filter. In VVC text, bS is set equal to 1 for the boundary between IBC and inter block on both luma edge and chroma edge. On the other hand, in VTM, bS is set equal to 1 for the boundary between IBC and inter block only on luma edge. The proponent of this contribution thinks there are two solutions, solution1: fix VTM to align to VVC text, solution2: fix VVC text to align to VTM. This contribution shows the difference of coding performance and text changes for both solutions.

No ticket yet.

It is mentioned by the proponents that solution 1 would have small impact on coding results, therefore they would better suggest aligning the text with software.

Several experts expressed support for solution 2, as also in the past it had been agreed that MV differences should not be checked for chroma deblocking.

Recommendation (BF text): The proposed text changes should be adopted. There may however be some interaction with a related issue in R0228, where palette mode is also considered.

JVET-R0372 Crosscheck of JVET-R0168 (Issue on bS derivation of deblocking filter) [T. Hashimoto, T. Ikai (Sharp)]

JVET-R0228 AHG11: Bugfix to deblocking filter boundary strength setting for palette [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba)]

In VVC draft 8, the deblocking filter boundary strength is set according to the coding modes of two neighboring blocks along a deblocking edge. However, it is reported that, when one of neighboring blocks is coded in palette and the other is coded in IBC or inter mode, the boundary filtering strength is not clearly defined in the spec. It is also reported that in VTM-8.0, this ambiguity partially exists as well, and can cause the software mismatch in encoder and decoder in some cases. In this contribution, it is proposed to clearly define the boundary filtering strength for the aforementioned case by setting it to one of 1 or 2. It is reported that, as compared to VTM-8.0 with palette off, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- VTM8:{-6.25%,-6.42%,-6.26%}for AI,{-5.04%,-6.81%,-6.86%}for RA,{-3.18%,-4.39%,-4.60%}for LB

- bS = 1:{-6.25%,-6.42%,-6.26%}for AI,{-5.03%,-6.82%,-6.87%}for RA,{-3.20%,-4.34%,-4.16%}for LB

- bS = 2:{-6.25%,-6.42%,-6.26%}for AI,{-5.02%,-6.76%,-6.81%}for RA,{-3.14%,-4.26%,-4.41%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-1.24%,-0.43%,-0.47%}for AI,{-1.29%,-0.47%,-0.56%}for RA,{-0.56%,-0.24%,-1.24%}for LB

- bS = 1:{-1.24%,-0.43%,-0.47%}for AI,{-1.29%,-0.48%,-0.51%}for RA,{-0.61%,-0.14%,-0.76%}for LB

- bS = 2:{-1.24%,-0.43%,-0.47%}for AI,{-1.32%,-0.53%,-0.52%}for RA,{-0.60%,-0.38%,-0.98%}for LB

For class SCC in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- VTM8:{-6.50%,-5.18%,-4.94%}for AI,{-3.78%,-2.45%,-2.42%}for RA,{-1.22%,-0.59%,-0.66%}for LB

- bS = 1:{-6.50%,-5.18%,-4.94%}for AI,{-3.76%,-2.42%,-2.43%}for RA,{-1.23%,-0.57%,-0.60%}for LB

- bS = 2:{-6.50%,-5.18%,-4.94%}for AI,{-3.76%,-2.36%,-2.39%}for RA,{-1.28%,-0.68%,-0.69%}for LB

In terms of subjective quality, no significant visual difference was observed in all three cases based on the informal subjective viewing conducted.

It I agreed during the discussion that the spec text does not clearly define the bS in case where palette is used at the other side of the block. For the case of not using local dual tree, the VTM mode of operation is clearly defined. For this case, it is agreed that the text should be aligned with the software (setting Bs=1 when the other side is a palette block, and operated in single tree mode). For local dual tree, more investigation is necessary to understand if the SW is covering all possible cases (e.g. when luma is palette and chroma in IBC, etc.)

Recommendation (BF text): The proposed text change on more clearly specifying bS in case of palette should be adopted. There may however be some interaction with a related issue in R0168

Revisit: Further offline study needed for the case of local dual tree.

JVET-R0440 Crosscheck of JVET-R0228 (AHG11: Bugfix to deblocking filter boundary strength setting for palette) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0289 [AHG16] On deblocking filter process [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In Ticket #899, it was claimed that there is a mismatch between VVC draft 8 and VTM8.0 on deblocking filter process when CU size is greater than maximum transform block size and cu_coded_flag is equal to 0.

In VVC draft 8, implicit TU tiling is applied even though cu_coded_flag is equal to 0 for inter prediction mode. These internal transform block boundaries within the CU is considered as transform block boundary in deblocking filter process. However, in the VTM8.0, the above case is treated as a TU which has the same size as CU therefore there is no internal transform block boundary. Although the mismatch has already resolved by the ticket #899, this cause misalignment in terms of filter length on the each block boundary with same property.

In this proposal, to fix abovementioned behavior two solutions are suggested.

- Method1: Implicit TU tiling is restricted when cu_coded_flag is equal to 0.

- Method2: Implicit TU tiling is retained but edges inside a CU are not treated as transform edge when cu_coded_flag is equal to 0.

Since Ticket #899 is reverted by Method 1 and deblocking filter process is fixed to be performed same as VTM8.0 by Method 2, the experimental results for both solutions are same as results of VTM8.0.

In revision 1, it is added the simulation results based on Ticket #899 bugfix.

After the fix of ticket #899, there is no mismatch between text and software.

The suggested solution 1 would revert the solution of the ticket, and require additional checks.

Also the solution 2 would be requiring additional logic.

There is no problem with the current design, no need for action.

JVET-R0395 Crosscheck of JVET-R0289 ([AHG16] On deblocking filter process) [R.-L. Liao (Alibaba)] [late]

JVET-R0300 Additional fix for long luma deblocking decisions [K. Andersson, J. Enhorn (Ericsson)]

The contribution proposes to fix the long deblocking decision such that all lines of respective 4 samples boundary segment are checked to avoid over filtering of lines 1 and 2 due to decision based only on line 0 and line 3. It is asserted that the proposal ensures that the deblocking filtering is robust and that the fix does not increase worst case complexity for deblocking decisions.

The BD rate impact for luma for CTC SDR:

AI: 0.00%, RA: -0.01%, LDB: -0.05%, LDP: -0.01%.

The BD rate impact for CTC HDR:

AI:  DE:   0.01% PSNRL: 0.00% wPSNRY: 0.00%, PSNRY: 0.00%

RA: DE: -0.04% PSNRL: -0.03% wPSNRY: -0.02%, PSNRY: 0.00%

Similar encoding and decoding time as the anchor.

The number of operations increases, but is still less than worst case for deblocking.

The target is rather a corner case, it is reported that the effect was visible in the sequence slide editing. Other experts mentioned that deblocking for screen content is a very special case, where an encoder might want to align parameters.

Not obvious that there is need for action on this issue.

JVET-R0279 AHG9: On decoupling luma deblocking parameters [K. Misra, F. Bossen, J. Samuelsson, S. Deshpande, A. Segall (Sharp Labs of America)]

In the current VVC draft, the deblocking activation and clipping threshold values (beta and tc) are common for the luma long filters and luma strong filters. This contribution proposes that the threshold values for each type of filtering be separate. This is achieved by signaling separate beta offset and tc offset for luma long filters and luma strong filters. The proposed modification enables the decoupling of activation and clipping control thresholds for the two types of filtering. The proposed change is asserted to be a desirable improvement over the signalling in the current VVC draft.

Question: Is there evidence that this is needed? Can it be expected that the values would be so different?

It also requires some additional logic at low level, switching the offset values between two different options.

No action.

JVET-R0388 AHG9: Cleanups on deblocking signalling [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, J. Xu (Bytedance)]

TBP

JVET-R0403 On the boundary strength derivation of IBC coded blocks [B. Ray, G. Van der Auwera, M.Karczewicz (Qualcomm)] [late]

TBP

JVET-R0454 Cross-check of JVET-R0403 (On the boundary strength derivation of IBC coded blocks) [K. Andersson (Ericsson)]

JVET-R0437 Combination of JVET-R0168 and JVET-R0228 on deblocking filter boundary strength setting [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba), K. Abe, T. Toma (Panasonic)] [late]

TBP

#### Adaptive loop filter (7)

Initially reviewed in AHG session 2.2 Thu 9 April 1705-1720 and session 2.3 2100- UTC (chaired by JRO)

JVET-R0133 AHG16: On Clipping values for Non-linear ALF [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [AHG late]

In VVC WD, clipping values for Non-linear ALF filtering process are defined in a lookup table, as a form of “2N” depending on BitDepth and clipIdx, where N is an integer value. When BitDepth is equal to 16 and clipIdx is equal to 0, clipping values become 216; Thus, the lookup table needs up to 17 bits per element.

This contribution proposes to replace the clipping values in a form of “2N” to a form of “2N - 1”; Any clipping values are kept within 16 bits and can be derived by simple logical operation without the lookup table. It is asserted that the proposed method has negligible bdrate changes of (0.00%,0.00%, 0.00%) for AI, (0.00%, -0.05%, 0.01%) for RA and (0.01%, -0.06%, -0.17%) for LDB under CTC.

In v2, results of IBDI equal to 8/12 are attached.

It is observed that:

- For IBDI=8, average bdrate changes are (0.02%, 0.10%, 0.11%) for AI, (0.03%, -0.19%, -0.21%) for RA and (0.02%, -0.40%, -0.44%) for LB.

- For IBDI=12, average bdrate changes are (0.00%, -0.01%, 0.01%) for AI, (0.00%, -0.03%, 0.07%) for RA and (0.05%, 0.03%, 0.04%) for LB.

Presented Thu April 9 1705 UTC (chaired by JRO)

The advantage would only apply to case of profiles beyond 15 bit, where the cost of lookup table storage seems almost irrelevant

There was a contribution in Gothenburg (JVET-O0188) which proposed the same approach. It was not adopted by that time

Several experts expressed that this change is not needed, as in the only case that would require 17 bit implementation clipping would have no effect and could be skipped.

No action.

JVET-R0208 AHG16: Rounding correction for ALF virtual boundary processing [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

In VTM-8.0 [1], to avoid extreme padding for the sample rows which are immediately adjacent to the adaptive loop filter (ALF) virtual boundary, the correction value applied during the filtering is quantized by a larger value 1024 (210) instead of 128(27). However the rounding value used during the filtering is still 64. The current proposal proposes a fix by changing the rounding value to 512 when the quantization value used in the ALF filtering is 1024.

The objective results, over VTM8.0 Anchor for CTC configuration are as follows:

Config.	Y	U	V	EncT	DecT

AI	0.00%	0.01%	0.01%	100%	99%

RA	0.01%	-0.01%	0.00%	100%	100%

LDB	0.01%	0.02%	-0.07%	100%	102%

LDP	0.01%	-0.25%	-0.14%	100%	103%

By modifying filters at virtual boundaries per adoption of Q0150 solution 2, the rounding operation in case of shift 10 is no longer doing the nearest integer rounding. This appears as an inconsistency rather than a bug. This issue had been detected during software integration. Proponents of Q0150 also support this change. It is agreed that the change is minor and there is no harm that it would introduce any problems.

R0231 method 1 and R0291 target the same problem, basically the same solution but different specification text. There may be more elegant ways of expressing the change than suggested in R0208, which introduces another column in the table 45/46, e.g. by an equation.

Recommendation(cleanup/text+software): The rounding operation in case of the modified filter at virtual boundary should be aligned. Editors should decide the best way of expressing it in text.

JVET-R0231 AHG2: Rounding offsets for adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

Virtual boundary (VB) processing is adopted to VVC to avoid line buffer increment for adaptive loop filter (ALF) and cross component adaptive loop filter (CC-ALF). Symmetrical sample padding is applied when VB processing is applied to ALF and CC-ALF. In some cases, extreme padding for the closest row on each side of a VB may introduce visual artifacts. In VVC, to account for that in ALF when ALF is applied to samples on the rows adjacent to a VB, filter strength is reduced by increasing the right shift for ALF filtering. However, the rounding offset of the right shift is not changed and is kept the same for all values of the right shift. In addition, the filter strength for samples on the rows adjacent to a virtual boundary is not changed in CC-ALF. In this contribution, at first, the rounding offset is changed for different right shift values for ALF. In another aspect, the filter strength is reduced when applying CC-ALF to the samples on the rows adjacent to a VB. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

| Results over VTM-8.0 | All Intra |  |  | Random Access |  |  | Low Delay B |  |  | Low Delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Method 1 | 0.00% | 0.01% | 0.01% | 0.01% | -0.01% | 0.00% | 0.01% | 0.02% | -0.07% | 0.01% | -0.25% | -0.14% |
| Method 2 | 0.00% | 0.05% | 0.05% | 0.00% | -0.01% | 0.00% | -0.05% | 0.00% | -0.09% | -0.01% | -0.31% | 0.20% |
| Method 3 | 0.00% | 0.07% | 0.07% | 0.01% | 0.01% | 0.04% | -0.04% | 0.09% | 0.05% | 0.01% | -0.32% | -0.17% |

Method 1 is identical to R0208.

Method 2 applies the modified boundary processing of Q0150 to CCALF

Method 3 combines methods 1 and 2.

It is asked if there is evidence that a similar problem of artifacts is present in CCALF which was shown in ALF in context of the Q0150 adoption. Currently, there is no such evidence.

The main argument is for design consistency. ALF and CCALF could share the table 45, but otherwise equations would be different. In implementations, different logic would likely be used.

Benefit not obvious – no action on Method 2/3.

JVET-R0291 AHG16: On ALF attenuation near virtual boundaries [F. Bossen (Sharp)]

TBP?  It was verbally reported in AHG session that all issues raised in this document are also covered by other contributions (R0208, R0231, R0233, R0312).

JVET-R0444 Crosscheck of JVET-R0291 (AHG16: On ALF attenuation near virtual boundaries) [N. Hu (Qualcomm)] [late]

JVET-R0299 Additional fix for ALF virtual boundary processing [K. Andersson, J. Ström, Z. Zhang, J. Enhorn (Ericsson)]

At the last previous meeting, a low complexity fix for ALF virtual horizontal CTU boundary was adopted from JVET-Q0150. An alternative approach proposed in the same contribution was rejected since the increase of two luma line buffers and two chroma line buffers for each component was undesirable. This contribution proposes a combination of the two approaches in JVET-Q0150 as follows: Filtering of a row just above the virtual horizontal CTU boundary is performed as currently using the low complexity technique, i.e., not using samples below the virtual horizontal CTU boundary. When filtering a row just below the virtual horizontal CTU boundary on the other hand, this contribution proposes to change the filtering process so as to let it use also one row just above the virtual horizontal CTU boundary. This combined approach is asserted to further reduce visual artifacts from virtual horizontal CTU boundary processing. Proposal 1 of this contribution only changes the ALF filtering. It is claimed that the memory cost for proposal 1 is 60% of one 10-bit line buffers for luma samples and 60% of one 10-bit line buffer for each chroma channel for chroma samples. Proposal 2 of this contribution combines proposal 1 with an approach that avoids filtering across the virtual boundary also for SAO when filtering samples just below the virtual boundary, by employing padding. Samples just above the virtual boundary are SAO-filtered as currently. It is claimed that proposal 2 comes at no memory cost in terms of line buffers over the current draft of VVC.

The claimed benefit of the proposal is suppression of coding artifacts from virtual horizontal CTU boundary processing. The BD rate impact for luma for CTC is as follows:

Proposal 1: AI: -0.01%, RA: -0.03%, LDB: -0.09%

Proposal 2: AI: -0.01%, RA: -0.xx%. LDB: -0.xx%

Similar encoding and decoding times as the anchor are reported.

It is claimed that the method improves over the method from Q0150 adopted by last meeting, but requires approx. 0.6 additional line buffer (by using buffer jointly with SAO).

It is however pointed out that SAO does not need to store sample values, so it would be more like 1 line buffer.

The proposal would require a substantial amount of changes, and the additional subjective benefit over the Q0150 method may not be too large.

No action.

JVET-R0312 AHG2/AHG16: A fix on chroma ALF virtual boundary position [Y. Wang, L. Zhang, H. Liu, K. Zhang, Z. Deng (Bytedance)]

In current VVC, luma and chroma ALF virtual boundaries are always set to four and two lines above the bottom luma CTB and chroma CTB boundaries, respectively. Such a design works well for 4:2:0 colour format since the height of a chroma CTB is half of that of a luma CTB. However, for 4:2:2 and 4:4:4 colour formats in which heights of a luma CTB and a chroma CTB are equal, it is asserted that the design could result in misaligned ALF virtual boundaries for luma and chroma samples. This contribution proposes to align ALF virtual boundaries of luma and chroma components for 4:2:2 and 4:4:4 colour formats. The simulation results for 4:4:4 colour format screen sequences, 4:4:4 and 4:2:2 natural sequences following the common test conditions are summarized as follows:

Dual tree on:

AI: {0.00%, -0.01%, 0.00%}; RA: {-0.02%, -0.02%, -0.03%}; LDB: {0.02%, -0.01%, 0.08%}

Dual tree off:

AI: {0.00%, 0.00%, 0.00%}; RA: {0.02%, 0.02%, -0.02%}; LDB: {0.01%, 0.07%, 0.05%}

Natural sequences:

YUV 4:4:4, AI: {0.00%, -0.02%, -0.04%}; RA: {0.01, -0.07%, 0.03%}; LDB: {}

YUV 4:2:2, AI: {0.00%, 0.00%, -0.02%}; RA: {0.01, -0.11%, -0.08%}; LDB: {0.00%, -0.15%, 0.01%}

It is proposed that for the cases of 4:4:4 and 4:2:0 the virtual boundary height should be aligned for luma and chroma.

It is pointed out that in terms of quality this may not be needed, as luma and chroma have different characteristics.

The motivation is about improving pipelining.

From the discussion, it is not fully clear if this would have consequences on the interaction with deblocking and SAO in the pipeline. There are different opinions on that.

Contribution 233 method is identical. See further discussion there.

JVET-R0363 Crosscheck of JVET-R0312 (AHG2/AHG16: A fix on chroma ALF virtual boundary position) [C.-M. Tsai (MediaTek)] [late]

JVET-R0322 ALF virtual boundary issue [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

TBP

#### CCALF (5)

Contributions initially presented in AHG session 2.6 Tuesday 14 April 0715-0815 except otherwise noted.

JVET-R0128 AHG16: On CCALF clipping [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

In VVC CCALF process, an 8-tap filter is applied to luma sample to generate a residual correction for the chroma samples. At first, an offset value is generated from the luma samples and then the offset value is clipped. Then, the clipped offset value is added to the chroma sample to generate filtered output. Another clipping operation is performed to generate final filtered sample.  This contribution proposes to remove the first clipping operation (i.e. clipping the offset value before sum) from the CCALF process. Following results are reported as compared to VTM-8.0.

	AI :  0.00% (Y), 0.00% (Cb), 0.00% (Cr)

	RA : 0.00% (Y), 0.00% (Cb), 0.00% (Cr)

	LB : 0.00% (Y), 0.00% (Cb), 0.00% (Cr)

In v2, the results of HDR sequences are added.

It is reported that the first clipping is never triggered in CTC.

One reason for introducing this clipping was saving memory for intermediate storage of luma data for later use in CCALF. There is also a conformance stream designed to check if the decoder implements the clipping.

No action.

JVET-R0443 Crosscheck of JVET-R0128 (AHG16: On CCALF clipping) [N. Hu (Qualcomm)] [late]

JVET-R0230 AHG2: Syntax clean-up for cross component adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. Filter coefficients of CC-ALF are signalled in adaptation parameter sets (APSs). In an APS, a Cb (Cr resp.) CC-ALF filter set with up to 4 filters could be signalled and a filter from this Cb (Cr resp.) filter set could be applied to a Cb (Cr resp.) coding tree block. On the other hand, in an APS, a chroma adaptive loop filter (ALF) filter set with up to 8 filters could be signalled and a filter from this chroma filter set could be applied to a chroma coding tree block. In this contribution, CC-ALF filters are unified for the two chroma components. Cb and Cr components share the same CC-ALF filter set in an APS, which is the same method used in a regular chroma ALF. Under common test conditions, compared with VTM-8.0, the average BD rate of the proposed method is

AI: 0.03%(Y), -0.17%(U), -0.20%(V)

RA: 0.01%(Y), -0.10%(U), 0.06%(V)

LDB: -0.04%(Y), -0.16%(U), -0.04%(V)

LDP: -0.06%(Y), -0.09%(U), 0.21%(V)

The intent of the proposal is unifying CCALF with ALF, using the same APS ID for Cb and Cr at slice.

There is nothing wrong with the current spec, and the proposal might give up some flexibility.

No action.

JVET-R0233 AHG16: Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In VVC draft 8, cross component adaptive loop filter (CC-ALF) is adopted to refine chroma components by using luma samples. To get an offset for a chroma sample, CC-ALF is applied to luma samples (after applying luma sample adaptive offset) where the centre of the filter is the co-located luma sample of the current chroma sample.

To reduce the line buffer requirement for adaptive loop filter (ALF) and CC-ALF, virtual boundary (VB) processing is applied to both ALF and CC-ALF. The position of a VB is 4 lines of luma samples and 2 lines of chroma samples above a horizontal coding tree unit (CTU) boundary. When ALF or CC-ALF is applied, a current to-be-filtered sample above (below resp.) a VB can not use samples below (above resp.) the VB.

However, when CC-ALF is applied to 4:2:2 and 4:4:4 video sequences, in some cases, when current to-be-filtered chroma sample is above a VB, its co-located luma sample is below the VB, which conflicts the design of VB.

In this contribution, three methods are proposed to solve the problem. Compared to VTM-8.0, the average BD-rate for the proposed methods is as follows:

Results over VTM-8.0	All Intra	Random Access	Low Delay B	Low Delay P

Y	U	V	Y	U	V	Y	U	V	Y	U	V

Method 1	0.00%	0.00%	0.01%	0.01%	-0.03%	0.02%	0.01%	-0.06%	0.01%	-0.02%	-0.02%	-0.04%

Method 2	0.00%	0.02%	0.06%	0.02%	0.04%	0.11%	0.00%	0.03%	0.08%	-0.06%	0.00%	0.04%

Method 3	0.00%	-0.01%	-0.02%	0.00%	-0.06%	-0.03%	0.01%	-0.07%	-0.05%	-0.02%	-0.04%	-0.13%

Discussed in session 2.3 Thu 9 Apr 2225-2300UTC (chaired by JRO)

Method 3 is conceptually identical with R0312. The problem is that in case of 4:4:4 and 4:2:2 the processing of chroma in CCALF can only be started when the VB processing of luma at co-located positions has been finished. As a consequence, two additional line buffers are required for each chroma component (above the 2 lines of chroma VB).

The problem only arises due to CCALF, but as the VB definition of ALF and CCALF is identical, cannot be separated. Method 3 is not saving any line buffers, but just redefines the height of the chroma VB.

It is not obvious (different opinions) that method 3 has a clear benefit. It claims to be more consistent between luma and chroma VB processing for 444 and 422, but on the other hand is less consistent with 420 chroma in those cases.

Methods 1 and 2 are saving the additional chroma line buffers in 444 and 422. Method 1 proposes to use luma samples from line above which are not co-located. Method 2 skips CCALF for the two rows where the additional line buffers would be necessary. Both methods would require some additional logic. It is not known whether they might impose subjective artifacts. Likely, the second method seems preferable in both aspects.

It is mentioned that for 444 (which requires more memory anyway) the four additional line buffers are not too critical. Clarify this aspect – revisit. If the line buffers are not asserted to be critical, the current design should be good enough. Otherwise, method 2 should be considered.

JVET-R0387 Crosscheck of JVET-R0233 (AHG16: Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences) [Y. Wang (Bytedance)] [late]

JVET-R0259 AHG7: On CCALF filtering of chroma sample location type-2 content [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

It is asserted that the CCALF filter shape is not optimal for chroma sample location type-2 content. Accordingly, this contribution proposes three cross shaped CCALF filters. Following results are reported.

For chroma sample location type-2 content,

	9 tap 5x5 cross shaped filter:

o	AI :  0.01% (Y), -0.77% (Cb), -0.81% (Cr)

o	RA:  0.07% (Y), -1.99% (Cb), -1.95% (Cr)

o	LB:  0.19% (Y), -5.85% (Cb), -9.09% (Cr)

	13 tap 7x7 cross shaped filter:

o	AI :  0.05% (Y), -1.40% (Cb), -1.34% (Cr)

o	RA:  0.04% (Y), -2.85% (Cb), -3.22% (Cr)

o	LB:   0.10% (Y), -7.28% (Cb), -11.34% (Cr)

	8 tap 5x4 cross shaped filter:

o	AI :  0.00% (Y), -0.35% (Cb), -0.39% (Cr)

o	RA:  0.05% (Y), -1.43% (Cb), -1.52% (Cr)

o	LB:   0.17% (Y), -4.10% (Cb), -7.06% (Cr)

When the proposed cross-shape filters are applied on chroma sample location type-0 content, it is reported that some coding gain can also be achieved:

	9 tap 5x5 cross shaped filter:

o	AI :  0.02% (Y), -0.26% (Cb), -0.68% (Cr)

o	RA:  0.01% (Y), -0.39 % (Cb), -0.48 % (Cr)

o	LB:  -0.04 % (Y), -1.06 % (Cb), -1.19 % (Cr)

	13 tap 7x7 cross shaped filter:

o	AI :  0.04% (Y), -0.82% (Cb), -1.16% (Cr)

o	RA:  0.00% (Y), -1.30% (Cb), -1.28% (Cr)

o	LB:   -0.02% (Y), -3.25% (Cb), -2.80% (Cr)

	8 tap 5x4 cross shaped filter:

o	AI :  0.01% (Y), -0.09% (Cb), -0.55% (Cr)

o	RA:  -0.01% (Y), -0.19% (Cb), -0.31% (Cr)

o	LB:   -0.01% (Y), -0.55% (Cb), -0.91% (Cr)

It is commented that the current filter shape could allow asymmetric tuning of coefficients regarding type 2 content.

Filters beyond 8-tap would be more complex than current design. Buffer requirements would not be increased. In hardware, also the 8-tap filter could be less regular.

Gains are mainly observed in HDR (where HDR H1 is the only type 2 sampling). Compared to the gain of CCALF in those sequences, the additional gain is approximately one tenth of that (or even less for the 8-tap filter).

Gains are largest in LB, which may not be the primary use case of CCALF.

It was asked for visual quality. It is reported by proponents that they inspected visual quality and did not find problems, nor differences compared to current CCALF.

It is commented that this is a quite substantial low level modification with the main intent of compression improvement, which only applies for certain type of content.

No action.

JVET-R0446 Crosscheck of JVET-R0259 (AHG7: On CCALF filtering of chroma sample location type-2 content) [F. Pu (Dolby)] [late]

JVET-R0313 AHG2/AHG16: Cleanups of chroma ALF and CC-ALF on/off control [Y. Wang, L. Zhang, H. Liu, K. Zhang (Bytedance)]

In current VVC, chroma ALF and CC-ALF are disabled implicitly when luma ALF is disabled at SPS/PH/SH as it is unlikely that chroma ALF/CC-ALF would be used when luma ALF is disabled. Such a design could benefit power consumption. However, luma ALF, chroma ALF and CC-ALF are controlled independently at CTU level. Therefore, for a slice, it is still possible that chroma ALF/CC-ALF is enabled in some CTUs, even luma ALF is disabled for all CTUs, which conflicts with the original intention. In this contribution, it is proposed to disable chroma ALF/CC-ALF implicitly when luma ALF is disabled for a CTU to keep the design consistent for all video processing units. Simulation results reportedly show that BD-rate changes are {0.00%, 0.02%, 0.04%}, {-0.01%, 0.18%, 0.14%}, and {-0.09%, 0.56%, 0.04%} with AI, RA, and LDB configurations under CTC, respectively.

It is commented that the reason of coupling the enabling at high level is rather an encoder choice, and it is not necessary to transfer that to the low level. The consistency argument is not necessarily applicable here.

No justification for a low level change. There is nothing conceptually broken. No action.

JVET-R0445 Crosscheck of JVET-R0313 (AHG2/AHG16: Cleanups of chroma ALF and CC-ALF on/off control) [N. Hu (Qualcomm)] [late]

#### Luma mapping with chroma scaling (3)

Initially discussed in AHG session 2.6 Tuesday 14 April 0825-0845

JVET-R0290 AHG16: LMCS constraint cleanup [F. Bossen (Sharp)]

It is asserted that the expression of constraints on LMCS parameters is needlessly convoluted. An alternative definition of constraints is proposed. While the proposed constraints are not strictly equivalent to the ones in VVC draft 8, no impact on coding efficiency is observed under common test conditions. It is asserted that the proposed constraint is much more straightforward.

The proposal would simplify the expression of the encoder restriction, but give up some flexibility of LMCS. No need to change a decoder implementation, though perhaps a decoder could be simplified by knowing the range is more restricted. There are however divergent opinions on this.

No urgent need of doing this change, nothing is broken, and it gives up some flexibility.

No action on this proposal

It is noted that conformance bitstreams should be made available which exercise the entire range of the current spec.

JVET-R0330 AHG16: On clipping average luma value for chroma residual scaling factor derivation [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

In VVC draft 8, the average of neighboring reconstructed luma samples above and left to one 64×64 region is used to calculate the chroma residual scaling factor for the coding units (CUs) inside the region. In the chroma sample reconstruction process 8.7.5.3 in VVC draft 8, one clipping operation is applied to clip the luma average to the full range of the internal bit-depth when deriving the chroma residual scaling factor. Additionally, the same clipping operation is also applied when generating chroma samples even if the chroma residual samples are zeros, i.e., chroma CBF is zero. It is asserted that those two clipping operations are redundant. For a cleaner design, this contribution proposes to remove those unnecessary clipping operations from the current VVC specification. Simulation results reportedly show that the proposed modification provides bit-exact BD-rate performance.

Agreed that the redundant clipping is not needed.

Recommendation (ed + SW cleanup): Remove the clipping from text, up to editor. Cleanup of software should be done for alignment with the text.

JVET-R0402 Crosscheck of JVET-R0330 AHG16: On clipping average luma value for chroma residual scaling factor derivation [J. Chen (Alibaba)] [late]

JVET-R0393 AHG9: On LMCS for GDR [L. Wang, S. Hong, K. Panusopone, M. M. Hannuksela (Nokia)]

TBP

### Transforms and transform signalling (16)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0345 Unified primary transform kernel for ISP mode [J.-Y. Huo, W.-H. Qiao, H.-X. Wang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-R0457 Crosscheck of JVET-R0345 (Unified primary transform kernel for ISP mode) [X. Zhao (Tencent)] [late]

JVET-R0056 LFNST complexity reduction [T.-D. Chuang, M.-S. Chiang, Z.-Y. Lin, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0426 Crosscheck of JVET-R0056 (LFNST complexity reduction) [T.-C. Ma (Kwai Inc.)] [late]

JVET-R0057 LFNST redundant syntax removal [T.-D. Chuang, M.-S. Chiang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0427 Crosscheck of JVET-R0057 (LFNST redundant syntax removal) [T.-C. Ma (Kwai Inc.)] [late]

JVET-R0167 Issue on LFNST index signaling condition [K. Abe, T. Toma (Panasonic)]

JVET-R0424 Crosscheck of JVET-R0167 (Issue on LFNST index signalling condition) [T. Tsukuba (Sony)] [late]

JVET-R0174 LFNST index signaling [C. Rosewarne, J. Gan (Canon)]

JVET-R0176 On chroma LFNST [C. Rosewarne, J. Gan (Canon)]

JVET-R0400 Crosscheck of JVET-R0176 (On chroma LFNST) [S. De-Luxán-Hernández (HHI)] [late]

JVET-R0234 Removal of redundant LFNST index signalling [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-R0360 Crosscheck of JVET-R0234 (Removal of redundant LFNST index signalling) [Z.-Y. Lin (MediaTek)] [late]

JVET-R0235 Removal of LFNST for chroma components [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-R0361 Crosscheck of JVET-R0235 (Removal of LFNST for chroma components) [Z.-Y. Lin (MediaTek)] [late]

JVET-R0236 Latency reduction in transformation process with TU-level signalling [H. E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-R0362 Crosscheck of JVET-R0236 (Latency reduction in transformation process with TU-level signalling) [Z.-Y. Lin (MediaTek)] [late]

JVET-R0303 Modified LFNST signalling for single tree blocks [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

JVET-R0416 Crosscheck of JVET-R0303 (Modified LFNST signalling for single tree blocks) [C. Rosewarne, J. Gan (Canon)] [late]

JVET-R0304 Restriction on LFNST signalling for local dual tree chroma coding blocks [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

JVET-R0318 Alternative methods of LFNST index signaling [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0430 Crosscheck of JVET-R0318 (Alternative methods of LFNST index signalling) [C. Rosewarne, J. Gan (Canon)] [late]

JVET-R0319 The interaction between LFNST and BDPCM [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0421 Crosscheck of JVET-R0319 (The interaction between LFNST and BDPCM) [J. Jung (WILUS)] [late]

JVET-R0331 Cleanup of LFNST signalling in single tree [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)]

JVET-R0352 On LFNST in shared tree [J. Jung, D. Kim, G. Ko, J.-H. Son, J. S. Kwak (WILUS)]

JVET-R0436 Crosscheck of JVET-R0352 (On LFNST in shared tree) [M. Koo (LGE)] [late]

JVET-R0358 A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST [H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm), T.-D. Chuang, M.-S. Chiang, Z.-Y. Lin, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, T.-C. Ma, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai), C. Rosewarne, J. Gan (Canon)]

JVET-R0401 Crosscheck of JVET-R0358 (A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST) [S. De-Luxán-Hernández (HHI)] [late]

### Partitioning (5)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0131 AHG2: On Chroma QT split in 4:2:2 format coding [H. Huang, W.-J Chien, V. Seregin, M. Karczewicz (Qualcomm), T.-D. Chuang, C.-M. Tsai, S.-T. Hsiang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0394 Crosscheck of JVET-R0131 (AHG2: On Chroma QT split in 4:2:2 format coding) [R.-L. Liao (Alibaba)] [late]

JVET-R0268 Implicit binary split at picture boundary [G. Li, X. Li, S. Liu (Tencent)]

JVET-R0269 Fix on minimum QT size value range [G. Li, X. Li, S. Liu (Tencent)]

JVET-R0316 AhG16: Normative constraints on BT and TT split under MER [Y. Wang, K. Zhang, L. Zhang, H. Liu, Z. Deng (Bytedance)]

JVET-R0431 Crosscheck of JVET-R0316 (AhG16: Normative constraints on BT and TT split under MER) [H. Huang (Qualcomm)] [late]

JVET-R0347 AHG2: On minimum QT size, maximum BT size and maximum TT size [H. Huang, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)] [AHG late]

### ACT related (6)

JVET-R0305 CU level transform size restriction for adaptive color transform [L.-F. Chen, X. Li, S. Liu (Tencent)]

In order to avoid large temporary buffer, it was decided to set max transform size to 32-point at SPS level when adaptive color transform (ACT) is enabled. However, disallowing 64-point transform leads to 1+% performance drop for RGB camera content. To compensate the coding loss, a simple fix is proposed in this contribution that max transform size is constrained to 32-point for CUs in ACT while for other CUs 64-point transform may still be used. The proposed method is implemented on top of VTM-8.0. Compared with VTM-8.0, the coding loss for RGB camera content is recovered. By using our method, the performance impact for RGB camera content is shown as following.

- AI: -1.5%/2.6%/0.1%

- RA: -1.5%/2.7%/-0.4%

- LB: -1.6%/3.4%/0.8%

Initially presented in AHG session 2.5 0630-0700 (chaired by JRO)

It is pointed out that a number of restrictions were introduced in the last meeting with regard to combining ACT with other tools.

It is also pointed out that the VTM until recently had various bugs in 444 and 422 modes, in particular regarding the QP setting for chroma. This may make some of the results less interpretable. For example, luma and chroma are showing different tendency here. See contribution R0321.

420 coding would not be affected since ACT is not used there.

The restriction is necessary due to buffering constraints.

Q0378 is also related to this topic.

Revisit – wait for new results with bug-fixed software

JVET-R0329 Mismatch on clipping input residuals to IACT [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai), J. Zhao, S.-H. Kim (LGE)]

TBP

JVET-R0336 Adaptive colour transform clean-ups [J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)]

TBP

JVET-R0355 On clipping input residuals to IACT [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai), J. Zhao, S.-H. Kim (LGE), J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)]

TBP

JVET-R0420 Crosscheck of JVET-R0355 (On clipping input residuals to IACT) [T. Tsukuba (Sony)] [late]

JVET-R0378 TU split for ACT [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

TBP

JVET-R0380 Scaling list for adaptive colour transform [S. Iwamura, S. Nemoto, A. Ichigaya (NHK), K. Naser, P. de Lagrange, F. Le Leannec, P. Bordes (InterDigital)]

TBP

### Other (1)

JVET-R0348 AHG16: On clipping operation for residual samples [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

TBP

## AHG6: 360° video coding tools, software and test conditions (1)

JVET-R0151 AHG6/AHG12: Uncoded subpictures and potential applications [J. Sauer (RWTH Aachen Univ.)]

Moved to cat. 1 – are the low-level aspects?

## AHG11: Screen content coding (10)

Initially presented in AHG sessions 2.3 Thu 9 April 1300-1500 and 2.5 Tue 14 April 0700-0900 (chaired by JRO)

JVET-R0145 AHG 11/15: On the use of limited EGk signalling [J. Gan, C. Rosewarne (Canon)]

Only four low level syntax elements remain (cu_qp_delta_abs, palette_predictor_run, num_signalled_palette_entries, and palette_escape_val) that use the general EGk binarisation. EGk is the only non-truncated binarisation used for low level signalling. This contribution proposes to replace the use of EGk with limited EGk for the four low level syntax elements. This change in binarisation has previously occurred for residual coding and mvd coding. It is asserted that the proposed changed binarisations remove unbounded codes from low-level syntax.

Presented Thu 9 1317 UTC (chaired by JRO)

There is no problem solved by this proposal, EGk has been used practically for a long time

It is pointed out that the EGk has advantages for syntax elements that change the valid range, such as delta QP

No benefit in terms of simplification

Other experts expressed concerns about adoption

No action.

It was later remarked by the proponents that an asserted benefit would be simplification of conformance testing.

JVET-R0397 Crosscheck of JVET-R0145 (AHG 11/15: On the use of limited EGk signalling) [H.-J. Jhu (Kwai Inc.)] [late]

JVET-R0146 AHG11: Context coded bin limits for palette coding [J. Gan, C. Rosewarne (Canon)]

To limit the required throughput of the CABAC engine, the number of context coded bins is limited to 1.75 bins per sample. This limit is explicitly calculated and enforced in the residual coding process. In palette coding, the number of context coded bins is not explicitly limited. With the adoption of JVET-Q0504, an “evil” bitstream can be constructed that would reach 1.984 context coded bins/sample. This contribution proposes a number of options for solving this bug. The preferred option explicitly calculates the context coded bins limit for palette mode CUs.

Presented Thu 9 1333 UTC (chaired by JRO)

More investigation is necessary to understand if the worst case is really exceeded, as

- The comparison is only made against the max number of transform coefficients per color component. As palette either codes all color comp jointly, this might only exceed the max number for monochrome case (which only would apply if there was a dedicated monochrome profile)

- In residual coding, additional context coded bins occur for motion, mode etc.

Revisit – more detailed analysis needed to understand if the problem exists, when the worst case for all context coded bins in residual coding is considered.

JVET-R0229 AHG11: Fixed number of reuse flags for palette mode [R.-L. Liao, Y. Ye, M. G. Sarwer (Alibaba)]

In the VVC draft 8, number of reuse flags to be decoded is related to the size of the palette predictor. In the case that two neighboring blocks are both coded in palette mode, the syntax of the second block cannot be parsed until the new palette predictor size is obtained. In this contribution, the number of reuse flags is set to a fixed value, that is, 63 and 31 for single tree slice and dual tree slice, respectively. Therefore, the syntax of second block can be parsed with no dependency on how large the previous block’s palette predictor is. Fixed number of reuse flags can be achieved by two methods: 1) initializing the palette predictor size to the fixed value; 2) add a requirement of bitstream conformance to palette_predictor_run signaling. It is reported that, as compared to VTM-8.0 with palette on, the overall coding performance impact for {Y, U, V} in 4:4:4 color format is:

- #1:{0.01%,0.01%,0.01%}for AI,{0.03%,0.05%,0.01%}for RA,{x.xx%,x.xx%,x.xx%}for LB

- #2:{0.01%,0.00%,0.02%}for AI,{0.03%,0.04%,-0.01%}for RA,{x.xx%,x.xx%,x.xx%}for LB

For class F in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{-0.01%,-0.07%,0.03%}for AI,{0.01%, 0.08%,0.07%}for RA,{x.xx%,x.xx%,x.xx%}for LB

- #2:{-0.02%,-0.04%,0.05%}for AI,{-0.02%,0.10%,0.08%}for RA,{x.xx%,x.xx%,x.xx%}for LB

For class TGM in 4:2:0 color format, the overall coding performance impact for {Y, U, V} is:

- #1:{0.00%,0.01%,0.00%}for AI,{0.05%,-0.06%,-0.04%}for RA,{0.01%,-0.09%,0.11%}for LB

#2:{0.01%,0.00%,-0.01%}for AI,{0.05%,-0.04%,-0.04%}for RA,{0.07%,-0.06%,0.07%}for LB

Presented Thu 9 1350 UTC (chaired by JRO)

It is not obvious that there is a severe issue. It is mentioned that method 1 could even be more complicated.

It is pointed out that JVET-R0310 is same as method 1. However, in that other contribution it is claimed that a parsing dependency exists. Several experts pointed out that there is no parsing dependency as it is typically defined.

Also the bit stream restriction does not seem to be necessary.

No action.

JVET-R0441 Crosscheck of JVET-R0229 (AHG11: Fixed number of reuse flags for palette mode) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0240 AHG11: On maximum palette size and palette predictor size [Y.-H. Chao, T. Hsieh, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

In this contribution, it is proposed to reduce maximum palette and palette predictor size. Three methods are proposed in this contribution:

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice for YUV420 format, while the size for other color formats stays the same as in VVC draft 8.

- Allow the signaling of maximum palette size and maximum palette predictor size in SPS, same syntax as in HEVC SCC extension. The allowed maximum palette and palette predictor size are restricted to 32 and 64.

The results of method 1 on YUV4:4:4 screen content sequences versus non-420 CTC anchor (PLT=1) are shown as:

1.48% AI, 1.05% RA and x.xx% LDB

The results of method 1 and 2 on YUV4:2:0 class F and TGM versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.71% AI, 0.59% RA, and 0.19% LDB

The results of method 3 with maximum palette and palette predictor size signaled to be 16 and 32 for YUV4:2:0 format versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.72% AI, 0.59% RA, and 0.19% LDB

Presented Thu 9 1407 UTC (chaired by JRO)

Presentation deck not uploaded.

Method 3 does not reduce decoder worst case complexity (and was also proposed previously, but not adopted). Also method 2 is not reducing worst case complexity, as 4:2:0 is not the worst case. Method 1 is reducing the complexity in terms of CABAC throughput, and the memory storage, but also has loss of up to 1.5% (AI)

Memory is not a significant issue here, according other experts’ opinion

It is not obvious that CABAC throughput of palette is the worst case.

No evidence that further reduction of the palette size is needed.

JVET-R0381 Crosscheck of JVET-R0240 (AHG11: On maximum palette size and palette predictor size) [H.-J. Jhu (Kwai Inc.)] [late]

JVET-R0309 [AHG16] Clean-up on palette predictor update for local dual tree [H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)]

This contribution proposes to clean-up regarding predictor palette update for local dual tree structure. Three alternative clean-up methods are suggested as described below:

- Clean-up 1. There is mismatch between SPEC and SW. Here, propose to fix SPEC to align with SW regarding local dual tree predictor palette update process.

- Clean-up 2. Update predictor palette entry only for luma component but not for chroma component in local dual tree structure.

- Clean-up 3. Remove palette predictor update process of local dual tree structure.

The suggested clean-up method 2 shows {0.00%, 0.00%, -0.06%} and method 3 shows{0.00%, 0.07%, 0.10%} coding performance respectively for average of ClassF/SCC under CTC with palette on. It is asserted that proposed clean-up methods save H/W power consumption with closed to zero coding loss and also cut worst case decoding latency by disabling predictor palette update for local dual tree.

Presented Thu 9 1427 UTC (chaired by JRO)

For “clean-up 1”, spec should be aligned with software. This is also included in JVET-R0333. See further notes there.

Items #2 and #3 try to simplify the interaction of palette with local dual tree structures. It is agreed that there is no problem in the spec such as unspecified decoder behaviour. Also in local dual tree the total predictor table size is identical with single tree. No need for action on these items.

JVET-R0310 [AHG16] Clean-up by removing parsing dependency for palette [H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)]

See notes under JVET-R0229.

JVET-R0320 AHG11: Maximum QP for escape value in palette coding [J. Xu, L. Zhang, W. Zhu, K. Zhang (Bytedance)]

Currently, EG(5) is used to code escape values in palette mode. It is observed that when QP is larger than or equal to 23, the bit length of escape values reaches its minimal, i.e. 6. Thus, it is proposed to limit the maximum QP to be 23 to reduce quantization distortion.

Presented in Session 2.5 Tue 14 April 0500 (chaired by JRO)

This does not appear to be a cleanup (another min check necessary). Small compression gain is observed.

Even small loss in LDB configuration (mainly coming from one sequence).

The relative small gain may be due to the fact that palette is less used in higher QP, and that also escape may be less selected.

No action.

JVET-R0396 Crosscheck JVET-R0320 (AHG11: Maximum QP for escape value in palette coding) [R.-L. Liao (Alibaba)] [late]

JVET-R0333 AHG11: Mismatches related to palette prediction [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

This contribution reports some mismatches between VVC draft 8 and test model software VTM-8.0 on updating the palette prediction in local dual tree cases. First, in the VVC draft 8, under local-dual tree case the update process of palette prediction is performed only for chroma CU while in the VTM-8.0 it is performed for both luma and chroma CUs. Secondly, in the VVC draft 8, the palette prediction for a luma CU under a local-dual tree does not use the chroma values of those entries in the palette predictor, and vice versa. But in the VTM-8.0, luma and chroma values of each entry in the palette predictor are used together in palette prediction.

In this contribution, one method is proposed to solve the first mismatch and two methods are proposed to solve the second mismatch in different ways. In the first change, it is proposed to change the specification text to align with the VTM-8.0 software for palette update in the first mismatch. The proposed change has no impact on coding performance. In the second change, it is proposed to change the specification text to align with the VTM-8.0 software for forming a palette in the second mismatch. The proposed change has no impact on coding performance. In the third change, in forming a palette, the same default values used for signaled palette entries are also used for predicted palette entries to fix the second mismatch. The proposed change reports negligible BD-rate changes compared to VTM8.0.

Presented Thu 9 1450 UTC (chaired by JRO)

Presentation deck not included.

It is obvious that the text spec is incomplete. The third change which modifies both text and software does not seem to be justified, as it is not simpler and has some small coding loss.

Recommendation (text BF): first and second change (as called in v2 of word or v3 zip) should be adopted.

Session 2.1 ended Thu April 9 1505 UTC

JVET-R0438 Crosscheck of JVET-R0333 (AHG11: Mismatches related to palette prediction) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0334 AHG11: Simplification of palette mode for local dual tree cases [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

TBP

JVET-R0439 Crosscheck of JVET-R0334 (AHG11: Disabling chroma CU palette mode under local dual tree) [Y.-H. Chao (Qualcomm)] [late]

JVET-R0412 AHG11: On maximum palette size and palette predictor size under local dual tree [Y.-H. Chao, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

TBP

## AHG14: Lossless and near-lossless coding (23)

Reviewed in AHG session 2.4 Thursday 9 April 2320 - Friday 10 April 0115 (chaired by BB), continued Tuesday 14 April in session 2.6 0850-0920 and session 2.7 1300-1515 (chaired by JRO and BB).

### Transform skip-related (22)

JVET-R0045 AHG15: cleanup for signaling of minimum QP of transform skip [J. Li, K. Abe (Panasonic)]

This contribution proposes to replace syntax “min_qp_prime_ts_minus4” (in range of [0-48]) with “internal_minus_input_bit_depth” (in range of[0-8]) to remove redundancy of signalling and make the design more compact.

It was commented that the name of the new syntax element could be changed and the semantics could be improved.

There was a question whether an encoder would choose one of the values of min_qp_prime_ts_minus4 not suited for lossless coding. Participants were generally in favor of this cleanup.

Recommendation (cleanup): adopt (with editorial improvements of the specification draft text).

JVET-R0049 AHG9: HLS on disabling TSRC [S.-T. Hsiang, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VVC Draft 8, extra bit costs are consumed for signalling the new syntax element slice_ts_residual_coding_disabled_flag for each of the coded slices. This contribution proposes three methods for high-level syntax modifications related to disabling transform skip residual coding, as follows:

- In Method 1, a new syntax element sps_ts_residual_coding_disabled_slice_present_flag is added to the sequence parameter set (SPS) to specify whether slice_ts_residual_coding_disabled_flag is present in the slice header.

- In Method 2, two new syntax elements sps_ts_residual_coding_disabled_slice_present_flag and sps_ts_residual_coding_disabled_slice_default_flag are added to the SPS. When slice_ts_residual_coding_disabled_flag is not present, it is inferred to be equal to the value of sps_ts_residual_coding_disabled_slice_default_flag.

- In Method 3, signalling slice_ts_residual_coding_disabled_flag in the slice header is conditioned on sps_transform_skip_enabled_flag equal to 1 without adding any new syntax element.

Method 1 same is in JVET-R0097.

Method 3 same as in JVET-R0097, JVET-R0068 (item 8) plus inference to 1 for slice_ts_residual_coding_disabled_flag, JVET-R0142, JVET-R0317 without the PPS flag, JVET-R0153 aspect 1, JVET-R0182 with inverse semantics.

The motivation behind method 1 is to not signal slice_ts_residual_coding_disabled_flag for lossy coding cases, which are considered to be the main application.

It was commented that at the last meeting, slice_ts_residual_coding_disabled_flag was adopted as a slice level flag instead of PPS or SPS flag. This would not prevent syntax as proposed here to gate its presence.

A participant questions whether the additional control syntax to save the signalling of slice_ts_residual_coding_disabled_flag is really needed. Functionality is not affected by any of the proposed methods.

It was agreed that it is desirable to not always send slice_ts_residual_coding_disabled_flag since it is expected to only be used for lossless coding scenarios.

It was further agreed that it makes sense to condition slice_ts_residual_coding_disabled_flag on sps_transform_skip_enabled_flag (Method 3).

Method 3:

SPS: sps_transform_skip_enabled_flag

SH: if(sps_transform_skip_enabled_flag )

slice_ts_residual_coding_disabled_flag

Method 1:

SPS: sps_transform_skip_enabled_flag

SPS: if(sps_transform_skip_enabled_flag )

sps_ts_residual_coding_disabled_slice_present_flag

SH: if(sps_ts_residual_coding_disabled_slice_present_flag)

slice_ts_residual_coding_disabled_flag

Recommendation (cleanup): method 3 (revisit to consider method 1).

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 8 of this contribution belongs to this category.

See notes under JVET-R0049.

JVET-R0083 AHG14: Residual coding constraints for transform skip blocks [A. Nalci, H.E. Egilmez, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

In 17th JVET meeting, the slice level flag “slice_ts_residual_coding_disabled_flag” was adopted into VVC as part of JVET-Q0089. This flag can bypass transform skip residual coding (TSRC) and enables the use of regular residual coding (RRC) for TS blocks. In VVC Draft 8, when slice_ts_residual_coding_disabled_flag=1 both dependent quantization (DQ) and sign data hiding (SDH) can be used for TS blocks. Additionally, as reported in [Spec Ticket #859], though VTM-8.0 software disables the dequantization part of DQ for TS blocks it keeps the DQ related state-based context derivation.

In variant (#1a), it is proposed to disable both DQ and SDH for TS blocks. In addition to this variant, (variant #1b) further encoder fixes for RDOQ are provided for both BDPCM and non-BDPCM TS blocks when slice_ts_residual_coding_disabled_flag=1 as discussed in [VVC Ticket #981].

- For lossless coding on YUV420 sequences, the simulation results show overall bit-rate savings of -0.16% AI, -0.17% RA, and -0.28% LDB with Class F: -0.47% AI, -0.56% RA, and -0.65% LDB and Class TGM: -0.47% AI, -0.57% RA, and -0.60% LDB.

- For lossless coding on YUV444 and RGB sequences, the simulation results show overall bit-rate savings about -0.32% AI, -0.43% RA, and -0.46% LDB.

- For lossy coding without encoder fixes, the simulation results show overall BD-rate savings (luma) of -0.03% AI, -0.03% RA, and -0.03% LDB with Class F: -0.80% AI, -0.70% RA, and -1.02% LDB and Class TGM: -0.86% AI, -0.86% RA, and -1.32% LDB.

- For lossy coding after encoder fixes, the simulation results show overall BD-rate savings (luma) of -0.05% AI, -0.05% RA, and -0.10% LDB with Class F: -0.99% AI, -0.83% RA, and -1.21% LDB and Class TGM: -1.23% AI, -1.08% RA, and -1.80% LDB.

In variant #2, only DQ is disabled for TS blocks and in variant #3 only SDH is disabled for TS blocks. The results for variant #2 is the same as variant #1, no results were provided for variant #3 since SDH is disabled under CTC and encoder crash occurs when SDH is enabled in current VVC software as reported in Ticket #981.

In variant #4, it is proposed to align the current spec text to the VVC software, in which dequantization part of DQ is disabled for TS blocks as in current VVC software however DQ related state transitions and contexts are kept.

Discussed in session 2.6 Tuesday April 14 0850-0920 (chaired by JRO and BB)

Question: Is anything wrong with the current spec? The ticket #981 refers to software. There is a spec related ticket #859.

To achieve lossless coding, an encoder has to disable both DQ and SDH (and other things) at high level. This also applies for mixed lossy/lossless coding, which might then be performing worse than in case of local disabling these tools. Currently, disabling is possible at picture level.

Currently, the SW modifies DQ reconstruction for TS with RRC blocks locally, whereas the spec does not have such an element. Otherwise, it would not be possible to get lossless reconstruction. It is noted that when introducing the switch between RRC and TSRC, the modification of DQ reconstruction was never mentioned. The context derivation of DQ is retained.

In the results for test 1 above, the anchor is not the CTC config for lossless, but a version that disables the context derivation of DQ as well as the reconstruction part. Results indicate that the SW mismatch (using context derivation of DQ) is not providing benefit.

There is no problem with TSRC, it is able to achieve lossless coding and there is no mismatch between spec and software.

An encoder could, with current spec, take the following options for mixed lossy/lossless

- Disable DQ, and enable RRC with TS, which would penalize the lossy coded parts

- Enable DQ, and disable RRC with TS, which would penalize the lossless coded parts (mainly for natural content, as per previous findings)

For lossless-only coding, there is no problem at all. For natural content, an encoder would just take the first choice if it is natural content.

No results are available for mixed lossy/lossless (as we don’t have CTC for this). Also, the current VTM encoder would need to modified, and realistic conditions (in terms of applications) are missing.

Aligning the text with software would introduce a block-level change that is not in the spec currently.

Another option would be a high-level restriction disallowing usage of DQ if TS/RRC is enabled (or the other way round). Also BDPCM should be considered in this context, as it can be enabled when TS is enabled.

It is mentioned that such a high-level restriction would be the cleanest approach with least danger of introducing even additional problems. See also further notes under R0119.

JVET-R0084 AHG14: On signaling for lossless coding [M. Karczewicz, M. Coban, A. Nalci, H.E. Egilmez, V. Seregin (Qualcomm), T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

No need to present according to proponents

JVET-R0417 Crosscheck of JVET-R0084 (AHG14: On signalling for lossless coding) [T. Tsukuba (Sony)] [late]

JVET-R0097 AHG9: Transform and transform-skip related HLS clean-up [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

See notes under JVET-R0049 for aspect 2.

JVET-R0116 AHG11/AHG14: On sign data hiding of transform skip block [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

It is asserted that, in VTM-8.0, if slice_ts_residual_coding_disabled_flag == 1, the encoder RDOQ process does not select proper residual coding method for transform skip (TS) and BDPCM blocks. In this contribution, the encoder RDOQ process is modified so that it can select correct residual coding method during RDOQ process.

It is further reported that, in VTM-8.0 software, an encoder-decoder mismatch is also observed if both slice_ts_residual_coding_disabled_flag == 1 and pic_sign_data_hiding_enabled_flag == 1. This contribution also proposes two methods to resolve the encoder-decoder mismatch issue. In the first method, sign data hiding (SDH) is disabled for BDPCM blocks only, whereas in the second method, SDH is disabled for both TS and BDPCM blocks. The encoder RDOQ bug fix (mentioned above) are included in both of the methods.

During the discussion, it is questioned whether the encoder/decoder mismatch is not just an issue of VTM implementation. The spec is asserted to clearly define how SDH works from a decoder perspective, regardless if it is regular transform, TS or TS/BDPCM. The encoder may have a problem of determining the hidden sign value, though. A normative change may not be needed to resolve the encoder/decoder mismatch.

The normative changes that are suggested: disable SDH at block level when BDPCM is on (method 2.1), or when TS is on (regardless if with or without BDPCM (method 2.2)

The problem only occurs with TS/RRC

The proposal also includes a cleanup, removing the condition on DQ/SDH at block level, as both are mutually exclusive at high level.

Relative to this cleanup, another condition would be introduced at block level.

R0141 also proposes method 2.2, which is also equivalent to R0083 variant 3. Document R0154 also proposes both methods 2.1 and 2.2, and also a high level solution additionally. R0144 method 2 is identical to 2.2. R0325 aspect 2 also proposes method 2.2. Add corresponding notes for those docs

It is the general opinion that the combination of TS/BDPCM with SDH/DQ and RRC is not beneficial and difficult to handle by an encoder. A high level disabling of this combination would also resolve the problem.

A solution resolving the problems with SDH/DQ should be consistent.

Options:

- Fix the software bugs, and keep the text unchanged

- High level disabling of the combination TS/RRC with either DQ or SDH (which are mutually exclusive)

- Block-level inhibiting that said combinations.

It is asserted that the second option (high level disabling the combination) is the safest solution for the problems raised, with least danger of introducing additional problems, making the software bug fixes most simple (except the fix of DQ context derivation).

Revisit

JVET-R0418 Crosscheck of JVET-R0116 (AHG11/AHG14: On sign data hiding of transform skip block) [T. Tsukuba (Sony)] [late]

JVET-R0139 AHG9: High-level control flag for lossless coding [T. Zhou, E. Sasaki, T. Ikai (Sharp)]

TBP

JVET-R0141 Disabling Dependent Quantization and Sign Data Hiding in Transform Skip blocks [T. Hashimoto, E. Sasaki, T. Aono, T. Ikai (Sharp)]

TBP

JVET-R0398 Crosscheck of JVET-R0141 (Disabling Dependent Quantization and Sign Data Hiding in Transform Skip blocks) [K. Abe (Panasonic)] [late]

JVET-R0142 AHG9: Slice header signalling clean up [T. Hashimoto, T. Aono, T. Ikai (Sharp)]

See notes under JVET-R0049.

JVET-R0144 AHG14: On lossless operation with RRC [J. Gan, C. Rosewarne (Canon)]

TBP

JVET-R0450 Crosscheck of JVET-R0144 (AHG14: On lossless operation with RRC) [A. Nalci (Qualcomm)] [late]

JVET-R0153 AHG9/AHG16: On slice_ts_residual_coding_disabled_flag [J. Choi, S. Yoo, J. Heo, J. Choi, J. Lim, S. Kim (LGE)]

See notes under JVET-R0049 for aspect 1.

JVET-R0154 AHG9/16: On sign data hiding for BDPCM blocks [S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

TBP

JVET-R0155 AHG9/14: On lossless coding granularity [S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

TBP

JVET-R0182 AHG9: Removed Redundant Slice Level TSRC Flag [K. Naser, F. Le Leannec, T. Poirier, M. Kerdranvat (InterDigital)]

See notes under JVET-R0049.

JVET-R0219 Alternative block size conditions for BDPCM [K. Unno, K. Kawamura, S. Naito (KDDI)]

TBP

JVET-R0419 Crosscheck of JVET-R0219 (Alternative block size conditions for BDPCM) [T. Tsukuba (Sony)] [late]

JVET-R0258 AHG9: Reduce redundant signalling in picture header [J. Enhorn, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang (Ericsson)]

The ph_dep_quant_enabled_flag aspect of item 1 of this contribution belongs to this category.

TBP

JVET-R0271 AHG9: High-level constraints of dependent quantization and sign data hiding [A. Nalci, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0317 AHG9: On slice transform skip residual coding method signalling [M. Coban, V. Seregin, Y. He, A. Nalci, M. Karczewicz (Qualcomm)]

See notes under JVET-R0049 except the PPS aspect.

JVET-R0325 AHG14: Disabling dependent quantization and sign bit hiding for transform skip mode [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

TBP

JVET-R0353 AHG14: On Interaction between ACT and BDPCM [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

TBP

JVET-R0354 AHG14: BDPCM for Inter/IBC-predicted residuals [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

TBP

JVET-R0451 Crosscheck of JVET-R0354 (AHG14: BDPCM for Inter/IBC-predicted residuals) [A. Nalci (Qualcomm)] [late]

### Other (1)

JVET-R0169 AHG14: Report of CABAC skip mode results on VTM-8.0 [K. Abe, T. Toma, V. Drugeon (Panasonic)]

TBP

## AHG15: Quantization control (4)

Discussed in AHG session 2.5 Tue 14 April 0520-0630

JVET-R0055 AHG15: On referencing a non-existent scaling list [C.-Y. Lai, O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, three methods to fix the issue of referencing a non-existent scaling list are presented. It is proposed to infer a non-existent scaling list to be the pre-defined matrix with all elements equal to 16 according to scaling_list_copy_mode_flag and scaling_list_pred_id_delta.

Problem occurs in 400 where chroma lists are not signalled

Method 1: define the non-existing scaling list as default

Method 2: always signal a scaling list for chroma (i.e. revert the decision of last meeting)

Method 3: bitstream constraint

Refers to ticket #926. In the discussion following that ticket. Methods 1 and 3 had been discussed there.

JVET-R0127 AHG15: On scaling list prediction [A. K. Ramasubramonian, B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

This document proposes changes to the derivation of scaling list reference ID when the chroma scaling lists are not signalled. Scaling lists may be explicitly signalled, copied or predicted from other scaling lists – the scaling list used for copying or predicting is referred to as the reference scaling list. The current specification allows luma scaling lists to be copied or predicted from lists that correspond to chroma scaling lists even when the chroma lists are absent; it is asserted that this behaviour is undesirable. The document proposes to modify the reference scaling list ID derivation by skipping the absent chroma lists. It is asserted that the proposed method fixes the issue of referring absent lists; due to the small bit savings, the proposed method is argued to be preferable compared to explicit constraints or inferring default lists.

Also related to ticket #926. Another method is proposed that disallows prediction from chroma scaling list when it is not present. This would also require a syntax change, reducing the range of code words to only allowing reference to luma values.

JVET-R0166 AHG15: Issue on chroma scaling matrix for 4:4:4 [K. Abe, T. Toma (Panasonic)]

This contribution points out the issue that VTM-8.0 does not work with enabling scaling matrix for 4:4:4 format and proposes two solutions. Solution1 proposes to fix VTM to strictly align to current VVC specification text, it needs to introduce size64 chroma scaling matrices reusing size32 matrices. Solution2 proposes alternative method of current VVC specification text by introducing size64 chroma scaling matrix using individual matrix id. It can simplify the specification and improve the tuning capability.

The proposed solution 2 would also require signalling the size 64 chroma matrices for the 420 case.

It is not fully clear that the separate scaling matrices for chroma block size 64 are really needed.

No support for solution 2.

Recommendation (SW BF): Align the VTM with text regarding the upscaling of chroma matrices for 444 64 size (adopt method1 of R0166).

JVET-R0326 AHG15: On Chroma Quantization Matrix Signaling [H. Zhang, X. Li, G. Li, L. Li, S. Liu (Tencent)]

This contribution proposes two fixes for the issue reported in ticket #926 that prediction may be from chroma quantization matrix which is not signaled.

Method #1 Values in non-signaled chroma QM is set to 16.

Method #2 Value of scaling_list_copy_mode_flag[id] is inferred to be equal to 1 when it is not present

Also related to ticket #926.

Both methods are using default value (similar to method 1 of R0055), but defining it differently. Method 2 of R0326 is identical to method 1 of R0055

Method 1 of R0326 was already proposed in the original Q0505, but not included in the last meeting’s decision. It is pointed out by the editor that the text description is not optimum.

It is agreed that method 3 of R0055 (encoder constraint) is not desirable.

It is agreed that method 2 of R0055 (reverting the decision of last meeting) is not desirable.

Compression efficiency is not an argument for scaling matrices.

Both method 1 of R0055 (same as method 2 of R0326), and the method of R0127 would solve the problem.

R0055 method 1 would have less impact on implementation logic changes, and had already been discussed in the reflector.

R0127 inhibits unnecessary codewords which seems to be more clean from the spec perspective.

Recommendation (BF): R0055M1/R0326M2 should be adopted.

# High-level syntax (HLS) proposals (253)

## AHG9: General high-level syntax (173)

### Combinations of subpictures and other features (3)

JVET-R0043 AHG9: On subpicture interaction with other tools [J. Li, K. Abe (Panasonic)]

Discussion began here with AHG Session 1.15 on Monday 13 April at 1300 UTC (GJS & YKW).

This contribution proposes to disable tool combinations between (1) subpicture and field coding (2) subpicture and Gradual Decoder Refresh (GDR). It is asserted that the concept between subpicture and field coding are mutually exclusive and that subpicture and GDR do not work well together.

Currently, the field coding indication is essentially metadata. It is not coupled with any aspects of coding. It was commented that it seemed undesirable to couple these only to disallow the combination, which could plausibly be used – e.g., splitting the left and right halves of the picture into two subpictures.

A virtual boundary can be used for GDR, and in the current draft, a virtual boundary can only be used with subpictures if the virtual boundary is in the SPS. However, GDR can also be used without a virtual boundary (e.g., with coding as would have been done for prior standards while providing GDR header information).

The proposed restrictions did not seem necessary and would prohibit potential uses, so no action was taken on this.

JVET-R0058 AHG8/AHG9/AHG12: On the combination of RPR, subpictures, and scalability [Y.-K. Wang, L. Zhang, K. Zhang, Z. Deng (Bytedance)]

Discussed in AHG Session 1.15 on Monday 13 April at 1320 UTC (GJS).

The latest VVC draft text includes constraints that basically disallow any other combination of subpictures and scalability with inter-layer prediction (ILP) than a restricted combination of subpictures and SNR scalability.

The contributor made the following comments and assertions:

- They could not figure out a reason why whether a layer is an output layer should make a difference herein. Rather, they thought the constraint should also apply when the layer containing the i-th subpicture is not an output layer of an OLS. The contributor asserted that the entire constraint should be specified in a manner that does not consider whether a layer is an output layer of an OLS.

- The requirement for the value of subpic_treated_as_pic_flag[ i ] to be aligned across layers should be included, as otherwise the extraction of the subpicture sequence with the same subpicture index across the layers won't be possible.

- The requirement for the value of loop_filter_across_subpic_enabled_flag[ i ] to be aligned across layers should be excluded, as regardless of the value of this flag, as long as the subpic_treated_as_pic_flag[ i ] is equal to 1 the subpicture sequence is extractable. Setting of the value of loop_filter_across_subpic_enabled_flag[ i ] should be left for the encoder to decide for trading-off the quality of single extractable subpicture sequences vs the quality of sets of extractable subpicture sequences, just as why the two flags are signalled independently from each other.

- The entire constraint should only apply when sps_num_subpics_minus1 is greater than 0, to avoid the cases of one subpicture per subpicture being covered by the constraint, unintentionally.

- The temporal scope, i.e., the set of AUs, in which the constraint applies is not clearly specified.

- The requirement for the value of each of the scaling window parameters scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset to be aligned across layers should be included, to make sure RPR for inter-layer reference pictures (ILRPs) is not needed when there are multiple subpictures per picture, as the latter is not supported in the latest VVC design.

- It is unnecessary to disallow a layer to use multiple subpictures per picture while its reference layer has just one subpicture per picture. As we know, a typical subpicture-based viewport-dependent 360o video delivery scheme is shown in the figure below, wherein a higher-resolution representation of the full video consists of subpictures, while a lower-resolution representation of the full video does not use subpictures and can be coded with less frequent random access points than the higher-resolution representation. The client receives the full video in the lower-resolution and for the higher-resolution video it only receives and decode the subpictures that cover the current viewport.

Another participant commented that the example use case would be highly beneficial for viewport-dependent streaming.

A typical subpicture-based viewport-dependent 360o video coding scheme

By allowing a layer to use multiple subpictures per picture while its reference layer has just one subpicture per picture, the coding scheme shown in the figure below would be allowed, where the only difference compared to the approach shown in the above figure is that ILP is allowed.

A typical subpicture-based viewport-dependent 360o video coding scheme based on subpictures and spatial scalability with ILP

A flag called res_change_in_clvs_allowed_flag is proposed to be added.

It was commented that negative scaling window offsets would be helpful for the suggested use (see R0114 and R0217).

The contribution proposes the following changes related to the combination of RPR, subpictures, and scalability:

- Change the constraint on the combination of subpictures and scalability (in the semantics of subpic_treated_as_pic_flag[ i ]) as follows:

  - To impose cross-layer alignment restrictions on all layers in each dependency tree, independent of whether any of the layers is an output layer of an OLS.

  - To require cross-layer alignment of subpic_treated_as_pic_flag[ i ]. (This is also proposed in R0118 aspect 2 and R0186 aspect 3.)

  - To not require cross-layer alignment of loop_filter_across_subpic_enabled_flag[ i ].

  - To not impose cross-layer alignment restrictions when sps_num_subpics_minus1 is equal to 0.

  - To clearly specify the applicable scope through the target set of AUs specified as follows: For each CLVS of a current layer referring to the SPS, let the target set of AUs targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order to the AU containing the last picture of the CLVS in decoding order, inclusive.

  - To require cross-layer alignment of the scaling window parameters (for pictures having the same spatial resolution).

  - To impose cross-layer alignment restrictions only on the current layer and all the higher layers that depend on the current layer, while not on the higher layers that do not depend on the current layer or on the lower layers.

- Change the following RPR aspects related to the combination of RPR, subpictures, and scalability as follows:

  - Instead of having just one SPS flag for controlling RPR, use two SPS flags (ref_pic_resampling_enabled_flag and res_change_in_clvs_allowed_flag), one for controlling the use of RPR as a tool, and the other for controlling whether the picture resolution can change within the CLVS.

  - Consequently, also specify two general constraint flags, one for each of these two flags.

  - Require all pictures with the same resolution within a CLVS to have the same scaling window.

- Disallow the collocated picture of a current picture to be an LTRP or ILRP.

- In the decoding processes involving the clipping operations for treating subpicture boundaries in motion compensation and motion prediction as picture boundaries, the condition on when to apply the clipping operations is slightly changed such that the clipping operation is not applied when the reference picture is not split into multiple subpictures.

The comments during the AHG review were generally positive. R0114 and R0217 are related.

It was commented that HEVC allows the collocated picture to be an LTRP or ILRP and was questioned whether this aspect is necessary.

Revisit after offline study of the details.

JVET-R0184 AHG9/AHG12: On reference picture wraparound for subpictures [S. Paluri, Hendry, S. Kim (LGE)]

Discussed in AHG Session 1.15 on Monday 13 April (GJS & YKW).

In the current VVC draft, reference picture wraparound for subpictures is only possible when the subpic_treated_as_pic_flag is equal to 0. This contribution asserted that the reference picture wraparound functionality could be extended for cases when subpic_treated_as_pic_flag equal to 1 with the subpicture width is equal to the picture width without having to have additional signalling. This contribution enables this functionality semantically.

In this contribution, the following is proposed:

- Allow reference picture wraparound for subpicture with subpic_treated_as_pic_flag[ i ] equal to 1 when the width of the subpicture is the same as the width of the picture, without additional signalling.

- At slice level, derive the boundaries to be applied for decoding of blocks in the slice. If the slice belongs to an independently coded subpiture, use the subpicture boundary; otherwise, use picture boundary. It is remarked that this 2nd proposal item can be considered as editorial updates.

Examples shown in Q0403 were discussed. Two examples are shown below.

Case 1

| Wrap off | Wrap off |
| --- | --- |
| Wrap on |  |
| Wrap on |  |

The above example would have different behaviour in different subpictures. The next figure would not.

Case 2

Wrap on

Wrap on

The second case could be used for, e.g., top-bottom stereoscopic ERP coding (or segmenting a single ERP).

Both of these are currently disallowed with subpictures treated as picture boundaries.

It was commented that we should avoid potential confusion in what is allowed and how it operates.

It was suggested to have wrap-around be a whole-picture property. This would invoke wrap-around as follows, with wrapping at the picture boundaries (thick boundaries, independent of the subpicture layout), as in case 3.

Case 3

The group seemed inclined to consider only the second of these three variations (case 2). The case just above and the case below would be prohibited:

Case 4

Wrap on

Wrap off

For signalling, wrap is in the SPS and the wrap offset is signalled in the PPS.

AHG Recommendation (cleanup): Allow case 2 (only). Revisit for text.

Discussion for AHG Session 1.15 ended on Monday 13 April at 1525 UTC.

### High level tool control (58)

#### Chroma deblocking tc and β offsets signalling (13)

JVET-R0338 AHG9: A summary of proposals on chroma deblocking tc and β offsets signalling [Y.-K. Wang (Bytedance)]

Discussed in AHG Session 1.1 Monday 6 April at 1315 UTC (GJS & YKW).

This contribution intends to provide a summary of the 12 proposals on signalling of chroma deblocking tc and β offsets submitted to this JVET meeting by the 3 April 2020 submission deadline.

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

- Skip the signalling of the chroma tc and β deblocking offset syntax elements (SEs) in the PPS when the chroma format is ( 4:0:0 or (4:4:4 and the separate color plane coding mode is in use) ) and/or when the parameter values for chroma are the same as for luma?

  - Yes, (R0077, R0078, R0095, R0106, R0152, R0172, R0206, R0218, R0232), and condition the SEs on

    - A new PPS flag for controlling the presence of chroma deblocking parameters (R0077, R0078, R0081, R0106, R0206)

    - The existing pps_chroma_tool_offsets_present_flag currently for controlling the presence of the QP offsets in the PPS (R0078, R0095, R0106, R0152, R0172, R0206, R0218, R0232) AHG Recommendation (cleanup): Recommended by AHG. (The editor may also consider renaming the flag.) If the flag is zero, the chroma offsets (if needed) are inferred from the luma offsets.

    - ChromaArrayType (R0172)

  - No. (R0048, R0079, R0081, R0232)

    - Impose semantics constraints that values shall be equal to 0 when ChromaArrayType is equal to 0. (R0079, R0081, R0232)

- Skip the signalling of the chroma tc and β deblocking offset syntax elements (SEs) in the PH and the SH when the chroma format is (4:0:0 or (4:4:4 and the separate color plane coding mode is in use) ) and/or when the parameter values for chroma are the same for luma?

  - Yes, (all the 12 contributions), and condition the SEs on

    - ChromaArrayType (R0048, R0078, R0079, R0081, R0095, R0106, R0152, R0172, R0206, R0218, R0232)

    - A new PPS flag for controlling the presence of chroma deblocking parameters (R0077, R0078, R0106)

    - The existing pps_chroma_tool_offsets_present_flag currently for controlling the presence of the QP offsets in the PPS (R0078, R0152, R0232) AHG Recommendation (cleanup): Recommended by AHG. (The editor may also consider renaming the luma beta and tc offset control syntax elements.) If the flag is zero, the chroma offsets (if needed) are inferred from the luma offsets.

    - A new PH flag and a new SH flag (R0081, R0206)

  - No, but impose semantics constraints that values shall be equal to 0 when ChromaArrayType is equal to 0. (R0079, R0232)

JVET-R0048 AHG9: On chroma deblocking parameters [C.-M. Tsai, C.-W. Hsu, S.-T. Hsiang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0077 AHG9: On chroma deblocking parameters signalling [J. Xu, L. Zhang, Y.-K. Wang, K. Zhang, Z. Deng (Bytedance)]

JVET-R0078 AHG9: On signalling of deblocking parameters for coding monochrome pictures [H.-W. Sun, H.-B. Teo, C.-S. Lim (Panasonic)]

JVET-R0079 AHG9: On signalling of chroma deblocking filter parameters for monochrome [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

JVET-R0081 AHG9: Chroma deblocking strength signalling [Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, K. Andersson, J. Ström, R. Sjöberg (Ericsson)]

JVET-R0095 AHG9: Clean-up of chroma deblocking control parameter signalling [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

JVET-R0106 AHG9: On Deblocking Control [S. Deshpande, J. Samuelsson, A. Segall, T. Zhou, T. Ikai (Sharp)]

Item 2 of this contribution belongs to this category.

JVET-R0152 AHG9: On signalling of chroma deblocking offsets in monochrome picture [J. Choi, J. Choi, J. Heo, S. Yoo, J. Lim, S. Kim (LGE)]

JVET-R0172 AHG9: Removed Redundant Coding of Chroma Deblocking Filter Parameters [K. Naser, F. Le Léannec, T. Poirier (InterDigital)]

JVET-R0206 AHG9: Modified signalling of Chroma deblocking control parameters [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

JVET-R0218 AHG9: Decoding conditions of deblocking control parameters for chroma [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-R0232 AHG9: APS, LMCS, deblocking and PPS constraints [N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

#### Deblocking control signalling - other aspects (5)

Discussion began here with AHG Session 1.8 on Tuesday 7 April at 2320 UTC (GJS & YKW).

JVET-R0072 AHG9: On signalling of deblocking control [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, J. Xu (Bytedance)]

Discussed in AHG Session 1.8 on Tuesday 7 April at 2320 UTC (GJS).

It is asserted that the deblocking control mechanism in the latest VVC text is pretty complicated, not straightforward, not easy to understand, and consequently prone to errors. This contribution proposes to change the deblocking signalling based on a 2-bit deblocking mode indicator in the PPS, summarized as follows:

- Replace the three PPS flags for deblocking signalling with a 2-bit deblocking mode indicator that specifies the following four modes: a) deblocking fully disabled and not used for all slices; b) deblocking used for all slices using 0-valued β and tC offsets; c) deblocking used for all slices using β and tC offsets explicitly signalled in the PPS; and d) deblocking further controlled at either picture or slice level.

- The two flags in PH/SH are renamed to be ph/slice_deblocking_filter_used_flag and ph/slice_deblocking_parameters_override_flag, with the use flag specifying whether deblocking is used for the current picture/slice, and the override flag specifying whether the β and tC offsets are overridden by the values signalled in the PH/SH.

There was discussion of the various cases described in the document.

It was commented that deblocking is the only case where a disabling in the PPS can be overridden at the picture level. Some participants said this is OK, as it enables PPS sharing, and noted that we have such an override in HEVC. Others said this is the only place in the text that allows a disabling to be overridden.

It was agreed that there is no clear bug in the current design, although it does appear that there are a number of editorial bugs in the current semantics of the text.

No clear need for action was identified by the AHG on this, so the AHG did not recommend action.

JVET-R0183 AHG9: On deblocking control signalling [S. Paluri, Hendry, S. Kim (LGE)]

Discussed in AHG Session 1.8 (GJS & YKW).

This contribution asserted that the current signalling for deblocking filter control is complex and may be simplified. For example, the current design allows the deblocking to be disabled at the PPS level and then later to be enabled either in picture header or the slice header. It is asserted that such design may be confusing and make the signalling more difficult to understand.

This contribution proposed the following changes:

- Signal a flag in the PPS, i.e., pps_deblocking_enabled_flag to specify whether or not deblocking is enabled / applied to pictures that refer to the PPS.

- When deblocking is enabled (e.g., the value of pps_deblocking_enabled_flag is equal to 1, additional flags can be signalled as follows:

  - pps_deblocking_override_enabled_flag, which is an existing flag.

  - pps_deblocking_parameter_present_flag to specify whether the PPS deblocking parameter is present.

This is very similar in spirit to R0072, and is a smaller change.

No clear need for action was identified by the AHG on this, so the AHG did not recommend action.

JVET-R0159 AHG9: On high level syntax of deblocking filter [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Discussed in AHG Session 1.8 (GJS & YKW).

This contribution includes two syntax and semantics changes of deblocking filter (DBF) as follows.

- It is proposed to signal an SPS enabled flag for DBF as done for other loop filters. It was asked whether there are any other features that do not have an SPS-level enabling flag; CU QP delta and CU chroma QP delta enabling were noted. It was commented that the reason there is no such flag is because it was expected that bitstreams would generally have the DBF enabled (and there is no constraint flag for this). Another participant commented that we may have wanted to have control flags for things either in the SPS or PPS but not both. It was commented that in other in-loop filter cases, there is syntax in the PH that is gated by the SPS flag. No clear need for action was identified by the AHG on this, so the AHG did not recommend action.

- Fixing basically editorial bugs of the semantics of DBF control related syntax elements. AHG Recommendation (editorial BF): Adopt (with editor discretion on exact form of expression).

JVET-R0106 AHG9: On Deblocking Control [S. Deshpande, J. Samuelsson, A. Segall, T. Zhou, T. Ikai (Sharp)]

Discussed in AHG Session 1.8 (GJS & YKW).

Item 1 of this contribution belongs to this category.

Proposal 1: It is proposed to move the signalling location of syntax element dbf_info_in_ph_flag to locate it near the other deblocking control parameters signalling.

This is a small proposed change to group together and logically nest the deblocking syntax in the PPS. AHG Recommendation (cleanup): Adopt.

Discussion stopped here for AHG Session 1.8 on Wednesday 8 April 0115 UTC.

JVET-R0388 AHG9: Cleanups on deblocking signalling [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, J. Xu (Bytedance)] [late]

TBP

#### Quantization control signalling (6)

JVET-R0050 AHG9: HLS on dependent quantization and sign data hiding [S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 7 of this contribution belongs to this category.

JVET-R0073 AHG9: Some cleanups on QP delta signalling [Z. Deng, L. Zhang, Y.-K. Wang, J. Xu, K. Zhang (Bytedance)]

JVET-R0076 AHG9: Chroma QP mapping table cleanups [J. Xu, L. Zhang, Y.-K. Wang, K. Zhang, Z. Deng (Bytedance)]

JVET-R0272 AHG9: On chroma QP offsets in picture header [K. Misra, J. Samuelsson, S. Deshpande, F. Bossen, A. Segall (Sharp)]

JVET-R0302 AHG12: On signalling of chroma QP [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

#### High-level control of features that use APSs: LMCS, scaling lists, and ALF (21)

Discussion began here in AHG Session 1.16 on Monday 13 April at 1540 (GJS & YKW).

JVET-R0404 AHG9: A summary of proposals on high level control of LMCS, Scaling list, ALF and SAO [L. Zhang, Y.-K. Wang (Bytedance)] [late]

Discussed in AHG Session 1.16 (GJS).

This contribution intends to provide a summary of the 21 proposals on high level control of LMCS, scaling list, ALF and SAO submitted to this JVET meeting by the 3 April 2020 submission deadline.

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

For high-level control and semantics changes of LMCS, the following aspects are proposed:

- Controlling of presence of the SH LMCS enabled flag slice_lmcs_enabled_flag

  - Conditionally add a new SPS SE to indicate whether slice_lmcs_enabled_flag is present, and when not present, infer the value to be equal to the PH LMCS enabled flag. (JVET-R0051)

This is to save a flag at the slice header level if all slices have LMCS enabled if the PH has LMCS enabled.

It was commented that saving a bit at the SH level doesn’t seem especially important for LMCS, so no action was taken on this.

  - Replace the PH flag ph_lmcs_enabled_flag with a 2-bit ph_lmcs_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2); and only signal slice_lmcs_enabled_flag for mode 2. (JVET-R0063)

This is similar in spirit to item a, but using a bit at the PH level instead of at the SPS level to distinguish the cases.

It was commented that saving a bit at the SH level doesn’t seem especially important for LMCS, so no action was taken on this.

  - Skip the signalling of the SH LMCS enabled flag for the case when the PH is in the SH. (JVET-R0089, JVET-R0098, JVET-R0210, JVET-R0200, JVET-R0202)

AHG Recommendation (cleanup): Adopt. Text is in R0098 and software will be provided by that proponent.

  - Move the SH flag slice_lmcs_enabled_flag to be just after the ALF parameters (JVET-R0200) so that the header information for LMCS is grouped in a similar way as in the picture header. (It was commented that there may also be another parsing simplification from this.)

AHG Recommendation (cleanup): Adopt.

- Slice-level control of chroma residual scaling (currently only controlled in the PH).

  - Remove the PH control flag (ph_chroma_residual_scale_flag) and add one flag in SH. (method 1 of JVET-R0096, method 2 of JVET-R0171)

  - Add a control flag in slice level under the condition "if( slice_lmcs_enabled_flag  &&  ph_chroma_residual_scale_flag )". (proposal 2 of JVET-R0089, method 2 of JVET-R0096, method 1 of JVET-R0171)

Currently, luma and chroma processing are switched on and off together at the SH level. This would enable controlling them separately.

The proposed chroma flag would only be sent if the luma flag is turned on.

It was commented that we generally have separate control of chroma at the same degree of local level as luma. Others commented that LMCS was brought in as a single tool and the design is somewhat unified (with the balance between luma and chroma somewhat maintained by changing chroma together with luma) and they thought currently logical.

In the absence of sufficient support, no action was recommended on this.

- Add a constraint to disable chroma residual scaling of LMCS for pictures within a GDR period (JVET-R0393).

The proponent indicated that enabling the chroma scaling can cause a GDR “leak”.

It was asked whether there would be a leak if the virtual boundary is at a CTU boundary. The proponent responded that this would not cause a leak.

It was commented that another approach could be to just add a NOTE to caution the reader that enabling chroma residual scaling could cause a GDR problem if there is a virtual boundary that is not aligned with a CTU boundary. AHG Recommendation (Ed.): It is suggested to add such a NOTE. The editor is requested to consider this. (No normative effect.)

- Revised semantics (italics for report emphasis only):

- Change the semantics of sps_lmcs_enabled_flag equal to 1 to use the wording of "may be used" instead of "is used". (JVET-R0051, JVET-R0160)

sps_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling may be used in the CLVS. sps_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not used in the CLVS.

- Revise the current semantics of ph_lmcs_enabled_flag to the following: (JVET-R0051, JVET-R0160, JVET-R0210)

ph_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling may be enabled for slices associated with the PH. ph_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is disabled for all slices associated with the PH. When not present, the value of ph_lmcs_enabled_flag is inferred to be equal to 0.

- Revise the current semantics of ph_chroma_residual_scale_flag to the following:

- ph_chroma_residual_scale_flag equal to 1

    - specifies that chroma residual scaling may be enabled for slices associated with the PH. (JVET-R0051, JVET-R0160)

    - specifies that chroma residual scaling is enabled for all slices associated with the PH and whether it is applied for each slice is further controlled by the slice_lmcs_used_flag signalled in the slice header. (JVET-R0063)

- ph_chroma_residual_scale_flag equal to 0

    - ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is disabled for all slices associated with the PH. (JVET-R0051, JVET-R0063, JVET-R0160)

- Change the semantics of slice_lmcs_enabled_flag equal to 1 to use the wording of "luma mapping is enabled for the current slice and chroma scaling may be enabled for the current slice" instead of "luma mapping with chroma scaling is enabled for the current slice" (JVET-R0160):

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping is enabled for the current slice and chroma scaling may be enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0.

AHG Recommendation (Ed. BF / expression of existing intent): Agreed as detailed above (editor has discretion over exact expression).

AHG Recommendation (Ed.): It is suggested to remove “one, more, or all” phrases in the text.

Discussion ended here for AHG Session 1.16 on 13 April at 1715.

The following design questions on high level control of scaling lists were proposed:

- Controlling of presence of the SH explicit scaling list enabled flag slice_explicit_scaling_list_used_flag

- Conditionally add a new SPS SE to indicate whether slice_explicit_scaling_list_used_flag is present, and when not present, infer the value to be equal to the PH explicit scaling list enabled flag. (JVET-R0051)

- Replace the PH one-bit flag by a 2-bit ph_explicit_scaling_list_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). and only signal slice_explicit_scaling_list_used_flag for mode 2. (JVET-R0064)

- Skip the signalling of the SH explicit scaling list enabled flag for the one slice per picture case. (JVET-R0089, JVET-R0098, JVET-R0202)

- Move the SH flag slice_explicit_scaling_list_used_flag to be just after the ALF parameters (but after slice_lmcs_enabled_flag). (JVET-R0200)

For high level control and semantics changes of ALF/SAO, the following aspects are proposed:

- Control ALF and SAO at SPS, PH (on/off control, ALF APS information for ALF) and SH level (on/off control) and remove the slice level ALF parameter adaptation. (JVET-R0160)

- Indication of chroma ALF

  - Use two separate SEs (one for Cb, one for Cr) to replace ph_alf_chroma_idc in PH, and use two separate SEs to replace slice_alf_chroma_idc in SH. (JVET-R0225)

- Indication of CC-ALF

  - Use two separate syntax (alf_ctb_cc_cb_flag and alf_ctb_cc_cr_idx) to replace alf_ctb_cc_cb_idc in CTU level. (JVET-R0225)

- In PH/SH, add a constraint such that if CCALF is disabled in SPS, an ALF_APS cannot contain any CCALF filters. (JVET-R0232)

- Revised semantics

- Change the semantics of ph_alf_enabled_flag equal to 0 to use the wording of "is disabled for all slices" instead of "may be disabled for one, or more, or all slices" (JVET-R0068, JVET-R0251)

ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled for all slices associated with the PH. (JVET-R0068)

ph_alf_enabled_flag being present and equal to 0 specifies that adaptive loop filter is disabled for all colour components in all slices associated with the PH. When not present, ph_alf_enabled_flag is inferred to be equal to 0. (JVET-R0251)

- Change the semantics of sps_alf_enabled_flag equal to 1 to use the wording of "may be enabled" instead of "is enabled" (JVET-R0160):

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is enabled.

- Change the semantics of sps_alf_enabled_flag equal to 1 to use the wording of "may be enabled" instead of "is enabled" (JVET-R0160):

- Change the semantics of sps_sao_enabled_flag equal to 1 to use the wording of "may be applied" instead of "is applied" (JVET-R0160):

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process may be applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process.

For APS related aspects, the following are proposed:

- Move the flag scaling_matrix_for_lfnst_disabled_flag from the scaling_list_data( ) syntax to the SPS. (JVET-R0064)

- Updating, cross-layer sharing, and decoding order of APSs

  - Update to the content of an ALF APS NAL unit within a PU is allowed. (JVET-R0070)

  - To change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture. (JVET-R0149)

  - Sharing of an APS NAL unit across layers is disallowed. (JVET-R0070)

  - Add the following constraints: When both one or more prefix APS NAL units and one or more suffix APS NAL units are present between two consecutive VCL NAL units in decoding order, the VCL NAL units shall belong to different subpictures and all the suffix APS NAL units shall precede, in decoding order, all the prefix APS NAL units. (JVET-R0149)

  - To constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU. (JVET-R0201)

  - To allow prefix and suffix APS NAL units with particular APS identifier and type to have different content. (JVET-R0201)

  - To constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU. (JVET-R0201)

- Signalling APS information in PH/SH

  - Add additional signalling of alf_data()/scaling_list_data()/lmcs_data() in SH, and add a constraint such that all presence flags of ALF/Scaling list/LMCS in SH shall be 1 when no_aps_constraint_flag is equal to 1. (JVET-R0180)

  - Add a mode of directly inclusion of the APS data structure inside a PH NAL. (JVET-R0273)

- Constratint for APS types:

  - Add a constraint for APS type based on the enabled tools in SPS. If a tool that uses an APS is disabled, then the APS with the corresponding APS type should not be present in a bitstream. (JVET-R0232)

- Constratint for alf_data in ALF APS:

  - Add a constraint to CC-ALF based on sps_ccalf_enabled_flag. When sps_ccalf_enabled_flag is equal to 0, an ALF_APS cannot contain any CCALF filters. (JVET-R0232)

- Remove BitDepth constratint for lmcs_data in LMCS APS (JVET-R0232):

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to 14, inclusive.

PH:

ph_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

The value of lmcs_delta_cw_prec_minus1 of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be in the range of 0 to BitDepth – 2, inclusive.

JVET-R0051 AHG9: HLS on LMCS and scaling list [S.-T. Hsiang, Z.-Y. Lin, C.-Y. Lai, O. Chubach, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0063 AHG9: Signalling of LMCS control [L. Zhang, Y.-K. Wang, K. Zhang (Bytedance)]

JVET-R0064 AHG9: Signalling of scaling list control [Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 4 of this contribution belongs to this category.

JVET-R0089 AHG9: On slice level control of LMCS and explicit scaling list [J. Jung, D. Kim, G. Ko, J.-H. Son, J. S. Kwak (WILUS)]

JVET-R0096 AHG9: On signalling of chroma residual scaling [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

JVET-R0098 AHG9: On Slice Header Signalling of LMCS and Scaling Lists Information [	S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0070 AHG9: On repetition and update of non-VCL data units [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

Items 4 and 5 of this contribution belongs to this category.

This relates to R0149 and R0201 aspects 2 and 3.

JVET-R0149 AHG9/AHG12: Relaxing an APS constraint [M. M. Hannuksela, M. Homayouni, A. Hallapuro, A. Aminlou (Nokia)]

This relates to R0070 and R0201 aspects 2 and 3.

JVET-R0201 AHG9: On prefix and suffix APSs [N. Ouedraogo, G. Laroche, P. Onno (Canon)]

JVET-R0160 AHG9: High level syntax cleanup for LMCS, ALF and SAO [J. Chen, J. Luo, M. G. Sarwer, Y. Ye, R.-L. Liao (Alibaba)]

JVET-R0171 AHG9: Slice-Level Chroma Residual Scaling Flag [K. Naser, E. François, F. Hiron, C. Chevance (InterDigital)]

JVET-R0180 AHG9: On ALF, LMCS and Scaling List Parameters Signalling [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

The first aspect (on slice_lmcs_enabled_flag) of item 2 of this contribution belongs to this category.

JVET-R0202 AHG9: Syntax cleanups when Picture Header is in the Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

The slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag aspects of item 1 of this contribution belong to this category.

JVET-R0210 AHG9: Cleanup of Picture Header Syntax Structure in Slice Header [S. Esenlik, B. Wang, A. Kotra, E. Alshina (Huawei)]

The slice_lmcs_enabled_flag aspect this contribution belongs to this category.

JVET-R0225 AHG9: On ALF/CC-ALF high level syntax [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-R0232 AHG9: APS, LMCS, deblocking and PPS constraints [N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

All aspects excluding the deblocking aspect of this contribution belong this this category.

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

Item 2 of this contribution belongs to this category.

JVET-R0393 AHG9: On LMCS for GDR [L. Wang, S. Hong, K. Panusopone, M. M. Hannuksela (Nokia)] [late]

#### High level control of other tools (13)

JVET-R0049 AHG9: HLS on disabling TSRC [S.-T. Hsiang, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

To be revisited for HLS review. This and related contributions were initially discussed in the Category 2 (coding tools) AHG pre-meeting, as reported in JVET-R0340.

JVET-R0150 AHG9/AHG12: Moving joint chroma coding sign flag from picture header to slice header [M. M. Hannuksela, J. Lainema (Nokia)]

JVET-R0175 AHG9: An SPS Flag for IBC-AMVR [K. Naser, M. Kerdranvat, T. Poirier, A. Robert (InterDigital)]

JVET-R0214 AHG9: MMVD syntax modifications [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0215 AHG9: Max num of subblock merge candidate signalling [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0216 AHG9: Signalling the parallel merge level relative to the minimum coding block size [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn, J. Ström (Ericsson)]

JVET-R0237 AHG9: Constraints based on the minimum coding block size [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, Y. Wang, J. Xu, H. Liu (Bytedance)]

JVET-R0252 AHG9: On high-level signalling of mvd_l1_zero_flag [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn, D. Liu (Ericsson)]

JVET-R0137 On mvd_l1_zero_flag and NoBackwadPredFlag [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

Item 2 of this contribution belongs to this category.

JVET-R0258 AHG9: Reduce redundant signalling in picture header [J. Enhorn, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang (Ericsson)]

Excluding the ph_dep_quant_enabled_flag aspect of item 1, all other aspects of this contribution belong to this category.

JVET-R0287 AHG9: On high level control parameters [H. Huang, Y.-J. Chang, M. Coban, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-R0371 AHG2/9: On max num of subblock merge candidates [H. Huang, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

JVET-R0373 AHG9: On Maximum Number of Subblock Merge Candidates Y.-C. Yang, C.-Y. Teng (Foxconn) [late]

### General and misc. HLS topics (9)

Discussion started here for AHG Session 1.9 on Wednesday 8 April at 1300 UTC (GJS & YKW).

JVET-R0041 AHG8/AHG9: On picture types and related constraints [Y.-K. Wang (Bytedance)]

Discussed in AHG Session 1.9 (GJS).

This contribution proposes some changes related to definitions of the terms "associated IRAP picture", "associated GDR picture", and "trailing picture" and the constraints regarding different types of pictures and their relationships in terms of decoding order, output order, and prediction relationship.

The proposed changes are summarized as follows:

- The definition of associated GDR picture is added and the definition of associated IRAP picture is updated, such that each picture of a layer, except the first picture in the layer in the bitstream, is specified to be associated with the previous IRAP or GDR picture of the same layer in decoding order, whichever is closer.

- The definition of trailing picture is updated, such that a trailing picture may also be associated with a GDR picture.

- The following existing constraints are updated such that they only impose restrictions to pictures within the same layer:

  - On the output order of pictures preceding an IRAP picture in decoding order

  - On the decoding order of pictures associated with an IRAP picture and some non-leading pictures

  - On RPLs for a CRA picture

- Constraints for an STSA picture, in terms of relative decoding order, output order, and prediction relationship with the associated IRAP picture and the same-layer pictures in the preceding and succeeding AUs, are specified, similarly as a trailing picture.

- Similar constraints for IRAP pictures and the same-layer pictures in the preceding and succeeding AUs in terms of relative decoding order, output order, and prediction relationship are specified for GDR pictures.

It was suggested to explicitly distinguish a “single-layer bitstream” from a “multi-layer bitstream”. As proposed, there may not be a need for such a distinction.

Some action is needed in this area; this is basically filling in gaps and proposing bug fixes for expression of the existing technical intent; no new functionalities are introduced.

Contribution R0226 is related.

It was commented that there may need to be some further cleanup regarding mixed NAL unit types; there are other contributions on that issue.

It was commented that it may be desirable to find a way to simplify / unify the discussions of GDR and IRAP if feasible (esp. if the recovery POC count is zero for the GDR).

It was noted that GDR characteristics are less restricted than IRAP, e.g. in regard to IRAP. The proponent said they had not intended to add constraints to GDR pictures, beyond what is necessary.

It was noted that various definitions depend on NAL unit types, and this could make the definitions difficult to comprehend outside the context of the document (a characteristic preferred by ISO editing guidelines).

AHG Recommendation: Adopt. (Further offline review is encouraged, and interacting aspects with other contributions remain in need of consideration.)

JVET-R0226 AHG9: Proposal to change the definition of trailing picture [R. Sjöberg, M. Pettersson, M. Damghanian, Z. Zhang, J. Enhorn, R. Yu (Ericsson)]

Discussed in AHG Session 1.9 (GJS & YKW).

The proponents of this proposal assert that STSA pictures are trailing pictures in HEVC but not in VVC, and that the rules for trailing pictures therefore do not apply for STSA pictures. It is proposed to include STSA pictures in the group of trailing pictures so that the rules do apply also for STSA pictures.

The proponents also claim that bitstreams starting with a GDR picture may contain pictures that do not have any associated IRAP picture and that the current VVC draft disallows the use of TRAIL_NUT pictures for this case. The proponents believe that TRAIL_NUT pictures should be allowed for pictures that do not have any associated IRAP picture.

The proposal is summarized as:

- Include STSA pictures and pictures with no associated IRAP picture into the trailing picture type.

- Only allow pictures having an associated IRAP picture to be “leading pictures”.

- Condition the conformance requirements regarding relationships between trailing pictures and associated IRAP pictures to only apply to trailing pictures that do have an associated IRAP picture.

- Add the following rule: “Any picture that precedes a GDR picture in decoding order shall precede the GDR picture in output order”

It was questioned whether “trailing picture” is a good term if there are cases where this picture is not trailing anything else.

A difference with R0041 is that this associates a trailing picture with the most recent IRAP, whereas R0041 associates it with the most recent picture that is either an IRAP or GDR picture. The intent here is to have as few restrictions as possible that apply around a GDR picture. Revisit for this question.

Can a trailing picture reference a RADL picture (a RADL of an IRAP)? No (although there is a trick for one picture corresponding to a field coding case). This is the same in R0041 and R0226 (and HEVC).

Both contributions allow a picture that follows a GDR picture in output order to refer to another picture that follows the GDR picture in decoding order that precedes the GDR picture in output order (in a way that is not allowed if the GDR picture was an IRAP picture).

Both this contribution and R0041 constrain a picture that precedes a GDR picture in decoding order to also precede it in output order. It was discussed whether such a constraint should apply only after the recovery point is reached. Revisit for this question.

JVET-R0065 AHG8/AHG9: On IRAP and GDR AUs [Y.-K. Wang (Bytedance)]

Discussed in AHG Session 1.9 (GJS & YKW).

Item 1 of this contribution belongs to this category.

This contribution proposes the following changes related to IRAP and GDR AUs:

- Each GDR AU is required to be “complete” (i.e., to have a picture for each of the layers present in the CVS). That means, an incomplete AU consisting of GDR pictures is not a GDR AU, similarly as in the current VVC text that an incomplete AU consisting of IRAP pictures is not an IRAP AU. This aspect seems to be straightforward completion of the specification of GDR to account for multi-layer usage.

- Add a flag named irap_or_gdr_au_flag to the AUD to specify whether the AU is an IRAP or GDR AU, and mandate the presence of an AUD NAL unit in each IRAP or GDR AU when vps_max_layers_minus1 is greater than 0.

The following issues were reportedly observed in the existing scalability design in the latest VVC text (in JVET-Q2001-vE/v15):

- Currently, an IRAP AU may start a new CVS and is required to be complete (i.e., to have a picture for each of the layers present in the CVS), while a GDR AU may also start a new CVS but is NOT required to be complete. It is believed that this is an oversight, as otherwise some layer-wise startup decoding process would have to be specified for such GDR case.

- A CVSS AU, which starts a new CVS, is required to be complete (i.e., to have a picture for each of the layers present in the CVS). However, according to the current design, the decoder is not able to check whether an AU includes a picture "for each of the layers present in the CVS" before it receives the last picture of the CVS, while on the other hand, even the last picture of the CVS is not easy to be determined because it is not easy to determine the start of any of CVS except for the very first CVS of the bitstream. Basically, that means, the decoder can only figure out the boundaries of CVSs after receiving the entire bitstream.

This contribution tries to address the above issues.

An OLS may contain more layers than are present in the bitstream.

It was discussed to what extent there may (or should) be out-of-band signalling to inform the decoder of the start of a new CVS or the establishment of a new target OLS. It was commented that the bitstream should at least be decodable without needing to rely on some such external means.

Another contribution R0274 proposes to not require an IRAP AU to be complete.

Revisit after offline study.

JVET-R0070 AHG9: On repetition and update of non-VCL data units [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

Discussed in AHG Session 1.9 (GJS & YKW).

Items 1-3, and 6 of this contribution belong to this category.

This contributions proposes the following aspects regarding repetition and update of non-VCL data units and sharing of APSs across layers:

- VPS, SPS, and DCI NAL units are proposed to be disallowed to be present in an AU that has no nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive. It was asked why we would do this – e.g., an SPS could be sent early before it is used. No need to impose such a constraint was identified, so no action was recommended on this.

- A PU is proposed to be disallowed to have more than one VPS, SPS, or PPS NAL unit with a particular VPS, SPS, or PPS ID, and is proposed to be disallowed to have more than one DCI NAL unit. No need to impose such a constraint was identified, so no action was recommended on this.

- A slice unit (SU) is proposed to be defined as a set of NAL units that are consecutive in decoding order and contain exactly one coded slice and all its associated non-VCL NAL units.

  - Within an SU it is proposed to be disallowed to have more than one APS NAL unit with a particular APS ID and of particular APS type.

  - Within an SU it is proposed to be disallowed to have more than one SEI payload with particular type and a particular content.

- Update to the content of an ALF APS NAL unit within a PU is proposed to be allowed. This aspect should be discussed as a matter of tool control (section 4.1.2.4), as it is not just a matter of HLS, and a similar change is proposed in R0149.

- Sharing of an APS NAL unit across layers is proposed to be disallowed. This aspect is discussed in section 4.1.2.4.

- The same types of APSs are proposed to share the same value space for the APS ID, regardless of whether the APSs are prefix or suffix APS NAL units. It was commented that this has been the existing design intent. AHG Recommendation (expression of existing intent): Adopt this aspect.

Discussion stopped here for AHG Session 1.9, with item 3 yet to be reviewed.

JVET-R0082 AHG9: Byte alignment modifications [Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, J. Ström, R. Sjöberg (Ericsson)]

JVET-R0122 AHG9: On picture output for non-reference pictures [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0147 AHG9: On picture order count and output order [M. M. Hannuksela (Nokia)]

JVET-R0263 AHG9: On TemporalId and sublayer [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

### Profile, tier, level (PTL) (6)

Discussion started here for AHG Session 1.5 on Tuesday 7 April at 1300 (GJS, YKW, JRO & JB).

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Discussed in AHG Session 1.5 (GJS, JRO & JB).

Item 2 of this contribution belongs to this category.

Discussed UTC Tuesday 7 April, 2020, 13:00. Chaired by JRO and JB.

It was commented that the encoder could use TemporalId 0 for IRAP pictures. However, it was commented that will make one less sublayer to be used.

It was commented that in intra based trick play, some IRAP pictures would be output/displayed multiple times.

It was commented that trick play can also use non-intra pictures, and scene cuts may appear in arbitrary positions and can be coded as IRAP pictures.

It was commented that such information, if useful, should use an SEI message.

It was commented that the information is useful, somewhat like marking non-reference pictures etc.

It was commented that DASH already makes use of this information, as a bitstream property.

It was commented that the information may be hard for the encoder to figure it out and set it. However, it was counter argued that this is similar for setting the level for temporal scalable layers.

What do people think about having an SEI message for this?

Revisit.

JVET-R0108 AHG9: Decoding Capability Information and PTL Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

Discussed in AHG Session 1.5 Tuesday 7 April at 1410 UTC (GJS, YKW, JRO & JB).

Following is proposed related to DCI and PTL signalling:

- Proposal 1: It is proposed that dci_max_sublayers_minus1 syntax element be removed and instead those bits and the reserved zero bit be used for the syntax element dci_num_ptls_minus1.

- Proposal 2: It is proposed to rearrange the syntax elements in profile-tier-level signalling structure such that general_level_idc syntax element, which is unconditionally signalled, is first in the structure and the other conditional signalling, which is all based on profileTierPresentFlag is together, thus requiring only a single if check.

- Proposal 3: It is proposed to conditionally signal sps_ptl_dpb_hrd_params_present_flag only when sps_video_parameter_set_id is not equal to 0.

Proposal 1:

It was commented that the added semantics constraint on vps_num_ptls_minus1 has a problem.

It was suggested to use 4 reserved bits instead of having 8 bits for the number of PTL structures.

Comment: The current extension mechanism for DCI is a bit heavy.

Suggestion: Reserve a value of dci_num_ptls_minus1, e.g., 15.

AHG recommendation (cleanup): Remove the dci_max_sublayers_minus1 SE, but to use 4 reserved bits (at the begin of the DCI syntax) instead of having 8 bits for the number of PTL structures (as proposed), and reserve the value 15 of dci_num_ptls_minus1.

Proposal 2:

Comments: This makes level goes before profile and tier, while the interpretation of level typically depends on profile.

Comments: The GCI syntax structure can be of variable length. The proposed change makes level at fixed position.

No action on this item was taken.

Proposal 3: See notes in 4.1.6.1.

JVET-R0243 AHG9: 4:4:4 vs. 4:2:0 bit-rate in VTM [S. Keating, A. Browne, K. Sharman (Sony)]

Discussed in AHG Session 1.5 (GJS, YKW, JRO & JB).

This contribution compares bit rates for 4:4:4 and 4:2:0 encoding. It is for information/discussion only.

Another contribution, JVET-R0244, proposes changes to the CpbVclFactor and MinCrScaleFactor for Main 4:4:4 10 profile. JVET-R0244 proposes to specify that the maximum bit-rate of 4:4:4 should be twice the maximum bit-rate of 4:2:0 (instead of 2.5 as currently specified, and as in AVC and HEVC).

Comments:

In some cases, unless low-pass filtering is applied, the bit rate is higher. However, on the other hand, low-pass filtering to chroma seems not good as it blurs the chroma.

Chroma QP offset or lambda adjustment are another ways of adjusting the bit-rate balance.

A reason for having some extra bit rate header room for 4:4:4 is that the quality expectation for 4:4:4 is higher, and the GOP length or intra refresh period may be shorter.

Sometimes there is RGB coding for 4:4:4, which is generally less efficient than YCbCr.

Discussion stopped here for AHG Session 1.5 Tuesday 7 April at 1500 UTC.

JVET-R0244 AHG9: Coded Picture Buffer sizes and MinCr in VVC [S. Keating, A. Browne, K. Sharman (Sony)]

JVET-R0245 AHG9: Level coding in VVC [S. Keating, A. Browne, K. Sharman (Sony)]

JVET-R0246 AHG9: Max Luma Picture Size in VVC [S. Keating, A. Browne, K. Sharman (Sony)]

### General constraints information (GCI) (9)

JVET-R0086 AHG9: Modification of general constraints flags [W. Lim, G. Bang (ETRI)]

JVET-R0173 AHG9: Cleanup of Constraint Flags [K. Naser, F. Le Léannec, M. Kerdranvat, P. de Lagrange (InterDigital)]

JVET-R0178 AHG9: On APS and GDR constraint Flags [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

See also 4.1.6.1 regarding GDR.

JVET-R0179 AHG9: Constraint Flag for TSRC [K. Naser, F. Le Léannec, T. Poirier, M. Kerdranvat (InterDigital)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Item 5 of this contribution belongs to this category.

JVET-R0207 AHG9: General constraint information for LFNST [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0227 AHG9: General constraint information semantics constraints and a flag for PH in SH [R. Sjöberg, R. Yu, M. Pettersson, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0286 AHG9: On general constraint information syntax [Y.-J. Chang, V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0341 AHG9: on constraint flag for local chroma QP control [Philippe de Lagrange, Karam Naser, Philippe Bordes, Fabrice Le Léannec (interdigital)]

### Parameter sets cleanups (22)

#### General (1)

JVET-R0343 AHG9: A summary of proposals on parameter sets cleanups [Hendry (LGE)]

This contribution is intended to provide a summary of the proposals on parameter sets cleanups submitted to this JVET meeting by the 3 April 2020 submission deadline.

It was suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

Summary of proposals on SPS cleanups:

This section was discussed in AHG Session 1.1 Monday 6 April at 1440 UTC (GJS & YKW).

- New condition for signalling of syntax elements

  - When sps_ptl_dpb_hrd_params_present_flag is equal to 1, inter_layer_ref_pics_present_flag is not present and inferred to be equal to 0 (JVET-R0156 proposal 2)

Is it possible for such a SPS to be referred to by a layer that has reference layers?

It was commented that JVET-R0205 is related.

This item was moved to 4.1.10.

  - Condition the presence of sps_sublayer_dpb_params_flag on the value of sps_ptl_dpb_hrd_params_present_flag, in addition to sps_max_sublayer_minus1 (JVET-R0156 proposal 3) (JVET-R0170) (JVET-R0222 proposal 2) AHG Recommendation: Adopt.

  - Condition that sps_independent_subpics_flag is present only when there are at least two subpictures. (JVET-R0156 proposal 4)

MC wrap-around was discussed.

It was commented that item 1 of R0284 and item 1 of R0071 are identical or similar to this.

This item was moved to 4.2.1.1.

Discussed stopped for AHG Session 1.2 here on Monday 6 April at 1500 UTC (GJS & YKW), and resumed at the start of AHG Session 1.4 Monday 6 April at 2330 UTC (GJS & YKW).

  - Condition the presence of subpic_info_present_flag by res_change_in_clvs_allowed_flag (JVET-R0266 proposal 3). These cannot be used together currently, although there had been proposals to allow them to be used together. No support was expressed by non-proponents for this.

  - Add new flag in SPS to indicate that intra-only (i.e., whether inter-coding is allowed). Use this flag to condition the presence of inter-coding related tools. (JVET-R0283 proposal 1) (JVET-R0335). The proponent said this could skip about 40 syntax elements (more than 5 times more than in HEVC), and drew an analogy to monochrome. It was commented that low-resolution still-picture coding might be the strongest argument for this. Another participant suggested that skipping the irrelevant syntax would ease encoder design. After seeing the syntax table, several participants expressed support for this, while others said this adds extra syntax that is not necessary for video and the bit savings seems too small to make a special provision for it. It was asked whether the syntax structure logic should be different if we do this. Data on this (with the encoder minimizing the necessary amount) was requested. An initial estimate was 23 bits per SPS.

It was later confirmed (Wednesday 8 April) that 23 bits in the SPS (and 4 in the PPS and 1 in the PH) could be saved in such a case. The quantity of data is not compelling; the argument is more a matter of the analogy to monochrome. It was commented whether the analogy is really apt, since there was already the chroma array type information in the SPS to use for that.

There is an all-intra constraint flag in the PTL structure, which currently controls only the slice level.

After discussion, there was not a consensus for this change.

See also R0332 on syntax grouping.

  - In a similar train of thought as point above, do the same for PPS (JVET-R0283 proposal 2). Revisit for data.

  - Add a constraint that sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when there is at least one OLS which has only one layer or VPS is not present? (JVET-R0275)?

It was said that this relates to some other proposals on SPS cleanup (R0191, R0156 aspect 1, R0108 proposal 3).

If VPS is not present, PTL would be absent if the flag is 0. R0156 aspect 1 and R0108 proposal 3 propose that if the sps_video_parameter_set_id is equal to 0, not to signal the flag and infer it to be equal to 1. It was commented that this could affect byte alignment of the PTL information. JVET-R0275 propose to constrain the flag for this. The motivation for not sending it was said to be to prevent the possibility of not having the PTL information at all. AHG Recommendation: To avoid changing byte alignment, the constraint approach was agreed in this case.

When the VPS is present and there is some OLS that has only one layer and the layer ID is the current layer’s ID, this case is proposed to be constrained in R0191 and R0275. AHG Recommendation: The constraint was also agreed to apply in this case.

V. Seregin agreed to provide text in an update of R0275 and to provide the software.

- Infer the value of sps_ccalf_enabled_flag to be equal to 0 when not present (i.e., when sps_alf_enabled_flag is equal to 1 and ChromaArrayType is equal to 0) (JVET-R0105). This seemed to be what was already the intended behaviour. AHG Recommendation (expression of existing intent): Adopt.

- Constraint the semantics of subpic_info_present_flag such that when it is equal to 1, at least either pic_width_max_in_luma_samples or the value of pic_height_max_in_luma_samples shall be larger than the CTB size CtbSizeY. (JVET-R0156 part 2 of item 4). After study, this might affect extraction, so no action was taken on this.

- Change sps_reserved_zero_4bits to sps_reserved_one_4bits to prevent the SPS start code emulation. (JVET-R0266 proposal 1). The case would be with an SPS ID of zero and monochrome and PTL info not present (only in a dependent layer). It was commented that our other reserved bits are zero and this seems somewhat ad hoc. There was no non-proponent interest in this, so no action was taken on it.

Discussion stopped for AHG session 1.4 here on Tuesday 7 April at 0115 UTC, and resumed for AHG Session 1.11 on Wednesday 8 April at 2100 UTC (GJS & YKW).

- Consolidate two syntax elements, sps_poc_msb_flag and poc_msb_len_minus1 into a single syntax element poc_msb_len (JVET-R0266 proposal 2)

It was commented that this seems almost editorial. Another participant commented that having a flag could be desired editorially for having a clear way to disable the feature, and that there may have been a similar prior situation.

It was questioned whether the current semantics are really correct, i.e., whether the signalled MSBs are intended to be all of the MSBs of the POC or only some of them. Others confirmed that all “missing” MSBs are inferred to be 0, and that this is intentional. AHG Recommendation (ed.): The editor is asked to review whether this aspect of the semantics of poc_msb_val is sufficiently clear.

It was noted that with the proposal, the value 0 would be overloaded to have a different and special meaning. It would mean more than the name of the syntax element would imply. (The proposed semantics would need some clarification in this regard.) Since this could be confusing, no action was recommended on the proposal.

- gdr_enabled_flag value constrained by no_gdr_constraint_flag (JVET-R0266 proposal 5, JVET-R0178)

There was discussion of the possibility of having some NUTs that are GDR and some that are not. This is already disallowed.

AHG Recommendation (editorial expression of existing intent): Specify that no_gdr_constraint_flag equal to 1 specifies that gdr_enabled_flag shall be equal to zero. no_gdr_constraint_flag equal to 0 does not impose such a constraint.

- Grouping syntax elements in SPS based on slice type (i.e. intra or inter) (JVET-R0332)

It was discussed whether we would want to do such a rearrangement regardless of whether we want to gate presence on whether inter pictures are present or not (see JVET-R0283 proposal 1 and JVET-R0335). Some participants said that some minor rearrangements might be OK, but wholesale restructuring would be undesirable. It was commented that software implementation would be desirable to make sure there are no overlooked dependencies. The proponent said they did implement it and could provide software for checking. It was asked for such software to be provided in a revision of the contribution. Support for this was expressed, as a more logical structuring of the syntax – the prior syntax may have been rather randomly ordered. Revisit after offline checking of the software.

Summary of proposals on PPS cleanups:

- Require the value of pps_conformance_window_flag to be equal to 0 when the picture width and height are the maximum picture width and height, and infer the values of the PPS conformance window syntax elements to be the same as those signalled in the SPS if the picture width and height are the maximum picture width and height and to be equal to 0 otherwise. (JVET-R0068 proposal 6) (JVET-R0262 proposal 1 and 2)

It was commented that there is already a constraint that in this case the window at the PPS level needs to be the same as the one at the SPS level; the question is only whether to require the flag to be 0 and to infer from the SPS level in this case. In the current draft, in this case, the window parameters are required to be sent in every PPS when non-zero and are required to always be the same.

There is also already a requirement that if the picture size in two PPSs is the same, their cropping windows must be the same. (These constraints are intended to ease RPR operation.)

One participant commented that having an inference rule that is conditional on a particular special case seems potentially confusing to implementers. Others said it only makes sense that if the parameters are required to have a particular value, that is the value that should be inferred and there shouldn’t be syntax capable of violating that constraint.

It was noted that this inference from the SPS prevent complete self-contained interpretation of the PPS content, although we already have some such dependencies.

AHG Recommendation: Adopt.

- Add a new syntax element pps_res_change_allowed_flag in the PPS and use it to condition the presence of the conformance window and scaling window syntax elements (JVET-R0262 proposal 3). This is related to the previous item above. With the action taken on the previous item, this reduces to adding a flag that would skip two flags in the PPS. No action was taken on this, due to the action taken on the previous item.

- Add a constraint for the cropping window offsets and scaling window offsets that at least one of the offsets is different than its default value when the flag that controls their presence is equal to 1 (JVET-R0115). It was commented that this extra constraint doesn’t really seem necessary, so no action was recommended on this.

- Change the signalling for wraparound offset. Signal “picture width minus wraparound offset” instead of “wraparound offset” (JVET-R0162 proposal 1). In the 360° ERP case, this corresponds to sending the padding width rather than the pre-padded picture width. For the 360° CTC, this would save about 16 bits per PPS (sending a value of 16 instead of 4448). The current syntax seems obviously inefficient, so this was supported without any expressed misgivings. AHG Recommendation: Adopt.

- Change the signalling of the PPS ID from ue(v) to u(6), as proposed in R0266 proposal 4. It was noted that this is the only parameter set ID that uses ue(v) coding for its ID. AHG Recommendation: Adopt.

Summary of proposals on APS cleanups:

- Handling chroma related syntax elements in APS when ChromaArrayType is equal to 0

  - To avoid having APS semantics depend on the SPS, move the constraints from the APS semantics to the PH and SH semantics of the relevant APS ID, in, such that the value of scaling_list_chroma_present_flag shall be equal to 0 when the value of ChromaArrayType is equal to 0.

It is also proposed to similarly move the constraint for alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, alf_cc_cr_filter_signal_flag, and add a similar constraint for lmcs_delta_abs_crs (JVET-R0074).

This is asserted to remove APS-to-SPS dependency in the semantics.

This is related to parts of contribution JVET-R0232.

It was asked whether there is really a problem with constraining APS content based on the SPS content. It was commented that this is probably not desirable, although only an editorial matter.

The contribution proposes a constraint move for the scaling list and ALF (purely editorial), and adding a constraint for LMCS.

LMCS contains only two variables (three bits) relevant to chroma. ALF contains only three flags relevant to chroma.

There was a comment about this approach forcing crs_offset not to be 0.

It was commented that the constraints are not really necessary, as the presence of the chroma data in the APS is not necessarily harmful (although we have been trying to avoid sending irrelevant chroma syntax).

  - Add flags (i.e., alf_chroma_present_flag and lmcs_chroma_present_flag) to the APS and constraint them to be equal to 0 when ChromaArrayType is equal to 0 (JVET-R0177 proposal 1)

The LMCS part of this is related to part of contribution JVET-R0232.

  - Repurpose the chroma scaling list presence flag in the APS (i.e., aps_chroma_present_flag) and use this flag to condition the presence of chroma presence flags in the APS. (JVET-R0177 proposal 2 and JVET-R0301)

The difference between “b” and “c” is basically only editorial.

Revisit for further discussion.

Discussion stopped here for AHG Session 1.12 Thursday 9 April at approximately 0115 UTC.

- Change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture (JVET-R0149 proposal 1)

- Disallow interleaving of APS NAL units of different subpictures (JVET-R0149 proposal 2)

- Constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU (JVET-R0210)

- Allow prefix and suffix APS NAL units with particular APS identifier and type to have different content (JVET-R0210)

- Constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU(JVET-R0210)

- Add a mode in PH to allow APS to be signalled within PH. Just like the mode of signalling PH in SH (JVET-R0273)

Later-added SPS cleanups:

- Change the value range of sps_max_sublayers_minus1 from 0..vps_max_sublayers_minus1 to 0..(sps_video_parameter_set_id ? vps_max_sublayers_minus1 : 6). (JVET-R0125)

- Add a constraint on the value of sps_max_sublayers_minus1 such that when sps_video_parameter_set_id is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, sps_max_sublayers_minus1 shall be equal to vps_max_sublayers_minus1. (JVET-R0125)

#### SPS cleanups (10)

JVET-R0105 AHG9: On CC-ALF Signalling in SPS [S. Deshpande, A. Segall, J. Samuelsson, P. Cowan (Sharp)]

JVET-R0125 AHG8/AHG9: On signalling max number of sublayers [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Items 1 and 3 this contribution belong to this category.

JVET-R0170 AHG9: Removed Coding Redundant DPB Related Flag [K. Naser, F. Le Léannec, T. Poirier (InterDigital)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Item 1 of this contribution belongs to this category.

JVET-R0222 AHG9: SPS sublayer syntax cleanup [J. Luo, J. Chen, Y. Ye (Alibaba)]

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Items 1-3, 5 of this contribution belong to this category.

JVET-R0283 AHG9: Cleanup of inter predication HLS syntax elements [K. Naser, F. Le Léannec, M. Kerdranvat, P. de Lagrange (InterDigital)]

JVET-R0332 AHG9: On syntax signalling order in SPS [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-R0408 Crosscheck of JVET-R0332 (AHG9: On syntax signalling order in SPS) [Z.-Y. Lin (MediaTek)] [late]

JVET-R0335 AHG9: On SPS inter slice related syntaxes [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

#### PPS cleanups (5)

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 6 of this contribution belongs to this category.

JVET-R0115 AHG9: On signalling of cropping windows and scaling windows [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-R0162 AHG9: PPS and SH syntax cleanup [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Item 1 (wraparound offset signalling) of this contribution belongs to this category.

JVET-R0262 AHG9: On PPS syntax [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Item 4 of this contribution belongs to this category.

#### APS cleanups (6)

JVET-R0074 AHG9: Removal of APS semantics dependencies on SPS [Z. Deng, L. Zhang, Y.-K. Wang, K. Zhang (Bytedance)]

JVET-R0177 AHG9: APS Cleanup [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-R0301 AHG12: on scaling_list_chroma_present_flag in APS [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0433 AHG 9: Combination of JVET-R0177/R0301 and JVET-R0074 on APS Signalling and Semantics Cleanup [late]

JVET-R0273 AHG9: APS signalled in picture header [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

JVET-R0132 On signalling of chroma related APS [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)] [late]

### Syntax for one slice per picture (14)

JVET-R0406 AHG9: A summary of proposals on syntax for one slice per picture [Y.-K. Wang (Bytedance)] [late]

Discussion began here for AHG Session 1.14 on Monday 13 April at 0500 UTC (GJS & YKW).

This contribution intends to provide a summary of the 13 proposals on syntax for one slice per picture submitted to this JVET meeting.

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

- Add an SPS flag sps_one_slice_per_picture_flag (or a different name with the same semantics: sps_picture_header_in_slice_header_flag). (R0060, R0118)

  - When sps_one_slice_per_picture_flag is equal to 1, skip the signalling of sps_num_subpics_minus1 and sps_independent_subpics_flag and infer the values. (R0060)

  - When sps_one_slice_per_picture_flag is equal to 1, skip the signalling of subpic_info_present_flag. (R0118)

Currently we have a flag in the SH to indicate that all pictures in the CLVS have one slice per picture.

We also have a one_slice_per_pic_constraint_flag (and similar for one tile, and one subpicture).

It was asked whether we have parsing dependencies on the general constraint flags. We do not.

In the current semantics of one_slice_per_pic_constraint_flag equal to 1 there is no constraint that the PH be combined with the SH.

AHG Recommendation (cleanup): Add a “general_pic_header_in_slice_header_constraint_flag” (or similar name). (constrained for sensibility with the existing flag, and constrain the combinations with the above syntax elements for sensibility, but do not add further constraints that are not necessary for sensibility). Text will be provided in a revision of R0118 and software is to be provided by B. D. Choi.

- Skip the signalling of the 6 PPS flags rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag, qp_delta_info_in_ph_flag under certain condition. (R0060, R0113, R0124)

  - Skip them when the existing PPS flag no_pic_partition_flag is equal to 1, and infer their values (to be equal to something TBD) under this condition. (R0113)

  - Skip them when a new PPS flag pps_one_slice_per_picture_flag is equal to 1, and infer their values to be equal to 0 under this condition. (R0060)

    - Also skip the PPS SEs pps_num_subpics_minus1, rect_slice_flag, single_slice_per_subpic_flag, num_slices_in_pic_minus1, and loop_filter_across_slices_enabled_flag (if this flag remains in the PPS) under this condition and infer the values. (R0060)

  - Skip them when a new PPS flag all_pic_coding_info_present_in_ph_flag is equal to 1, and infer their values to be equal to 1 under this condition. (R0124). This can be considered supplemental to item a.

Subitem “a” is straightforward sensibility cleanup. Subitems b and c are basically for coding efficiency savings (saving 5–10 bits) or bypassing unnecessary flexibility in the PPS level.

It was initially agreed to disallow random settings of these flags when the PH is in the SH.

When no_pic_partition_flag is equal to 1 we already don’t send rect_slice_flag, single_slice_per_subpic_flag, num_slices_in_pic_minus1, and loop_filter_across_slices_enabled_flag. And subpic_id_mapping_in_pps_flag prevents sending pps_num_subpics_minus1. One distinction that was pointed out is that the no_pic_partition_flag does not distinguish the case with many tiles and one slice per picture.

It was suggested that the common case would be to send info in the PH unless flexibility is needed in the SH. However, for the RPL and WP, it (currently) matters where the data is signalled; for RPL, it helps to have the slice type, and for WP it helps to have the number of RPL active entries. For the other four, it doesn’t really matter one way or the other. Interacting with this question, there are proposals for signalling the number of active entries in the PH.

AHG recommendation (cleanup): When (pps_)no_pic_partition_flag is equal to 1, skip the 6 PPS flags rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag, qp_delta_info_in_ph_flag and infer them to be equal to 0. Text is provided in R0113 and software to be provided by its proponent. Any missing sensibility constraints may be added by the editor.

- When slice headers referring to the PPS have (sh_)picture_header_in_slice_header_flag equal to 1, require alf_info_in_ph_flag to be equal to 1. (R0200)

Having this be equal to 1 instead of 0 is requested due to wanting the APS information to be early in the NAL unit. However, it was commented that the proposed requirement would be different than discussed for item 2 above.

No action was recommended for this.

- When slice headers referring to the PPS have (sh_)picture_header_in_slice_header_flag is equal to 1, require rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, wp_info_in_ph_flag, qp_delta_info_in_ph_flag to be equal to 1. (R0202)

AHG recommendation (cleanup): When (sh_)picture_header_in_slice_header_flag is equal to 1, require rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, wp_info_in_ph_flag, qp_delta_info_in_ph_flag to be equal to 0.

(Consistency with item 2 suggests the value 0 rather than 1.)

- When (sh_)picture_header_in_slice_header_flag is equal to 1, regardless of the value of wp_info_in_ph_flag, pred_weight_table syntax structure is signalled in slice header and not as part of picture_header syntax structure. (R0220)

  - Or add a constraint such that (sh_)picture_header_in_slice_header_flag and wp_info_in_ph_flag shall not be both equal to 1 (technically equivalent to requiring the value of wp_info_in_ph_flag to be equal to 0 when slice headers referring to the PPS have picture_header_in_slice_header_flag equal to 1). (R0220)

This is resolved by the action taken on item 4.

- Skip the signalling of the SH syntax element (SE) slice_address when the picture contains only one slice and infer its value. (R0060, R0104, R0162, R0189, R0202, R0210)

  - Skip it when a new PPS flag pps_one_slice_per_picture_flag is equal to 1. (R0060)

  - Skip it when the existing SH flag picture_header_in_slice_header_flag is equal to 1. (R0104, R0162, R0189, R0202, R0210)

This is already skipped when rect_slice_flag is 1 and the number of slices in the subpicture is 1. However, it is not skipped when the rect_slice_flag is equal to 0 (i.e., raster scan slices) and NumTilesInPic > 1.

It was suggested to only allow (sh_)picture_header_in_slice_header_flag is equal to 1 when (pps_)rect_slice_flag is 1? It was commented that there is some other PPS syntax associated with that case that would need to be signalled. But that other syntax is minimal (just a PPS flag).

AHG recommendation (cleanup): Only allow (sh_)picture_header_in_slice_header_flag is equal to 1 when (pps_)rect_slice_flag is 1.

- Skip the signalling of the SH SE num_tiles_in_slice_minus1when the picture contains only one slice and infer its value. (R0060, R0104, R0202, R0210)

  - Skip it when a new PPS flag pps_one_slice_per_picture_flag is equal to 1. (R0060)

  - Skip it when the existing SH flag picture_header_in_slice_header_flag is equal to 1. (R0104, R0202, R0210)

This was resolved by the action recommended for item 6.

- Skip the SH SE num_tiles_in_slice_minus1 when NumTilesInPic − slice_address is not greater than 1. (R0210).

R0248 includes the same change (among other proposed changes).

AHG recommendation (cleanup): Adopt. Text and software are to be provided in a revision of R0210.

- Even when skipping of signalling of the SH SE num_tiles_in_slice_minus1 as in the item above is not done, infer num_tiles_in_slice_minus1, when not present, to be equal to NumTilesInPic − 1. (R0060, R0104)

This was resolved by the action recommended for item 8.

- Consider one of the following

  - Skip the signalling of the SH SE slice_subpic_id when the SH flag picture_header_in_slice_header_flag is equal to 1 infer its value. (R0189), or

  - Add a constraint such that when subpic_info_present_flag is equal to 1, the value of picture_header_in_slice_header_flag shall be equal to 0 (technically equivalent to "When picture_header_in_slice_header is equal to 1, the value of subpic_info_present_flag shall be equal to 0." but editorially the constraint should be expressed on picture_header_in_slice_header_flag). (R0189, R0202)

AHG recommendation (cleanup): Adopt approach b. (There was some discussion of whether this is already part of the action taken on item 1.)

- When picture_header_in_slice_header_flag is equal to 1, skip the signalling of the SH SE num_ref_idx_active_override_flag and infer its value to be equal to 0. (R0202)

This would be overriding the default number of active entries signalled in the PPS content.

It was discussed whether there is really a connection between the number of slices in the picture and the need to be able to change the number of active reference pictures used by that picture. It was commented that these are somewhat different issues.

It was said and confirmed that we have designed the PH syntax structure to be the same regardless of whether the PH is combined with the SH or not.

No action was recommended on this since the coupling seems unnecessary and the issue seems minor.

- Add a constraint such that when slice headers referring to the SPS contain the PH syntax structure, separate_colour_plane_flag shall be equal to 0. (technically equivalent to "When separate_colour_plane_flag is equal to 1, the value of picture_header_in_slice_header_flag shall be equal to 0.", but editorially the constraint should be expressed on picture_header_in_slice_header_flag). (R0202)

It was remarked that this is somewhat hypothetical, since no profile supports this.

AHG recommendation (basically editorial cleanup): Adopt.

- Change the text for determination of the first VCL NAL unit of an AU:

  - As follows: (R0163)

If a PH NAL unit is present in a PU, let firstVclNalUnitInPic be the first VCL NAL unit that follows the PH NAL unit; otherwise let firstVclNalUnitInPic be the only one VCL NAL unit in a PU. firstVclNalUnitInPic is the first VCL NAL unit of an AU (and consequently the PU containing the VCL NAL unit is the first PU of the AU) when one or more of the following conditions are true:

    - The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order.

    - The value of ph_pic_order_cnt_lsb of the VCL NAL unit differs from the ph_pic_order_cnt_lsb of the previous picture in decoding order.

    - PicOrderCntVal derived for the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

  - As follows: (R0124)

A VCL NAL unit is the first VCL NAL unit of an AU (and consequently the PU containing the VCL NAL unit is the first PU of the AU) when the VCL NAL unit is the first VCL NAL unit that follows a PH NAL unit or has picture_header_in_slice_header_flag equal to 1 and one or more of the following conditions are true:

...

This issue had also been discussed in the ticket system for ticket #979, and a couple of approaches were discussed in that system. It was agreed that the current text has a bug. The three ways to fix it are all technically equivalent; the differences are only editorial.

AHG Recommendation (BF / expression of existing intent): Correct the text as described (with the editorial detail delegated to the editor).

Discussion stopped here for AHG Session 1.14 on Monday 13 April at 0915 UTC.

- Mandate the EOS NAL unit for easy detection of the first VCL NAL unit of a coded picture. (R0163)

  - Replace the SH flag picture_header_in_slice_header_flag with a variable derived based on the presence of the PH NAL unit. (R0163)

- Change the semantics of the 6 PPS flags rpl_info_in_ph_flag, dbf_info_in_ph_flag, sao_info_in_ph_flag, alf_info_in_ph_flag, wp_info_in_ph_flag, qp_delta_info_in_ph_flag as follows (R0251):

rpl_info_in_ph_flag equal to 1 specifies that reference picture list information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. rpl_info_in_ph_flag equal to 0 specifies that reference picture list information is not present in the PH syntax structure and may be present in slice headers referring to the PPS.

dbf_info_in_ph_flag equal to 1 specifies that deblocking filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. dbf_info_in_ph_flag equal to 0 specifies that deblocking filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS. When not present, the value of dbf_info_in_ph_flag is inferred to be equal to 0.

sao_info_in_ph_flag equal to 1 specifies that SAO filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. sao_info_in_ph_flag equal to 0 specifies that SAO filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS.

alf_info_in_ph_flag equal to 1 specifies that ALF information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. alf_info_in_ph_flag equal to 0 specifies that ALF information is not present in the PH syntax structure and may be present in slice headers referring to the PPS.

wp_info_in_ph_flag equal to 1 specifies that weighted prediction information may be present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. wp_info_in_ph_flag equal to 0 specifies that weighted prediction information is not present in the PH syntax structure and may be present in slice headers referring to the PPS. When not present, the value of wp_info_in_ph_flag is inferred to be equal to 0.

qp_delta_info_in_ph_flag equal to 1 specifies that QP delta information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. qp_delta_info_in_ph_flag equal to 0 specifies that QP delta information is not present in the PH syntax structure and shall be present in slice headers referring to the PPS.

JVET-R0060 AHG9/AHG12: On CLVSs with one slice per picture [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang, J. Xu (Bytedance)]

JVET-R0104 AHG9: On Raster-scan Slice Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0113 AHG9: On Picture Parameter Set [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Item 2 of this contribution belongs to this category.

Item 2 proposes that, when there is no picture partitioning, not to signal the PPS-level flags that indicate whether parameters are signalled in picture header or in slice header (e.g., rpl_info_in_ph_flag, sao_info_in_ph_flag, etc. – six flags) always send these in the SH. Infer them to be equal to 0. (JVET-R0113 proposal 2).

This is related to R0251 item 3.

JVET-R0118 AHG9/AHG12: On signalling of subpicture partitioning in SPS [B. Choi, S. Wenger, S. Liu (Tencent)]

Item 1 of this contribution belongs to this category.

JVET-R0162 AHG9: PPS and SH syntax cleanup [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Item 3 (skip signalling of slice_address) of this contribution belongs to this category.

JVET-R0163 AHG9: On Picture Header [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

JVET-R0189 AHG9: On picture_header_in_slice_header_flag syntax element [Hendry, S. Kim, J. Nam, H. Jang, J. Lim (LGE)]

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

The second aspect (on alf_info_in_ph_flag) of item 2 of this contribution belongs to this category.

JVET-R0202 AHG9: Syntax cleanups when Picture Header is in the Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

Items 2-4, and the num_tiles_in_slice_minus1 and slice_address aspects of item 1 of this contribution belong to this category.

JVET-R0210 AHG9: Cleanup of Picture Header Syntax Structure in Slice Header [S. Esenlik, B. Wang, A. Kotra, E. Alshina (Huawei)]

The num_tiles_in_slice_minus1 and slice_address aspects of this contribution belong to this category.

JVET-R0220 AHG9: Weight prediction syntax cleanup [J. Luo, J. Chen, Y. Ye (Alibaba)]

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

Item 3 of this contribution belongs to this category.

JVET-R0124 AHG9: Clean-ups on picture header [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Items 1 and 2 of this contribution belong to this category.

### Picture header and slice header (12)

JVET-R0410 AHG9: A summary of proposals on PH and SH syntax [Y.-K. Wang, L. Zhang (Bytedance)] [late]

JVET-R0052 AHG9: Overhead reduction for picture header and slice header [S.-T. Hsiang, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0061 AHG9: On allowed slice types in a picture [L. Zhang, Y.-K. Wang, K. Zhang, Z. Deng (Bytedance)]

JVET-R0250 AHG9: On B-slice signalling in the PH and derivation of slice_type [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0112 AHG9: On picture header [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-R0192 AHG9: On signalling recovery point picture [Hendry (LGE)]

JVET-R0198 AHG9: On signalling of IDR or GDR picture flag in picture header [J. Nam, H. Jang, J. Lim, Hendry, S. Kim (LGE)]

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

Item 1 of this contribution belongs to this category.

JVET-R0324 AHG9: On syntax signalling conditions in picture header [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, W. Chen, X. Wang (Kwai Inc.)]

JVET-R0165 AHG12: Entry point offsets avoiding start code emulation prevention byte [K. Abe, T. Toma, V. Drugeon (Panasonic)]

JVET-R0298 AHG9: On order of syntax elements for entry point offsets [K. Suehring (HHI), R. Foray (Allegro DVT)]

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

Item 1 of this contribution belongs to this category.

### Mixed NAL unit types within a coded picture (11)

JVET-R0414 AHG9: A summary of proposals on mixed NAL unit types within a coded picture [Y.-K. Wang (Bytedance)] [late]

JVET-R0042 AHG8/AHG9/AHG12: On mixed subpicture types within a picture [Y.-K. Wang (Bytedance)]

JVET-R0136 AHG9/AHG12: Improvements on sps_independent_subpics_flag and nal_unit_type constraint [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

Item 2 of this contribution belongs to this category.

JVET-R0276 AHG9: On IRAP NAL constraint for reordered subpictures [V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0085 AHG9: On signalling the mixed NAL unit type flag [L. Chen, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0203 AHG9/AHG12: On combination of NAL unit types in a picture [Hendry, S. Kim (LGE)]

JVET-R0267 AHG9/AHG12: On mixed NAL unit types [Y. He, M. Coban, V. Seregin, A.K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-R0270 AHG9: On mixing of RASL and RADL NAL unit types [Hendry, S. Kim (LGE), R. Skupin, Y. Sanchez, K. Suehring (HHI)]

JVET-R0315 AHG9: On mixed nal unit type signalling and PPS cleanup [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

JVET-R0120 AHG9: On mixed NAL unit types [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-R0124 AHG9: Clean-ups on picture header [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Item 3 of this contribution belongs to this category.

### RPL, WP, and collocated picture signalling (10)

JVET-R0411 AHG9: A Summary of Proposals Related to Reference Picture Lists, Weighted Prediction, and Collocated Picture Signalling [S. Deshpande (Sharp)] [late]

JVET-R0059 AHG9: Cleanups on RPL and related signalling [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang, J. Xu (Bytedance)]

JVET-R0323 AHG9: On TMVP enabling flag in picture header [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, W. Chen, X. Wang (Kwai Inc.)]

JVET-R0102 AHG9: On Reference Picture List Override Signalling [S. Deshpande, T. Chujoh, T. Ikai, J. Samuelsson, A. Segall (Sharp)]

JVET-R0138 AHG9: Some constraints of num_ref_entries [T. Chujoh, T. Ikai (Sharp)]

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 2 of this contribution belongs to this category.

JVET-R0205 AHG9: On signalling of inter_layer_ref_pics_present_flag [T. Nishi, K. Abe, V. Drugeon (Panasonic)]

JVET-R0253 AHG9: Three restrictions when RPL is present in PH [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0255 AHG9: Fixes related to RPL [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0277 AHG9: On reference picture list signalling [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

### Signalling of virtual boundaries (4)

JVET-R0121 AHG9/AHG12: On virtual boundary signalling with subpictures [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Item 4 of this contribution belongs to this category.

JVET-R0256 AHG9: Virtual boundaries in increasing order using u(v) [M. Damghanian, M. Pettersson, R. Sjöberg, Z. Zhang, J. Enhorn, R. Yu, J. Ström (Ericsson)]

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Item 6 of this contribution belongs to this category.

### Hypothetical reference decoder (HRD) (9)

JVET-R0342 AHG9: A Summary of Proposals Related to HRD S. [Deshpande (Sharp)]

JVET-R0094 AHG9: DPB output time offsets for temporal sublayers [V. Drugeon, K. Abe (Panasonic)]

JVET-R0100 AHG9: On Decoding Unit Information Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0101 AHG9: On Alternative Timing Information Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0103 AHG9: On Picture Timing Information Signalling and HRD [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0264 AHG9: On sub-bitstream extraction [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0295 AHG12: On subpicture conformance [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-R0297 AHG9: HRD bug-fixes and editorial clarifications [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-R0413 AHG9: On Parameters for HRD Timing Information [S. Deshpande (Sharp)] [late]

### DCI, VUI, and SEI (6)

JVET-R0090 AHG9: On Video Usability Information [V. Drugeon (Panasonic)]

JVET-R0190 Post-filter hint based on ALF classification [H.-B. Teo, H.-W. Sun, C.-S. Lim (Panasonic)]

JVET-R0242 AHG9/AHG12: Decoded subpicture hash SEI message [J. Boyce, L. Xu (Intel)]

JVET-R0260 AHG9: On decoding capability information [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0307 AHG8/AHG9: Positioning information SEI message of output independent layers [E. Thomas (TNO)]

JVET-R0308 AHG8: Implementation of multi-layer decoding and output independent layer composition in VTM [E. Thomas (TNO)]

### HLS editorial inputs (1)

JVET-R0249 AHG9: Proposed structural text changes to HLS in the VVC specification [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

## AHG12: high-level parallelism and coded picture regions (51)

### Subpictures (25)

#### General (1)

JVET-R0415 AHG12: A summary of proposals on subpictures [Hendry (LGE)] [late]

#### General and misc. subpicture aspects (11)

JVET-R0071 AHG12: Some cleanups on subpicture signalling [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

JVET-R0091 AHG9: Issue with slice indexing [V. Drugeon (Panasonic)]

JVET-R0151 AHG6/AHG12: Uncoded subpictures and potential applications [J. Sauer (RWTH Aachen Univ.)]

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 4 of this contribution belongs to this category.

JVET-R0238 AHG12: A fix on subpicture-level slice indexing [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, K. Fan, J. Xu, H. Liu (Bytedance)]

JVET-R0093 AHG12: Subpictures and conformance cropping window [V. Drugeon (Panasonic)]

Item 1 of this contribution belongs to this category.

JVET-R0136 AHG9/AHG12: Improvements on sps_independent_subpics_flag and nal_unit_type constraint [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

Item 1 of this contribution belongs to this category.

JVET-R0186 AHG12: On misc updates for picture partitioning signalling [Hendry, S. Paluri, S. Kim (LGE)]

JVET-R0257 AHG12: Raster scan order flag for subpictures [M. Damghanian, R. Sjöberg, M. Pettersson, Z. Zhang, J. Enhorn, J. Ström, R. Yu (Ericsson)]

JVET-R0284 AHG12/AHG9: On independent subpicture signalling [Y.-J. Chang, Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0337 AHG12: Subpicture with filler slice for merged stream [K. Kawamura, S. Naito (KDDI)]

#### Subpicture layout signalling (4)

JVET-R0117 AHG9/AHG12: On signalling of subpicture and slice in PPS [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0118 AHG9/AHG12: On signalling of subpicture partitioning in SPS [B. Choi, S. Wenger, S. Liu (Tencent)]

Item 2 of this contribution belongs to this category.

JVET-R0135 AHG12: On subpicture layout signalling [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

JVET-R0239 AHG9: Cleanups on signalling of tiles, slices, and subpictures [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, J. Xu, H. Liu (Bytedance)]

#### Subpicture ID signalling (4)

JVET-R0087 AHG12: Modification of subpicture information in slice header [W. Lim, G. Bang (ETRI)]

JVET-R0088 AHG12: Modification of subpicture information in PPS [W. Lim, G. Bang (ETRI)]

Item 1 of this contribution belongs to this category.

JVET-R0126 AHG9/AHG12: On signalling of subpicture ID [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0265 AHG9/AHG12: On subpicture ID mapping signalling [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

#### Subpicture based bitstream extraction and merging (5)

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 5 of this contribution belongs to this category.

JVET-R0092 AHG12: Subpicture size calculation for subpicture extraction [V. Drugeon (Panasonic)]

JVET-R0093 AHG12: Subpictures and conformance cropping window [V. Drugeon (Panasonic)]

Item 2 of this contribution belongs to this category.

JVET-R0294 AHG12: On subpicture extraction [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JVET-R0148 AHG12: Subpicture merging experiments [A. Hallapuro, M. Homayouni, A. Aminlou, M. M. Hannuksela (Nokia)]

### Slices and tiles (20)

#### Tile signalling (7)

JVET-R0053 AHG9: Signalling tile partitioning [S.-T. Hsiang, C.-M. Tsai, Y.-W. Huang, S.-M. Lei (MediaTek)]

Items 1 and 2 of this contribution belong in this category.

This was discussed in AHG Session 1.2 Monday 6 April at 1600 UTC (GJS & YKW).

This contribution proposes three high-level syntax modifications related to signalling tile partitioning of the coded picture, summarized as follows:

- The number of explicitly provided tile column widths or tile row heights is proposed to be allowed to be equal to 0. When no_pic_partition_flag is equal to 0 and the coded picture contains only one tile, the proposed method can infer the tile column width and the tile row height to be equal to the picture width and the picture height, respectively, without signalling the syntax elements tile_column_width_minus1[ 0 ] and tile_row_height_minus1[ 0 ] for deriving the tile column width and the tile row height.

R0285 and R0080 are said to contain the same proposal. It is a syntax optimization to avoid signalling tile width or height if the picture is not split into tiles in the corresponding dimension. It was commented that this is consistent with another use of avoiding explicit signalling. It was commented that for uniform tile size signalling, this would increase the signalling that is needed. No action on this was recommended; although it saves a few bits in the PPS in some cases, it adds more in another case that some participants expect to be more common.

- pps_log2_ctu_size_minus5 is proposed to be signalled in the picture parameter set (PPS) only when rect_slice_flag is equal to 0, single_slice_per_subpic_flag is equal to 0 and num_slices_in_pic_minus1 is greater than 0.

A participant commented that it is nice to know pps_log2_ctu_size_minus5, even in other conditions where it is not necessary for parsing. Since it is only two bits, it does not seem important to avoid sending this. No action was recommended on this.

- loop_filter_across_tiles_enabled_flag is proposed to be signalled only when the number of the tiles in the coded picture is greater than 1.

At the previous meeting, it was planned that we would move this flag to the SPS (see notes for JVET-Q0120).

R0113 and an aspect of R0197 are proposing this in principle as well.

It was asked whether bitstream merging would be affected by this, and it was commented that PPSs need to be rewritten in that case anyway.

AHG Recommendation (cleanup/previous plan): confirm the move to the SPS.

JVET-R0062 AHG12: A cleanup on uniform tile and rectangular slice partitioning [L. Zhang, Z. Deng, K. Zhang, Y.-K. Wang (Bytedance)]

Discussed in AHG Session 1.2 Monday 6 April at 1515 UTC (GJS & YKW).

This contribution proposes the following changes related to uniform tile and rectangular slice partitioning:

- In the equation (Eqn. 23) for derivation of tile columns parameters, replace the loop count "i < num_exp_tile_columns_minus1" with "i  <=  num_exp_tile_columns_minus1", such that the value of the last explicitly signalled tile_column_width_minus1[ i ] specifies the width of at least one tile column. The semantics of tile_column_width_minus1[ i ] is updated accordingly.

- In the equation (Eqn. 24) for derivation of tile rows parameters, replace the loop count "j < num_exp_tile_rows_minus1" with "j  <=  num_exp_tile_rows_minus1", such that the value of the last explicitly signalled tile_row_height_minus1[ i ] specifies the height of at least one tile row. The semantics of tile_row_height_minus1[ i ] is updated accordingly.

- In the equation (Eqn. 30) for derivation of in-tile rectangular slices parameters, replace the loop count "j < num_exp_slices_in_tile[ i ] − 1" with "j < num_exp_slices_in_tile[ i ]", such that for each value of i, the value of the last explicitly signalled exp_slice_height_in_ctus_minus1[ i ][ j ] specifies the height of at least one rectangular slice in the tile containing the i-th rectangular slice.

The basic desire is to prevent the ability of the encoder to express syntax that seems strange and confusing. It was said that the strange case is not uncommon.

This proposal was viewed favourably. One participant said this is essentially editorial. There is not really a bug – just a lack of prohibiting something strange that is asserted to be useless. The proposal would just prohibit the encoder from signalling silly values that would result in the same decoded result as sensible values. It was said there are other related proposals. AHG Recommendation (cleanup): Adopt.

JVET-R0080 AHG12: On signalling of tile and slice [Y.-U. Yoon, D. H. Park (KAU), J. H. Do (ETRI), J.-G. Kim (KAU)]

Discussed in AHG Session 1.2 Monday 6 April at 1650 UTC (GJS & YKW).

VVC (Draft 8) includes signalling for tile and slices in PPS.

This contribution proposes two modifications on signalling of tile and slice information in the PPS as follows.

- Proposal 1: It is proposed to change the condition for signalling the syntax element of tile_idx_delta_present_flag. When the value of num_slices_in_pic_minus1 is greater than 1 instead of 0, the syntax element of tile_idx_delta_present flag is signalled.

This would save one bit when there are only two rectangular slices in the entire picture. Although the savings is very minor, the change is trivial. JVET-R0211 item 1 proposes this as well.

AHG Recommendation: Adopt.

- Proposal 2: It is proposed to replace the syntax element of num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 as num_exp_tile_columns and num_exp_tile_rows, respectively. When a picture is not partitioned into multiple tiles in rows or columns, the value of num_exp_tile_columns or num_exp_tile_rows is signaled as 0. Then, the syntax element of tile_column_width_minus1 or tile_rows_height_minus1 is not signaled and inferred to be equal to PicWidthInCtbsY-1 or PicHeightInCtbsY-1.

Proposal 2 is the same as item 1 of R0053; see the notes for that item.

JVET-R0054 AHG12: On combination of wavefront parallel processing and tile partitioning [C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Discussed in AHG Session 1.2 Monday 6 April at 1700 UTC (GJS & YKW).

[Not really an HLS proposal.]

In VVC Draft 8, parallel processing can be achieved by using wavefront parallel processing (WPP) or tile partitioning, and they are allowed to be simultaneously used within the same picture. However, allowing the combination of WPP and tile partitioning not only increases the effort of decoder verification but also introduces functionality redundancy between WPP and horizontal tile partitioning. In order to reduce the verification effort and remove the functionality redundancy, two methods are proposed in this contribution.

- In Method 1, if WPP is used in the current picture, the number of tile rows in the current picture shall be equal to 1. As a result, the functionality redundancy between WPP and horizontal tile partitioning is removed, and the behavior of CABAC context variable inheritance is also simplified.

- In Method 2, same as in most HEVC profiles (those other than the high-throughput profiles), WPP and tile partitioning are disallowed to be simultaneously used within the same picture. It is claimed that Method 2 is simpler than Method 1 and simplifies the decoder verification significantly.

The proposed restriction is motivated by verification effort. It is a functionality change and was suggested to be too substantial to be able to agree to in the AHG. One participant said that “method 1” would be undesirably restrictive in the case of rectangular slices with wavefronts.

Left open by the AHG.

JVET-R0157 AHG9/AHG12: Signalling cleanup on PPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 2 of this contribution belongs to this category.

Discussed in AHG Session 1.6 Tuesday 7 April at 1520 UTC (GJS & YKW).

It is proposed to skip signalling of tile width and height when the picture width or height is less than or equal to the CTU size. It was noted that a similar provision is applied for subpicture signalling and in the SPS syntax. This would save two bits in each relevant dimension.

It was remarked that something this is also proposed in the 2nd aspect of proposal R0239, with a somewhat simpler editorial expression.

One participant said this seemed like an unnecessary complication for a corner case. In the subpicture case the syntax element is a u(v) rather than ue(v), and it was a somewhat different circumstance.

However, another participant said it was strange to send something and have semantics saying it shall be in the range of 0 to 0.

Software had been provided and the proponent said they had tested it.

No action was recommended on this since the issue is for a very minor corner case.

An editorial bug fix is also proposed for when num_exp_tile_columns_minus1 is equal to 0; however this aspect was no longer relevant due to an action taken on R0062.

JVET-R0221 AHG9: Clean-up of tile signalling [J. Luo, J. Chen, Y. Ye (Alibaba)]

Discussed in AHG Session 1.6 Tuesday 7 April at 1555 UTC (GJS & YKW).

In VVC draft 8, the syntax allows indicating a sum of tile widths/heights that is wider than the picture width/height. It is asserted that the constraints of tile partitioning is not straightforward. If the sum of signalled tile widths/heights is larger than picture width/height, the current derivation could cause invalid CTU addresses being added to a slice. In this contribution, it is proposed to add two conformance constraints to make the conformance requirements on tile partitioning cleaner, such that invalid CTU address would not be included in a slice. In the second version, the conformance requirement on tileColBd and tileRowBd is added.

Q0359 was a related proposal of the last meeting. The proponent reported that there was still an editorial error in the constraint expression.

This is an editorial bug fix proposal.

AHG Recommendation (expression of existing intent): The editor is asked to ensure that the text adequately expresses the necessary constraints, such that tiles, slices, and subpictures are a proper partitioning of the picture (no overlaps, no gaps, no CTUs that are outside the picture).

JVET-R0285 AHG12: On tile information signalling [Y.-J. Chang, Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Discussed in AHG Session 1.6 Tuesday 7 April at 1625 UTC (GJS & YKW).

In this contribution, there are two proposed changes to the signalling of tile information:

- Replace num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 with num_exp_tile_columns and num_exp_tile_rows.

- Change the range of num_exp_tile_columns_minus1 to be 0 to PicWidthInCtbsY − 2, inclusive, and the range of num_exp_tile_rows_minus1 to be 0 to PicHeightInCtbsY − 2, inclusive.

The first aspect is the same as in R0053 item 1 and R0080; see notes elsewhere on that.

The second aspect has a somewhat similar spirit to R0062. It is intended to prohibit the encoder from sending something explicitly that could be inferred instead. However, it would not be a strictly necessary change.

“Method 1” would tighten the constraint on num_exp_tile_columns(rows)_minus1. It was commented that “Method 2” seemed like unnecessary complication for a corner case. In “Method 2”, a shortcut is proposed for when the number of tiles columns or rows is equal to the number of CTUs in the picture width or height.

No clear need for action was identified, so no action was taken.

#### Rectangular slice signalling (11)

JVET-R0088 AHG12: Modification of subpicture information in PPS [W. Lim, G. Bang (ETRI)]

Item 2 of this contribution belongs to this category.

Discussed in AHG Session 1.6 Tuesday 7 April at 1655 UTC (GJS & YKW).

This contribution proposes to modify syntax elements related to subpicture and slice in PPS. The number of subpictures and slices are dependent according to the current VVC specification. In PPS, those syntax elements are signalled regardless of each other. The following two proposals are described in this document.

Proposal 1) Subpicture-related syntax elements are signalled in PPS when a picture partitioned which refers to the PPS.

Proposal 2) Signaling difference between the number of slices and the number of subpictures instead of the number of slices.

Proposal 2 is to save some bits for signalling in the PPS. It was commented that this is for something sent only once per PPS. It was commented that the number of subpictures, which this uses, is not always available in the PPS.

Proposal 2 was said to be the same as the second item of R0117, in which item 1 proposes to make the number of subpictures unconditionally present in the PPS.

Several participants commented that it seems undesirable to couple the subpicture and slice signalling and use differential signalling, e.g., as the number of bits saved is minimal. Even without the issue of whether the number of subpictures is always present or not, this was expressed.

Another contribution R0162 was said to also be related, which proposes to change num_slices_in_pic_minus1 to num_slices_in_pic_minus2.

Q0332 was also somewhat similar, and it was concluded at the time that it would not provide a bit savings or be substantially beneficial.

The AHG recommended no action on R0088 item 2.

JVET-R0162 AHG9: PPS and SH syntax cleanup [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Item 2 (num_slices_in_pic_minus2 signalling) of this contribution belongs to this category.

Discussed in AHG Session 1.6 Tuesday 7 April at 1710 UTC (GJS & YKW).

This proposes to change num_slices_in_pic_minus1 to num_slices_in_pic_minus2 in PPS. Aside from a small bit savings (which is not the main motivation), this would prevent a duplicate way of expressing the same thing.

A participant commented that the semantics of single_slice_per_subpic_flag is “one way”, and if this was adopted that would need to be changed, so there would be only one way to express that. It was commented that this seems similar in spirit to the action on each_layer_is_an_ols_flag. Both of these flags are intended as shortcuts for particular cases and making the constraint two-way might make it easier to understand. However, another participant said that although we wanted to have the shortcut, we should not force it to be used just because it is applicable. Another participant said there was a difference between the situation for the each_layer_is_an_ols_flag.

At least one subpicture in all pictures referring to the PPS would need to have at least two slices in it if this is adopted. It is noted that the slice layout is determined in the PPS, so this may not be a significant burden.

There seemed to be no clear need for action, and some participants disliked the removal of flexibility of expression. Others (including the original proponent of the shortcut) thought the two-way constraint would be more sensible and consistency with each_layer_is_an_ols_flag is desirable.

Discussion stopped here for AHG session 1.6 Tuesday 7 April at 1715 UTC, and resumed here with AHG Session 1.10 on Wednesday 8 April at 1530 UTC]

After offline study and further discussion, there continued to be mixed opinions.

JVET-R0111 AHG9/AHG12: Vertical slice boundaries [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Discussed in AHG Session 1.10 Wednesday 8 April (GJS & YKW).

This contribution provides an analysis of using subpictures for server-side composition. It is asserted that it would be beneficial to be able to merge bitstreams from several different sources into a combined bitstream composed of multiple different subpictures. The contribution details some of the parameters and settings that needs to be aligned between different sources in order to support this use case and includes a proposal for one modification asserted to improve feasibility and reduce implementational burden; to allow a tile to include multiple slices either vertically or horizontally (which is different from the current VVC draft where only horizontally structured slices are allowed within a tile).

In summary, the following changes are proposed:

- A new syntax element, vertical_slice_boundaries_flag, for indicating if a tile is split vertically or horizontally.

- Modification to CTU scan derivation to include two cases depending on the value of the proposed flag.

- Update to syntax and decoding process since a vertical slice boundary is no longer required to be aligned with a vertical tile boundary.

- Replace the level limit on number of vertical tile boundaries with a limit on the sum of vertical tile boundaries and vertical slice boundaries.

The proposal would avoid having the encoder need to pre-segment the content into more tiles when authoring the content (which also has a coding efficiency penalty).

This would be a significant design change, as it would require the decoder to track vertical boundary positions that would be different for different rows. There had been an objection to this at the previous meeting as a burden on decoder implementation. It was commented that this would have a significant impact on hardware design.

It was commented that the previously proposed dependent slices concept of N0497 is an alternative approach to provide some degree of similar functionality.

It was commented that there has been usage of MCTSs and a need for highly coordinated encoding in past practice.

Especially given the late stage at which we are in this design process, there were strong objections to the proposed change, so no action was taken on this.

JVET-R0129 AHG9/AHG12: On CTU row based slice chunks of a slice within a tile [L. Chen, C.-W. Hsu, C.-C. Chen, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-M. Tsai, Y.-W. Huang, S.-M. Lei (MediaTek)]

Discussed in AHG Session 1.10 Wednesday 8 April at 1630 UTC (GJS & YKW).

This contribution proposes to specify CTU row-based slice chunks of a slice within a tile in decoding order. The slice chunks in sequence of a slice are delivered and decoded sequentially. Each slice chunk is proposed to be contained in a single NAL unit. It is claimed that no essential new requirements are needed in the decoding process. It is claimed that the proposed slice chunks can meet ultra-low latency requirements with better coding efficiency than conventional slices. It is reported the BD-rate savings of the proposed slice chunks compared against the conventional slices are 4.52%, 3.55%, 3.64%, and 4.93% for short length (1 sec) Class A1 (4K), Class A2 (4K), Class B (1080p), and Class E (720p), respectively, when the encoding latency is one CTU row. Proposed reference software was claimed to be ready and cross-checked and was attached in the uploaded proposal package.

This would be something like a special case of the dependent slice segment concept of HEVC. The proponent said dependent slice segments was a useful feature of HEVC, while another participant said they had seldom encountered this feature in practice.

Especially given the late stage at which we are in this design process, there were strong objections to the proposed change, so no action was taken on this.

JVET-R0349 Crosscheck of JVET-R0129: AHG9/AHG12: On CTU row based slice chunks of a slice within a tile [J. Chen, J. Luo, Y. Ye (Alibaba)] [late]

JVET-R0157 AHG9/AHG12: Signalling cleanup on PPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 1 of this contribution belongs to this category.

Discussed in AHG Session 1.10 Wednesday 8 April (GJS & YKW).

Item 1 proposes the following cleanups for the PPS, when considering mixed NAL unit types: When mixed_nalu_types_in_pic_flag is equal to 1, no_pic_partition_flag is proposed not to be signalled but inferred to be equal to 0 and rect_slice_flag is proposed not to be signalled but inferred to be equal to 1.

The motivation was said to be primarily to ensure that an invalid combination is not indicated in the PPS.

There was discussion of the desired relative order of mixed_nalu_types_in_pic_flag and no_pic_partition_flag.

It was commented that mixed_nalu_types_in_pic_flag does not affect the decoding process, and this would make the syntax depend on it. It was also commented that no_pic_partition_flag is an important property and it would be undesirable to omit it even when this is hypothetically possible. It was commented that it would be common in a BEAM application to not need to change any aspect of the PPS other than to flip that flag, so semantic constraints were suggested to be sufficient, so no action was recommended on this.

Editor action item: The editor was asked to check and make sure that the constraints (which we believe are already expressed in some form) are sufficiently clear to the reader.

JVET-R0187 AHG12: On signalling for picture with one tile and multiple slices [Hendry, S. Paluri, J. Zhao, S. Kim (LGE)]

Discussed in AHG Session 1.10 Wednesday 8 April (GJS & YKW).

It is asserted that in the current picture partitioning signalling scheme when there is only one tile and the slices are rectangular slices, the syntax elements num_slices_in_pic_minus1 and tile_idx_delta_present_flag are not needed. In such situation, the number of slices in the picture can easily be known from the derived variable NumSlicesInTile[ 0 ] and the value of tile_idx_delta_present_flag is never be used.

Furthermore, it is asserted that by not signalling num_slices_in_pic_minus1 in the described scenario above, it would be possible to avoid having the encoder signal an incorrect value for num_slices_in_pic_minus1), i.e. a value that is different from the derived value (i.e., NumSlicesInTile[ 0 ]).

This contribution proposed to omit the signalling of num_slices_in_pic_minus1 and tile_idx_delta_present_flag when no_pic_partition_flag is equal to 0, NumTilesInPic is equal to 1, and rect_slice_flag is equal to 1.

It was commented that this might prevent having tiles that are split into a signalled number of slices.

No action was planned on this unless offline study determines otherwise.

Discussion stopped here for AHG Session 1.10 on Wednesday 8 April at 1715 UTC.

JVET-R0188 AHG12: On signalling of rectangular slice height and width [Hendry, S. Kim, S. Paluri (LGE)]

JVET-R0209 AHG12/AHG9: On signalling of rectangular slices [S. Esenlik, B. Wang, A. M. Kotra, E. Alshina (Huawei)]

JVET-R0211 AHG12: Cleanups on rectangular slices signalling [B.-K. Lee (Xris)]

JVET-R0241 AHG12: A direct signalling method of rectangular slice partitioning [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, J. Xu, H. Liu (Bytedance)]

JVET-R0247 AHG9: Signalling rectangular slice partitioning [S.-T. Hsiang, C.-W. Hsu, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

#### Raster-scan slices (2)

JVET-R0047 AHG9/AHG12: On slice address for raster scan slices in a picture [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0248 AHG9: Supporting multiple slices within one tile for raster-scan slice mode [S.-T. Hsiang, L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

### Control of loop filtering across subpicture/tile/slice boundaries (6)

JVET-R0044 AHG9: On subpicture boundary handling [J. Li, K. Abe (Panasonic)]

JVET-R0053 AHG9: Signalling tile partitioning [S.-T. Hsiang, C.-M. Tsai, Y.-W. Huang, S.-M. Lei (MediaTek)]

Item 3 of this contribution belongs to this category.

JVET-R0113 AHG9: On Picture Parameter Set [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Item 1 of this contribution belongs to this category.

JVET-R0069 AHG12: Control of loop filtering across subpicture/tile/slice boundaries [L. Zhang, Y.-K. Wang, K. Zhang (Bytedance), Hendry, N. Park, H. Jang, J. Nam, S. H. Kim, J. Lim (LG Electronics)]

JVET-R0109 AHG9/AHG12: On tile, slice, and related loop filter control flags [L. Chen, C.-W. Hsu, C.-M. Tsai, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0197 AHG12: On signalling of loop filter across tiles and slices enabled flags [N. Park, J. Nam, H. Jang, J. Lim, Hendry, S. Kim (LGE)]

## AHG8: layered coding and resolution adaptivity (29)

### Scalability specific HLS (27)

#### General scalability HLS topics (10)

JVET-R0046 AHG8: Temporal sublayer requirements for multi-layer referencing [C.-Y. Lai, O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0065 AHG8/AHG9: On IRAP and GDR AUs [Y.-K. Wang (Bytedance)]

Item 2 of this contribution belongs to this category.

JVET-R0274 AHG8: On CVSS AU [V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0066 AHG8/AHG9: On DPB memory allocation and derivation of NoOutputOfPriorPicsFlag [Y.-K. Wang (Bytedance)]

JVET-R0067 AHG8/AHG9: On the derivation of PictureOutputFlag [Y.-K. Wang (Bytedance), M. M. Hannuksela (Nokia)]

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 1 of this contribution belongs to this category.

JVET-R0278 AHG8: On SPS sharing and slice type constraint [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0070 AHG9: On repetition and update of non-VCL data units [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

Item 5 of this contribution belongs to this category.

JVET-R0194 AHG8/AHG9: On parameter set sharing in multi-layered bitstream [Hendry (LGE)]

JVET-R0123 AHG9: On derivation of picture output flag [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

#### Scalability information signalling and related (17)

JVET-R0344 AHG9: A Summary of Proposals Related to Scalability Information Signalling [S. Deshpande (Sharp)]

Discussed in AHG Session 1.3 Monday 7 April at 2100 (GJS, YKW & JRO).

This contribution intends to provide a summary of proposals on scalability information signalling.

Seventeen proposals from the category “4.3.1.2 Scalability information signalling and related” listed in a revision of JVET-R0339-v4 are included in this summary. Thus in v3, summary is added for JVET-R0158, JVET-R0199, and JVET-R0222 aspect 1.

It is suggested that this summary be used for the reviewing of these proposals, such that the discussions may be done in a more structured and efficient manner.

List of design questions

Related to PTL information signalling

- Omit signaling of index to the list of PTL structures for output layer sets when number of signalled PTL structures is equal to total number of output layer sets and instead infer its value? (JVET-R0161 PTL part of proposal 2, JVET-R0185 proposal 4, JVET-R0204, JVET-R0275 aspect 4)

Note: There may not be an OLS that contains all layers (regardless).

Something similar is in the draft for HRD.

Is the number of OLSs always less than or equal to the number of PTL structures? There is a constraint like that already (regardless of this proposal).

It was commented that this usage seems like it would be common. This is a syntax shortcut for that case.

AHG Recommendation (cleanup): Adopt. Text and software were provided by R0161 proponent (J. Chen).

Related to DPB information signalling

- Modify the upper range of vps_num_dpb_params to allow signalling of DPB parameters for all OLSs from current fixed upper value of 16:

  - upper limit is equal to total number of OLSs minus the number of single-layer OLSs (JVET-R0099 Proposal 1, JVET-R0191 aspect 3). AHG Recommendation (expression of existing intent): Adopt. Text was provided by Hendry and he is also to supply software.

  - upper limit is equal to total number of OLSs − 1 (JVET-R0196)

- Update the range value for num_ols_hrd_params_minus1

  - similarly as in (a) in previous item? (JVET-R0191 aspect 3) AHG Recommendation (expression of existing intent): Adopt. Text was provided by Hendry and he is also to supply software.

  - to total number of OLSs − 2 (JVET-R0204)

- Don't signal and instead infer the index of the dpb_parameters( ) syntax structure that applies to the i-th OLS when a condition is met (JVET-R0099 Proposal 2, JVET-R0204, JVET-R0275 aspect 4)

  - The condition is total number of output layer sets minus number of single layer output layer sets is equal to number of signalled dpb parameters (JVET-R0099 proposal 2). AHG Recommendation (expression of existing intent): Adopt, and also apply to HRD parameters. Text was to be provided by S. Deshpande and he is also to supply software.

  - The condition is total number of output layer sets is equal to number of signalled dpb parameters (JVET-R0161 proposal 2, JVET-R0275 aspect4)

  - The condition is total number of output layer sets is equal to number of signalled dpb parameters + 1 (JVET-R0204)

- Start the for loop which signals ols_dpb_pic_width[ i ], ols_dpb_pic_height[ i ], and ols_dpb_params_idx[ i ] to start at 1 instead of at 0, since 0-th OLS is single layer? (JVET-R0099 Proposal 1, JVET-R0196). AHG Recommendation (expression of existing intent): Adopt (unless affected by proposals to redefine the 0-th OLS).

- Replace if( !vps_all_independent_layers_flag ) condition on vps_num_dpb_params syntax element with if(!each_layer_is_an_ols_flag) (JVET-R0185 proposal 1, JVET-R0196, JVET-R0275 aspect 3). AHG Recommendation (bug fix): Adopt.

  - If above main item is agreed, additionally change vps_num_dpb_params to vps_num_dpb_params_minus1? (JVET-R0185 proposal 2, JVET-R0196, JVET-R0275 aspect 3). It was commented the semantics of each_layer_is_an_ols_flag is a “one-way” constraint. It was asked why we would want to allow the flag to be 0 and still have each layer be an OLS – all of this is in the VPS, so the encoder should know what it is doing when it writes the VPS. AHG Recommendation (bug fix): Adopt and change the semantics to a “two-way” constraint (so if the flag is zero, there must be at least one multilayer OLS specified by the VPS).

  - Additionally signal DPB parameters for OLS in this case only if(!each_layer_is_an_ols_flag) (JVET-R0185 proposal 3). AHG Recommendation (cleanup): Adopt.

Text was provided by Hendry (to be modified for the “two-way” constraint) and he is also to supply software.

Related to HRD information signalling

- Allow control separately if HRD parameters are signalled for an OLSs or not on individual basis? (JVET-R0195).

The proponent indicated that this was motivated by the syntax allowing some HRD parameters to be present and some not for single-layer OLSs. This flexibility is not provided for other cases.

It was commented that the situation for single-layer OLSs was just a consequence of where the data is sent, and noted that each single-layer OLS could be extracted and become a single-layer stand-alone bitstream. So no action was recommended by the AHG on this.

  - If want separate control then signal a separate new flag for each OLS to specify if index to HRD parameters structure is signalled or not? OR

  - Designate 0-th index to mean HRD parameters are not specified for an OLS?

- Change condition for omitting signalling of ols_hrd_idx[ i ] from “num_ols_hrd_params_minus1 + 1  !=  TotalNumOlss” to “num_ols_hrd_params_minus1 + 2  !=  TotalNumOlss”? (JVET-R0204). This item no longer needed to be considered due to the action taken on item 4.a.

- Add a constraint that vps_general_hrd_params_present_flag shall be equal to 1 when more than one layer is included into any OLS? (JVET-R0275). It was commented that HRD parameters presence is optional in the single-layer case and has been optional in AVC and HEVC and their extensions and should be optional, so no action was taken on this.

Common or Combination aspects of PTL, DPB, HRD signalling:

- Constrain that each DPB, HRD, parameter structure signalled in VPS shall be associated with at least one OLS (in the VPS) that contains more than one layer and each PTL structure that is signalled is associated with at least one OLS? (JVET-R0191 Aspect 3). These are just “sensibility” constraints. AHG Recommendation (expression of existing intent): Adopt. Text was provided by Hendry and he is also to supply software.

- Define and use a common gating flag vps_dpb_hrd_params_present_flag and use this to condition presence of dpb_parameters() and ols_hrd_parameters()? (JVET-R0275 aspect 2). The proposal is motivated by a desire for a consistent approach in the VPS and SPS. However, it was commented that the circumstances in the VPS and SPS are different. This item no longer needed to be considered due to the action taken on item 6.a.

- Include PTL signalling in VPS under a common gating flag along with DPB and HRD signalling in VPS? (JVET-R0275 aspect 3). This is similar in spirit to item 11, so no action was taken on this.

Related to max_tid and number of sublayers:

- Signal the syntax elements max_tid_ref_present_flag[ i ], max_tid_il_ref_pics_plus1[ i ] only when ols_mode_idc is not equal to 1 and each_layer_is_an_ols_flag is not equal to 1? (JVET-R0107 Proposal 1). This is a “sensibility” issue – avoiding sending information that is not used. A participant questioned the aspect about ols_mode_idc, and it was discussed whether this information is intended to be metadata or only for sub-bitstream extraction. The proponent said that the syntax in the VPS is intended only to be non-metadata syntax. It was commented that the two syntax elements are already gated by a !vps_independent_layer_flag[ i ] condition, and each_layer_is_an_ols_flag can only be true if all layers are independent. No action seemed needed unless offline study indicates otherwise.

Discussion stopped here for AHG Session 1.3 on Monday 6 April at 2300 UTC, and resumed here in AHG Session 1.7 on Tuesday 7 April at 2100 UTC.]

- Assertedly simplify the condition checking for signalling ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ] to only use the flag vps_all_layers_same_num_sublayers_flag instead of using the flag vps_all_layers_same_num_sublayers_flag and vps_max_sublayers_minus1 syntax element. Also assertedly simplify the inference rules for ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ], when not present? (JVET-R0107 Proposal 2)

It was commented that this appears purely editorial – it is just removing checks that are unnecessary.

AHG Recommendation (editorial simplification): The editor is asked to confirm this and remove checks that are unnecessary.

- Change the inferred value of max_tid_il_ref_pics_plus1[] when not present from 7 to vps_max_sublayers_minus1 + 1, to avoid an asserted wrong derivation case for the value of the variable NumSubLayersInLayerInOLS? (JVET-R0119 item 1)

AHG Recommendation (cleanup): Adopt this item. Text was provided by B. Choi, and he is also to supply the software.

- Don't derive the NumSubLayersInLayerInOLS[] and layerIncludedInOlsFlag[][] values, when vps_all_independent_layers_flag is equal to 1? (JVET-R0119 item 2).

It was commented that this appears purely editorial – it is just removing an unnecessary derivation.

AHG Recommendation (editorial simplification): The editor is asked to confirm this and remove the derivation if confirmed editorially undesirable.

- Fix an asserted bug in the iteration loop in eq. (40)? (JVET-R0119 item 3)

AHG Recommendation (editorial bug fix): Correct the error.

- Signal max_tid_il_ref_pics_plus1 value separately for each direct reference layer of a layer, i.e. max_tid_il_ref_pics_plus1[ i ][ j ] for each direct reference layer j less than i, instead of single  max_tid_il_ref_pics_plus1[ i ] as currently? (JVET-R0193)

It was commented that HEVC has a two-dimensional array similar to what is proposed. With the one-dimensional approach, in some cases there may be unnecessary sublayers present after operation of the specified extraction process. The issue is whether the maximum number of sublayers used for interlayer prediction could be different for different layers.

It was commented that if some kind of hypothetical extra metadata is available (e.g. in a system environment or some SEI message), it could provide a more highly optimized extraction capability.

It was commented that the one-dimensional approach was chosen at the previous meeting (see the notes for Q0398), with an understanding that it involved some loss of generality, although there had not been much careful consideration of the question at the time. The amount of complication needed for supporting the greater generality did not seem substantial.

In HEVC, the generality is present in the syntax, and this functionality is used for reference picture list construction but it is not used in the extraction process.

It was commented that the HRD parameters in the bitstream are for the “thin” bitstream – i.e., the bitstream from which all pictures not needed for an OLS have been removed.

AHG Recommendation (cleanup): Adopt. Text was provided in the contribution, and the authors are to supply the software.

- Signal a flag in VPS to indicate that all dependent layers share the same value of max_tid_il_ref_pics_plus1. If the flag is set, signal a common vps_max_tid_il_ref_pics_plus1 for all layers. Otherwise conditionally signal separate values for max_tid_il_ref_pics_plus1[ i ]? (JVET-R0261/ aspect 2)

This is a proposed signalling shortcut in the VPS. We have shortcuts for “vps_all_layers_same_num_sublayers_flag” and “vps_all_independent_layers_flag”. This proposes an additional shortcut “vps_all_layers_same_tid_il_flag” to save repetition of values of max_tid_ref_present_flag[ i ] and max_tid_il_ref_pics_plus1[ i ]. With the adoption of JVET-R0193, this would save some more max_tid_il_ref_pics_plus1 values, since that becomes two-dimensional.

It was commented that we should have conformance bitstreams to test the shortcuts.

The proponent said this does address a common case. Others thought this was unnecessary complication, and the most common case would not use this part of the syntax at all. No action was taken on this.

- Fix an asserted bug for semantics of max_tid_il_ref_pics_plus1[ i ] for special value 0? (JVET-R0107 proposal 3, JVET-R0296 aspect 1)? AHG Recommendation (editorial text bug): Adopt.

Additionally define the semantics for special value 0 to include GDR pictures with recovery_poc_cnt equal to 0 (JVET-R0107 Proposal 3)? It was said that such a GDR picture is functionally equivalent to an IRAP picture. Another participant commented that RPL constraints are different for such a GDR picture, and there was discussion of whether difference is appropriate or not. AHG Recommendation (editorial text bug): Adopt (assuming we don’t disallow GDR pictures with recovery_poc_cnt equal to 0).

- Modify the sub-bitstream extraction process to account for GDR pictures with recovery_poc_cnt equal to 0? (JVET-R0107 Proposal 3) AHG Recommendation (editorial text bug): Adopt (assuming we don’t disallow GDR pictures with recovery_poc_cnt equal to 0).

- Fix an asserted bug in the derivation of NumSubLayersInLayerInOLS by separating the cases for each_layer_is_an_ols_flag is equal to 1 and ols_mode_idc is equal to 0? (JVET-R0296 aspect2). AHG Recommendation (bug fix): Adopt.

Related to Output layer sets and layer dependency:

- Re-define 0-th OLS to include all independent layers when present and every included layer is output? (JVET-R0261 aspect 3). Currently, the 0-th OLS is conceptually a base layer, and there did not seem to be a strong need to change that, so no action was recommended on this by the AHG.

Discussion stopped here in AHG Session 1.7 on Tuesday 7 April at 2300 UTC.

- Keep the design that the 0-th OLS contains only the lowest layer when each_layer_is_an_ols_flag is equal to 1, the output layer set mode is equal to 0 or the output layer set mode is equal to 1, but relax this when output layer set mode equal to 2 and if so modify the loop and derivation? (JVET-R0306)

- Change vps_all_independent_layers_flag to 2-bit vps_layer_dependency_idc to indicate common layer dependency to align with VPS OLS mode signaling (0 means all layers independently coded, 1 means all non-base layers use ILP, with immediate lower layer as direct reference layer, 2 means general referencing, 3 is reserved)? (JVET-R0261 aspect 1)

- Add a constraint that for each independent layer (i.e., vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1), there shall be an OLS that contains that layer only? (JVET-R0191 item 2).

Other VPS clean-ups:

- Change the coding of ols_ptl_idx[ i ] from u(8)? (JVET-R0161 proposal 1)

  - Option 1: Change to u(v) with length equal to Ceil(Log2(vps_num_ptls_minus1+1))

  - Option 2: Change to ue(v)

- Change the coding of num_output_layer_sets_minus1 from u(8) to u(v) with length eqaul to min( 8, vps_max_layers_minus1 + 1 ) (JVET-R0161 proposal 3)

- Infer vps_layer_id [0] to be equal to nuh_layer_id of the first VCL NAL unit in a bitstream when vps_layer_id [0] is not signaled? (JVET-R0158 aspect 1)

- When VPS is not present:

  - Directly require sps_max_sublayers_minus1 to be in the range of 0 to 6, inclusive? (JVET-R0158 aspect 2)

  - Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). (JVET-R0222 aspect 1)

  - If DCI is present infer vps_max_sublayers_minus1 to be dci_max_sublayers_minus1 or 6 otherwise. (JVET-R0199 aspect 2)

- Constrain the maximum value of vps_max_sublayers_minus1 to be less than or equal to dci_max_sublayers_minus1? (JVET-R0199 aspect 1)

JVET-R0099 AHG8/AHG9: On Output Layer Sets Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0107 AHG8/AHG9: On Temporal Sublayers Information [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0119 AHG8/AHG9: On derivation of sublayer number in output layer set [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0158 AHG9: Semantic bug fixes for syntax elements in VPS and SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-R0161 AHG8/AHG9: On VPS syntax signalling [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

JVET-R0185 AHG9: On syntax elements signalling in VPS [S. Paluri, Hendry, S. Kim (LGE)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Items 2, 3 of this contribution belong to this category.

JVET-R0193 AHG8/AHG9: On signalling of syntax element max_tid_il_ref_pics_plus1 [Hendry, S. Paluri, S. Kim (LGE)]

JVET-R0195 AHG8/AHG9: On HRD structure and OLS mapping signalling in VPS [Hendry (LGE)]

JVET-R0196 AHG8: On signalling of DPB parameters in the VPS [T. Nishi, K. Abe, V. Drugeon (Panasonic)]

JVET-R0199 AHG9: On vps_max_sublayers_minus1 [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak(WILUS)]

JVET-R0204 AHG8: On inference of index for PTL/DPB/HRD parameters in the VPS [T. Nishi, K. Abe, V. Drugeon (Panasonic)]

JVET-R0261 AHG9: On VPS syntax [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0275 AHG8: On PTL, HRD, and DPB structures signalling in VPS and SPS [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

JVET-R0296 AHG9: On sublayer references [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-R0306 AHG8/AHG9: On the 0-th OLS for multi-layer bitstream [E. Thomas (TNO)]

### Reference picture resampling (RPR) specific HLS (2)

JVET-R0217 AHG8: On signalling PH RPR scaling window offsets [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby), J. Boyce (Intel), J. N. Shingala (Ittiam)]

JVET-R0382 Crosscheck of JVET-R0217: AHG8: On signalling PH RPR scaling window offsets [J. Luo (Alibaba)] [late]

JVET-R0114 AHG9: On scaling window offsets [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Initially discussed in AHG Session 1.12 on 8 April at 2340 UTC in PPS syntax discussion (GJS & YKW).

It is proposed to allow signalling of negative scaling window offsets so that negative vertical and horizontal offsets can be derived even when the referenced picture did not include a scaling window (JVET-R0114).

It was asserted that this would improve support of zooming. An example use of this was illustrated in the contribution. The change to the text would be just changing ue(v) to se(v) and having a different value range.

It was asked whether this would increase the bit width needed for reference picture referencing. The proponent indicated that this should not be an issue.

It was commented that R0217 is related to the use cases for this.

Revisit with R0217.

# Complexity analysis (0)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

# Encoder optimization (6)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

JVET-R0110 AHG14: Mixed lossy/lossless coding of VTM reference software [M. G. Sarwer, Y. Ye, J. Luo (Alibaba)]

JVET-R0428 Crosscheck of JVET-R0110 (AHG14: Mixed lossy/lossless coding of VTM reference software) [T.-C. Ma (Kwai Inc.)] [late]

JVET-R0140 AHG14: Max BT/TT size restriction for lossless coding encoder configuration [T. Zhou, E. Sasaki, T. Ikai (Sharp)]

JVET-R0143 AHG14: Configuration parameter to enable TSRC for lossless coding [C. Hollmann, M. Damghanian, L. Litwic, M. von Strauss (Ericsson)]

JVET-R0164 AHG10: Mean-scaled SATD for VTM encoder [J. Lainema, A. Hallapuro (Nokia)]

JVET-R0453 Crosscheck of R0164 (AHG10: Mean-scaled SATD for VTM encoder) [J. Enhorn, R. Sjöberg (Ericsson)] [late]

JVET-R0327 CCALF encoder optimization X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-R0328 ALF encoder optimization [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

# Metrics and evaluation criteria (0)

Contributions in this category were discussed XXday X Apr. XXXX–XXXX in Track X (chaired by XXX).

# Withdrawn (7)

Section kept for future use.

JVET-R0075 Withdrawn

JVET-R0181 Withdrawn

JVET-R0346 Withdrawn

JVET-R0348 Withdrawn

JVET-R0377 Withdrawn

JVET-R0377 Withdrawn

JVET-R0409 Withdrawn

# Plenary meetings, joint meetings, BoG reports, and summary of actions taken

## High-level syntax / systems relation meeting

This planned session was cancelled due to a lack of identified need.

## Plenary meeting Sunday 12 Jan. 0800-1215

Reports of the tracks were presented as follows:

The status of Tracks A and B was presented and discussed, which particularly included the following aspects:

Track A:

Track B:

Decisions recommended from trackA and B were agreed and approved, unless otherwise noted:

Conformance testing was discussed (see section 4.6).

Profile, tier and level were discussed (see section 4.6).

## Joint meeting Tuesday 14 January 0900-1000

## Joint meeting Wednesday 15 January 1115-1215

## Plenary meeting Thursday 16 January 1530-1800

Reports of the tracks were presented as follows:

The status of Tracks A and B was presented and discussed, which particularly included the following aspects:

## Closing plenary meeting Friday 17 January 0800-

… .

## BoGs (X)

## List of actions taken affecting the draft text of VVC, the VTM, and 360Lib

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the VVC draft text, VTM or 360Lib description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part. The description given in the “Tool” column is a best effort for the sake of understanding but may not precisely reflect the functionality of the tool. It is also noted that in cases where several contributions proposed the same method, usually only one of the is listed as adoption below; refer to the meeting notes about the adoption to see which other contributions are related.

| Category | Rationale | Tool | Document |
| --- | --- | --- | --- |

# Project planning

## Core experiment planning

No CEs planned at this meeting.

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- VTM algorithm description text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XXday XX Apr 2020.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized during the opening plenary on January 9 that those rules which had been set up or refined during the 12th meeting should be observed. In particular, for some CEs, results were available late, and some changes in the experimental setup (particularly in CE4) were not discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the draft standard by the next meeting.

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure tha the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials; the method for members to obtain the credentials is TBA on the reflector).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-N1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

[Add info on software access.]

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to marked as “withdrawn”.

T2 = Test model software release + 2 weeks or X XX, whichever is earlier: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline – 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforwared combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology.

Availability of spec text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the spec will be. There must also be sufficient time to study it in detail. CE contributions without sufficiently mature draft spec text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

## Software development and anchor generation (update)

The planned timeline for software releases was established as follows:

- VTM8.0 will be released by 2020-02-17 including all adoptions necessary for CTC. VTM8.1 with non-CTC adoptions will be released 2020-03-16. Further versions of VTM may be released for additional bug fixing, as appropriate.

- Preparation of the VTM software will include immediate removal of macros that were added in the previous meeting cycle. The software coordinator has the discretion to retain some such macros.

- 360lib software is to be revised for the modified generalized cubemap, which was requested by 2019-02-28

- No change of HDRTools software was noted in response to meeting.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise CE and AHG studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. | J.-R. Ohm, G. J. Sullivan (co-chairs) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-Q2001 VVC text specification draft 8 and JVET-Q2007 SEI text draft 3. - Produce and finalize JVET-Q2002 VVC Test Model 8 (VTM 8) Algorithm and Encoder Description. - Gather and address comments for refinement of these documents. - Coordinate with test model software development AhG to address issues relating to mismatches between software and text. | B. Bross, J. Chen (co-chairs), J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test model (VTM) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Discuss and make recommendations on the software development process. - Propose improvements to the guideline document for developments of the test model software. - Perform tests of VTM behaviour relative to HEVC and the previous VTM using the VTM common test conditions. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Coordinate with AHG6 for integration with 360lib software. | F. Bossen, X. Li, K. Sühring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Produce the draft verification test plan JVET-Q2009 and develop proposed improvements for verification testing of VVC capability. - Maintain the video sequence test material database for development of the VVC standard. - Identify and recommend appropriate test materials for use in the development of the VVC standard. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Evaluate new test sequences. - Maintain and update the directory structure for the test sequence repository as necessary. - Prepare availability of viewing equipment and facilities arrangements for the next meeting, and prepare testing upon consultation with CE coordinators. - Coordinate with AHG11 on test material for screen content coding. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), R. Chernyak, A. Norkin (vice-chairs) | N |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Produce the JVET-Q2008 draft conformance testing specification and develop proposed improvements. - Study the requirements of VVC conformance testing to ensure interoperability. - Propose a work plan, including timeline, for preparation of a conformance testing specification and conformance bitstream database. - Study potential testing methodology to fulfil the requirements of VVC conformance testing. | J. Boyce and W. Wan (co-chairs), E. Alshina, I. Moccagatta, K. Kawamura, S. McCarthy, K. Sühring, X. Xu (vice-chairs) | N |
| 360° video coding tools, software and test conditions (AHG6) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Produce and finalize JVET-Q2004, Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 10). - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Study coding tools dedicated to 360° video, their impact on compression, and implications to the core codec design, including consideration of subpicture segmentations and adaptive viewport usage. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Study complexity of GPU rendering of projection formats. - Study syntax for signalling of projection formats, cubeface layouts, spherical rotations. - Prepare and deliver the 360Lib-10 software version and common test condition configuration files according to JVET-Q1012. - Generate CTC anchors and PERP results for the VTM according to JVET-Q1012 within two weeks of availability of SDR CTC anchors. - Produce documentation of software usage for distribution with the software. | J. Boyce and Y. He (co-chairs), K. Choi, J.-L. Lin, Y. Ye (vice-chairs) | N |
| Coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and evaluate available HDR/WCG test content. - Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results. - Compare the performance of the VTM and HM for HDR/WCG content. - Generate CTC anchors for the VTM according to JVET-P2011 within two weeks of availability of SDR CTC anchors. - Prepare for expert viewing of HDR content at the next JVET meeting if feasible. - Coordinate implementation of HDR anchor aspects in the test model software with AHG3. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, W. Husak, S. Iwamura, D. Rusanovskyy (vice-chairs) | N |
| Layered coding and resolution adaptivity (AHG8) (jvet@lists.rwth-aachen.de) - Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including subpicture-based resampling, reference picture management and related scope and signalling. - Study approaches for temporal scalability to avoid temporal judder when temporal scalability sub-bitstream extraction is used for achieving lower frame rate, and consider whether this should have a normative impact. - Coordinate with AHG2 and AHG3 for text drafting and software development for the layered coding and resolution adaptivity aspects of the VVC design. - Produce, study and develop improvements of the JVET-Q2015 functionality testing condition description. - Propose common test conditions for layered coding and resolution adaptivity. - Study approaches for support of layered coding scalability including spatial, temporal, quality, view, and region-of-interest scalability; and analyse their coding efficiency and complexity characteristics | S. Wenger and A. Segall (co-chairs), M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun, P. Topiwala, M. Zhou (vice-chairs) | N |
| High-level syntax (AHG9) (jvet@lists.rwth-aachen.de) - Study NAL unit header, decoding parameter set, video parameter set, sequence parameter set, picture parameter set, adaptation parameter set, picture header, and slice header syntax designs. - Study reference picture buffering and list construction. - Study random access signalling and random access approaches. - Study detection of AU and picture boundaries and properties. - Study the appropriate syntax level and signalling approaches for high-level signalling of control information for lower-level coding tools. - Coordinate with AHG2 and AHG3 for text drafting and software development for the high-level syntax in the VVC design. - Study syntax approaches for interoperability point signalling. - Study selection of constraint flags and their impact on syntax, semantics, and decoding process. | R. Sjöberg, J. Boyce (co-chairs), B. Choi, S. Deshpande, M. M. Hannuksela, R. Skupin, A. Tourapis, Y.-K. Wang, W. Wan P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization. - Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores. - Study the impact of adaptive quantization on individual tools in the test model. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and their impact on performance, subjective and objective quality. | A. Duenas, A. Tourapis (co-chairs), S. Ikonin, A. Norkin, R. Sjöberg, J. Le Tanou, J.-M. Thiesse (vice-chairs) | N |
| Screen content coding (AHG11) (jvet@lists.rwth-aachen.de) - Investigate coding tools targeted at screen content in terms of compression benefit and implementation complexity. - Identify test materials, discuss testing conditions for screen content coding, and propose associated updated common test conditions. - Study the impact of loop filters on screen content coding. | S. Liu (chair), J. Boyce, A. Filippov, Y.-C. Sun, J. Xu (vice-chairs) | N |
| High-level parallelism and coded picture regions (AHG12) (jvet@lists.rwth-aachen.de) - Study wavefront processing including the relationship with tiles and low delay characteristics. - Study flexible loop filter control and tile size restrictions, including identifying implications on coding tools and implementation. - Study support of independently coded picture regions, including easy extraction and merging of such regions into conforming bitstreams. - Coordinate with AHG2 and AHG3 for text drafting and software development for the high-level parallelism and coded picture regions aspects of the VVC design. - Study the coding efficiency impact of parallel processing and coded picture regions. | S. Deshpande (chair), B. Choi, M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, B. Wang, Y.-K. Wang (vice-chairs) | N |
| Tool reporting procedure and testing (AHG13) (jvet@lists.rwth-aachen.de) - Prepare output document JVET-Q2005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers, including non-CTC configurations as appropriate. - Produce, study and develop improvements of the JVET-Q2013 testing condition description for non-4:2:0 colour format coding. - Provide configurations files, bitstreams, and results of tool-on/tool-off testing. - Develop and collect test results for additional testing of VVC capabilities. - Maintain VTM software aspects for memory bandwidth analysis in coordination with AHG3. - Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing. - Prepare a report with results of the tests. | W.-J. Chien, J. Boyce (co-chairs), Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu (vice-chairs) | N |
| Lossless and near-lossless coding (AHG14) (jvet@lists.rwth-aachen.de) - Produce, study and develop improvements of the JVET-Q2014 testing condition description. - Study lossless and near-lossless coding, including transform skip, BDPCM, and other potential technologies. - Consider the interaction between coding tools and other processing such as loop filtering and LMCS for lossless and near-lossless coding. - Consider throughput bottlenecks for lossless and near-lossless coding at high resolutions and frame rates. | T. Nguyen and T.-C. Ma (co-chairs), M. Ikeda, H. Jang, X. Zhao (vice-chairs) | N |
| Quantization control (AHG15) (jvet@lists.rwth-aachen.de) - Identify methods for quantization step size control for luma and chroma, including spatially-adaptive and frequency-adaptive approaches. - Develop methods for evaluating quantization step size control operation. - Study the association between transforms and quantization scaling matrices. - Develop testing conditions for evaluating QP signalling improvements including rate control and perceptual optimization strategies as appropriate. - Evaluate the performance of the current VVC QP design using the adaptive quantization control techniques currently available in the VTM. | R. Chernyak (chair), E. François, C. Helmrich, S. McCarthy, A. Segall (vice-chairs) | N |
| Implementation studies (AHG16) (jvet@lists.rwth-aachen.de) - Study current and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty. - Solicit hardware analysis of complex tools. - Provide feedback on potential solutions to address identified issues. | M. Zhou (chair), J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu (vice-chairs) | N |
| Film Grain Synthesis (AHG17) (jvet@lists.rwth-aachen.de) - Study the proposed and existing (as in HEVC) film grain synthesis methods in the context of VVC. - Provide evidence for the efficacy of film grain synthesis technology in the context of VVC. - Develop proposed text (syntax, semantics, and process description) for film grain synthesis technology. - Study methodologies for subjective evaluation of film grain synthesis technology. - Develop software that includes parsing of film grain synthesis control syntax, application of synthesized film grain to reconstructed video, and (if feasible) encoder-side film grain analysis and grain removal filtering. | A. Norkin, A. Tourapis (co-chairs), D. Grois, P. de Lagrange, X. Li, S. McCarthy, R. Sjöberg (vice-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

JVET-Q2000 Meeting Report of the 17th JVET Meeting [G. J. Sullivan, J.-R. Ohm] (2020-04-07, near next meeting)

Initial versions of the meeting notes (d0 … dB) were made available on a daily basis during the meeting.

JVET-Q2001 Versatile Video Coding (Draft 8) [B. Bross, J. Chen, S. Liu, Y.-K. Wang] [WG 11 N19117] (2020-02-28)

(Initial version planned to be made available by 2020-01-24.)

See the list of elements under section 11.7, [revisit to check].

JVET-Q2002 Algorithm description for Versatile Video Coding and Test Model 8 (VTM 8) [J. Chen, Y. Ye, S. Kim] [WG 11 N 19118] (2019-04-03)

(Initial version planned to be made available by 2020-03-06.)

Remains valid – not updated: JVET-N1003 Guidelines for VVC reference software development [K. Sühring] (2019-04-01)

JVET-Q2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 10) [Y. Ye, J. Boyce] (2020-02-28)

This includes updates for the generalized cubemap projection format

JVET-Q2005 Methodology and reporting template for coding tool testing [W.-J. Chien and J. Boyce] (2020-03-02)

Initial version to be available by 2020-02-17; final version expected by two weeks after VTM 8 availability.

Remains valid – not updated: JVET-M1006 Methodology and reporting template for neural network coding tool testing [Y. Li, S. Liu, K. Kawamura] (2019-02-01)

This output was produced to capture aspects specific to enable study of neural network techniques.

JVET-Q2007 Supplemental enhancement information messages for coded video bitstreams (Draft 3) [J. Boyce, V. Drugeon, G. J. Sullivan, Y.-K. Wang] [WG 11 N19119] (2020-02-28)

(Resolution impact: Adding V. Drugeon as editor)

See the list of elements under section 11.8 [revisit to check].

JVET-Q2008 Conformance testing for versatile video coding (Draft 2) [J. Boyce, E. Alshina, K. Kawamura, S. McCarthy, I. Moccagatta, W. Wan] [WG 11 N18927] (2020-03-20)

JVET-Q2009 Preliminary plan for VVC verification testing (Draft 1) [M. Wien, V. Baroncini] [WG 11 N19155] (2020-02-14)

(The final testing should use naïve viewers.)

Work was done for selection of QP values with approximately comparable quality for various sequences.

Expert subjective viewing was conducted at the current meeting for the 6 UHD CTC sequences for various candidate QP values, to get an understanding of what should be selected for formal testing.

First there was some testing with all of these test sequences, then some testing with higher QP values. The second round of testing did not use the CampFire sequences, since it had significant artefacts even with the smaller QP values.

[add notes of such results, and include these in an annex to the plan document]

It was commented that it might be harder to determine equal quality when both cases have rather poor quality and the comparison becomes somewhat a selection between preferred type of artefacts. This could also, perhaps, cause an increase in the size of confidence intervals.

Possibly, the VT could be selected with quality at particular rate points rather than / instead of particular QP points.

It was commented that it likely that the VT would involve per-sequence customization of QP values.

The output document should describe the desire to select operating points, and the initial experiment results can be an annex to the document.

There was also a bit of experimenting with the new 8K material offered by HHI, both with downsampling and with cropping. Some had a lot of texture.

There were comments about sequences that had mixed content characteristics, e.g., areas of sky and water.

The fact that the encoder uses constant QP was discussed. Real encoders might have special tricks in them for particular characteristics. It is most valid to compare encoders that use similar types of configuration and optimization, and perhaps similar coding architectures.

Remains valid – not updated: JVET-N1010 JVET common test conditions and software reference configurations for SDR video [F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring] (2019-04-12)

Remains valid – not updated: JVET-P2011 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy] (2019-07-31)

Remains valid – not updated: JVET-L1012 JVET common test conditions and evaluation procedures for 360° video [P. Hanhart, J. Boyce, K. Choi, J.-L. Lin] (2018-10-26)

JVET-Q2013 JVET common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu] (2020-03-02)

JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen] (2020-03-02)

JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin] (2020-03-02)

JVET-Q2016 Summary information on BD-rate experiment evaluation practices [K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan] [WG 11 N19168] (2020-01-17)

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (ordinarily starting meetings on the Tuesday of the first week and closing it on the Wednesday of the second week of the SG 16 meeting – a total of 9 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (ordinarily starting meetings on the Wednesday prior to such meetings and closing it at lunchtime on the last day of the WG 11 meeting – a total of 9.5 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Tue. 23 June – Wed. 1 July 2020, 19th meeting under ITU-T auspices in Geneva, CH.

- Wed. 7 – Fri. 16 October 2020, 20th meeting under WG 11 auspices in Rennes, FR.

- Wed. 6 – Fri. 15 January 2021, 21st meeting under WG 11 auspices in Capetown, ZA.

- Tue. 20 – Wed. 28 April 2021, 22nd meeting under ITU-T auspices in Geneva, CH.

The agreed document deadline for the 19th JVET meeting was planned to be XXday X June 2020. Only HLS topics will be considered on the first XX days.

University of Brussels (ULB) was thanked for the excellent hosting of the 17th meeting of the JVET, and particularly for accommodating evening meeting hours, especially thanking Prof. Gauthier Lafruit for his efforts.

HHI was thanked for offering new 8K video test sequences that could be used in experiments and testing of video coding technology for standardization.

Philips, Sharp Labs of America, and ULB were thanked for providing equipment used for subjective viewing during the 17th JVET meeting. Kenneth Andersson, Vittorio Baroncini, Andrey Norkin, Andrew Segall, and Mathias Wien were thanked for preparing and conducting expert subjective viewing during the meeting. The experts who participated in the role of test subjects were also thanked.

The 18h JVET meeting was closed at approximately XXXX hours UTC on Friday 24 April 2020.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the eighteenth meeting of the JVET, according to an attendance sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: