| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0002-v1 |
| --- | --- |

| Title: | JVET AHG report: Draft text and test model algorithm description editing (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jianle Chen Jill Boyce Seung-Hwan Kim Shan Liu Ye-Kui Wang Yan Ye | Email: | benjamin.bross@hhi.fraunhofer.de cjianle@qti.qualcomm.com jill.boyce@intel.com seunghwan3.kim@lge.com shanl@tencent.com yekui.wang@bytedance.com yan.ye@alibaba-inc.com |
| Source: | AHG2 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on draft text and test model algorithm description editing (AHG2) between the 17th meeting in Brussels, BE, (7–17 January 2020) and the 18th meeting by teleconference, (15–24 April 2020).

A joint ad hoc group meeting of AHGs 2, 3, 11, 14, 15 and 16 was held by teleconference on April 9 and April 14, involving 8 sessions of two hours each.  The report from the joint AHG meeting sessions can be found in JVET-R0340.

# Mandates

At the 17th JVET meeting, AHG2 on draft text and test model algorithm description editing was established with the following mandates:

- Produce and finalize JVET-Q2001 VVC text specification draft 8.

- Produce and finalize JVET-Q2002 VVC Test Model 8 (VTM 8) Algorithm and Encoder Description.

- Gather and address comments for refinement of these documents.

- Coordinate with test model software development AhG to address issues relating to mismatches between software and text.

# Background

At the 17th JVET meeting, it was decided to include additional coding features as well as simplify, clean up and fix the existing features for intra picture-prediction, inter-picture prediction, transform, CABAC engine and in-loop filter in the eighth draft of Versatile Video Coding (VVC D8) and the VVC Test Model 8 (VTM8) encoding.

The normative decoding process for Versatile Video Coding is specified in the VVC draft 8 text specification document [1]. The VVC Test Model 8 (VTM 8) Algorithm and Encoder Description document provides an algorithm description as well as an encoder-side description of the VVC Test Model 8, which serves as a tutorial for the algorithm and encoding model implemented in the VTM8.x software [2].

An issue tracker (https://jvet.hhi.fraunhofer.de/trac/vvc) was used to facilitate the reporting of errata with the VVC documents.

# JVET-Q2001 VVC text specification (Draft 8)

Fourteen versions of JVET-Q2001 were published by the Editing AHG between the 17th meeting in Brussels, BE, (7–17 January 2020) and the 18th meeting by teleconference, (15–24 April 2020).

JVET-Q2001 has been established based on JVET-P2001 and now contains the following:

- Fixed the bugs #672, #700, #711, #715, #716, #720, #729, #737, #739.

- Fixed subpicture figure together with accompanying text (#740).

- Removed the editing note comment in the semantics of tile_idx_delta[ i ], which was concluded not needed after a study (input: Hendry).

- Fixed quite some format and style consistency issues in syntax tables, equations, etc. as well as some bugs in those places.

- Made some fixes in the semantics of DCI and VPS.

- Fixed the semantics of sps_loop_filter_across_virtual_boundaries_disabled_present_flag and ph_loop_filter_across_virtual_boundaries_disabled_present_flag, and changed the syntax element names to sps_virtual_boundaries_present_flag and ph_virtual_boundaries_present_flag.

- Fixed the following issues:

  - The definition of the sub-bitstream extraction process still referred to a target LayerId: "A specified process by which NAL units in a bitstream that do not belong to a target set, determined by a target OLS index and a target highest TemporalId and a target LayerId, are removed from the bitstream, …". However, the process in clause C.6 only mentions a target OLS index and a target highest TemporalId in the inputs, but no target LayerId.

  - In the VPS syntax: "ols_hrd_parameters( firstSubLayer, hrd_max_temporal_id[ i ] )". hrd_max_temporal_id is not defined anywhere. It should be "hrd_max_tid" instead.

  - In the VPS semantics, when deriving LayerUsedAsRefLayerFlag, it was initialised as follows: "LayerUsedAsRefLayerFlag[ j ] = 0." But j is not defined at this point, since it gets defined in the following loop.

  - Fixed a bug in Equation 40 by changing "layerIncludedFlag[ i ][ j ] = 1" to "layerIncludedFlag[ i ][ k ] = 1" and then changed the variable name to layerIncludedInOlsFlag[ ][ ] to be more intuitive.

  - In the semantics of dpb_size_only_flag, only the value 1 was specified, twice, and in a contradicting way. The second sentence should be: "dpb_size_only_flag[ i ] equal to 0 specifies that…".

  - In the semantics of num_ols_hrd_params_minus1: "When TotalNumOlss is greater than 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0". However, num_ols_hrd_params_minus1 is signalled when TotalNumOlss is greater than 1. It should be "When TotalNumOlss is equal to 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0".

  - In the semantics of hrd_max_tid, there should be an inference for this syntax element when the value of vps_max_sublayers_minus1 is greater than 0, but vps_all_layers_same_num_sublayers_flag is 1, similarly as in the semantics of ptl_max_temporal_id[ i ] and dpb_max_temporal_id[ i ].

  - In the semantics of sps_ptl_dpb_hrd_params_present_flag: "The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ nuh_layer_id ]." Everywhere else vps_independent_layer_flag is indexed by the layer index rather than the layer identifier. It should be the following instead: "The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ]."

  - Annex A still mentioned bit_depth_luma_minus8 and bit_depth_chroma_minus8, although these two syntax elements have been replaced by bit_depth_minus8.

  - Annex A still mentioned num_tile_columns_minus1 and num_tile_rows_minus1, although these syntax elements do not exist anymore.

  - The list NestingLayerId[ ] and the variable NestingNumLayers specified in the semantics of the scalable nesting SEI message are not used in other places, thus two variable names were changed to be local variables, i.e., nestingLayerId[ ] and nestingNumLayers.

- Fixed the bugs #600, #713 and some typos.

- Fixed the bugs #604, #613, #710, #717, #723 and some typos.

- Fixed the bugs #742, #751 and some other editorial bugs.

- Changed numrous occurences of recPictureL[ hx, vy ] etc. to recPicture[ hx ][ vy ] etc. (removal of 'L' and to be in the form of two-dimention array; input: Yang Wang).

- Fixed the bugs #765, #709, #306, #567, #791.

- Incorporated JVET-Q0819 on PH and SH changes, including text for the following adoption items:

  - JVET-Q0482: Remove the mechanism for signalling of constant SH parameter values.

  - JVET-Q0259 aspect 5 (indicate information about collocated picture in picture header when RPL information is signalled in the PH)

  - JVET-Q0781: A two-flag approach. One flag that indicates the presence of intra parameters and one that indicates the presence of inter parameters. When the first flag is 0, no inter slices can be present. When the second flag is 0, no intra slices can be present. If the first flag is 0, the second flag is not present and inferred to be equal to 1. When the first flag is 0, the SH would not contain a slice_type.

  - JVET-Q0414 Option 2: constrain the value of gdr_pic_flag based on gdr_enabled_flag.

  - JVET-Q0154 option 1: irap_or_gdr_pic_flag in the beginning of PH and use it to condition the presence of gdr_pic_flag in the PH. The semantics should be a “one way” indication, not a guarantee that the picture is not an IRAP or GDR.

  - JVET-Q0115: Move POC LSB signalling from SH to PH.

  - JVET-Q0155 aspect 1: Move colour_plane_id to SH.

  - JVET-Q0775: A mechnism to allow picture header in slice header. Adopt. Additionally, it was agreed to condition the no_output_of_prior_pics_flag on the indication that the picture is a GDR or IRAP picture and to put ph_pic_parameter_set_id before ph_pic_order_cnt_lsb, which would be followed by recovery_poc_cnt. If there is something that can be sent either in the PH NAL unit or in the SH, it shall be in the PH structure when the PH is in the SH. It was agreed that we will keep the PH syntax structure the same in the PH NAL unit and the SH.

  - JVET-Q0200: The flags that specify whether a syntax element of a related coding tool is present in PH or in SH (but not both) are moved from the PH to the PPS. This includes reference picture lists, SAO, ALF, and deblocking.

  - JVET-Q0247: Make the prediction weight table a fifth type of data that can be signalled either in the PH or SH (like ALF, deblocking, RPL, and SAO).

  - JVET-Q0270: Add a PPS flag to determine whether qp delta is sent in the PH or SH, like other things (e.g., ALF, deblocking, SAO).

  - JVET-Q0358: Add a TemporalId constraint between the ALF_APS NAL unit and the picture associated with PH.

- Made various fixes, including removal of unnessary brackets and merging of some syntax conditions.

- Incorporated JVET-Q0500 on CCLM predictors derivation cleanup.

- Incorporated JVET-Q0194 on CTU row boundary location fix in CCLM.

- Incorporated JVET-Q0275 on bug fixes for CCLM filtering.

- Fixed the bug #606 in CCLM derivation process for neighbouring block availability.

- Incorporated JVET-Q0468: Limit the range of min CU size in SPS to 4,8,16,32,64. This is an editorial cleanup of semantics. The subsequent clipping operation min (64, ctusize) shall be kept.

- Incorporated JVET-Q0471: When chroma sampling is 4:2:2, the QT split limit of chroma block would be rectangular (larger in height than in width). The proposed solution is to scale the limit check by multiplying the subsampling ratio factor.

- Incorporated JVET-Q0330: Bug fix in implicit split at a picture boundary.

- Incorporated JVET-Q0267: Resetting CuQpOffsets to 0 at the start of each QG.

- Incorporated JVET-Q0505: Quantization matrix signalling for 4:0:0.

- Incorporated JVET-Q0089: TSRC slice-level switch and BDPCM for chroma in 4:2:0.

- Incorporated JVET-Q0368: Bug fix on MVP and refIdx context models.

- Incorporated JVET-Q0209: Align chroma QP offset between palette and TU signalling.

- Incorporated JVET-Q0293: Removal of chroma N×2 blocks in PDPC.

- Incorporated JVET-Q0446: MIP with constant shifts and offsets.

- Incorporated JVET-Q0371: Bug fix in MIP intra reference sample generation.

- Fixed minor issues and the bugs reported in #687, #761, #762, #797.

- Incorporated JVET-Q0044: Align the picture-level slice index and the subpicture-level slice index to be in the same order which also corresponds to the decoding order.

- Incorporated JVET-Q0113: Editorial changes regarding the specification of NAL unit decoding order.

- Incorporated JVET-Q0119: Modifications to the coding and semantics of sps_num_subpics_minus1.

- Incorporated JVET-Q0169: The maximum value of subpicture ID derived by the signalled subpicture ID length shall be greater than or equal to the number of subpicture IDs, and for each signalled subpicture ID, it shall be different from all other subpicture IDs in the same picture.

- Incorporated JVET-Q0271: Add sps_independent_subpic_flag.

- Incorporated JVET-Q0816: Omit the signalling of subpicture layout when only one subpicture is present in the picture.

- Incorporated JVET-Q0817: Remove constraint on single_slice_per_subpic_flag when subpic_info_present_flag is 0.

- Incorporated JVET-Q0222: Skip the signalling of the top-left position of the first subpicture and the size of the last subpicture.

- Incorporated JVET-Q0787: Condition the presence of the syntax elements for signalling of subpictures on their length being non-zero.

- Incorporated JVET-Q0157: Replace the current picture size with the reference picture size in the clipping (motion padding).

- Incorporated JVET-Q0119: Condition subpic_id_mapping_explicitly_signalled_flag on "if( subpic_info_present_flag )" and infer the value of subpic_id_mapping_explicitly_signalled_flag to be equal to 0 when it is not present.

- Incorporated JVET-Q0119: Remove signalling of the subpicture ID mapping from PH.

- Incorporated JVET-Q0119: Require signalling in the SPS or PPS when subpic_id_mapping_explicitly_signalled_flag is equal to 1.

- Incorporated JVET-Q0119: When subpic_info_present_flag is equal to 1, the subpicture ID length is explicitly signalled in the SPS regardless of whether subpicture IDs are explicitly signalled, and the length of slice_subpic_id is always specified by that length.

- Incorporated JVET-Q0157: Add the following constraint: "When the current picture is not the first picture of the CLVS, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, if the value of SubpicId[ i ] is not equal to the value of SubpicId[ i ] of previous picture in decoding order in the same layer, the nal_unit_type for all coded slice NAL units of the subpicture with subpicture index i shall be in the range of IDR_W_RADL to CRA_NUT, inclusive."

- Incorporated JVET-Q0246: Add a flag in the SPS for Virtual Boundary enable flag. When the flag is equal to 1, VB signalling may be present in either SPS or PH, but not in both.

- Incorporated JVET-Q0210: Constrain that when subpicture signalling is present, the signalling of virtual boundary position, if present, shall be in SPS.

- Incorporated JVET-Q0417: Constrain that when RPR is enabled, the signalling of virtual boundary position, if present, shall be in PH.

- Incorporated JVET-Q0359: Provide value range for syntax elements tile_column_width_minus1 and tile_column_width_minus1.

- Incorporated JVET-Q0218: Condition the presence of tile_idx_delta_present_flag on the value of num_slices_in_pic_minus1.

- Incorporated JVET-Q0218: Add the default value for single_slice_per_subpic_flag when it is not present.

- Incorporated JVET-Q0244: Signal rectangular slice width in tile only when number of tile column is greater than 1; likewise, signal rectangular slice height only when the number of tile row is greater than 1.

- Incorporated JVET-Q0480: Only specify slice_height_in_tiles_minus1 syntax element at the start of each rectangular slice row.

- Incorporated JVET-Q0289: Editorial update to clarify the semantics of tile_idx_delta[ i ].

- Incorporated JVET-Q0289: If NumTilesInPic is equal to 1, don’t signal the rect_slice_flag and infer its value to be 1.

- Incorporated JVET-Q0203: Only signal explicit number of slice and explicit slice height for slices that are within a tile.

- Incorporated JVET-Q0244: Condition the signalling of syntax elements for rectangular slices that are less than one tile only when tile row height is greater than 1.

- Incorporated JVET-Q0119: Condition the presence of the slice_address on NumSlicesInSubpic[ CurrSubpicIdx ] being greater than 1 and update the semantics, including the inference of the value of slice_address accordingly.

- Incorporated JVET-Q0055: Zero-out dependent MTS index signalling.

- Incorporated JVET-Q0516: MTS signalling based on last significant coefficient position.

- Incorporated JVET-Q0784: LFNST signalling, latency reduction and scaling process.

- Incorporated JVET-Q0183: High-level syntax related to transform skip mode.

- Incorporated JVET-Q0820: ACT common text for bug fixes and transform change.

- Fixed the bugs #601, #685, #702, #725, #768, #769, #789, #805, #810.

- Incorporated JVET-Q0435: Bug fix in palette coding syntax structure.

- Incorporated JVET-Q0629: Palette mode excluding small (4×4) blocks.

- Incorporated JVET-Q0504: Palette mode for non-4:4:4 colour format in 4:4:4 profile.

- Incorporated JVET-Q0501: Palette predictor initialization in WPP.

- Incorporated JVET-Q0491: Palette escape binarization.

- Incorporated JVET-Q0291: Reduce palette size for dual tree.

- Incorporated JVET-Q0210: Remove uek(v).

- Incorporated JVET-Q0147: Conditional signalling of sps_joint_cbcr_enabled_flag.

- Incorporated JVET-Q0265: Conditional signalling of sps_act_enabled_flag.

- Incorporated JVET-Q0152: Move sps_seq_parameter_set_id to beginning of the SPS.

- Incorporated JVET-Q0420: Conditional signalling of chroma QP-related elements.

- Incorporated JVET-Q0043: Disallow for both subpic_info_present_flag and res_change_in_clvs_allowed_flag to be equal to 1.

- Incorporated JVET-Q0764: Ref wraparound signalling modifications.

- Incorporated JVET-Q0117: Parmeter sets cleanups.

- Incorporated JVET-Q0280: SPS constraint on VPS id.

- Moved mixed_nalu_types_in_pic_flag location in the PPS.

- Incorporated JVET-Q0399: Inference of scaling window parameters.

- Incorporated JVET-Q0416: Wraparound offset range.

- Move filler data SEI message to SEI specification.

- Incorporated JVET-Q0818: Move frame-field SEI message to SEI spec.

- Incorporated JVET-Q0172: Restrict chroma format and bit depth for inter-layer referencing.

- Incorporated JVET-Q0488: Limit of 4 repetitions of most SEI messages.

- Incorporated JVET-Q0128: Disable DMVR and BDOF when either luma WP or chroma WP is applicable to a coding unit.

- Incorporated JVET-Q0297: Merge estimation regions.

- Incorporated JVET-Q0483: Add clipping to restored MV.

- Incorporated JVET-Q0449: Disable smoothing filter in RPR.

- Incorporated JVET-Q0517: RPR down-sampling filters for affine mode.

- Incorporated JVET-Q0806: Geometric partitioning for merge mode.

- Incorporated JVET-Q0242: Cleanup for CIIP and Geo for 4:0:0 chroma format.

- Fixed the bugs #723, #802, #808, #811.

- Incorporated JVET-Q0441: SAO Modification for 12-bit.

- Incorporated JVET-Q0495: Clip ranges for NL-ALF.

- Incorporated JVET-Q0150: Fix for ALF virtual boundary processing.

- Incorporated JVET-Q0249: Clipping flag clean-up for chroma.

- Incorporated JVET-Q0795: Cross-component adaptive loop filter.

- Incorporated JVET-Q0054: Bug fix on long-tap deblocking filter.

- Incorporated cleanups and fixes including the ones reported in #698, #746, #757, #798, #809, #815, #816, #817.

- Incorporated JVET-Q0786:

  - Repeat PTL info of OLSs containing only one layer in the VPS (in addition to signalling them in the SPS).

  - Not to repeat HRD parameters info of OLSs containing only one layer in the VPS (in addition to signalling them in the SPS).

- Incorporated JVET-Q0118:

  - Require the slice_type to be 2, not only for the first picture in an AU that is an IRAP picture, but also for IRAP pictures in independent layers.

  - Specify the variable LayerUsedAsOutputLayerFlag[ i ] and use it in the constraint for expressing that there shall be no layer that is both not used for reference and not an output layer in any OLS.

- Incorporated JVET-Q0814: DPB operations based on the conceptual decoded picture size in the DPB is the largest value of max_width * max_height among all the SPSs in the OLS that is signalled in VPS.

- Incorporated JVET-Q0151/Q0205: Mandate the tile offsets signalling, and move entropy_coding_sync_enabled_flag and entry_point_offsets_present_flag from the PPS to the SPS.

- Incorporated JVET-Q0256:

  - It was agreed that the phrase “sublayer representation” should be removed from the language as proposed. Also “identified by a specific value of general_level_idc” (and similar tier language) should be removed from the description of a decoder.

  - Add the following statements and rephrase the wording of level accordingly:

    - Decoders are not required to extract a subset of the bitstream; any such extraction process that may be a part of the system is considered to be outside of the scope of the decoding process.

    - The profile of the bitstream is indicated by general_profile_idc.

    - The level of the bitstream is indicated by Min( general_level_idc, sublayer_level_idc[ Htid ].

    - The values of Htid and sublayer_level_idc[ Htid ] are not necessary for the operation of the decoding process, and if it is desirable for the decoder to check the conformance of the bitstream, these may be provided by external means.

- Incorporated JVET-Q0112:

  - The first item (using the global maximum picture size to determine the DPB size).

  - The first part of the third item (using the global maximum picture size in computing limits instead of current picture size).

  - The fourth item on using the cumulative worst-case picture size for all pictures in an AU to derive constraints on CPB removal time, etc.

- Incorporated JVET-Q0048: Mandate a bitstream order for BP/PT/DUI SEI message that obeys the parsing dependency.

- Incorporate JVET-Q0216: Not signal and infer cpb_removal_delay_delta_idx[ i ] when there is only one cpb_removal_delay_delta[ i ] in the BP SEI message.

- Incorporate JVET-Q0219: Signal the alternative times in the PT SEI message per sub-layer.

- Incorporate JVET-Q0394: Non-scalale nested SEI messages apply to bitstreamToDecode, extraction propcess handling scalable-nested SEI messages, add flag to HRD parameter syntax structure to indicate that non-scalable-nested PT SEI messages apply to all OLS in the bitstream.

- Incorporate JVET-Q0221: Add a presence flag to BP SEI message about DPB output delay in PT/DUI SEI message.

- Incorporate JVET-Q0397: Subpicture HRD conformance testing with subpicture extraction process and scalable-nesting HRD timing information.

- Incorporate JVET-Q0404: Add a CBR flag to the subpicture level info SEI message, and change the semantics and the extraction process based on the assocation of VCL and non-VCL NAL units.

- Incorporate JVET-Q0406: CABAC zero word constraint for subpictures.

- Incorporate JVET-Q0630: Resolve parsing dependency of subpicture level SEI message on SPS.

- Incorporate JVET-Q0395: Add constraints for Bitrate and number of tiles to subpicture level info SEI message.

- Incorporate JVET-Q0443: Add MinCR constraint for subpictures, and fix identified bugs in the subpicture level SEI message text, such as incorrect array indices.

- Incorporated JVET-Q0257: Impose scaling ratio constraint after scaling window offset is applied.

- Incorporated JVET-Q0179: Add scaling ratio constraint adressing peak memory bandwith concerns.

- Incorporated JVET-Q0487: aspect 1 - clarify TMVP constraint in slice_collocated_ref_idx semantics.

- Incorporated JVET-Q0487: aspect 3 - modify RefPicIsScaled derivation and rename to RprConstraintsActive to account for 1:1 scaling.

- Incorporated JVET-Q0487: aspect 4 - Signal scaling window in units of chroma samples

- Incorporated JVET-Q0262: Rename ref_pic_resampling_enabled_flag to res_change_in_clvs_allowed_flag.

- Incorporated JVET-Q0260: Add conformance window signalling to SPS.

- Incorporated JVET-Q0751: Including the following agreed aspects:

  - Allow mixing of leading with trailing NAL unit types and mixing of RASL with RADL NAL unit types.

  - Not including STSA in the list that can be mixed with IRAP.

  - Allow RASL / RADL to be mixed with STSA.

  - Require that all VCL NAL units within one subpicture have the same NAL unit type.

  - Adding a NOTE expressing that the value of sps_idr_rpl_present_flag must be equal to 1 in the mixed NAL unit types case.

- Incorporated JVET-Q0217: RPL cleanups.

- Fixed some bugs and made some other editorial changes.

- Incorporated JVET-Q0379: Move the aps_id related syntax elements in the PH and SH. For the PH, these would immediately follow the POC information. For the SH, it would immediately follow slice_type.

- Incorporated JVET-Q0121:

  - Extend abs range of beta offset and tc offset from 6 to 12, PPS and SH as proposed, but also in PH.

  - To have separate deblocking parameters for the two chroma components.

- Incorporated JVET-Q0114:

  - Remove frame_only_constraint_flag, which is redundant to general_frame_only_constraint_flag.

  - Add a general constraint on no tile partitioning (i.e., one tile only for each picture).

  - Add a general constraint on no slice partitioning (i.e., one slice only for each picture).

  - Add a general constraint on no subpicture partitioning (i.e., one subpicture only for each picture).

  - Add a general constraint on no reference picture resampling.

  - Add general_non_projected_constraint_flag, the value equal to 1 specifies that there is no omnidirectional projection SEI messages, and change the semantics of general_non_packed_constraint_flag to cover only frame packing, as frame packing and no omnidirectional projection can be independently applied.

- Incorporated JVET-Q0398: Sublayer wise dependency in multi-layer: when there is a dependent layer, there is an indication of the max_tid_il_ref_pics_plus1 that the layer depends on, and if that value is 0, inter-layer prediction uses only IRAP pictures.

- Incorporated JVET-Q0400: Extra PH and SH bits in early positions.

- Incorporated JVET-Q0197: Miscellaneous editorial fixes to texts related to RPL signalling.

- Added pt_display_elemental_periods_minus1 plus 1 to the PT SEI message, to complete the incorporation of JVET-Q0818 for moving of the frame-field SEI message to the SEI spec.

- Incorporated JVET-Q0156: Enabling inter-layer prediction for STSA pictures.

- Incorporated JVET-Q0237: Allow STSA pictures in a dependent layer to have TemporalId equal to 0.

- Incorporated JVET-Q0277: Only allow references to SPSs/PPSs/APSs that are in the current or lower layer that is in an OLS that includes the VCL NAL unit.

- Incorporated JVET-Q0346: Add slice_lmcs_enabled_flag.

- Incorporated JVET-Q0346: Enable the functionality of slice-level scaling list enabling/disabling.

- Incorporated JVET-Q0444: Make the presence of the sps_affine_amvr_enabled_flag in the SPS conditioned on the value of sps_amvr_ enabled_flag.

- Incorporated JVET-Q0481: Reorder the syntax elements in the SPS and PH to group intra syntax elements and inter syntax elements

- Incorporated JVET-Q0402: Establish the semantics of subpic_treated_as_pic_flag[ ] to allow SNR scalability with independent subpictures when subpictures are aligned.

- Incorporated JVET-Q0317: When the two sides of a boundary have different values of loop_filter_across_subpic_enabled_flag[ ], disable on both sides when either side has it disabled.

- Incorporated the following decision: Add an enable flag for dependent quantization in the SPS, constrain the SPS flag according to the general_constraint flag, and condition the presence of the PH flag on the enable flag (and there are no other flags in other places). And do basically the same thing for sign data hiding.

- Incorporated JVET-Q0798: Merge candidate syntax accounting for picture-level TMVP control. Decision (cleanup): Determine the number of subblock merge candidates at the picture level based on the SPS number (sent at the SPS level) and the SPS subblock TMVP enable flag and the PH TMVP enable flag. Text is in Q0798 (v4) option 2.

- Incorporated JVET-Q0273: Use "ph_" consistently for most PH syntax elements; use a shared syntax structure for RPL syntax in the PH and SH.

- Removed/rephrased all instances of "and that" in the text.

- Incorporated JVET-Q0164: Further clarified that each vertical slice boundary (and consequently each vertical subpicture boundary) is always a vertical tile boundary, including when single_slice_per_subpic_flag is equal to 1.

- Fixed the bugs #747, #752, #801, #823, #824, #826, #828, #830.

- Fixed the bugs #315, #777https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/747, #318, #488, #819, #825, #829, #832, #833.

- Fixed the bugs #834, #835, and #836, and made various editorial changes, including merging of Annexes D and E, and putting the syntax and semantics of each SEI message next to each other.

- Fixed some bugs, incuding on the setting of PictureOutputFlag in clause 8.1.2, the typo reported in bug ticket #841, and some other typo corrections.

- Fixed bugs #843, #848, #850, #852, #860, #861, #874.

- Made some editorial corrections and fixed bugs #878, #881, #883, #884, #888, #890, #893, #896, #898, #905, #906, and #907.

- Updated the names of the SPS, PH, and SH syntax elements on enabling/disabling/using of explicit scaling list, clarified their semanitics, and fixed their use for controlling of explicit scaling list usage.

- Updated the names of some binary-valued sytnax elements to have "_flag" at the end of their syntax element names, including cu_cbf_flag -> cu_coded_flag, tu_cbf_luma -> tu_y_coded_flag, tu_cbf_cb -> tu_cb_coded_flag, tu_cbf_cr -> tu_cr_coded_flag, coded_sub_block_flag-> sb_coded_flag, sao_offset_sign -> sao_offset_sign_flag, and intra_mip_transposed  -> intra_mip_transposed_flag.

- Changed "predMode = MODE_INTER (INTER, IBC)" to "predMode = MODE_INTER or MODE_IBC".

- Removed "It is a requirement of bitstream conformance that, when present, the next AU after an AU that contains an EOS NAL unit shall be a CVSS AU." which is a leftover that confilicts with the design intent that EOS is layer specific.

- Fixed places involving the general_hrd_parameters( ) syntax structure that contained some obsolete texts with the assumption of the ols_hrd_parameters( ) syntax structure being included within the general_hrd_parameters( ) syntax structure.

- Fixed texts on mixed nal_unit_type values for VCL NAL units within a picture to be more aligned with the agreed design intent noted in JVET meeting minutes.

- Added the condition "When sps_video_parameter_set_id is greater than 0" to the constraints that pic_width/height_max_in_luma_samples shall be less than or equal to ols_dpb_pic_width/height[ i ], because when the condition is false there is no VPS and ols_dpb_pic_width/height[ i ] don't exist.

- Fixed bugs #908, #909, #910, #914, #915, #921, and #940.

- Fixed bugs #837, #838, #839, #847, #894, #916, #931, #932, #934, and #935.

# JVET-Q2002 VVC Test Model 8 (VTM 8) Algorithm and Encoder Description.

Three versions of JVET-Q2002 were published by the Editing AHG between the 17th meeting in Brussels, BE, (7–17 January 2020) and the 18th meeting by teleconference, (15–24 April 2020).

JVET-Q2002 has been established based on JVET-P2002. It provides the algorithm description for majority of coding tools in VVC. In this editing period, the following changes were included:

- Editorial improvements of the JCCR mode

- Added encoder algorithm description for the palette mode, as well as editorial improvements of the palette mode

- Incorporated JVET-Q0291: CE2-related: On maximum palette size of VVC

- Incorporated JVET-Q0503: CE2-related: Encoder improvement for palette mode

- Incorporated JVET-Q0504: CE2-related: Palette mode for non 4:4:4 color format

- Incorporated JVET-Q0712: Non-CE2: Extension of error limit table in JVET-Q0503 to high QP

- Incorporated JVET-Q0493: Non-CE2: Palette encoder improvements for lossless coding

- Incorporated JVET-Q0695: Combined encoder improvements of JVET-Q0101/JVET-Q0408/JVET-Q0514 on JCCR with chroma transform skip

- Incorporated JVET-Q0054: CE1-1.1: Fixes for long luma deblocking filter decision

- Incorporated JVET-Q0150: Fix for ALF virtual boundary processing

- Incorporated JVET-Q0806: Geometric partitioning mode

- Incorporated JVET-Q0495: Clip ranges for NL-ALF

- Incorporated JVET-Q0297: Merge estimation regions

- Incorporated JVET-Q0330: Use QT splitting if other methods are not allowed for picture boundary implicit partitioning

- Incorporated JVET-Q0795: Cross component adaptive loop filter (CC-ALF)

- Incorporated JVET-Q0491: Palette escape binarization

- Incorporated JVET-Q0501: Palette predictor initialization in WPP

- Incorporated JVET-Q0629: Palette mode excluding small blocks

- Incorporated JVET-Q0293: Removal of chroma Nx2 blocks in PDPC

- Incorporated JVET-Q0820: ACT common text for bug fixes and transform change

- Incorporated JVET-Q0516: MTS signaling based on last significant coefficient position

- Incorporated JVET-Q0784: LFNST signalling, latency reduction and scaling process

- Incorporated JVET-Q0512: Enable TS for ACT blocks

- Incorporated JVET-Q0089: TSRC slice-level switch and BDPCM for chroma in 4:2:0

- Incorporated JVET-Q0267: Resetting CuQpOffsets to 0 at the start of each QG

# Recommendations

The AHG recommends to:

- Approve the edited JVET-Q2001 and JVET-Q2002 documents as JVET outputs,

- Continue to edit the VVC draft and Test Model documents to ensure that all agreed elements of VVC are fully described,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification draft and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC WD and Test Model documents,

- Ensure that, when considering the addition of new feature to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC Working Draft is made available in a timely manner,

- Review the remaining contributions of the joint AHG meeting (JVET-R0340) and act on them, on the revisits and on the recommendations.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” document JVET-Q2001, 17th JVET meeting: Brussels, BE, 7–17 January 2020.

- J. Chen, Y. Ye, S.-H. Kim, “Algorithm description for Versatile Video Coding and Test Model 8 (VTM 8),” document JVET-Q2002, 17th JVET meeting: Brussels, BE, 7–17 January 2020.

- J.-R. Ohm, B. Bross, A. Segall, Y. Ye, “Agenda and report of the category 2 AHG pre-meeting of the 18th JVET meeting,” document JVET-R0340, 18th JVET meeting: by teleconference, 15–24 April 2020.