| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0007-v1 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. Francois W. Husak S. Iwamura D. Rusanovskyy | Email: | asegall@sharplbas.com Edouard.Francois@InterDigital.com WJH@dolby.com iwamura.s-gc@nhk.or.jp> dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 17th meeting in Brussels, BE (7 – 17 January 2020) and the 18th meeting by teleconference (14 – 24 April 2020).

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Generate CTC anchors for the VTM according to JVET-P2011 within two weeks of availability of SDR CTC anchors.

- Prepare for expert viewing of HDR content at the next JVET meeting if feasible.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Study additional aspects of coding HDR/WCG content.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers.  The primary activity of the AhG was related to the mandates of (i) generating CTC anchor for the VTM according to JVET-P2011 and (ii) comparing the performance of the VTM for HDR/WCG content. This work is described in the following subsection.

## Anchor Generation

The AhG generated CTC anchors for the VTM according to JVET-P2011.  The performance of the anchors was reported to the reflector on March 23, 2020.  A summary of the performance is provided below, and more detailed information may be found in the included XLS data.

### VTM 8.0 versus VTM 7.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -7.59% | 0.15% | 0.25% | -12.22% | -22.12% | 0.26% | -13.48% | -22.94% | 104% | 125% |
| Class H2 |  |  |  |  |  | 0.16% | -14.70% | -15.00% | 104% | 128% |
| Overall | -7.59% | 0.15% | 0.25% | -12.22% | -22.12% | 0.22% | -13.93% | -20.05% | 104% | 126% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -8.81% | -0.18% | 0.01% | -12.80% | -26.47% | 0.05% | -13.94% | -26.39% | 110% | 110% |
| Class H2 |  |  |  |  |  | -0.23% | -18.80% | -19.22% | 111% | 119% |
| Overall | -8.81% | -0.18% | 0.01% | -12.80% | -26.47% | -0.05% | -15.71% | -23.78% | 110% | 113% |

### VTM 7.0 versus HM 16.18

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.13% | -26.63% | -26.13% | -56.61% | -51.95% | -23.45% | -52.54% | -45.14% |  |  |
| Class H2 |  |  |  |  |  | -21.16% | -47.43% | -48.90% |  |  |
| Overall | -41.13% | -26.63% | -26.13% | -56.61% | -51.95% | -22.62% | -50.68% | -46.50% |  |  |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -31.39% | -31.44% | -31.13% | -46.08% | -38.43% | -28.09% | -41.03% | -30.85% |  |  |
| Class H2 |  |  |  |  |  | -28.53% | -56.27% | -58.58% |  |  |
| Overall | -31.39% | -31.44% | -31.13% | -46.08% | -38.43% | -28.25% | -46.57% | -40.93% |  |  |

## Tool-On / Tool-Off Comparisons

In addition to evaluating the performance of VTM 8.0, the AhG also studied the performance of individual coding tools in the context of HDR content.  This was accomplished by conducting a Tool-On/Tool-Off test according to the methodology established in AhG13.

Results are summarized in the tables below.  Additionally, more detailed results are provided in the included XLS data.

The AhG would like to thank the following companies for contributing to the Tool-On tests: Alibaba, Dolby, InterDigital, LG, MediaTek, NHK, and Sharp.

### Class H1 (PQ)

Simulation Results for AI (Class H1)

|  |  |  |  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 16.01% | 0.96% | 0.87% | 14.45% | 18.89% | 150% | 101% | 148% | 102% |
| DQ | 0.10% | 1.37% | 1.46% | 0.19% | 0.26% | 95% | 107% | 96% | 105% |
| CCLM | 18.72% | 2.34% | 2.14% | 46.68% | 52.49% | 101% | 101% | 101% | 101% |
| MTS | 1.15% | 1.30% | 1.29% | 0.99% | 0.86% | 87% | 95% | 86% | 101% |
| ALF | 10.21% | 2.75% | 2.17% | 18.51% | 37.46% | 95% | 89% | 96% | 92% |
| MRLP | 0.20% | 0.32% | 0.30% | 0.16% | 0.01% | 99% | 102% | 99% | 101% |
| IBC | -0.08% | -0.34% | -0.32% | -0.11% | -0.11% | 141% | 102% | 183% | 101% |
| ISP | 0.04% | 0.63% | 0.72% | -0.28% | -0.26% | 90% | 99% | 85% | 99% |
| LMCS | 1.04% | 0.89% | 4.28% | 1.58% | 4.62% | 99% | 97% | 97% | 97% |
| BDPCM | 0.01% | 0.02% | -0.03% | -0.12% | -0.16% | 101% | 99% | 106% | 101% |
| MIP | 0.47% | 0.75% | 0.59% | 0.40% | 0.10% | 93% | 99% | 89% | 101% |
| LFNST | 0.90% | 0.93% | 0.82% | 1.25% | 2.31% | 106% | 98% | 103% | 101% |
| JCCR | 0.22% | 0.52% | 0.55% | 0.37% | -1.69% | 99% | 100% | 98% | 102% |
| SAO | 0.92% | 0.07% | 0.00% | 1.09% | 2.31% | 100% | 97% | 100% | 98% |
| CCALF | 7.40% | -0.16% | -0.17% | 13.29% | 33.96% | 97% | 98% | 99% | 99% |

Simulation Results for RA (Class H1)

|  |  |  |  |  | VTM RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 13.45% | 0.55% | 0.48% | 10.56% | 14.53% | 104% | 101% | 105% | 103% |
| DQ | -0.32% | 1.54% | 1.48% | -1.15% | -0.75% | 101% | 94% | 100% | 104% |
| CCLM | 15.06% | 1.16% | 1.04% | 35.92% | 39.48% | 100% | 99% | 100% | 102% |
| MTS | 0.78% | 1.10% | 1.08% | 1.03% | 1.18% | 99% | 96% | 95% | 102% |
| ALF | 11.48% | 2.08% | 1.72% | 17.60% | 45.54% | 93% | 92% | 95% | 95% |
| AFF | 0.91% | 1.13% | 1.12% | 0.59% | 0.61% | 79% | 101% | 79% | 102% |
| SbTMVP | 0.41% | 0.37% | 0.39% | 0.36% | 0.38% | 101% | 99% | 98% | 97% |
| AMVR | 0.95% | 0.76% | 0.78% | 1.26% | 1.73% | 88% | 101% | 86% | 104% |
| GPM | 0.65% | 0.40% | 0.45% | 0.99% | 1.10% | 97% | 101% | 98% | 104% |
| BDOF | 0.72% | 0.93% | 1.00% | 0.38% | 0.33% | 96% | 102% | 97% | 101% |
| CIIP | -0.02% | 0.14% | 0.17% | -0.15% | -0.04% | 97% | 101% | 98% | 103% |
| MMVD | 0.34% | 0.31% | 0.29% | 0.21% | 0.39% | 91% | 100% | 90% | 103% |
| BCW | 0.66% | 0.24% | 0.21% | 0.46% | 0.70% | 94% | 106% | 94% | 104% |
| MRLP | 0.21% | 0.21% | 0.17% | -0.11% | 0.27% | 100% | 105% | 100% | 102% |
| IBC | 0.16% | -0.07% | 0.02% | 0.23% | 0.53% | 104% | 101% | 108% | 103% |
| ISP | -0.12% | 0.43% | 0.46% | -0.15% | 0.26% | 99% | 99% | 96% | 102% |
| DMVR | 1.12% | 1.09% | 0.95% | 1.28% | 1.24% | 100% | 97% | 101% | 99% |
| SBT | 0.13% | 0.17% | 0.38% | 0.00% | 0.28% | 96% | 102% | 96% | 100% |
| LMCS | -1.17% | 0.26% | 4.55% | 0.76% | 3.42% | 98% | 98% | 101% | 100% |
| SMVD | 0.15% | 0.14% | 0.16% | 0.15% | 0.26% | 100% | 99% | 96% | 101% |
| BDPCM | 0.00% | 0.03% | 0.01% | -0.13% | 0.12% | 104% | 101% | 101% | 103% |
| MIP | 0.32% | 0.53% | 0.38% | 0.40% | 0.19% | 100% | 98% | 98% | 103% |
| LFNST | 0.38% | 0.59% | 0.47% | 0.91% | 1.89% | 98% | 97% | 97% | 102% |
| JCCR | -0.36% | 0.35% | 0.37% | -0.71% | -1.12% | 100% | 97% | 100% | 102% |
| SAO | 0.94% | -0.03% | -0.08% | 1.13% | 2.37% | 100% | 100% | 99% | 100% |
| PROF | 0.25% | 0.26% | 0.28% | 0.15% | 0.22% | 95% | 105% | 95% | 101% |
| CCALF | 9.12% | -0.47% | -0.48% | 14.56% | 41.31% | 101% | 97% | 101% | 102% |

Figure 1. wPSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 2. wPSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 3. wPSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (Class H1)

### Class H2 (HLG)

Simulation Results for AI (Class H2)

|  |  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.60% | 12.89% | 19.26% | 162% | 104% | 158% | 104% |
| DQ | 1.69% | 0.64% | 0.72% | 99% | 104% | 99% | 106% |
| CCLM | 1.81% | 26.99% | 16.96% | 102% | 100% | 101% | 100% |
| MTS | 1.87% | 2.62% | 2.14% | 87% | 97% | 86% | 99% |
| ALF | 2.70% | 18.58% | 20.81% | 97% | 89% | 94% | 91% |
| MRLP | 0.03% | -0.06% | -0.13% | 98% | 101% | 97% | 101% |
| IBC | -0.11% | 0.04% | 0.04% | 209% | 100% | 183% | 100% |
| ISP | 0.31% | -0.61% | -0.26% | 86% | 100% | 84% | 100% |
| LMCS | 0.06% | -0.90% | -0.74% | 99% | 101% | 97% | 100% |
| MIP | 0.72% | 0.83% | 0.35% | 91% | 100% | 89% | 101% |
| LFNST | 0.53% | 1.40% | 1.58% | 110% | 101% | 107% | 100% |
| JCCR | 0.28% | 0.82% | 5.64% | 99% | 101% | 97% | 101% |
| SAO | 0.06% | 0.27% | 0.66% | 100% | 97% | 98% | 98% |
| CCALF | -0.12% | 16.12% | 16.65% | 98% | 97% | 99% | 98% |

Simulation Results for RA (Class H2)

|  |  |  | VTM RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.22% | 7.47% | 13.71% | 103% | 99% | 106% | 105% |
| DQ | 1.72% | 0.94% | 0.77% | 102% | 103% | 105% | 108% |
| CCLM | 0.84% | 24.41% | 18.56% | 100% | 101% | 104% | 106% |
| MTS | 1.15% | 1.47% | 0.88% | 96% | 101% | 98% | 104% |
| ALF | 3.40% | 26.85% | 28.54% | 97% | 91% | 96% | 97% |
| AFF | 0.72% | 0.51% | 0.43% | 77% | 99% | 80% | 102% |
| SbTMVP | 0.36% | 0.29% | 0.33% | 101% | 100% | 104% | 104% |
| AMVR | 0.72% | 1.04% | 1.22% | 84% | 101% | 86% | 106% |
| GPM | 0.65% | 0.77% | 0.76% | 97% | 101% | 101% | 106% |
| BDOF | 0.60% | 0.29% | 0.27% | 96% | 99% | 99% | 103% |
| CIIP | 0.21% | -0.18% | -0.35% | 98% | 100% | 101% | 105% |
| MMVD | 0.20% | 0.32% | 0.27% | 91% | 101% | 93% | 105% |
| BCW | 0.22% | 0.15% | 0.14% | 95% | 102% | 97% | 107% |
| MRLP | 0.03% | -0.03% | -0.25% | 101% | 101% | 102% | 106% |
| IBC | 0.11% | 0.00% | -0.46% | 109% | 101% | 109% | 105% |
| ISP | 0.23% | 0.24% | 0.11% | 97% | 101% | 99% | 105% |
| DMVR | 0.86% | 1.08% | 1.09% | 100% | 97% | 103% | 100% |
| SBT | 0.31% | -0.14% | -0.15% | 97% | 101% | 99% | 105% |
| LMCS | 0.96% | 0.69% | 0.64% | 99% | 103% | 99% | 100% |
| SMVD | 0.19% | 0.15% | 0.20% | 97% | 101% | 96% | 105% |
| BDPCM | 0.02% | 0.05% | -0.10% | 102% | 100% | 104% | 105% |
| MIP | 0.50% | 0.62% | 0.19% | 97% | 100% | 101% | 105% |
| LFNST | 0.48% | 0.96% | 1.12% | 97% | 101% | 100% | 104% |
| JCCR | 0.17% | 0.53% | 6.62% | 100% | 101% | 103% | 105% |
| SAO | 0.06% | 0.34% | 1.61% | 100% | 99% | 103% | 104% |
| PROF | 0.33% | 0.19% | 0.33% | 95% | 99% | 96% | 104% |
| CCALF | -0.12% | 22.93% | 24.40% | 100% | 104% | 103% | 105% |

Figure 4. PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (Class H2)

Figure 5. PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (Class H2)

Figure 6. PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (Class H2)

# Contributions

There are three contribution related to HDR video coding.

| JVET-R0259 | AHG7: On CCALF filtering of chroma sample location type-2 content | M.G. Sarwer, Y. Ye, J. Luo (Alibaba) |
| --- | --- | --- |
| JVET-R0365 | Proposals on VVC extension for higher fidelity video | T. Suzuki, M. Ikeda, Y. Yagasaki (Sony), T. Toma, K. Abe (Panasonic), M. Shima (Canon) |
| JVEt-R0446 | Crosscheck of JVET-R0256 (AHG7: On CCALF filtering of chroma sample location type-2 content) | F. Pu (Dolby) |

# Recommendations

The AHG recommends the following:

- Review all input contributions