| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0051-v1 |
| --- | --- |

| Title: | AHG9: HLS on LMCS and scaling list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Zhi-Yi Lin, Chen-Yen Lai, Olena Chubach, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, zhi-yi.lin, jenny.lai, olena.chubach, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes three high-level syntax modifications related to luma mapping with chroma scaling (LMCS) and scaling list, summarized as follows:

- When sps_lmcs_enabled_flag is equal to 1, a new sequence parameter set (SPS) syntax element sps_lmcs_enabled_slice_present_flag is further signalled to indicate whether slice_lmcs_enabled_flag is present in the slice header.

- When sps_explicit_scaling_list_enabled_flag is equal to 1, a new SPS syntax element sps_explicit_scaling_list_used_slice_present_flag is further signalled to indicate whether slice_explicit_scaling_list_used_flag is present in the slice header.

- Semantic corrections are proposed for the syntax elements sps_lmcs_enabled_flag, ph_lmcs_enabled_flag, and ph_chroma_residual_scale_flag.

Introduction

In Versatile Video Coding (VVC) Draft 8 as specified in [1], the new syntax elements slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag are added to the slice header. The first syntax element is used for indicating whether LMCS is enabled in the current slice.  The second syntax element is used to specify that the explicit scaling list is used in the scaling process for transform coefficients when decoding a current slice. A video encoder is allowed to select whether to enable slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag adaptively for each coded slice. However, extra bits are needed for signalling the two new syntax elements for each of the coded slices.

Proposed modifications

Modification 1: Further signalling the new SPS syntax element sps_lmcs_enabled_slice_present_flag

In Modification 1, when the syntax element sps_lmcs_enabled_flag is equal to 1, a new syntax element sps_lmcs_enabled_slice_present_flag is further signalled in the SPS. When sps_lmcs_enabled_slice_present_flag is equal to 0, slice_lmcs_enabled_flag is not coded in the current slice header and it is inferred to be equal to the value of ph_lmcs_enabled_flag. In this way, when a video encoder chooses to always enable LMCS for all slices in one picture or in a CLVS, the bits required for signalling slice_lmcs_enabled_flag can be saved for all slice headers referring to the SPS by setting the value of sps_lmcs_enabled_slice_present_flag equal to 0.

Modification 2: Further signalling the new SPS syntax element sps_explicit_scaling_list_used_slice_present_flag

In Modification 2, when the syntax element sps_explicit_scaling_list_enabled_flag is equal to 1, a new syntax element sps_explicit_scaling_list_used_slice_present_flag is further signalled in the SPS. When sps_explicit_scaling_list_used_slice_present_flag is equal to 0, slice_explicit_scaling_list_used_flag is not coded in the current slice header and it is inferred to be equal to the value of ph_explicit_scaling_list_enabled_flag. In this way, when a video encoder chooses to always enable explicit scaling list for all slices in one picture or in a CLVS, the bits required for signalling slice_explicit_scaling_list_used_flag for all slices can be saved for all slice headers referring to the SPS by setting the value of sps_explicit_scaling_list_used_slice_present_flag equal to 0.

Modification 3: Semantic corrections

Several semantic corrections are proposed for the following syntax elements: sps_lmcs_enabled_flag, ph_lmcs_enabled_flag, and ph_chroma_residual_scale_flag, as highlighted in yellow in Section 3.

Modified syntax tables and semantics

The modified parts of the syntax tables based on VVC Draft 8 are written in red and provided below:

- 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_lmcs_enabled_flag | u(1) |
| if( sps_lmcs_enabled_flag ) |  |
| sps_lmcs_enabled_slice_present_flag | u(1) |
| …. |  |
| sps_explicit_scaling_list_enabled_flag | u(1) |
| if( sps_explicit_scaling_list_enabled_flag ) |  |
| sps_explicit_scaling_list_used_slice_present_flag | u(1) |
| …. |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header ( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| ….. |  |
| if( ph_lmcs_enabled_flag && sps_lmcs_enabled_slice_present_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag && sps_explicit_scaling_list_used_slice_present_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| …. |  |
| } |  |

The related semantics are as follows:

- 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is may be used in the CLVS. sps_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not used in the CLVS.

sps_lmcs_enabled_slice_present_flag equal to 1 specifies that the syntax element slice_lmcs_enabled_flag may be present in slice headers referring to the SPS. sps_lmcs_enabled_slice_present_flag equal to 0 specifies that the syntax element slice_lmcs_enabled_flag is not present in slice headers referring to the SPS. When sps_lmcs_enabled_pic_present_flag is not present, it is inferred to be equal to 0.

….

sps_explicit_scaling_list_enabled_flag equal to 1 specifies that the use of an explicit scaling list, which is signalled in a scaling list APS, in the scaling process for transform coefficients when decoding a slice is enabled for the CLVS. sps_explicit_scaling_list_enabled_flag equal to 0 specifies that the use of an explicit scaling list in the scaling process for transform coefficients when decoding a slice is disabled for the CLVS.

sps_explicit_scaling_list_used_slice_present_flag equal to 1 specifies that the syntax element slice_explicit_scaling_list_used_flag is present in slice headers referring to the SPS. sps_explicit_scaling_list_used_slice_present_flag equal to 0 specifies that the syntax element slice_explicit_scaling_list_used_flag is not present in slice headers referring to the SPS. When sps_explicit_scaling_list_used_slice_present_flag is not present, it is inferred to be equal to 0.

….

- 7.4.3.3 Picture header RBSP semantics

….

ph_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is may be enabled for all slices associated with the PH. ph_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling may be is disabled for one, or more, or all slices associated with the PH. When not present, the value of ph_lmcs_enabled_flag is inferred to be equal to 0.

ph_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

ph_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is may be enabled for all slices associated with the PH. ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is disabled for one, or more, or all slices associated with the PH. When ph_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

ph_explicit_scaling_list_enabled_flag equal to 1 specifies that the use of the explicit scaling list, which is signalled in the referenced scaling list APS (i.e., the APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_explicit_scaling_list_aps_id), in the scaling process for transform coefficients when decoding a slice is enabled for the picture. ph_explicit_scaling_list_enabled_flag equal to 0 specifies that the use of an explicit scaling list in the scaling process for transform coefficients when decoding a slice is disabled for the picture. When not present, the value of ph_explicit_scaling_list_enabled_flag is inferred to be equal to 0.

ph_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS. The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

….

- 7.3.7.1 General slice header syntax

….

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is disabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0 the value of ph_lmcs_enabled_flag.

slice_explicit_scaling_list_used_flag equal to 1 specifies that the explicit scaling list, which is signalled in the referenced scaling list APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id, is used in the scaling process for transform coefficients when decoding of the current slice. slice_explicit_scaling_list_used_flag equal to 0 specifies that no explicit scaling list is not used in the scaling process for transform coefficients when decoding of the current slice. When not present, the value of slice_explicit_scaling_list_used_flag is inferred to be equal to 0 the value of ph_explicit_scaling_list_enabled_flag.

….

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).