| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0061-v1 |
| --- | --- |

| Title: | AHG9: On allowed slice types in a picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang1, Ye-Kui Wang1, Kai Zhang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | lizhang.idm@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a SPS flag, sps_b_slice_allowed_flag, indicating permission of B slices, to better support applications with low-complexity encoders and reduce overhead cost. On top of that, a PH flag, ph_b_slice_allowed_flag, is further added to specify whether B slice is allowed in a picture. In addition, the following changes are proposed accordingly:

- When sps_b_slice_allowed_flag is equal to 0, the syntax elements sps_weighted_bipred_flag, sps_bdof_enabled_flag, sps_smvd_enabled_flag, sps_dmvr_enabled_flag, sps_bcw_enabled_flag, and sps_gpm_enabled_flag are not signalled and their values are inferred.

- When ph_b_slice_allowed_flag is equal to 0, the syntax elements ph_collocated_from_l0_flag, mvd_l1_zero_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag, and num_l1_weights are not signalled and their values are inferred, and, consequently, weighted prediction parameters for reference picture list 1 are not signalled in the PH or SHs of the picture.

- When both ph_b_slice_allowed_flag and ph_intra_slice_allowed_flag are equal to 0, the syntax element slice_type is not signalled and the value is inferred to be equal to 1.

# Introduction

In the latest VVC draft text, two PH syntax elements related to allowed slice types are included, i.e., ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag, as tabulated in the sub-clause 7.3.2.7 below. With these two flags, syntax elements related to I slice coding are signalled only when the ph_intra_slice_allowed_flag is true, and syntax elements related to inter slice coding are signalled only when the ph_inter_slice_allowed_flag is true. However, when ph_inter_slice_allowed_flag is equal to 1, the decoder does not know whether the picture contains B slices. Some applications, e.g., online gaming, video conference, video surveillance, often use only P and I slices. Therefore, if there is an indication of whether B slices are allowed, the decoders of such applications would be able to choose to only request/use bitstreams that do not B slices included, and in addition, the indication can be used to avoid sending of multiple unnecessary parameters.

7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| non_reference_picture_flag | u(1) |
| … |  |
| if( ph_intra_slice_allowed_flag ) { |  |
| … |  |
| } |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| … |  |
| } |  |
| … |  |

# Proposal

## Summary of the proposed changes

The following changes are proposed:

- In the SPS, add sps_b_slice_allowed_flag, and sps_b_slice_allowed_flag equal to 1 specifies that the CLVS may contain one or more B slices, and sps_b_slice_allowed_flag equal to 0 specifies that the CLVS does not contain B slices.

  - When sps_b_slice_allowed_flag is equal to 0, the syntax elements sps_weighted_bipred_flag, sps_bdof_enabled_flag, sps_smvd_enabled_flag, sps_dmvr_enabled_flag, sps_bcw_enabled_flag, and sps_gpm_enabled_flag are not signalled and their values are inferred.

- In the PH, add ph_b_slice_allowed_flag, and ph_b_slice_allowed_flag equal to 1 specifies that the picture may contain one or more B slices, and ph_b_slice_allowed_flag equal to 0 specifies that the picture does not contain B slices.

  - When ph_b_slice_allowed_flag is equal to 0, the syntax elements ph_collocated_from_l0_flag, mvd_l1_zero_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag, and num_l1_weights are not signalled and their values are inferred, and, consequently, weighted prediction parameters for reference picture list 1 are not signalled in the PH or SHs of the picture.

  - When both ph_b_slice_allowed_flag and ph_intra_slice_allowed_flag are equal to 0, the syntax element slice_type is not signalled and the value is inferred to be equal to 1.

## Proposed spec text changes

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| sps_b_slice_allowed_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| … | u(1) |
| sps_weighted_pred_flag | u(1) |
| if( sps_b_slice_allowed_flag ) |  |
| sps_weighted_bipred_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | u(4) |
| sps_poc_msb_flag | u(1) |
| … |  |
| sps_amvr_enabled_flag | u(1) |
| if( sps_b_slice_allowed_flag ) { |  |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| } |  |
| sps_mmvd_enabled_flag | u(1) |
| … |  |
| if( sps_b_slice_allowed_flag ) |  |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| if( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_merge_cand | ue(v) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( sps_b_slice_allowed_flag && MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| … |  |
| } |  |

...

sps_b_slice_allowed_flag equal to 0 specifies that none of the coded slices of the CLVS has slice_type equal to 0. sps_b_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices of the CLVS that have slice_type equal to 0.

sps_weighted_bipred_flag equal to 1 specifies that explicit weighted prediction may be applied to B slices referring to the SPS. sps_weighted_bipred_flag equal to 0 specifies that explicit weighted prediction is not applied to B slices referring to the SPS. When not present, the value of sps_weighted_bipred_flag is inferred to be equal to 0.

sps_bdof_enabled_flag equal to 0 specifies that the bi-directional optical flow inter prediction is disabled. sps_bdof_enabled_flag equal to 1 specifies that the bi-directional optical flow inter prediction is enabled. When not present, the value of sps_bdof_enabled_flag is inferred to be equal to 0.

sps_smvd_enabled_flag equal to 1 specifies that symmetric motion vector difference may be used in motion vector decoding. sps_smvd_enabled_flag equal to 0 specifies that symmetric motion vector difference is not used in motion vector coding. When not present, the value of sps_smvd_enabled_flag is inferred to be equal to 0.

sps_dmvr_enabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is enabled. sps_dmvr_enabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is disabled. When not present, the value of sps_dmvr_enabled_flag is inferred to be equal to 0.

sps_bcw_enabled_flag specifies whether bi-prediction with CU weights can be used for inter prediction. If sps_bcw_enabled_flag is equal to 0, the syntax shall be constrained such that no bi-prediction with CU weights is used in the CLVS, and bcw_idx is not present in coding unit syntax of the CLVS. Otherwise (sps_bcw_enabled_flag is equal to 1), bi-prediction with CU weights can be used in the CLVS. When not present, the value of sps_bcw_enabled_flag is inferred to be equal to 0.

...

7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) { |  |
| ph_intra_slice_allowed_flag | u(1) |
| if( sps_b_slice_allowed_flag ) |  |
| ph_b_slice_allowed_flag | u(1) |
| } |  |
| non_reference_picture_flag | u(1) |
| … |  |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| if( ph_b_slice_allowed_flag ) |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| … |  |
| if( ph_b_slice_allowed_flag ) |  |
| mvd_l1_zero_flag | u(1) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| ph_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag && ph_b_slice_allowed_flag ) |  |
| ph_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag && ph_b_slice_allowed_flag ) |  |
| ph_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| ph_disable_prof_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && wp_info_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| ... |  |

...

ph_intra_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 0 or 1. ph_intra_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 2. When not present, the value of ph_intra_slice_allowed_flag is inferred to be equal to 1.

ph_b_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 1 or 2. ph_b_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0. When not present, the value of ph_b_slice_allowed_flag is inferred to be equal to 0.

NOTE  – ph_b_slice_allowed_flag equal to 0 and ph_intra_slice_allowed_flag equal to 0 indicates that all coded slices of the picture have slice_type equal to 1.

...

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When ph_temporal_mvp_enabled_flag and rpl_info_in_ph_flag are both equal to 1 and ph_b_slice_allowed_flag is equal to 0, the value of ph_collocated_from_l0_flag is inferred to be equal to 1.

ph_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When ph_collocated_from_l0_flag is equal to 1, ph_collocated_ref_idx refers to an entry in reference picture list 0, and the value of ph_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] − 1, inclusive.

When ph_collocated_from_l0_flag is equal to 0, ph_collocated_ref_idx refers to an entry in reference picture list 1, and the value of ph_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] − 1, inclusive.

When not present, the value of ph_collocated_ref_idx is inferred to be equal to 0.

...

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] and MvdCpL1[ x0 ][ y0 ][ cpIdx ][ compIdx ] are set equal to 0 for compIdx = 0..1 and cpIdx = 0..2. mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed. When ph_inter_slice_allowed_flag is equal to 1 and ph_b_slice_allowed_flag is equal to 0, the value of mvd_l1_zero_flag is inferred to be equal to 1.

...

ph_disable_bdof_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled in the slices associated with the PH. ph_disable_bdof_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction may or may not be enabled in the slices associated with the PH.

When ph_disable_bdof_flag is not present, the following applies:

- If sps_bdof_enabled_flag is equal to 1 and ph_b_slice_allowed_flag is equal to 1, the value of ph_disable_bdof_flag is inferred to be equal to 0.

- Otherwise (sps_bdof_enabled_flag is equal to 0 or ph_b_slice_allowed_flag is equal to 0), the value of ph_disable_bdof_flag is inferred to be equal to 1.

ph_disable_dmvr_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled in the slices associated with the PH. ph_disable_dmvr_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction may or may not be enabled in the slices associated with the PH.

When ph_disable_dmvr_flag is not present, the following applies:

- If sps_dmvr_enabled_flag is equal to 1 and ph_b_slice_allowed_flag is equal to 1, the value of ph_disable_dmvr_flag is inferred to be equal to 0.

- Otherwise (sps_dmvr_enabled_flag is equal to 0 or ph_b_slice_allowed_flag is equal to 0), the value of ph_disable_dmvr_flag is inferred to be equal to 1.

...

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_inter_slice_allowed_flag && ( ph_b_slice_allowed_flag ¦ ¦ ph_intra_slice_allowed_flag ) ) |  |
| slice_type | ue(v) |
| … |  |

...

slice_type specifies the coding type of the slice according to Table 9.

Table 9 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When not present, the value of slice_type is inferred to be equal to ( ph_inter_slice_allowed_flag ? 1: 2 ) 2.

When ph_b_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 1 or 2. When ph_b_slice_allowed_flag equal to 1 and ph_intra_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 0 or 1. When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

...

7.3.7.2	Weighted prediction parameters syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_weighted_bipred_flag && wp_info_in_ph_flag && ph_b_slice_allowed_flag ) |  |
| num_l1_weights | ue(v) |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < NumWeightsL1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

7.4.8.2	Weighted prediction parameters semantics

...

num_l1_weights specifies the number of weights signalled for entries in reference picture list 1 when pps_weighted_bipred_flag and wp_info_in_ph_flag are both equal to 1. The value of num_l1_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] ), inclusive.

The variable NumWeightsL1 is derived as follows:

if( !pps_weighted_bipred_flag  | |  !ph_b_slice_allowed_flag )
	NumWeightsL1 = 0
else if( wp_info_in_ph_flag )		(148)
	NumWeightsL1 = num_l1_weights
else
	NumWeightsL1 = NumRefIdxActive[ 1 ]

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).