| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0081-v1 |
| --- | --- |

| Title: | AHG9: Chroma deblocking strength signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Martin Pettersson Mitra Damghanian Jack Enhorn Kenneth Andersson Jacob Ström Rickard Sjöberg | Email: | zhi.a.zhang@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com kenneth.r.andersson@ericsson.com jacob.strom@ericsson.com rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution contains two proposals related to deblocking strength control for the Cb and Cr components.

Proposal 1 makes the presence of chroma β and tC offset syntax elements in the picture header (PH) and slice header (SH) conditional such that they are not present when ChromaArrayType is equal to 0. It is also proposed to add bitstream constraints on the chroma β and tC offset syntax elements in the picture parameter set (PPS) to enforce that they are equal to 0 when ChromaArrayType is equal to 0. The proponents considered to propose to condition the presence of these PPS syntax elements on ChromaArrayType but concluded that this would cause a parsing dependency since the value of ChromaArrayType is derived from SPS syntax elements.

Proposal 2 is to add one flag to each of the PPS, PH and SH for controlling the presence of chroma β and tC offset syntax elements. The three new flags are proposed to be called pps_deblocking_chroma_offsets_present_flag, ph_deblocking_chroma_offsets_present_flag and slice_deblocking_chroma_offsets_present_flag, respectively.

The proposed behaviour of the flags is as follows:

- When one of the proposed flags is equal to 1, chroma β and tC offset values are signalled as in the current VVC draft.

- When one of the proposed flags is equal to 0, the chroma β and tC offset values are not signalled but inferred to be equal to the values of the luma β and tC offset values signaled in the same syntax table.

- When one of the proposed flags is not present, the chroma β and tC offset values are inferred as in the current VVC draft.

The proponents claim that proposal 2 provides a short-cut for the claimed common case when the chroma β and tC offset values are identical to the luma β and tC offset values.

# Introduction

At the 17th JVET meeting, it was decided to include separate deblocking (DB) β and tC offset syntax elements for the three colour components Y, Cb and Cr. We call these syntax elements deblocking offsets in this document. In the current version of VVC, the deblocking offsets may be signalled in the picture parameter set (PPS), picture header (PH) or slice header (SH).

- When the deblocking offsets are present, there are 6 signed integer 0-th order Exp-Golomb-coded syntax elements with range [-12, 12] signaled for the three colour components Y, Cb and Cr. In VVC, the parameter separately controlled for the luma and chroma components is applied for ALF, SAO and DB. However, the signaling cost for the parameter separately controlled is much higher for DB than ALF and SAO. DB has a signaling bit cost between 6 and 54 bits, while the SAO cost is 1 bit and the ALF cost is 2 bits.

- When the deblocking offsets are not present, the values of the β and tC offset values are inferred as shown in Table 1. In VVC, the presence of the deblocking offsets is controlled by the values of pps_deblocking_filter_disabled_flag, ph_deblocking_filter_disabled_flag and slice_deblocking_filter_disabled_flag in the PPS, PH and SH respectively. However, in the PH and SH it is possible to make the presence of the deblocking offsets for the Cb and Cr components (called chroma deblocking offsets in this document) conditional based on the value of ChromaArrayType.

|  | Not present in PPS | Not present in PH | Not present in SH |
| --- | --- | --- | --- |
| beta_offset_div2 | pps_beta_offset_div2 = 0 | ph_beta_offset_div2 = pps_beta_offset_div2 | slice_beta_offset_div2 = ph_beta_offset_div2 |
| tc_offset_div2 | pps_tc_offset_div2 = 0 | ph_tc_offset_div2 = pps_tc_offset_div2 | slice_tc_offset_div2 = ph_tc_offset_div2 |
| cb_beta_offset_div2 | pps_cb_beta_offset_div2 = 0 | ph_cb_beta_offset_div2 = pps_cb_beta_offset_div2 | slice_cb_beta_offset_div2 = ph_cb_beta_offset_div2 |
| cb_tc_offset_div2 | pps_cb_tc_offset_div2 = 0 | ph_cb_tc_offset_div2 = pps_cb_tc_offset_div2 | slice_cb_tc_offset_div2 = ph_cb_tc_offset_div2 |
| cr_beta_offset_div2 | pps_cr_beta_offset_div2 = 0 | ph_cr_beta_offset_div2 = pps_cr_beta_offset_div2 | slice_cr_beta_offset_div2 = ph_cr_beta_offset_div2 |
| cr_tc_offset_div2 | pps_cr_tc_offset_div2 = 0 | ph_cr_tc_offset_div2 = pps_cr_tc_offset_div2 | slice_cr_tc_offset_div2 = ph_cr_tc_offset_div2 |

Table 1. The value inference of the deblocking offsets

# Proposal

## Proposal 1: Presence of the chroma deblocking offsets based on ChromaArrayType

It is proposed to make the presence of the chroma deblocking offsets in the PH and SH conditional based on the value of ChromaArrayType since it makes no sense to include chroma offset syntax elements when there are no chroma components (e.g. monochrome format or separate_colour_plane_flag is equal to 1, where, in both cases ChromaArrayType is equal to 0). When ChromaArrayType is equal to 0, the chroma deblocking offsets in the PH and SH are not present, and the values are inferred as in the current VVC draft, shown in Table 1.

It is also proposed to add bitstream constraints in the PPS since it makes no sense to increase the signaling cost for the chroma deblocking offsets when there are no chroma components. When ChromaArrayType is equal to 0, the values of the chroma deblocking offsets for the Cb and Cr components shall be equal to 0. The proponents considered to propose to condition the presence of these PPS syntax elements on ChromaArrayType but concluded that this would cause a parsing dependency since the ChromaArrayType value is derived from SPS syntax elements.

Proposed changes on top of the VVC draft (JVET-Q2001-vD) for proposal 1 are shown below with added text in yellow.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … | … |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … | … |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … | … |
| } |  |

pps_beta_offset_div2 and pps_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to the luma component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the picture headers or the slice headers of the slices referring to the PPS. The values of pps_beta_offset_div2 and pps_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of pps_beta_offset_div2 and pps_tc_offset_div2 are both inferred to be equal to 0.

pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the picture headers or the slice headers of the slices referring to the PPS. The values of pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 are both inferred to be equal to 0. When ChromaArrayType is equal to 0, the values of pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 shall both be equal to 0.

pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the picture headers or the slice headers of the slices referring to the PPS. The values of pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 are both inferred to be equal to 0. When ChromaArrayType is equal to 0, the values of pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 shall both be equal to 0.

## Proposal 2: Adding a present flag in each of the PPS, PH and SH to control the presence of the chroma deblocking offsets

We propose to add present flags to control the presence of the chroma deblocking offsets in each of the PPS, PH and SH. The three new flags are proposed to be called pps_deblocking_chroma_offsets_present_flag, ph_deblocking_chroma_offsets_present_flag and slice_deblocking_chroma_offsets_present_flag, respectively.

When one of the proposed flags is equal to 1, the chroma deblocking offsets are signalled as in the current VVC draft.

When one of the proposed flags is equal to 0, the chroma deblocking offsets are not signalled but set equal to the values of the luma β and tC offset values signaled in the same syntax table.

When one of the proposed flags is not present, for example pps_deblocking_filter_disabled_flag being equal to 1, the chroma deblocking offsets are not signalled and the values of the chroma deblocking offsets are inferred as in the current VVC draft.

Given the deblocking offsets are signed integer 0-th order Exp-Golomb-coded syntax elements with range [-12, 12], the bit cost is as shown in Table 2:

| Bit cost | Value of the syntax elements |
| --- | --- |
| 1 | { 0 } |
| 3 | { -1, 1 } |
| 5 | { -3, -2, 2, 3 } |
| 7 | { -7, -6, -5, -4, 4, 5, 6, 7 } |
| 9 | { -12, -11, -10, -9, -8, 8, 9, 10, 11, 12 } |

Table 2. Bit cost for each of the deblocking offsets

The proposed method reduces the signaling cost when the chroma β and tC offsets are equal to the luma β and tC offsets. The signaling cost reduction may be up to 35 bits in the PPS, PH and SH respectively. Where the 35 bit is derived by ( BitCost of the value of deblocking offsets * number of chroma deblocking offsets syntax elements – BitCost of the present flag) = ( 9 * 4 – 1) = 35.

Proposed changes on top of the VVC draft (JVET-Q2001-vD) are shown below with added text in yellow and removed text in red strike-through.

### PPS changes

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | … |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_deblocking_chroma_offsets_present_flag | u(1) |
| if( pps_deblocking_chroma_offsets_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … | … |
| } |  |

pps_beta_offset_div2 and pps_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to the luma component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the picture headers or the slice headers of the slices referring to the PPS. The values of pps_beta_offset_div2 and pps_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of pps_beta_offset_div2 and pps_tc_offset_div2 are both inferred to be equal to 0.

pps_deblocking_chroma_offsets_present_flag equal to 1 specifies the presence of deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb and Cr components in the PPS. pps_deblocking_chroma_offsets_present_flag equal to 0 specifies the absence of deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb and Cr components in the PPS. When not present, the value of pps_deblocking_chroma_offsets_present_flag is inferred to be equal to 0. When ChromaArrayType is equal to 0, the value of pps_deblocking_chroma_offsets_present_flag shall be equal to 0.

pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the picture headers or the slice headers of the slices referring to the PPS. The values of pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 are inferred as follows:both inferred to be equal to 0.

- When pps_deblocking_chroma_offsets_present_flag is present, the values of pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

- When pps_deblocking_chroma_offsets_present_flag is not present, the values of pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 are both inferred to be equal to 0.

pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the picture headers or the slice headers of the slices referring to the PPS. The values of pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 are inferred as follows: both inferred to be equal to 0.

- When pps_deblocking_chroma_offsets_present_flag is present, the values of pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

- When pps_deblocking_chroma_offsets_present_flag is not present, the values of pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 are both inferred to be equal to 0.

### PH changes

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … | … |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| ph_deblocking_chroma_offsets_present_flag | u(1) |
| if( ph_deblocking_chroma_offsets_present_flag ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

ph_beta_offset_div2 and ph_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the luma component for the slices associated with the PH. The values of ph_beta_offset_div2 and ph_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_beta_offset_div2 and ph_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

ph_deblocking_chroma_offsets_present_flag equal to 1 specifies the presence of deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb and Cr components in the PH. ph_deblocking_chroma_offsets_present_flag equal to 0 specifies the absence of deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb and Cr components in the PH. When not present, the value of ph_deblocking_chroma_offsets_present_flag is inferred to be equal to 0.

ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the slices associated with the PH. The values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred as follows: to be equal to pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2, respectively.

- When ph_deblocking_chroma_offsets_present_flag is present, the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred to be equal to ph_beta_offset_div2 and ph_tc_offset_div2, respectively.

- When ph_deblocking_chroma_offsets_present_flag is not present, the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred to be equal to pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2, respectively.

ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the slices associated with the PH. The values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred as follows: to be equal to pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2, respectively.

- When ph_deblocking_chroma_offsets_present_flag is present, the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred to be equal to ph_beta_offset_div2 and ph_tc_offset_div2, respectively.

- When ph_deblocking_chroma_offsets_present_flag is not present, the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred to be equal to pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2, respectively.

### SH changes

| slice_header( ) { | Descriptor |
| --- | --- |
| … | … |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| slice_deblocking_chroma_offsets_present_flag | u(1) |
| if( slice_deblocking_chroma_offsets_present_flag ) { |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … | … |
| } |  |

slice_beta_offset_div2 and slice_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the luma component for the current slice. The values of slice_beta_offset_div2 and slice_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of slice_beta_offset_div2 and slice_tc_offset_div2 are inferred to be equal to ph_beta_offset_div2 and ph_tc_offset_div2, respectively.

slice_deblocking_chroma_offsets_present_flag equal to 1 specifies the presence of deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb and Cr components in the slice. slice_deblocking_chroma_offsets_present_flag_equal to 0 specifies the absence of deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb and Cr components in the slice. When not present, the value of slice_deblocking_chroma_offsets_present_flag is inferred to be equal to 0.

slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the current slice. The values of slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 are inferred as follows: to be equal to ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2, respectively.

- When slice_deblocking_chroma_offsets_present_flag is present, the values of slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 are inferred to be equal to ph_beta_offset_div2 and ph_tc_offset_div2, respectively.

- When slice_deblocking_chroma_offsets_present_flag is not present, the values of slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 are inferred to be equal to ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2, respectively.

slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the current slice. The values of slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 are inferred as follows: to be equal to ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2, respectively.

- When slice_deblocking_chroma_offsets_present_flag is present, the values of slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 are inferred to be equal to slice_beta_offset_div2 and slice_tc_offset_div2, respectively.

When slice_deblocking_chroma_offsets_present_flag is not present, the values of slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 are inferred to be equal to ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2, respectively.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).