| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0099-v2 |
| --- | --- |

| Title: | AHG8/ AHG9: On Output Layer Sets Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains proposals related to the DPB parameters for output layer sets signalled in VPS. Following is proposed for cleanup and bug-fix:

- Proposal 1: A more accurate and/or tighter conformance constraint is proposed on the value of vps_num_dpb_params.

- Proposal 2: It is proposed that if the total number of output layer sets with more than one layer is equal to the number of dpb_parameters( ) syntax structures in the VPS, the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS is not signalled, instead it is inferred. In v2 additional text is provided for similar aspect for HRD signalling as agreed in Category 1 AHG meeting.

- Proposal 3: It is proposed that since the 0-th OLS has one layer, the for loop which signals syntax elements  ols_dpb_pic_width[ i ], ols_dpb_pic_height[ i ], and ols_dpb_params_idx[ i ] should start at 1 instead of at 0.

# Introduction

In VVC WD8 [1] Video Parameter Set (VPS) includes signalling of Decoded Picture Buffer (DPB) parameters for Output Layer Sets (OLSs).

Following modifications are proposed for cleanup and bug-fix:

- Proposal 1: A more accurate and/or tighter conformance constraint is proposed on the value of vps_num_dpb_params.

- Proposal 2: It is proposed that if the total number of output layer sets with more than one layer is equal to the number of dpb_parameters( ) syntax structures in the VPS, the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS is not signalled, instead it is inferred.

- Proposal 3: It is proposed that since the 0-th OLS has one layer, the for loop which signals syntax elements  ols_dpb_pic_width[ i ], ols_dpb_pic_height[ i ], and ols_dpb_params_idx[ i ] should start at 1 instead of at 0.

# Proposal

## Proposal 1\

Currently value of vps_num_dpb_params is required to be in the range 0 to 16. It is proposed to modify the value range of the syntax element vps_num_dpb_params so that it is possible to signal more than 16 (up to TotalNumOlss) DPB parameters. This is to ensure that it is possible to provide the correct DPB parameters for each OLS. Thus a more accurate and/or tighter conformance constraint is proposed on the value of vps_num_dpb_params.

The proposed modification is shown below.

…

ols_output_layer_flag[ i ][ j ] equal to 1 specifies that the layer with nuh_layer_id equal to vps_layer_id[ j ] is an output layer of the i-th OLS when ols_mode_idc is equal to 2. ols_output_layer_flag[ i ][ j ] equal to 0 specifies that the layer with nuh_layer_id equal to vps_layer_id[ j ] is not an output layer of the i-th OLS when ols_mode_idc is equal to 2.

The variable NumOutputLayersInOls[ i ], specifying the number of output layers in the i-th OLS, the variable OutputLayerIdInOls[ i ][ j ], specifying the nuh_layer_id value of the j-th output layer in the i-th OLS, and the variable LayerUsedAsOutputLayerFlag[ k ], specifying whether the k-th layer is used as an output layer in at least one OLS, are derived as follows:

NumOutputLayersInOls[ 0 ] = 1
OutputLayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
LayerUsedAsOutputLayerFlag[ 0 ] = 1
for( i = 1, i  <=  vps_max_layers_minus1; i++ ) {
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc < 2 )
		LayerUsedAsOutputLayerFlag[ i ] = 1
	else /*( !each_layer_is_an_ols_flag  &&  ols_mode_idc  = =  2 ) */
		LayerUsedAsOutputLayerFlag[ i ] = 0
}
for( i = 1; i < TotalNumOlss; i++ )
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
	} else if( ols_mode_idc  = =  1 ) {
		NumOutputLayersInOls[ i ] = i + 1
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ )
			OutputLayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
	} else if( ols_mode_idc  = =  2 ) {
		for( j = 0; j  <=  vps_max_layers_minus1; j++ )
			layerIncludedInOlsFlag[ i ][ j ] = 0
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ )	(40)
			if( ols_output_layer_flag[ i ][ k ] ) {
				layerIncludedInOlsFlag[ i ][ k ] = 1
				LayerUsedAsOutputLayerFlag[ k ] = 1
				OutputLayerIdx[ i ][ j ] = k
				OutputLayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
			}
		NumOutputLayersInOls[ i ] = j
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			idx = OutputLayerIdx[ i ][ j ]
			for( k = 0; k < NumRefLayers[ idx ]; k++ )
				layerIncludedInOlsFlag[ i ][ RefLayerIdx[ idx ][ k ] ] = 1
		}
	}

For each value of i in the range of 0 to vps_max_layers_minus1, inclusive, the values of LayerUsedAsRefLayerFlag[ i ] and LayerUsedAsOutputLayerFlag[ i ] shall not be both equal to 0. In other words, there shall be no layer that is neither an output layer of at least one OLS nor a direct reference layer of any other layer.

For each OLS, there shall be at least one layer that is an output layer. In other words, for any value of i in the range of 0 to TotalNumOlss − 1, inclusive, the value of NumOutputLayersInOls[ i ] shall be greater than or equal to 1.

The variable NumLayersInOls[ i ], specifying the number of layers in the i-th OLS, and the variable LayerIdInOls[ i ][ j ], specifying the nuh_layer_id value of the j-th layer in the i-th OLS, are derived as follows:

NumLayersInOls[ 0 ] = 1
LayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
for( i = 1, NumSingleLayerOlss=1; i < TotalNumOlss; i++ ) {
	if( each_layer_is_an_ols_flag ) {
		NumLayersInOls[ i ] = 1
		LayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]		(41)
	} else if( ols_mode_idc  = =  0  | |  ols_mode_idc  = =  1 ) {
		NumLayersInOls[ i ] = i + 1
		for( j = 0; j < NumLayersInOls[ i ]; j++ )
			LayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
	} else if( ols_mode_idc  = =  2 ) {
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ )
			if( layerIncludedInOlsFlag[ i ][ k ] )
				LayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
		NumLayersInOls[ i ] = j
	}
	if( NumLayersInOls[ i ] == 1)
		NumSingleLayerOlss++
}

NOTE 1 – The 0-th OLS contains only the lowest layer (i.e., the layer with nuh_layer_id equal to vps_layer_id[ 0 ]) and for the 0-th OLS the only included layer is output.

The variable OlsLayerIdx[ i ][ j ], specifying the OLS layer index of the layer with nuh_layer_id equal to LayerIdInOls[ i ][ j ], is derived as follows:

for( i = 0; i < TotalNumOlss; i++ )
	for j = 0; j < NumLayersInOls[ i ]; j++ )		(42)
		OlsLayerIdx[ i ][ LayerIdInOls[ i ][ j ] ] = j

The lowest layer in each OLS shall be an independent layer. In other words, for each i in the range of 0 to TotalNumOlss − 1, inclusive, the value of vps_independent_layer_flag[ GeneralLayerIdx[ LayerIdInOls[ i ][ 0 ] ] ] shall be equal to 1.

Each layer shall be included in at least one OLS specified by the VPS. In other words, for each layer with a particular value of nuh_layer_id nuhLayerId equal to one of vps_layer_id[ k ] for k in the range of 0 to vps_max_layers_minus1, inclusive, there shall be at least one pair of values of i and j, where i is in the range of 0 to TotalNumOlss − 1, inclusive, and j is in the range of NumLayersInOls[ i ] − 1, inclusive, such that the value of LayerIdInOls[ i ][ j ] is equal to nuhLayerId.

…

vps_num_dpb_params specifies the number of dpb_parameters( ) syntax strutcures in the VPS. The value of vps_num_dpb_params shall be in the range of 0 to 16 (TotalNumOlss − NumSingleLayerOlss), inclusive. When not present, the value of vps_num_dpb_params is inferred to be equal to 0.

## Proposal 2

It is proposed that if the total number of output layer sets with more than one layer is equal to the number of dpb_parameters( ) syntax structures in the VPS, the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS is not signalled, instead it is inferred.

In other words when (TotalNumOlss − NumSingleLayerOlss) is equal to vps_num_dpb_params, ols_dpb_params_idx[ i ] is not signalled and instead it is inferred.

The proposed changes are shown highlighted below compared to VVC WD.

#### Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| … | u(6) |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( (vps_num_dpb_params > 1) && (vps_num_dpb_params != (TotalNumOlss – NumSingleLayerOlss))) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

#### …

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. If the value of vps_num_dpb_parameters is equal to (TotalNumOlss− NumSingleLayerOlss), the value of ols_dpb_params_idx[ i ] is inferred as follows: 
	for( i = 0,  j = 0; i < TotalNumOlss; i++ ) 
		if( NumLayersInOls[ i ] > 1 )	
			Infer ols_dpb_params_idx[ i ] to be equal to j++
Otherwise, Wwhen ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

Additional text for similar signalling and inference for HRD is provided below as per following quoted agreement in Category 1 AHG meeting from JVET-R0339 notes:

“Don't signal and instead infer the index of the dpb_parameters( ) syntax structure that applies to the i-th OLS when a condition is met (JVET-R0099 Proposal 2, JVET-R0204, JVET-R0275 aspect 4)

  - The condition is total number of output layer sets minus number of single layer output layer sets is equal to number of signalled dpb parameters (JVET-R0099 proposal 2). AHG Recommendation (expression of existing intent): Adopt, and also apply to HRD parameters. Text was to be provided by S. Deshpande and he is also to supply software.”

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !each_layer_is_an_ols_flag ) |  |
| vps_general_hrd_params_present_flag | u(1) |
| if( vps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_cpb_params_present_flag | u(1) |
| num_ols_hrd_params_minus1 | ue(v) |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| hrd_max_tid[ i ] | u(3) |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |
| } |  |
| if( num_ols_hrd_params_minus1 + 1 != (TotalNumOlss − NumSingleLayerOlss) && num_ols_hrd_params_minus1 > 0 ) |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |
| if( NumLayersInOls[ i ] > 1 ) |  |
| ols_hrd_idx[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

num_ols_hrd_params_minus1 plus 1 specifies the number of ols_hrd_parameters( ) syntax structures present in the general_hrd_parameters( ) syntax structure when vps_general_hrd_params_present_flag is equal to 1. The value of num_ols_hrd_params_minus1 shall be in the range of 0 to TotalNumOlss − NumSingleLayerOlss − 1, inclusive.

hrd_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the HRD parameters are contained in the i-th ols_hrd_parameters( ) syntax structure. The value of hrd_max_tid[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of hrd_max_tid[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of hrd_max_tid[ i ] is inferred to be equal to vps_max_sublayers_minus1.

ols_hrd_idx[ i ] specifies the index, to the list of ols_hrd_parameters( ) syntax structures in the VPS, of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

If the value of num_ols_hrd_param_minus1 + 1 is equal to (TotalNumOlss− NumSingleLayerOlss), the value of ols_hrd_idx[ i ] is inferred to be equal to i as follows: 
	for( i = 1,  j = 0; i < TotalNumOlss; i++ ) 
		if( NumLayersInOls[ i ] > 1 )	
			Infer ols_hrd_idx[ i ] to be equal to j++

Otherwise, when NumLayersInOls[ i ] is greater than 1 and num_ols_hrd_params_minus1 is equal to 0, the value of ols_hrd_idx[ i ] is inferred to be equal to 0.

## Proposal 3

It is proposed that since the 0-th OLS has one layer, the for loop which signals syntax elements  ols_dpb_pic_width[ i ], ols_dpb_pic_height[ i ], and ols_dpb_params_idx[ i ] start at 1 instead of at 0.

| for( i = 01; i < TotalNumOlss; i++ ) { |  |
| --- | --- |
| if( NumLayersInOls[ i ] > 1 ) { |  |
| ols_dpb_pic_width[ i ] | ue(v) |
| ols_dpb_pic_height[ i ] | ue(v) |
| if( vps_num_dpb_params > 1 ) |  |
| ols_dpb_params_idx[ i ] | ue(v) |
| } |  |
| } |  |

# Conclusion

This document proposes modifications to DPB parameters signalling for output layer sets. It is proposed to adopt the proposals in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001-vE, Versatile Video Coding (Draft 8), Brussels, January 2020