| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0110 |
| --- | --- |

| Title: | AHG14: Mixed lossy/lossless coding of VTM reference software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye, Jiancong Luo 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | + 1 408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

One of the important use cases of VVC is mixed lossy/lossless coding where one part of the image is lossless coded and another part of the image is lossy coded. However, current VTM reference software does not support mixed lossy/lossless coding. In order to study the performance of VVC in mixed content, a reference software implementation is necessary. This contribution proposes encoder only implementation to support slice-level mixed lossy/lossless coding where a slice can be configured to be coded either in lossy or lossless mode.

# Problem Statement

In VTM reference software, lossless coding is achieved at sequence level (entire sequence is lossless or lossy) by selecting the transform-skip mode for all of the coding blocks. In order to achieve the lossless coding at sequence level, the parameter “CostMode” is set to “lossless” in the encoder config file. In addition, all lossy coding tools such as in-loop filters are disabled for the entire sequence.

The current version of the VTM reference software does not support mixed lossy/lossless coding where one part of the image is lossless and another part of the image is lossy coded. Therefore, current reference software can not be used to study the performance of VVC in mixed lossy/lossless scenario.

# Proposed method

At the 17th JVET meeting, slice level residual coding flag slice_ts_residual_coding_disabled_flag is adopted to improve the compression performance of the lossless coding. This slice level residual coding flag inspired us to support slice level lossy/lossless coding. In this contribution, we have implemented encoder only method to achieve slice level mixed lossy/lossless coding. In the proposed method, if a picture contains multiple slices, one slice can be lossy and another slice can be lossless coded.

Following are the major features of the proposed implementation:

- The mixed lossy/lossless coding can be enabled in the encoder side by setting “CostMode=lossless”

- A slice can be defined either a lossy or lossless in the encoder config file.

- If a slice is lossless coded, all in-loop filters are disabled. For lossy slice, deblocking filter, ALF, and SAO are enabled.

- For a lossless slice, slice_ts_residual_coding_disabled_flag is set to 1. For a lossy slice, slice_ts_residual_coding_disabled_flag = 0.

# Simulation

In order to verify the proposed method, a picture is divided into multiple slices and extensive experiments were conducted. Below is the example of a slice configuration where the size of the slice is set to 3CTUx3CTU (i.e. 384x384).

#============ Tiles / Slices ================

EnablePicPartitioning         : 1

TileColumnWidthArray          : 3 3 3 3 3 3

TileRowHeightArray            : 3 3 3 3

RasterScanSlices              : 0

RectSliceFixedWidth           : 1

RectSliceFixedHeight          : 1

#slice index in raster-scan order: 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14

SliceLosslessArray          : 0 0 0 0 0 0 0 1 0 0 0  0  0  0 0

In the above configuration, “SliceLosslessArray” defines the lossy/lossless slices of a given picture in raster scan order. “0” means lossy; “1” means lossless. For instance, in the above example, 7th slice in raster scan order is lossless and all other slices are lossy.

Figure 1 shows an example of a decoded frame of mixed lossy/lossless coding generated by the proposed software. In Figure 1, a picture is divided into 15 slices and the middle slice is lossless coded and marked as “L”. All other slices are lossy.

Figure 1: Middle slice is lossless coded (Marketplace sequence,  frame 542, LB, lossy slice QP = 37)

Figure 2: Lossy/lossless slices are interleaved in raster scan order (BasketballDrillText sequence, LDB, lossy slice QP = 37)

In Figure 2, the picture is divided into 6 slices and the lossy and lossless slices are coded as interleaved manner. The lossless slices are marked as “L”. The decoded image of Figure 2 is generated by setting the “SliceLosslessArray” value as follows:

#slice index in raster-scan order: 0 1 2 3 4 5

SliceLosslessArray          : 0 1 0 1 0 1

# Conclusion

This contribution proposes the software implementation of slice level mixed lossy/lossless coding of VTM software. In the proposed implementation, a slice can be coded either in lossy or lossless mode. A config parameter “SliceLosslessArray” is added to define a slice is lossy or lossless. We recommend to adopt the proposed software implementation in next VTM version so that performance of VVC in case of mixed lossy/lossless coding scenario can be properly studied. It is further suggested to add mixed lossy/lossless coding conditions into “JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding” (JVET-Q2014).

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).