| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0118-v2 |
| --- | --- |

| Title: | AHG9/AHG12: On signaling of subpicture partitioning in SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following HLS clean-ups in SPS:

- Proposal-1: Adding pic_header_in_slice_header_constraint_flag

- Proposal-2: Alignment on subpic_treated_as_pic_flag value across layers

# Proposal-1: Adding pic_header_in_slice_header_constraint_flag

It is proposed to add the constaint flag pic_header_in_slice_header_constraint_flag in general constraint info(). When pic_header_in_slice_header_constraint_flag is equal to 1, the value of picture_header_in_slice_header_flag in PH shall be equal to 1.

Proposed Text of Proposal-1:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| one_tile_per_pic_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| pic_header_in_slice_header_constraint_flag | u(1) |
| … |  |
| } |  |

pic_header_in_slice_header_constraint_flag equal to 1 specifies that PH syntax structure is present in slice header. pic_header_in_slice_header_constraint_flag equal to 0 does not impose such a constraint. When pic_header_in_slice_header_constraint_flag shall be equal to 1, the value of picture_header_in_slice_header_flag shall be equal to 1.

# Proposal-2: Alignment on subpic_treated_as_pic_flag value across layers

For subpicture extraction from a multilayer bitstream, when subpic_treated_as_pic_flag[ i ] is equal to 1,  for each output layer and its reference layers in an OLS, all pictures in the output layer and its reference layers shall have the value of subpic_treated_as_pic_flag[ j ] equal to 1 for each value of j in the range of 0 to sps_num_subpics_minus1. Therefore, we propose to add the following constraint.

Proposed Text of Proposal-2:

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag.

When subpic_treated_as_pic_flag[ i ] is equal to 1, it is a requirement of bitstream conformance that all of the following conditions are true for each output layer and its reference layers in an OLS that includes the layer containing the i-th subpicture as an output layer:

–	All pictures in the output layer and its reference layers shall have the same value of pic_width_in_luma_samples and the same value of pic_height_in_luma_samples.

–	All the SPSs referred to by the output layer and its reference layers shall have the same value of sps_num_subpics_minus1 and shall have the same values of subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], and loop_filter_across_subpic_enabled_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	All pictures in each access unit in the output layer and its reference layers shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	All pictures in the output layer and its reference layers shall have the value of subpic_treated_as_pic_flag[ j ] equal to 1 for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).