| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0133-v2 |
| --- | --- |

| Title: | AHG16: On Clipping values for Non-linear ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In VVC WD, clipping values for Non-linear ALF filtering process are defined in a lookup table, as a form of “2N” depending on BitDepth and clipIdx, where N is an integer value. When BitDepth is equal to 16 and clipIdx is equal to 0, clipping values become 216; Thus, the lookup table needs up to 17 bits per element.

This contribution proposes to replace the clipping values in a form of “2N” to a form of “2N - 1”; Any clipping values are kept within 16 bits and can be derived by simple logical operation without the lookup table. It is asserted that the proposed method has negligible bdrate changes of (0.00%,0.00%, 0.00%) for AI, (0.00%, -0.05%, 0.01%) for RA and (0.01%, -0.06%, -0.17%) for LDB under CTC.

In v2, results of IBDI equal to 8/12 are attached.

It is observed that:

- For IBDI=8, average bdrate changes are (0.02%, 0.10%, 0.11%) for AI, (0.03%, -0.19%, -0.21%) for RA and (0.02%, -0.40%, -0.44%) for LB.

- For IBDI=12, average bdrate changes are (0.00%, -0.01%, 0.01%) for AI, (0.00%, -0.03%, 0.07%) for RA and (0.05%, 0.03%, 0.04%) for LB.

Introduction

In VVC WD, clipping values for Non-linear ALF filtering process are defined in a lookup table, as a form of “2N” depending on BitDepth and clipIdx as shown in Table 1-1. When BitDepth is equal to 16 and clipIdx is equal to 0, clipping values become 216; Thus, the lookup table needs up to 17 bits per element, which seems redundant.

Note that any clipping value in Table 1-1 can be derived by following equation without lookup table.

AlfClip = ( 1 << ( BitDepth – 2 * clipIdx – ( clipIdx > 0 ) ) )

Table 1-1- Clipping values in VVC WD

| BitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 28 | 25 | 23 | 21 |
| 9 | 29 | 26 | 24 | 22 |
| 10 | 210 | 27 | 25 | 23 |
| 11 | 211 | 28 | 26 | 24 |
| 12 | 212 | 29 | 27 | 25 |
| 13 | 213 | 210 | 28 | 26 |
| 14 | 214 | 211 | 29 | 27 |
| 15 | 215 | 212 | 210 | 28 |
| 16 | 216 | 213 | 211 | 29 |

Proposal

To keep bitdepth of clipping values within 16 bits, it is proposed to replace the clipping values in a form of “2N” to a form of “2N - 1” as shown in Table 2-1. In addition, any clipping values in Table 2-1 can be derived by following simple equation.

AlfClip = ( 1 << ( BitDepth – 2 * clipIdx – ( clipIdx > 0 ) ) ) – 1

Table 2-1- Proposed clipping values

| BitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 28–1 | 25–1 | 23–1 | 21–1 |
| 9 | 29–1 | 26–1 | 24–1 | 22–1 |
| 10 | 210–1 | 27–1 | 25–1 | 23–1 |
| 11 | 211–1 | 28–1 | 26–1 | 24–1 |
| 12 | 212–1 | 29–1 | 27–1 | 25–1 |
| 13 | 213–1 | 210–1 | 28–1 | 26–1 |
| 14 | 214–1 | 211–1 | 29–1 | 27–1 |
| 15 | 215–1 | 212–1 | 210–1 | 28–1 |
| 16 | 216–1 | 213–1 | 211–1 | 29–1 |

For spec changes, two options are provided:

Option#1: Remove the lookup table and derive clipping values by the proposed logical operation

Option#2: Change clipping values to “2N - 1” in the lookup table and take a note that any clipping values can be derived by the proposed logical operation.

Two options are identical from algorithm point of view, which differ in expression.

Authors prefer Option#1 since it can also cleanup text by removing the lookup table.

Simulation Results

The proposed method is implemented on the VTM-8.0 and tested under the common test condition [2] as shown in Table 3-1 (Test#1).

To see impacts to coding performance under other IBDI setting, additional simulations are conducted:

- Test#2: VTM-8.0 vs JVET-R0133 with IBDI=8

- Test#3: VTM-8.0 vs JVET-R0133 with IBDI=12

For Test#2, bugfix for ticket#987 is applied to both the anchor and the tested.

Note that runtimes for Test#2/Test#3 are not reliable due to heterogeneous grids.

Table 3-1- Coding performance under CTC (Test#1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 104% | 101% |
| Class A2 | 0.00% | 0.01% | 0.00% | 103% | 102% |
| Class B | 0.00% | -0.01% | 0.01% | 102% | 102% |
| Class C | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class E | 0.00% | 0.01% | -0.01% | 98% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.01% | 101% | 101% |
| Class F | 0.01% | 0.00% | -0.02% | 103% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | 0.01% | 100% | 98% |
| Class A2 | -0.01% | -0.05% | 0.13% | 100% | 99% |
| Class B | 0.01% | -0.07% | -0.02% | 101% | 100% |
| Class C | 0.00% | -0.02% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | 0.01% | 100% | 99% |
| Class D | 0.03% | 0.18% | -0.02% | 101% | 101% |
| Class F | 0.04% | 0.02% | 0.01% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | -0.05% | 102% | 100% |
| Class C | -0.05% | -0.13% | -0.17% | 98% | 98% |
| Class E | 0.11% | -0.24% | -0.38% | 99% | 99% |
| Overall | 0.01% | -0.06% | -0.17% | 100% | 99% |
| Class D | -0.06% | -0.29% | -0.44% | 101% | 101% |
| Class F | -0.03% | 0.47% | -0.24% | 101% | 101% |

Table 3-2: Coding performance under CTC (Test#2, IBDI=8)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.17% | 0.16% |  |  |
| Class A2 | 0.02% | 0.10% | 0.09% |  |  |
| Class B | 0.02% | 0.09% | 0.10% |  |  |
| Class C | 0.03% | 0.07% | 0.12% |  |  |
| Class E | 0.02% | 0.10% | 0.07% |  |  |
| Overall | 0.02% | 0.10% | 0.11% |  |  |
| Class D | 0.02% | 0.08% | 0.09% |  |  |
| Class F | 0.08% | 0.05% | 0.09% |  |  |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.29% | -0.17% |  |  |
| Class A2 | 0.02% | -0.25% | -0.17% |  |  |
| Class B | 0.02% | -0.18% | -0.33% |  |  |
| Class C | 0.05% | -0.08% | -0.10% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.19% | -0.21% |  |  |
| Class D | 0.00% | -0.21% | 0.01% |  |  |
| Class F | 0.11% | -0.15% | -0.07% |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.35% | -0.72% |  |  |
| Class C | 0.07% | -0.48% | -0.56% |  |  |
| Class E | 0.03% | -0.37% | 0.17% |  |  |
| Overall | 0.02% | -0.40% | -0.44% |  |  |
| Class D | -0.09% | -0.79% | -0.91% |  |  |
| Class F | 0.17% | -0.04% | 0.08% |  |  |

Table 3-3: Coding performance under CTC (Test#2, IBDI=12)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.02% |  |  |
| Class A2 | 0.00% | -0.01% | 0.00% |  |  |
| Class B | 0.00% | -0.01% | -0.01% |  |  |
| Class C | 0.00% | -0.01% | 0.01% |  |  |
| Class E | 0.00% | 0.00% | 0.01% |  |  |
| Overall | 0.00% | -0.01% | 0.01% |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |
| Class F | 0.00% | 0.02% | -0.03% |  |  |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | -0.09% |  |  |
| Class A2 | -0.01% | -0.10% | 0.08% |  |  |
| Class B | -0.02% | -0.02% | 0.06% |  |  |
| Class C | 0.00% | -0.01% | 0.21% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.07% |  |  |
| Class D | 0.03% | 0.15% | -0.04% |  |  |
| Class F | -0.02% | 0.05% | -0.08% |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.08% | -0.14% |  |  |
| Class C | 0.03% | 0.17% | 0.29% |  |  |
| Class E | 0.19% | -0.25% | 0.00% |  |  |
| Overall | 0.05% | 0.03% | 0.04% |  |  |
| Class D | 0.00% | 0.51% | -0.29% |  |  |
| Class F | 0.00% | -0.19% | -0.74% |  |  |

As can be seen, the proposed method has negligible impacts to coding performance under various IBDI settings.

Conclusion

In this contribution, modified clipping values for Non-linear ALF are proposed, which has negligible coding performance changes. It is recommended to adopt either of the proposed methods into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8), ” JVET-Q2001, Jan. 2020

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video, ” JVET-N1010, May 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Spec Changes

Proposed changes are highlighted in yellow over JVET-Q2001vE.

Option#1 (Remove LUT)

- 7.4.3.19 Adaptive loop filter data semantics

…

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The luma filter clipping values AlfClipL[ adaptation_parameter_set_id ] with elements AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as specified in Table 8 depending on BitDepth and clipIdx set equal to alf_luma_clip_idx[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ] as below:.

AlfClipL [ adaptation_parameter_set_id ][ filtIdx ][ j ] =
                                        (1<<(BitDepth – 2*clipIdx – (clipIdx>0))) – 1	(X1)

…

alf_chroma_clip_idx[ altIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the alternative chroma filter with index altIdx. It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ altIdx ][ j ] with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 shall be in the range of 0 to 3, inclusive.

The chroma filter clipping values AlfClipC[ adaptation_parameter_set_id ][ altIdx ] with elements AlfClipC[ adaptation_parameter_set_id ][ altIdx ][ j ], with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 are derived as specified in Table 8 depending on BitDepth and clipIdx set equal to alf_chroma_clip_idx[ altIdx ][ j ] as bellow:.

AlfClipC[ adaptation_parameter_set_id ][ altIdx ] = 
                                 (1<<(BitDepth – 2*clipIdx – (clipIdx>0))) – 1  	(X2)

Table 8 –Specification AlfClip depending on BitDepth and clipIdx

| BitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 28 | 25 | 23 | 21 |
| 9 | 29 | 26 | 24 | 22 |
| 10 | 210 | 27 | 25 | 23 |
| 11 | 211 | 28 | 26 | 24 |
| 12 | 212 | 29 | 27 | 25 |
| 13 | 213 | 210 | 28 | 26 |
| 14 | 214 | 211 | 29 | 27 |
| 15 | 215 | 212 | 210 | 28 |
| 16 | 216 | 213 | 211 | 29 |

Option#2 (Only change values in LUT)

- 7.4.3.19 Adaptive loop filter data semantics

…

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The luma filter clipping values AlfClipL[ adaptation_parameter_set_id ] with elements AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as specified in Table 8 depending on BitDepth and clipIdx set equal to alf_luma_clip_idx[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ].

…

alf_chroma_clip_idx[ altIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the alternative chroma filter with index altIdx. It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ altIdx ][ j ] with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 shall be in the range of 0 to 3, inclusive.

The chroma filter clipping values AlfClipC[ adaptation_parameter_set_id ][ altIdx ] with elements AlfClipC[ adaptation_parameter_set_id ][ altIdx ][ j ], with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 are derived as specified in Table 8 depending on BitDepth and clipIdx set equal to alf_chroma_clip_idx[ altIdx ][ j ].

Table 8 – Specification AlfClip depending on BitDepth and clipIdx

| BitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 28 -1 | 25-1 | 23-1 | 21-1 |
| 9 | 29-1 | 26-1 | 24-1 | 22-1 |
| 10 | 210-1 | 27-1 | 25-1 | 23-1 |
| 11 | 211-1 | 28-1 | 26-1 | 24-1 |
| 12 | 212-1 | 29-1 | 27-1 | 25-1 |
| 13 | 213-1 | 210-1 | 28-1 | 26-1 |
| 14 | 214-1 | 211-1 | 29-1 | 27-1 |
| 15 | 215-1 | 212-1 | 210-1 | 28-1 |
| 16 | 216-1 | 213-1 | 211-1 | 29-1 |

NOTE X – Clipping values AlfClip in Table 8 are derived depending on BitDepth and clipIdx as below:

AlfClip = ( 1 << ( BitDepth – 2 * clipIdx – ( clipIdx > 0 ) ) ) – 1  				(X1)