| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0140_r1 |
| --- | --- |

| Title: | AHG14: Max BT/TT size restriction for lossless coding encoder configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Eiichi Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a max BT/TT size restriction for lossless coding encoder configuration, which set MAX_BT_SIZE and MAX_TT_SIZE being equal to 8 Comparing to the current VTM-8.0 lossless anchor, the proposed modification provides average bitsaving difference of 0.32 %, 0.22 % and 0.24 %  with 44 %, 22 % and 44 % encoding runtime on AI, RA and LB configuration respectively.

Introduction

At the 17th JVET meeting in Brussels, it was reported that compared to the HM-16.20 lossless coding results, the VTM-7.0 lossless anchor results show 3133% and 1339% encoding runtime increase in AI and RA [1] respectively.

The authors tested several encoding configurations and found that significant encoding runtime saving can be possible by using BT/TT size restriction, which shows similar performance of bit ratio.

Proposal

In this contribution, it is proposed to set MAX_BT_SIZE and MAX_TT_SIZE equal to 8 in lossless coding mode. The proposed encoder cfg is attached.

The following conditions were added to the lossless configuration file.

MaxBTLumaISlice              : 8

MaxBTNonISlice               : 8

MaxTTLumaISlice              : 8

MaxTTNonISlice               : 8

Test results

The test results of the proposal is summarised in Table 1. The test is performed using the JVET lossless test condition [2] with the proposed change. The software was made on top of VTM 8.0.

Table 1: Results of proposed modification, MAX_BT/TT_SIZE = 8 for lossless VVC

| Reference: | vtm80 lossless |
| --- | --- |
| Test: | Proposed method |

Conclusion

This contribution proposes a max BT/TT size restriction for lossless coding encoder configuration. It is recommended to adopt this configuration change in the next VTM software.

Reference

[1]	T. Nguyen, T. C. Ma, M. Ikeda, H. Jang, X. Zhao, “JVET AHG report: Lossless and near-lossless coding tools,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2014, 17th Meeting: Brussels, BE, 7–17 January 2020.

[2]	T. Nguyen, T. C. Ma, A. Nalci, “JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2014, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).