| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0182 |
| --- | --- |

| Title: | AhG 9: Removed Redundant Slice Level TSRC Flag Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Tangi Poirier Michel Kerdranvat | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In the 17th JVET meeting, a slice level flag for disabling transform skip residual coding (TSRC) in JVET-Q0082. Whether transform skip itself is activated or not, this flag is independently coded. This contribution proposes coding this flag conditionally to transform skip, which leads to removed redundant coding at slice level.

# Introduction

The residuals of transform skip stage can either be coded in regular residual coding (RRC) or transform skip residual coding (TSRC) using the slice level flag slice_ts_residual_coding_disabled_flag. This is mainly used for lossless coding where RRC provides better coding performance for camera captured contents.

At SPS level, transform skip can be disabled by the flag sps_transform_skip_enabled_flag. That is, all residuals pass through the transform stage and only RRC is used if sps_transform_skip_enabled_flag is set to zero. In this case, it becomes redundant and does not need to be coded. Therefore, this contribution proposes coding slice_ts_residual_coding_disabled_flag only if transform skip is enabled.

# Proposal

slice_ts_residual_coding_disabled_flag is conditionally coded if sps_transform_skip_enabled_flag equals to one. That is, switching between RRC and TSRC is allowed if transform skip is allowed. The corresponding specification changes are shown below (added part is in blue):

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (sps_transform_skip_enabled_flag) { |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).