| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0199-v1 |
| --- | --- |

| Title: | AHG9: On vps_max_sublayers_minus1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim, Jaehong Jung, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 dongcheol.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes the following changes:

- The maximum value of vps_max_sublayers_minus1 is constrained by dci_max_sublayers_minus1 when present.

- It is proposed to specify an inference rule for the syntax element vps_max_sublayers_minus1 when sps_video_parameter_set_id is equal to 0.

Introduction

On vps_max_sublayers_minus1

The syntax element dci_max_sublayers_minus1 in DCI indicates the maximum allowed number of temporal sublayers in entire bitstreams when present.

The semantics of vps_max_sublayers_minus1 is defined as follows:

- vps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in a layer in each CVS referring to the VPS. The value of vps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive.

In the current draft [1], the maximum number of temporal sublayers may be signaled in DCI, VPS, or SPS. The maximum value of sps_max_sublayers_minus1 in SPS is constrained by the value of vps_max_sublayers_minus1. This applies to the same syntax element in SPSs referring to the VPS. However, there is no constraint on the maximum value of vps_max_sublayers_minus1. It may incur interoperability problem when the value of vps_max_sublayers_minus1 is greater than the value of dci_max_sublayers_minus1 that is present in DCI.

An inference rule for vps_max_sublayers_minus1

When sps_video_parameter_set_id is equal to 0, the syntax element vps_max_sublayers_minus1 is not present. This syntax element in VPS is used to specify the maximum range of sps_max_sublayers_minus1 in SPS referring to the VPS as follows:

- sps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in each CLVS referring to the SPS. The value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive.

Therefore, it’s needed to specify the inference rule for vps_max_sublayers_minus1 when not present.

Proposed spec text changes

Proposed spec text changes are highlighted in green below.

On vps_max_sublayers_minus1

7.4.3.2	Video parameter set RBSP semantics

…

vps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in a layer in each CVS referring to the VPS. When dci_max_sublayers_minus1 is present, Tthe value of vps_max_sublayers_minus1 shall be in the range of 0 to 6dci_max_sublayers_minus1, inclusive. When dci_max_sublayers_minus1 is not present, the value of vps_max_sublayers_minu1 shall be in the range of 0 to 6, inclusive.

…

An inference rule for vps_max_sublayers_minus1

7.4.3.3	Sequence parameter set RBSP semantics

…

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS.

When sps_video_parameter_set_id is equal to 0, the following applies:

–	The SPS does not refer to a VPS.

–	No VPS is referred to when decoding each CLVS referring to the SPS.

–	The value of vps_max_layers_minus1 is inferred to be equal to 0.

–	The CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id).

–	The value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0.

–	The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

–	When dci_max_sublayers_minus1 is present, the value of vps_max_sublayers_minus1 is inferred to be equal to dci_max_sublayers_minus1. When dci_max_sublayers_minus1 is not present, the value of vps_max_sublayers_minus1 is inferred to be equal to 6.

…

Reference

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8),” JVET-Q2001, Jan. 2020.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).