| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0226-v1 |
| --- | --- |

| Title: | AHG9: Proposal to change the definition of trailing picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian Zhi Zhang Jack Enhorn Ruoyang Yu | Email: | rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The proponents of this proposal claim that STSA pictures are trailing pictures in HEVC but not in VVC, and that the rules for trailing pictures therefore do not apply for STSA pictures. It is proposed to include STSA pictures in the group of trailing pictures so that the rules do apply also for STSA pictures.

The proponents also claim that bitstreams starting with a GDR picture may contain pictures that do not have any associated IRAP picture and that the current VVC draft disallows the use of TRAIL_NUT pictures for this case. The proponents believe that TRAIL_NUT pictures should be allowed for pictures that do not have any associated IRAP picture.

The proposal is summarized as follows by the proponents:

- Include STSA pictures and pictures with no associated IRAP picture into the trailing picture type using the following text changes shown in yellow highlights:

trailing picture: A non-IRAP picture that either follows the associated IRAP picture in output order or has no associated IRAP picture and is not an STSA picture.

NOTE – Trailing pictures associated with an IRAP picture also follow the IRAP picture in decoding order. Pictures that follow the associated IRAP picture in output order and precede the associated IRAP picture in decoding order are not allowed. A bitstream that starts with a GDR picture may contain pictures that have no associated IRAP picture.

- Only allow pictures having an associated IRAP picture to be “leading pictures”.

- Condition the conformance requirements regarding relationships between trailing pictures and associated IRAP pictures to only apply to trailing pictures that do have an associated IRAP picture.

- Add the following rule: “Any picture that precedes a GDR picture in decoding order shall precede the GDR picture in output order”

# Introduction

The VVC draft (JVET-Q2001-vE) contains the following definition of “trailing picture”:

trailing picture: A non-IRAP picture that follows the associated IRAP picture in output order and is not an STSA picture.

NOTE – Trailing pictures associated with an IRAP picture also follow the IRAP picture in decoding order. Pictures that follow the associated IRAP picture in output order and precede the associated IRAP picture in decoding order are not allowed.

The HEVC specification contains the following definition:

trailing picture: A non-IRAP picture that follows the associated IRAP picture in output order.

NOTE – Trailing pictures associated with an IRAP picture also follow the IRAP picture in decoding order. Pictures that follow the associated IRAP picture in output order and precede the associated IRAP picture in decoding order are not allowed.

From the texts above it is clear that the VVC definition excludes STSA pictures from trailing pictures while in HEVC they are included. Actually, in HEVC any STSA picture is also a trailing picture since all leading pictures shall be either a RADL or RASL picture.

Note the following definition and rule in the VVC specification:

leading picture: A picture that is in the same layer as the associated IRAP picture and precedes the associated IRAP picture in output order.

When a picture is a leading picture of an IRAP picture, it shall be a RADL or RASL picture.

From this definition and rule it is clear that an STSA picture in VVC must follow its associated IRAP picture in output order but the STSA picture is not a trailing picture.

# Analysis

We analyzed the current VVC specification and studied some particular picture type cases. As a reminder, the current VVC specification contains the following 8 VCL NAL unit types:

- TRAIL_NUT

- STSA_NUT

- RADL_NUT

- RASL_NUT

- IDR_W_RADL

- IDR_N_LP

- CRA_NUT

- GDR_NUT

## Including STSA pictures as trailing pictures

In the current VVC specification, the constraints for trailing pictures do not apply to STSA pictures since the STSA pictures are not trailing pictures. That means that is seems allowed for e.g. an STSA picture to use a RASL picture for reference while that is not allowed for a picture decoded from NAL units of type TRAIL_NUT. A straightforward solution is to include STSA pictures as trailing pictures, similar to what is done in HEVC. Then the STSA pictures will need to follow the rules for trailing pictures which seems desirable to us.

## Using TRAIL_NUT and STSA_NUT for pictures that follows a GDR picture in both output and decoding order

A VVC bitstream can start with a GDR picture. It seems desirable in this case to use the TRAIL_NUT for pictures that follow the GDR picture in decoding order, but that does not seem to be supported in the current VVC draft since a picture in a bitstream that started with a GDR picture may not have any associated IRAP picture. The current draft includes the following:

- A TRAIL_NUT is according to the draft a “Coded slice of a non-STSA trailing picture”

- A trailing picture is according to the draft “A non-IRAP picture that follows the associated IRAP picture in output order and is not an STSA picture”.

We propose that both the TRAIL_NUT and STSA_NUT types should be allowed for non-GDR pictures that do not have any associated IRAP picture. This is exemplified in the table below.

Example 1 – We propose that these two combinations should be allowed

| Decoding order | POC | NUT |
| --- | --- | --- |
| 0 | 0 | GDR_NUT |
| 1 | 1 | TRAIL_NUT or STSA_NUT |

Note that these two combinations would also be allowed in bitstreams in which an IRAP picture precedes the GDR picture in decoding order (given that the POC of the IRAP picture is smaller than 0).

## Picture types for pictures that follow a GDR picture decoding order and precede it in output order

TRAIL_NUT pictures and STSA_NUT pictures that follows a GDR picture decoding order and precede it in output order can be seen as leading pictures to the GDR picture. We see the following options to handle this case:

- Disallow pictures that follow a GDR picture in decoding order to precede it in output order

- See the GDR picture as an IRAP picture and mandate the use of RADL and RASL pictures

- See the GDR picture as a trailing picture and allow “leading pictures of a GDR picture” to have the STSA_NUT and TRAIL_NUT type

We propose to go with the third approach for the following reasons:

- It seems useful to allow open-GOP random access using GDR pictures where the GDR picture itself is the refresh picture. Therefore, we propose to allow pictures that follow a GDR picture in decoding order to precede it in output order.

- Including the GDR picture in the set of IRAP pictures seems to be a large specification change and may cause text instability problems at this late stage.

- Using the RADL and RASL picture types as suggested in approach 2 would either disallow the leading pictures to be used for reference for trailing pictures of the GDR picture or require the behavior of RADL and RASL to change depending on the associated picture type. Both seem undesirable to us.

This means that we suggest that the cases in Example 2 below should be allowed and the cases in Example 3 should not be allowed.

Example 2 – We propose that these two combinations should be allowed

| Decoding order | POC | NUT |
| --- | --- | --- |
| 0 | 0 | GDR_NUT |
| 1 | -1 | TRAIL_NUT or STSA_NUT |

Example 3 – We propose that these four combinations should not be allowed

| Decoding order | POC | NUT |
| --- | --- | --- |
| 0 | 0 | GDR_NUT |
| 1 | -1 or 1 | RADL_NUT or RASL NUT |

## TRAIL_NUT or STSA_NUT pictures that precede a GDR picture in decoding order and follows it in output order

Finally, we studied the case where a picture that precedes a GDR picture in decoding order follows it in output order. Since this is not allowed around IRAP pictures we propose to not allow it for GDR pictures either.

Example 4 – We propose that these two combinations should not be allowed

| Decoding order | POC | NUT |
| --- | --- | --- |
| 0 | 1 | TRAIL_NUT or STSA_NUT |
| 1 | 0 | GDR_NUT |

# Proposed specification text changes

We propose the following changes to the VVC specification. Added text is shown by yellow highlights and deleted text is shown by strikethrough yellow highlight.

# Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

...

  - associated IRAP picture (of a particular picture): The previous IRAP picture in decoding order (when present) having the same value of nuh_layer_id as the particular picture.

...

  - leading picture: A picture with an associated IRAP picture that is in the same layer as the associated IRAP picture and precedes the associated IRAP picture in output order.

...

  - trailing picture: A non-IRAP picture that either follows the associated IRAP picture in output order or has no associated IRAP picture and is not an STSA picture.

NOTE – Trailing pictures associated with an IRAP picture also follow the IRAP picture in decoding order. Pictures that follow the associated IRAP picture in output order and precede the associated IRAP picture in decoding order are not allowed. A bitstream that starts with a GDR picture may contain pictures that have no associated IRAP picture.

...

      - NAL unit header semantics

...

Table 5 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a non-STSA trailing picture slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |

...

For VCL NAL units of any particular picture, the following applies:

–	...

For a single-layer bitstream, the following constraints apply: [Ed. (YK): Further study the following constraints for mixed_nalu_types_in_pic_flag is equal to 1 and for multi-layer bitstreams.]

–	Each picture, other than the first picture in the bitstream in decoding order, is considered to be associated with the previous IRAP picture in decoding order.

–	When a picture is a leading picture of an IRAP picture, it shall be a RADL or RASL picture.

–	When a picture is a trailing picture of an IRAP picture, it shall not be a RADL or RASL picture.

–	No RASL pictures shall be present in the bitstream that are associated with an IDR picture.

–	No RADL pictures shall be present in the bitstream that are associated with an IDR picture having nal_unit_type equal to IDR_N_LP.

NOTE 6 – It is possible to perform random access at the position of an IRAP PU by discarding all PUs before the IRAP PU (and to correctly decode the IRAP picture and all the subsequent non-RASL pictures in decoding order), provided each parameter set is available (either in the bitstream or by external means not specified in this Specification) when it is referenced.

–	Any picture that precedes an IRAP picture in decoding order shall precede the IRAP picture in output order and shall precede any RADL picture associated with the IRAP picture in output order.

–	Any RASL picture associated with a CRA picture shall precede any RADL picture associated with the CRA picture in output order.

–	Any RASL picture associated with a CRA picture shall follow, in output order, any IRAP picture that precedes the CRA picture in decoding order.

–	If field_seq_flag is equal to 0 and the current picture is a leading picture associated with an IRAP picture, it shall precede, in decoding order, all non-leading pictures that are associated with the same IRAP picture. Otherwise, let picA and picB be the first and the last leading pictures, in decoding order, associated with an IRAP picture, respectively, there shall be at most one non-leading picture preceding picA in decoding order, and there shall be no non-leading picture between picA and picB in decoding order.

- Any picture that precedes a GDR picture in decoding order shall precede the GDR picture in output order.

...

    - Decoding process for reference picture lists construction

...

It is a requirement of bitstream conformance that the following constraints apply:

- For each i equal to 0 or 1, num_ref_entries[ i ][ RplsIdx[ i ] ] shall not be less than NumRefIdxActive[ i ].

- The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be present in the DPB and shall have TemporalId less than or equal to that of the current picture.

- The picture referred to by each entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall not be the current picture and shall have non_reference_picture_flag equal to 0.

- An STRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of a picture and an LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of the same slice or a different slice of the same picture shall not refer to the same picture.

- There shall be no LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] for which the difference between the PicOrderCntVal of the current picture and the PicOrderCntVal of the picture referred to by the entry is greater than or equal to 224.

- Let setOfRefPics be the set of unique pictures referred to by all entries in RefPicList[ 0 ] that have the same nuh_layer_id as the current picture and all entries in RefPicList[ 1 ] that have the same nuh_layer_id as the current picture. The number of pictures in setOfRefPics shall be less than or equal to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified in clause A.4.2, and setOfRefPics shall be the same for all slices of a picture.

- When the current slice has nal_unit_type equal to STSA_NUT, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture, there shall be no picture that precedes the STSA picture in decoding order, has TemporalId equal to that of the current picture, and has nuh_layer_id equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ].

- When the current picture is a CRA picture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present).

- When the current picture is a trailing picture with an associated IRAP picture, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture.

- When the current picture is a trailing picture with an associated IRAP picture that follows, in both decoding order and output order, one or more leading pictures associated with the same IRAP picture, if any, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture.

- When the current picture is a recovery point picture or a picture that follows the recovery point picture in output order, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that contains a picture that was generated by the decoding process for generating unavailable reference pictures for the GDR picture of the recovery point picture.

- When the current picture is a trailing picture with an associated IRAP picture, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order.

- When the current picture is a trailing picture with an associated IRAP picture that follows, in both decoding order and output order, one or more leading pictures associated with the same IRAP picture, if any, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order.

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following: [Ed. (YK): Further study for the case of a picture with mixed RASL and RADL NAL unit types.]

  - A RASL picture

  - A picture that was generated by the decoding process for generating unavailable reference pictures

  - A picture that precedes the associated IRAP picture in decoding order

- The picture referred to by each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall be in the same AU as the current picture.

- The picture referred to by each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall be present in the DPB and shall have nuh_layer_id less than that of the current picture.

- Each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

...

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).