| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0241 |
| --- | --- |

| Title: | AHG12: A direct signalling method of rectangular slice partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Li Zhang1, Ye-Kui Wang1, Zhipin Deng2, Jizheng Xu1, Hongbin Liu2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current VVC text, rectangular slice partitioning is signalled based on the tile partitioing. Because a rectangular slice may be a rectangular region comprising several tiles, or it may be a rectangular region inside a tile, the current signalling method of rectangular slice partitioning incorporates quite a lot of logics to handle the two cases in a single scheme. In this contribution, it is proposed to signal a slice directly by its top-left postion and width/height, in a similar way as signalling of subpictures.

# Problem Statement

In the current VVC text, rectangular slice partitioning is signalled based on the tile partitioing. Because a rectangular slice may be a rectangular region comprising several tiles, or it may be a rectangular region inside a tile, the current signalling method of rectangular slice partitioning incorporates quite a lot of logics to handle the two cases in a single scheme.

# Proposed solution

To simplify the signalling method of rectangular slice partitioning, it is proposed to signal a slice directly by its top-left postion and width/height, in a similar way as signalling of subpictures.

The proposed text changes based on JVET-Q2001-vE are as follows, with the deleted text and newly added text highlighted in turquois and yellow, respectively.

### 6.5.1 CTB raster scanning, tile scanning, and subpicture scanning processes

…

When rect_slice_flag is equal to 1, the list NumCtusInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the number of CTUs in the i-th slice, the list SliceTopLeftTileIdx[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the tile index of the tile containing the first CTU in the slice, and the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, and j ranging from 0 to NumCtusInSlice[ i ] − 1, inclusive, specifying the picture raster scan address of the j-th CTB within the i-th slice, and the variable NumSlicesInTile[ i ], specifying the number of slices in the tile containing the i-th slice, are derived as follows:

if( single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ ) {
		NumCtusInSlice[ i ] = 0 
		if( subpicHeightLessThanOneTileFlag[ i ] ) /* The slice consists of a number of CTU rows in a tile. */
			AddCtbsToSlice( i, subpic_ctu_top_left_x[ i ],
				subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1, subpic_ctu_top_left_y[ i ],
				subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 )
		else { /* The slice consists of a number of complete tiles covering a rectangular region. */
			tileX = CtbToTileColBd[ subpic_ctu_top_left_x[ i ] ]
			tileY = CtbToTileRowBd[ subpic_ctu_top_left_y[ i ] ]
			for( j = 0; j < SubpicHeightInTiles[ i ]; j++ )
				for( k = 0; k < SubpicWidthInTiles[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ], tileRowBd[ tileY + j ],
							tileRowBd[ tileY + j + 1 ] )
		}
	}
} else {
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
		if( slice_represented_in_ctb_flag[ i ] )
			AddCtbsToSlice( i, slice_top_left_x[ i ], slice_top_left_x[ i ] + slice_width_minus1[ i ] + 1,
				slice_top_left_y[ i ], slice_top_left_y[ i ] + slice_height_minus1[ i ] + 1 )
		else		
			AddCtbsToSlice( i, tileColBd[ slice_top_left_x[ i ] ], tileColBd[ slice_top_left_x[ i ] +
				slice_width_minus1[ i ] + 1 ], tileRowBd[ slice_top_left_y[ i ]], tileRowBd[ slice_top_left_y[ i ] +
				slice_height_minus1[ i ] + 1 ] )
		}	
	tileIdx = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
		NumCtusInSlice[ i ] = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
		SliceTopLeftTileIdx[ i ] = tileIdx
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		if( i < num_slices_in_pic_minus1 ) {
			sliceWidthInTiles[ i ] = slice_width_in_tiles_minus1[ i ] + 1
			sliceHeightInTiles[ i ] = slice_height_in_tiles_minus1[ i ] + 1
		} else {
			sliceWidthInTiles[ i ] = NumTileColumns − tileX
			sliceHeightInTiles[ i ] = NumTileRows − tileY
			NumSlicesInTile[ i ] = 1
		}
		if( slicWidthInTiles[ i ]  = =  1  &&  sliceHeightInTiles[ i ]  = =  1 ) {	(30)
			if( num_exp_slices_in_tile[ i ]  = = 0 ) {
				NumSlicesInTile[ i ] = 1
				sliceHeightInCtus[ i ] = RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ]
			} else {
				remainingHeightInCtbsY = RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ]
				for( j = 0; j < num_exp_slices_in_tile[ i ] − 1; j++ ) {
					sliceHeightInCtus[ i + j ] = exp_slice_height_in_ctus_minus1[ i ][ j ] + 1
					remainingHeightInCtbsY   −=  sliceHeightInCtus[ i + j ]
				}
				uniformSliceHeight = exp_slice_height_in_ctus_minus1[ i ][ j ] + 1
				while( remainingHeightInCtbsY  >=  uniformSliceHeight ) {
					sliceHeightInCtus[ i + j ] = uniformSliceHeight
					remainingHeightInCtbsY  −=  uniformSliceHeight
					j++
				}
				if( remainingHeightInCtbsY > 0 ) {
					sliceHeightInCtus[ i + j ] = remainingHeightInCtbsY
					j++
				}
				NumSlicesInTile[ i ] = j
			}
			ctbY = tileRowBd[ tileY ]
			for( j = 0; j < NumSlicesInTile[ i ]; j++ ) {
				AddCtbsToSlice( i + j, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + sliceHeightInCtus[ i + j ] )
				ctbY  +=  sliceHeightInCtus[ i + j ]
			}
			i  +=  NumSlicesInTile[ i ] − 1
		} else
			for( j = 0; j < sliceHeightInTiles[ i ]; j++ )
				for( k = 0; k < sliceWidthInTiles[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],
						tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )
		if( i < num_slices_in_pic_minus1 ) {
			if( tile_idx_delta_present_flag )
				tileIdx  +=  tile_idx_delta[ i ]
			else {
				tileIdx  +=  sliceWidthInTiles[ i ]
				if( tileIdx % NumTileColumns  = =  0 )
					tileIdx  +=  ( sliceHeightInTiles[ i ] − 1 ) * NumTileColumns
			}
		}
	}
}

Where the function AddCtbsToSlice( sliceIdx, startX, stopX, startY, stopY) is specified as follows:

for( ctbY = startY; ctbY < stopY; ctbY++ )
	for( ctbX = startX; ctbX < stopX; ctbX++ ) {
		CtbAddrInSlice[ sliceIdx ][ NumCtusInSlice[ sliceIdx ] ] = ctbY * PicWidthInCtbsY + ctbX	(31)
		NumCtusInSlice[ sliceIdx ]++
	}

It is a requirement of bitstream conformance that the values of NumCtusInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, shall be greater than 0. Additionally, it is a requirement of bitstream conformance that the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, and j ranging from 0 to NumCtusInSlice[ i ] − 1, inclusive, shall include each of all CTB addresses in the range of 0 to PicSizeInCtbsY − 1, inclusive, once and only once.

…

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| for( i = 0; num_slices_in_pic_minus1 > 0 && i <= num_slices_in_pic_minus1; i++ ) { |  |
| slice_represented_in_ctb_flag[ i ] | u(1) |
| if( i > 0 && pic_width_in_luma_samples > CtbSizeY ) |  |
| slice_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_in_luma_samples > CtbSizeY ) { |  |
| slice_top_left_y[ i ] | u(v) |
| if( i < num_slices_in_pic_minus1 && pic_width_in_luma_samples > CtbSizeY ) |  |
| slice_width_minus1[ i ] | u(v) |
| if( i < num_slices_in_pic_minus1 && pic_height_in_luma_samples > CtbSizeY ) |  |
| slice_height_minus1[ i ] | u(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

...

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0. When single_slice_per_subpic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1.

tile_idx_delta_present_flag equal to 0 specifies that tile_idx_delta[ i ] syntax elements are not present in the PPS and all pictures referring to the PPS are partitioned into rectangular slice rows and rectangular slice columns in slice raster order. tile_idx_delta_present_flag equal to 1 specifies that tile_idx_delta[ i ] syntax elements may be present in the PPS and all rectangular slices in pictures referring to the PPS are specified in the order indicated by the values of the tile_idx_delta[ i ] in increasing values of i. When not present, the value of tile_idx_delta_present_flag is inferred to be equal to 0.

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th rectangular slice in units of tile columns. The value of slice_width_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileColumns − 1, inclusive.

When i is less than num_slices_in_pic_minus1 and NumTileColumns is equal to 1, the value of slice_width_in_tiles_minus1[ i ] is inferred to be equal to 0.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of tile rows when num_exp_slices_in_tile[ i ] is equal to 0. The value of slice_height_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileRows − 1, inclusive.

When i is less than num_slices_in_pic_minus1 and slice_height_in_tiles_minus1[ i ] is not present, it is inferred to be equal to NumTileRows  = =  1 ? 0 : slice_height_in_tiles_minus1[ i − 1 ].

num_exp_slices_in_tile[ i ] specifies the number of explicitly provided slice heights for the slices in the tile containing the i-th slice (i.e., the tile with tile index equal to SliceTopLeftTileIdx[ i ]). The value of num_exp_slices_in_tile[ i ] shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1, inclusive. When not present, the value of num_exp_slices_in_tile[ i ] is inferred to be equal to 0.

NOTE 3 – If num_exp_slices_in_tile[ i ] is equal to 0, the tile containing the i-th slice is not split into multiple slices. Otherwise (num_exp_slices_in_tile[ i ] is greater than 0), the tile containing the i-th slice may or may not be split into multiple slices.

exp_slice_height_in_ctus_minus1[ i ][ j ] plus 1 specifies the height of the j-th rectangular slice in the tile containing the i-th slice in units of CTU rows. The value of exp_slice_height_in_ctus_minus1[ i ][ j ] shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1, inclusive.

tile_idx_delta[ i ] specifies the difference between the tile index of the tile containing the first CTU in the ( i + 1 )-th rectangular slice and the tile index of the tile containing the first CTU in the i-th rectangular slice. The value of tile_idx_delta[ i ] shall be in the range of −NumTilesInPic + 1 to NumTilesInPic − 1, inclusive. When not present, the value of tile_idx_delta[ i ] is inferred to be equal to 0. When present, the value of tile_idx_delta[ i ] shall not be equal to 0.

slice_represented_in_ctb_flag[ i ] equal to 0 specifies that the top-left position, width and height of the i-th slice are represented in the unit of tiles. Otherwise ( slice_represented_in_ctb_flag[ i ] equal to 1 specifies that the top-left position, width and height of the i-th slice are represented in the unit of CtbSizeY.

slice_top_left_x[ i ] specifies horizontal position of top left unit of i-th slice in unit of widthes of tiles or CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_in_luma_samples + CtbSizeY  − 1 ) >>  CtbLog2SizeY ) ) bits. When not present, the value of slice_top_left_x[ i ] is inferred to be equal to 0.

slice_top_left_y[ i ] specifies vertical position of top left unit of i-th slice in units of heights of tiles or CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of slice_top_left_y[ i ] is inferred to be equal to 0.

slice_width_minus1[ i ] plus 1 specifies the width of the i-th slice in units of widthes of tiles or CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of slice_width_minus1[ i ] is inferred to be equal to ( slice_represented_in_ctb_flag[ i ] ? ( ( pic_width_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) : NumTileColumns ) − slice_top_left_x[ i ] − 1.

slice_height_minus1[ i ] plus 1 specifies the height of the i-th slice in units of heights of tiles or CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of slice_height_minus1[ i ] is inferred to be equal to (slice_represented_in_ctb_flag[ i ] ? ( ( pic_height_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ): NumTileRows) − slice_top_left_y[ i ] − 1.

...

# Conclusion

In this contribution, a direct method is proposed to signal rectangular slice partitioning. It is recommended to adopt the proposed method into the next version of working draft.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.