| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0271-v2 |
| --- | --- |

| Title: | AHG9: High-level constraints on dependent quantization and sigh data hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Muhammed Coban, and Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In 17th JVET meeting, a slice level flag “slice_ts_residual_coding_disabled_flag” was adopted into VVC as part of JVET-Q0089 for lossless coding. This flag can bypass transform skip residual coding (TSRC) regardless of the residual type and transform or regular residual coding (RRC) is used for TS blocks. Since TS blocks can now be coded with RRC, dependent quantization (DQ) and sign data hiding (SDH) can introduce undesired behavior as pointed in Ticket #981, lossy results for lossless coding and potential coding loss. In this proposal several high-level aspects are proposed to alleviate this issue.

# Introduction

In VTM-8.0, lossless coding is possible via existing syntax with TS enabled both for luma and chroma [1], which uses regular residual coding (RRC) with a high-level TSRC bypass flag slice_ts_residual_coding_disabled_flag. The control with current design is at the slice level which restricts low-level control and granularity for switching between TSRC and RRC especially for mixed lossless and lossy coding. Several issues arise, whether to apply sign data hiding (SDH) and dependent quantization (DQ) for blocks that have TS residuals but are coded with RRC since slice_ts_residual_coding_disabled_flag=1. Furthermore, there are apparent bugs as reported in Tickets #981 and #988 that there are mismatch issues and encoder crashes when DQ and SDH are enabled with slice_ts_residual_coding_disabled_flag=1.

# Proposed Methods

## Variant #1: Disallow slice_ts_residual_coding_disabled_flag=1 for DQ and SDH at picture header

In variant #1, we disallow signaling of slice_ts_residual_coding_disabled_flag based on the picture level flags ph_dep_quant_enabled_flag and pic_sign_data_hiding_enabled_flag such that, if either flag is 1 then TS blocks always use TSRC. On the JVET-Q2001 VVC Draft Text 8, the necessary spec text changes are provided below. Additions and modifications are shown with yellow highlighter.

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !ph_dep_quant_enabled_flag && !pic_sign_data_hiding_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| … |  |
| byte_alignment( ) |  |
| } |  |

## Variant #2: Disallow DQ and SDH at slice level when slice_ts_residual_coding_disabled_flag=1

In variant #2, the picture header flags ph_dep_quant_enabled_flag and pic_sign_data_hiding_enabled_flag are first moved to slice header and signaling of slice level DQ and SDH flags are done depending on the slice_ts_residual_coding_disabled_flag. If slice_ts_residual_coding_disabled_flag=1 DQ and SDH flags are not signaled and inferred as 0. On the JVET-Q2001 VVC Draft Text 8, the necessary spec text changes are provided below. Additions and modifications are shown with yellow highlighter and removals with red strikethrough.

#### 7.3.2.7 Picture header structure syntax (Common to Variants #2, #3, #4)

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| … |  |
| } |  |

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag && !slice_ts_residual_coding_disabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| … |  |
| } |  |

#### 7.4.3.7 Picture header structure semantics (Common to Variants #2, #3, #4)

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

#### 7.4.8.1 General slice header semantics (Common to Variants #2, #3, #4)

slice_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. slice_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When slice_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

slice_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. slice_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When slice_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

## Variant #3: Disallow slice_ts_residual_coding_disabled_flag=1 for DQ and SDH at slice level

In variant #3, the picture header flags ph_dep_quant_enabled_flag and pic_sign_data_hiding_enabled_flag are moved to slice header before slice_ts_residual_coding_disabled_flag and the signaling of slice_ts_residual_coding_disabled_flag are conditioned on these flags. In this case, if slice_ts_residual_coding_disabled_flag=0 then DQ and SDH are automatically turned off at the slice level. On JVET-Q2001 VVC Draft Text 8, the necessary spec text changes are provided below. Additions and modifications are shown with yellow highlighter and removals with red strikethrough.

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( ! slice_dep_quant_enabled_flag && ! slice_sign_data_hiding_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| … |  |
| } |  |

## Variant #4: Move DQ and SDH from picture header to slice header

In variant #4, picture header flags ph_dep_quant_enabled_flag and pic_sign_data_hiding_enabled_flag are moved to slice header (as in variant #2) however no signaling constraint is enforced. These tools can be turned off at the slice level (using config settings) to ensure lossless coding. On the JVET-Q2001 VVC Draft Text 8, the necessary spec text changes are provided below. Additions and modifications are shown with yellow highlighter.

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| … |  |
| } |  |

# Recommendation

It is recommended to adopt one of these high-level constraint variants to ensure that DQ and SDH can work as originally intended for TS blocks.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th meeting, Brussels, BE, 7-17 Jan. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).