| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0272 |
| --- | --- |

| Title: | AHG9: On chroma QP offsets in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra1, Jonatan Samuelsson1, Sachin Deshpande1, Frank Bossen2 Andrew Segall1 | Email: | misrak@sharplabs.com |
| Source: | 1Sharp Labs of America, Inc., 2Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

The current VVC Draft does not support signalling chroma QP offsets in the picture header. Here, it is proposed to enable this support.  The change is asserted to be aligned with the current VVC design and to be beneficial when an encoder assigns chroma QP offsets on a picture basis. This is expected to be especially useful given the advanced chroma coding tools in the VVC design, as an encoder may use different chroma QP offsets for different picture types and/or temporal layer.

# Motivation

The current VVC Draft [1] does not support signalling of chroma QP offsets in the picture header. As a result, if an application makes use of the same chroma QP offset for a picture, the current signalling approach necessitates that the chroma QP offset be signalled in each slice header. Figure 1 below illustrates an example where different chroma QP offsets are used for pictures with different slice type, i.e. (CbI, CrI) for I_SLICE, (CbB, CrB) for B_SLICE, (CbP, CrP) for P_SLICE. In the Figure, each picture consists of multiple slices, and so the number of bits spent to signal the chroma QP offsets is proportional to the number of slices. This contribution proposes modifications that enable signalling of chroma QP offsets in a picture header. As a result, only one chroma QP offset need be signalled for each picture. With the proposed change, the number of bits spent to signal the chroma QP offsets is proportional to the number of picture headers.

Figure 1: Same chroma QP offset being signalled for each slice of a picture

The change is asserted to be aligned with how other syntax elements that may appear in both the picture header and slice header are supported in the current design.  And, it is asserted to be beneficial when multiple slices of a picture have the same chroma QP offset. Note, it is possible to send multiple PPSs to signal different chroma QP offsets at the picture level, but such a signalling approach is asserted to be undesirable and not aligned with the current design of signalling syntax elements in VVC Draft.

# Proposed changes

We propose two approaches to enable the desired functionality.  The first variant is our preferred approach, as it has minimal changes needed to address the application described in Section 1. The second is provided for completeness, as it offers more flexibility.  The key difference is that the first variant signals the QP offsets in either the picture header or slice header, while the second variant can signal the QP offsets in both the picture header and slice header.  In the second case, the resultant chroma QP offset is the summation of the chroma QP offsets signalled in the PPS, picture header and slice header.

Syntax is provided below, and the semantic changes are provided in the draft text attached to the contribution.

## Proposal: Signal presence of chroma QP offset in picture header or slice header

In the proposal, the chroma QP offset may be signalled in either the picture header or slice header.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_additional_chroma_qp_offsets_present_flag | u(1) |
| if ( pps_ additional_chroma_qp_offsets_present_flag ) |  |
| pps_slice_chroma_qp_offsets_present_flag chroma_qp_offsets_in_ph_flag | u(1) |
| … |  |
| } |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_ additional_chroma_qp_offsets_present_flag && chroma_qp_offsets_in_ph_flag) { |  |
| ph_cb_qp_offset | se(v) |
| ph_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| ph_joint_cbcr_qp_offset | se(v) |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_ additional_chroma_qp_offsets_present_flag && !chroma_qp_offsets_in_ph_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| … |  |
| } |  |

## Alternate: Allow signalling of chroma QP offset in both picture header and/or slice header

In this alternative, the chroma QP offset may be signalled in either the picture header, slice header, or both.  (Except when picture header is included in slice header.)  The chroma QP offset is then determined by summing the picture header and the slice header offsets with any offset signalled in the PPS.  This allows for a high degree in flexibility in signaling the offsets.

The syntax and decoding process changes are provided below.  Again, please note that the semantics are provided in the draft text provided with the contribution.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_ph_chroma_qp_offsets_present_flag | u(1) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_ph_chroma_qp_offsets_present_flag ) { |  |
| ph_cb_qp_offset | se(v) |
| ph_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| ph_joint_cbcr_qp_offset | se(v) |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(!(picture_header_in_slice_header_flag && pps_ph_chroma_qp_offsets_present_flag) ) |  |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| … |  |
| } |  |

…

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset + ph_cb_qp_offset + slice_cb_qp_offset + 
		CuQpOffsetCb ) + QpBdOffset	(1147)

Qp′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + ph_cr_qp_offset + slice_cr_qp_offset + 
		CuQpOffsetCr ) + QpBdOffset	(1148)

Qp′CbCr = Clip3( −QpBdOffset, 63, qPCbCr + pps_joint_cbcr_qp_offset_value + ph_joint_cbcr_qp_offset_value + 
	slice_joint_cbcr_qp_offset +CuQpOffsetCbCr ) + QpBdOffset	(1149)

# References

[1] Benjamin Bross, Jianle Chen, Shan Liu, Ye-Kui Wang, “Versatile Video Coding (Draft 8)”, 17th Meeting of Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Brussels, BE, 7–17 January 2020

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).