| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0332-v2 |
| --- | --- |

| Title: | AHG9: On syntax signaling order in SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to rearrange the syntax elements related to inter slices in the sequence parameter set (SPS) in a similar manner as the grouping of the inter-slice related syntaxes in picture header. More specifically, it is proposed to also group syntaxes in SPS based on slice type (i.e. intra or inter).

# Current syntax signaling in SPS

In the current draft text of the VVC standard [1], the syntax elements signaled in picture header are grouped based on slice type (i.e. intra or inter), which offers better clarity. It is noticed that similar syntax elements signaled in SPS are currently not grouped in a same way. Instead, they are signaled in an interleaved manner. It would be preferable to have them grouped in the similar manner. The following syntax tables show the detailed signaling in current VVC, with inter syntax elements highlighted in green:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| res_change_in_clvs_allowed_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_conformance_window_flag | u(1) |
| if( sps_conformance_window_flag ) { |  |
| sps_conf_win_left_offset | ue(v) |
| sps_conf_win_right_offset | ue(v) |
| sps_conf_win_top_offset | ue(v) |
| sps_conf_win_bottom_offset | ue(v) |
| } |  |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| bit_depth_minus8 | ue(v) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| if( sps_entropy_coding_sync_enabled_flag ) |  |
| sps_wpp_entry_point_offsets_present_flag | u(1) |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | u(4) |
| sps_poc_msb_flag | u(1) |
| if( sps_poc_msb_flag ) |  |
| poc_msb_len_minus1 | ue(v) |
| num_extra_ph_bits_bytes | u(2) |
| extra_ph_bits_struct( num_extra_ph_bits_bytes ) |  |
| num_extra_sh_bits_bytes | u(2) |
| extra_sh_bits_struct( num_extra_sh_bits_bytes ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( chroma_format_idc = = 1 ) { |  |
| sps_chroma_horizontal_collocated_flag | u(1) |
| sps_chroma_vertical_collocated_flag | u(1) |
| } |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| sps_palette_enabled_flag | u(1) |
| if( ChromaArrayType = = 3 && !sps_max_luma_transform_size_64_flag ) |  |
| sps_act_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |
| min_qp_prime_ts_minus4 | ue(v) |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| if( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_merge_cand | ue(v) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| sps_lmcs_enabled_flag | u(1) |
| sps_lfnst_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| sps_explicit_scaling_list_enabled_flag | u(1) |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_x[ i ] | u(13) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |
| sps_general_hrd_params_present_flag | u(1) |
| if( sps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_cpb_params_present_flag | u(1) |
| firstSubLayer = sps_sublayer_cpb_params_present_flag ? 0 : sps_max_sublayers_minus1 |  |
| ols_hrd_parameters( firstSubLayer, sps_max_sublayers_minus1 ) |  |
| } |  |
| } |  |
| field_seq_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) /* Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7 */ |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

This contribution proposes to rearrange and group the inter related syntax elements in the sequence parameter set. The changes to the VVC working draft is highlighted below. The added parts are highlighted in yellow while the deleted parts are highlighted in green.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| res_change_in_clvs_allowed_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_conformance_window_flag | u(1) |
| if( sps_conformance_window_flag ) { |  |
| sps_conf_win_left_offset | ue(v) |
| sps_conf_win_right_offset | ue(v) |
| sps_conf_win_top_offset | ue(v) |
| sps_conf_win_bottom_offset | ue(v) |
| } |  |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| bit_depth_minus8 | ue(v) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| if( sps_entropy_coding_sync_enabled_flag ) |  |
| sps_wpp_entry_point_offsets_present_flag | u(1) |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | u(4) |
| sps_poc_msb_flag | u(1) |
| if( sps_poc_msb_flag ) |  |
| poc_msb_len_minus1 | ue(v) |
| num_extra_ph_bits_bytes | u(2) |
| extra_ph_bits_struct( num_extra_ph_bits_bytes ) |  |
| num_extra_sh_bits_bytes | u(2) |
| extra_sh_bits_struct( num_extra_sh_bits_bytes ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| sps_bcw_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( chroma_format_idc = = 1 ) { |  |
| sps_chroma_horizontal_collocated_flag | u(1) |
| sps_chroma_vertical_collocated_flag | u(1) |
| } |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| sps_palette_enabled_flag | u(1) |
| if( ChromaArrayType = = 3 && !sps_max_luma_transform_size_64_flag ) |  |
| sps_act_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |
| min_qp_prime_ts_minus4 | ue(v) |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| if( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_merge_cand | ue(v) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| sps_lmcs_enabled_flag | u(1) |
| sps_lfnst_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| sps_explicit_scaling_list_enabled_flag | u(1) |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_x[ i ] | u(13) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |
| sps_general_hrd_params_present_flag | u(1) |
| if( sps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_cpb_params_present_flag | u(1) |
| firstSubLayer = sps_sublayer_cpb_params_present_flag ? 0 : sps_max_sublayers_minus1 |  |
| ols_hrd_parameters( firstSubLayer, sps_max_sublayers_minus1 ) |  |
| } |  |
| } |  |
| field_seq_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) /* Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7 */ |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, the 17th JVET meeting, Brussels, BE, 7–17 January 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).