| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0335-v2 |
| --- | --- |

| Title: | AHG9: On SPS inter slice related syntaxes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In current VVC, one flag ph_inter_slice_allowed_flag is signaled in the picture header to control the signaling of those syntax elements that are related to inter slices. In this proposal, a similar flag sps_inter_slice_allowed_flag is proposed to be added in SPS to control the signaling of those inter slice related syntax elements in SPS. When it is signaled as false, those inter slice related syntaxes in SPS are not signaled; additionally in this case the signaling of ph_inter_slice_allowed_flag in picture header can be omitted and inferred as false.

# Current syntax

In the current draft text of the VVC standard [1], one flag ph_inter_slice_allowed_flag is signaled in the picture header (PH) to indicate whether inter slice is allowed for the coded picture. When ph_inter_slice_allowed_flag is signaled as off, the signaling of the syntax elements related to the inter slices is omitted to reduce the signaling overhead. It is proposed to add the same mechanism in SPS to further reduce the signaling overhead at least for all intra condition.

# Proposal

This contribution proposes to add a flag, sps_inter_slice_allowed_flag, in SPS to indicate whether inter slice is allowed in coding the current video sequence. When sps_inter_slice_allowed_flag is signaled as off, inter slice related syntax elements are not signaled in SPS and they are inferred as off.

Moreover, when sps_inter_slice_allowed_flag is signaled as off, the flag ph_inter_slice_allowed_flag in picture header can be omitted for signaling and be inferred as 0. The proposed syntax changes are illustrated below and the changes to the VVC Draft are highlighted.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| res_change_in_clvs_allowed_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_conformance_window_flag | u(1) |
| if( sps_conformance_window_flag ) { |  |
| sps_conf_win_left_offset | ue(v) |
| sps_conf_win_right_offset | ue(v) |
| sps_conf_win_top_offset | ue(v) |
| sps_conf_win_bottom_offset | ue(v) |
| } |  |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| bit_depth_minus8 | ue(v) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| if( sps_entropy_coding_sync_enabled_flag ) |  |
| sps_wpp_entry_point_offsets_present_flag | u(1) |
| sps_inter_slice_allowed_flag | u(1) |
| if(sps_inter_slice_allowed_flag ) { |  |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | u(4) |
| sps_poc_msb_flag | u(1) |
| if( sps_poc_msb_flag ) |  |
| poc_msb_len_minus1 | ue(v) |
| num_extra_ph_bits_bytes | u(2) |
| extra_ph_bits_struct( num_extra_ph_bits_bytes ) |  |
| num_extra_sh_bits_bytes | u(2) |
| extra_sh_bits_struct( num_extra_sh_bits_bytes ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if(sps_inter_slice_allowed_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |
| if(sps_inter_slice_allowed_flag ) { |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| } |  |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( chroma_format_idc = = 1 ) { |  |
| sps_chroma_horizontal_collocated_flag | u(1) |
| sps_chroma_vertical_collocated_flag | u(1) |
| } |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| if(sps_inter_slice_allowed_flag ) |  |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| if(sps_inter_slice_allowed_flag ) { |  |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| } |  |
| sps_palette_enabled_flag | u(1) |
| if( ChromaArrayType = = 3 && !sps_max_luma_transform_size_64_flag ) |  |
| sps_act_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |
| min_qp_prime_ts_minus4 | ue(v) |
| if(sps_inter_slice_allowed_flag ) |  |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| if( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_merge_cand | ue(v) |
| if(sps_inter_slice_allowed_flag ) { |  |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| } |  |
| sps_lmcs_enabled_flag | u(1) |
| sps_lfnst_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if(sps_inter_slice_allowed_flag ) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| sps_explicit_scaling_list_enabled_flag | u(1) |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_x[ i ] | u(13) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |
| sps_general_hrd_params_present_flag | u(1) |
| if( sps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_cpb_params_present_flag | u(1) |
| firstSubLayer = sps_sublayer_cpb_params_present_flag ? 0 : sps_max_sublayers_minus1 |  |
| ols_hrd_parameters( firstSubLayer, sps_max_sublayers_minus1 ) |  |
| } |  |
| } |  |
| field_seq_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) /* Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7 */ |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

7.4.3.3	Sequence parameter set RBSP semantics

sps_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the video sequence have slice_type equal to 2 (which indicates that the coded slice is I slice). sps_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the video sequence that have slice_type equal to 0 (which indicates that the coded slice is P slice) or 1 (which indicates that the coded slice is B slice).

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_inter_slice_allowed_flag ) |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| … |  |
| } |  |

7.4.3.7	Picture header structure semantics

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1. When not present, the value of ph_inter_slice_allowed_flag is inferred to be equal to 0.

In JVET-R0332 [2], the syntax elements in SPS is rearranged to group together the syntax elements of similar functions. The combination of the proposed sps_inter_slice_allowed_flag and the reordered SPS syntax elements as proposed in JVET-R0332 is illustrated in the table below. The added parts are highlighted in yellow while the deleted parts are highlighted in green.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| res_change_in_clvs_allowed_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_conformance_window_flag | u(1) |
| if( sps_conformance_window_flag ) { |  |
| sps_conf_win_left_offset | ue(v) |
| sps_conf_win_right_offset | ue(v) |
| sps_conf_win_top_offset | ue(v) |
| sps_conf_win_bottom_offset | ue(v) |
| } |  |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| bit_depth_minus8 | ue(v) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| if( sps_entropy_coding_sync_enabled_flag ) |  |
| sps_wpp_entry_point_offsets_present_flag | u(1) |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | u(4) |
| sps_poc_msb_flag | u(1) |
| if( sps_poc_msb_flag ) |  |
| poc_msb_len_minus1 | ue(v) |
| num_extra_ph_bits_bytes | u(2) |
| extra_ph_bits_struct( num_extra_ph_bits_bytes ) |  |
| num_extra_sh_bits_bytes | u(2) |
| extra_sh_bits_struct( num_extra_sh_bits_bytes ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| sps_inter_slice_allowed_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if(sps_inter_slice_allowed_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |
| if(sps_inter_slice_allowed_flag ) { |  |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| sps_bcw_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| } |  |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( chroma_format_idc = = 1 ) { |  |
| sps_chroma_horizontal_collocated_flag | u(1) |
| sps_chroma_vertical_collocated_flag | u(1) |
| } |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| if(sps_inter_slice_allowed_flag ) |  |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| sps_sbt_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| sps_affine_type_flag | u(1) |
| if( sps_amvr_enabled_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| sps_palette_enabled_flag | u(1) |
| if( ChromaArrayType = = 3 && !sps_max_luma_transform_size_64_flag ) |  |
| sps_act_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |
| min_qp_prime_ts_minus4 | ue(v) |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| if( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_merge_cand | ue(v) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( MaxNumMergeCand >= 2 ) { |  |
| sps_gpm_enabled_flag | u(1) |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |
| max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |
| } |  |
| sps_lmcs_enabled_flag | u(1) |
| sps_lfnst_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| sps_explicit_scaling_list_enabled_flag | u(1) |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_x[ i ] | u(13) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |
| sps_general_hrd_params_present_flag | u(1) |
| if( sps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_cpb_params_present_flag | u(1) |
| firstSubLayer = sps_sublayer_cpb_params_present_flag ? 0 : sps_max_sublayers_minus1 |  |
| ols_hrd_parameters( firstSubLayer, sps_max_sublayers_minus1 ) |  |
| } |  |
| } |  |
| field_seq_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) /* Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7 */ |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, the 17th JVET meeting, Brussels, BE, 7–17 January 2020.

H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang, “ AHG9: On syntax signaling order in SPS”, JVET-R0332, the 18th JVET meeting, by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).