| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0337 |
| --- | --- |

| Title: | AHG12: Subpicture with filler slice for merged stream |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes to enable signalling subpicture with filler slice for simple and flexible subpicture merging. For enabling filler subpicture, a filler subpicture flag is introduced. A decoding process of the filler slice is defined as normative or non-normative approaches.

Introduction

One of the merge use cases is adaptive 360-cubemap streaming. This use case is shown in the JVET-P0233 at the previous JVET meeting. The following example is borrowed from JVET-P0233. Three resolution types such as high, middle, low are defined. One case is combination of high and middle resolution while another case is combination of middle and low resolution. The latter case contains filler subpicture because picture is partitioned into subpictures. In other words, filler picture is missing functionality of the VVC bitstream-extraction-and-merger use-cases.

Two benefits are realized by normatively introducing a filler subpicture flag in SPS. Firstly, a system layer can identify which subpicture contains the content or the filler. Secondly, some decoder can ignore the filler slice without decoding process while other decoder can decode bitstream as a whole picture.

The filler slice can be defined as a non-normative or a normative way. The details are shown in the proposed section.

Figure 1 360-cubemap example with three resolution types

Figure 1 Two cases of adaptive streaming; high & middle and middle and low.

Proposed method

Subpicture with filler slice signalling

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| sps_filler_slice_present_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| if ( sps_filler_slice_present_flag ) |  |
| subpic_treated_as_filler_slice_flag[ i ] | u(1) |
| } |  |
| … |  |
| } |  |

sps_filler_slice_present_flag equal to 1 specifies that subpicture with filler slice is present in the SPS. sps_filler_slice_present_flag equal to 0 specifies that subpicture with filler slice is not present in the SPS.

subpic_treated_as_filler_slice_flag [ i ] equal to 1 specifies that subpicture may be filler slice. subpic_treated_as_filler_slice_flag [ i ] equal to 0 specifies that subpicture is not a filler slice.

Non-normative case of filler slice decoding

In the case of non-normative way, slice data is generated as a gray region. For I slice, planar intra prediction mode, the most large transform size, no coded block are selected while for P/B slice, merge index is zero with the most large block with no coded-block flag. It is the smallest size of slice data in our experiments. It is noted that the slice data is not limited to the above but the coding performance is sacrificed.

Normative case of filler slice decoding

In the case of normative way, slide data is not signalled but decoded as planar intra prediction mode, no block partitioning, and no coded block flag. The benefit of this definition is that decoding process of CTU level is not touched.

Acknowledgement

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- M. Hirabayashi, M. Katsumata, T. Suzuki, “AHG12 Signaling subpicture without slice for merged and other use cases”, JVET-P0233, Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).