| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0365 |
| --- | --- |

| Title: | Proposals on VVC extensions for higher fidelity video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki, Masaru Ikeda, Yoichi Yagasaki 2-10-1 Osaki, Shinagawa-ku Tokyo, JAPAN Tadamasa Toma, Kiyofumi Abe 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, JAPAN Masato Shima Canon Inc. 70-1 Yanagicho, Saiwai-ku, Kawasaki, Kanagawa, JAPAN | Tel: Email: Tel: Email: Tel: Email: | +81-70-7598-7392 teruhiko.s@sony.com toma.tadamasa@jp.panasonic.com +81-3-3758-2111 shima.masato@mail.canon |
| Source: | Sony Corporation, Panasonic Corporation, Canon Inc. |  |  |

_____________________________

- Abstract

In the current discussion of profile in JVET, 4:2:0 10 bit and 4:4:4 10 bit profiles will be created in the version 1 of VVC. However due to the evolution of sensing technology, higher fidelity of image/video can be captured and HDR video is emerging to the market. At this moment, display can support 10 bit HDR video, e.g. HDR10, however C-MOS sensor can capture higher fidelity of image/video, e.g. 12, 14, 16 bit video. Especially for high end production equipment, it is important to support higher bit depth than that of display side. This contribution proposes to investigate the extension to support such higher bit depth video and proposes to develop new profiles in version 2 of VVC.

Background

Applications of bit depth higher than 10 bit:

Demand for higher bit depth video is higher than past.

- HDR

HDR display supports 1000 – 10,000 nit while SDR display supports 100 nit

In near future, display can support to 100, 000 nit

Professional camera can support higher than 10 bit

Since the distribution format is 10 bit, higher bit depth is required for mastering and production of contents.

- Medical imaging

Some medical imaging equipments, e.g. X-ray image, support 14-16 bit

High throughput and high bit rate:

For such professional equipment, operational bit rate is higher and therefore, higher throughput is required to reduce the implementation complexity.

- Content production

Professional camera supports all-intra encoding, bitrate can be 400 to 800 Mbps for UHD

Technical issues:

The technical issues that needs to extend the VVC v1 are proposed in the following contributions.

- JVET-R0351

Proposal

In the current discussion of profile in JVET, 4:2:0 10 bit and 4:4:4 10 bit profiles will be created in the version 1 of VVC. However due to the evolution of sensing technology, higher fidelity of image/video can be captured and HDR video is emerging to the market. At this moment, display can support 10 bit HDR video, e.g. HDR10, however C-MOS sensor can capture higher fidelity of image/video, e.g. 12, 14, 16 bit video. Especially for high end production equipment, it is important to support higher bit depth than that of display side. This contribution proposes to investigate the extension to support such higher bit depth video and proposes to develop new profiles in version 2 of VVC.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).