| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April. 2020 | Document: JVET-R0367-v1 |
| --- | --- |

| Title: | Adjustment of shiftHor calculation in GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma, Qihong Ran, Ruipeng Qiu, Menglin Zhang, Junyan Huo, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | yzma@mail.xidian.edu.cn qhRan@stu.xidian.edu.cn rpqiu@stu.xidian.edu.cn mlzhang_1@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

In VVC8, a variable shiftHor is derived to indicate the partitioning line shift direction (horizontal or vertical) in each CU using geometric partitioning mode (GPM). Given some GPM modes with specific angle being utilized for CUs with specific height/width ratio, the shift intervals between candidate partitioning lines are too narrow to be identified from each other. However, the interval can be increased by simply adjusting the shift direction in these cases. In this contribution, a quite simple modification is proposed in shiftHor calculation method, with which the selected shift direction always lead to wider intervals between the partitioning lines.

The experimental results are as below:

For RA configuration: -0.04%, -0.04%, and -0.07%;

For LDB configuration: -0.04 %, -0.04%, and -0.01%.

Introduction

In the VVC Draft [1], the partitioning line shift direction (horizontal or vertical) is indicated by a variable shiftHor, which is derived by the following equations.

hwRatio = nH / nW		                                              (1)

shiftHor = ( angleIdx % 16  = =  8  | |  ( angleIdx % 16  !=  0  &&  hwRatio > 0 ) ) ? 0 : 1 (2)

Then the variables offsetX and offsetY, who will determine the exact position of the partitioning line are calculated by the following equations:

- If shiftHor is equal to 0, the following applies:

offsetX = ( −nW )  >>  1		                                                            (3)

offsetY = ( ( −nH )  >>  1 ) + 
		 ( angleIdx < 16 ? ( distanceIdx * nH )  >>  3 : −( ( distanceIdx * nH )  >>  3 ) )   (4)

which means the partitioning line has an offset value offsetY related to the height of the block in the vertical direction.

- Otherwise (shiftHor is equal to 1), the following applies:

offsetX = ( ( −nW )  >>  1 ) + 
		( angleIdx < 16 ? ( distanceIdx * nW )  >>  3 : −( ( distanceIdx * nW )  >>  3 ) )   (5)

offsetY = ( − nH )  >>  1		                                                            (6)

which means the current block has an offset value offsetX related to the width of the block in the horizontal direction.

Problem statement

In current design, there are some cases, when some GPM modes with specific angle being utilized for CUs with specific height/width ratio, the shift intervals between candidate partitioning lines are too narrow to be identified from each other. Fig.1 gives several partitioning line candidates (shown as dash lines) examples, in which the left ones are derived by the current design in VVC draft, however, the right ones providing wider intervals should be preferred to be selected.

(a) AngleIdx2\18, nH==nW, shiftHor=0   (b) AngleIdx2\18, nH==nW, shiftHor=1

(c) AngleIdx2\18, nH==2nW, shiftHor=0   (d) AngleIdx2\18, nH==2nW, shiftHor=1

(e) AngleIdx3\19, nH==nW, shiftHor=0  (f) AngleIdx3\19, nH==nW, shiftHor=1

(g) angleIdx13\29, nH==nW, shiftHor=0  (h) angleIdx13\29, nH==nW, shiftHor=1

(i) angleIdx14\30, nH==nW, shiftHor=0  (j) angleIdx14\30, nH==nW, shiftHor=1

(k) angleIdx14\30, nH==2nW, shiftHor=0  (l) angleIdx14\30, nH==2nW, shiftHor=1

Fig.1 Different offset directions (horizontal with shiftHor==1, or vertical with shiftHor==0) under different block sizes and angles, left: current design, right: ideal design.

Proposed method

In this contribution, it is proposed to modify the derivation method of shiftHor in a same simple way to make the intervals between the partitioning lines with same lean angle always be the wider one.

The calculation of shiftHor is modified as following changes in spec text. The added texts are highlighted in yellow, deleted texts are described using texts with red strikethrough.

The variables nW, nH, shift1, offset1, hwRatio, displacementX, displacementY, partFlip and shiftHor are derived as follows: … hwRatio = nH / nW (1034) … shiftHor = ( angleIdx % 16 = = 8 ¦ ¦ ( angleIdx % 16 != 0 && hwRatio > 0 ) ) ? 0 : 1 shiftHor = (nH << (angleIdx%16 > 8?(16 - angleIdx%16) : angleIdx%16)) < (nW<<4) ?1:0 (1038) … The variables hwRatio, displacementX, displacementY, partIdx and shiftHor are derived as follows: hwRatio = cbHeight / cbWidth (1049) … shiftHor = ( angleIdx % 16 = = 8 ¦ ¦ ( angleIdx % 16 != 0 && hwRatio > 0 ) ? 0 : 1 shiftHor = (cbHeight << (angleIdx%16 > 8?(16 - angleIdx%16) : angleIdx%16)) < (cbWidth<<4) ?1:0 (1053)

It can be seen that the calculation complexity is at the same level with the original design.

Experimental results

The proposed method is integrated into VTM-8.0 and the results under CTC [2] are shown below compared to VTM8.0. The experimental results of the test reportedly show -0.04%, -0.04% BD-rate coding gain. As for the encoding and decoding run-time, it is not accurate due to the parallel running.

Table 1. Results of the proposed method in Random Access Configuration

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.09% | -0.10% | 100% | 99% |
| Class A2 | -0.05% | -0.02% | 0.09% | 100% | 100% |
| Class B | -0.02% | -0.02% | -0.07% | 100% | 100% |
| Class C | -0.08% | -0.06% | -0.18% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.04% | -0.07% | 100% | 100% |
| Class D | -0.04% | -0.04% | -0.08% | 100% | 100% |
| Class F | 0.02% | -0.02% | -0.08% | 100% | 99% |

Table 2. Results of the proposed method under Low delay B configuration

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.04% | 0.00% | -0.14% | 100% | 95% |
| Class C | -0.12% | -0.45% | -0.21% | 104% | 93% |
| Class E | 0.06% | 0.44% | 0.48% | 93% | 87% |
| Overall | -0.04% | -0.04% | -0.01% | 99% | 93% |
| Class D | -0.14% | -0.09% | -0.14% | 103% | 102% |
| Class F | -0.04% | 0.61% | 0.12% | 91% | 85% |

Conclusion

This contribution presents a shiftHor derivation method to select the partitioning line shift direction (horizontal or vertical) to wider intervals between partitioning lines with same lean angle. With the proposed simple modification, -0.04% and -0.04% coding gain can be achieved. It is recommended to adopt this method to VVC.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 8), Document JVET-Q2001, Version 14 , Brussels, Belgium, January 2019.

[2] “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).