| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April. 2020 | Document: JVET-R0368-v2 |
| --- | --- |

| Title: | GPM merge list construction modification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma, Qihong Ran, Ruipeng Qiu, Junyan Huo, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | yzma@mail.xidian.edu.cn qhRan@stu.xidian.edu.cn rpqiu@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

In this contribution, a new method is proposed to derive the geometric partitioning mode (GPM) unidirectional merge candidate list. In this method, regular merge candidate list is re-used by selecting the unique available candidates in it.

The experimental results are as below:

For RA configuration: -0.07 %, -0.11%, and -0.08%.

For LB configuration: x.xx %, x.xx%, and x.xx%.

Introduction

In VVC8[1], to maximize the common operations shared by different merge modes, the regular merge list is constructed and its motion vectors are re-used for geometric prediction. Once the regular merge list is constructed, for each merge candidate in the regular merge candidate list, one and only one of its L0 or L1 motion vector is used for GPM. In addition, the order of selecting the L0 vs. L1 motion vector is based on its merge index parity. More specifically, as shown in Figure 1, for a candidate with an even value index, its L0 motion vector is first chosen to be used for GPM. If it is not available, its L1 motion vector would be used instead. For a candidate with an odd value index, its L1 motion vector is first chosen to be used for GPM. If it is not available, its L0 motion vector would be used instead. In Figure 1, corresponding to each merge index, the motion vector marked with “x” is first selected for GPM.

Fig. 1. Uni-prediction MV selection for GPM in VTM8.0

Problem statement

The unidirectional motion vectors in the regular merge candidate list, including the ones took out from the bidirectional motion vectors, may have lot of repetition.

And at the same time, the two unidirectional motion vectors derived from a same bidirectional motion vector (same row in Fig.1) may has the same importance, either of them being excluded in the GPM mode motion vector candidates based on its merge index parity may cause inefficiency in prediction coding.

Proposed method

In this contribution, it is proposed to construct the GPM unidirectional merge list with all the unique candidates in regular merge list.

For each merge candidate in the regular merge candidate list, its L0 and L1 motion vectors are filled into the unidirectional GPM merge candidate list in the merge index increasing order. In addition, the unavailable unidirectional candidates and repeated candidates should be skipped.

Figure 2 gives an example, the blue area in the regular merge candidate list represents the available unidirectional candidates, the white area represents unavailable unidirectional candidates, and the colors with different shade represents different unidirectional MV values. At last, unidirectional zero motion vectors may be added to make the merge list full.

Figure 2. A unidirectional GPM merge candidate list construction example with MaxNumGpmMergeCand=5

With the proposed method, by reusing the regular merge candidates in a different way, the GPM merge candidate list is constructed with more representative and unique candidates.

Experimental results

The proposed method is integrated into VTM-8.0 and the results of the proposed implementation under CTC [2] are shown below against VTM8.0[3].

Table 1. Results of proposed method under Random Access configuration

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.09% | -0.07% | 101% | 100% |
| Class A2 | -0.04% | -0.10% | -0.02% | 100% | 99% |
| Class B | -0.05% | -0.07% | -0.04% | 100% | 100% |
| Class C | -0.17% | -0.19% | -0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.11% | -0.08% | 100% | 100% |
| Class D | -0.12% | -0.09% | -0.19% | 100% | 100% |
| Class F | -0.06% | -0.07% | -0.12 | 100% | 100% |

Table 2. Results of proposed method under Low delay B configuration

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.20% | -0.31% | -0.21% | 104% | 94% |
| Class E | -0.07% | -0.02% | -0.23% | 96% | 92% |
| Overall |  |  |  |  |  |
| Class D | -0.10% | -0.24% | -0.44% | 104% | 100% |
| Class F |  |  |  |  |  |

Conclusion

In this proposal, a new GPM merge candidate list construction process was proposed. The proposed scheme reuses the candidates in regular merge list with the representative and unique ones in it. The experimental results shows shows 0.07%(RA), x.xx%(LDB) BD-rate gain and similar encoding and decoding time compared to VTM8.0. It is recommended to consider adoption of the proposed method into VVC.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 8), Document JVET-Q2001, Version 15 , Brussels, Belgium, January 2020

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document JVET-N1010, Geneva, Switzerland, March 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-8.0

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).