| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0462 |
| --- | --- |

| Title: | Crosscheck of JVET-R0393 (AHG9: On LMCS for GDR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

____________________________

- Abstract

The contribution reports a cross-checking report. The method proposed in JVET-R0393 disables chroma residual scaling for CUs in clean area to solve a problem that clean area may reference dirty area in chroma residual scaling process, which would be not appropriate. The experimental results match with those provided by the proponents. The bdrate change is -0.02%, -0.03%, 0.33% in Y, U, V respectively in GDR LB condition.

Introduction

The LMCS’s chroma residual scaling uses an average of luma (invAvgLuma), which may use dirty area in clean area in GDR case. That would break GDR feature that needs clean area should be guaranteed by only referencing clean area. JVET-R0393 disables chroma residual scaling for CUs in clean area to solve this problem.

Experimental Results

Table 1 and Table 2 shows results of VVC-8 base solution (disabling LMCS or chroma residual scaling) while Table 3 shows results of JVET-R0393 solution.

Table 1. Disable LMCS for pictures within GDR periods.

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proposal 1 over VTM 8.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 1.70% | 1.32% | 1.54% | 94% | 97% |  |
| Class C | 0.71% | -1.25% | -1.65% | 92% | 97% |  |
| Class E | 0.18% | -1.37% | -0.53% | 95% | 98% |  |
| Overall | 0.99% | -0.21% | -0.04% | 93% | 97% |  |
| Class D | 0.88% | 0.73% | 1.16% | 94% | 98% |  |
| Class F | 0.50% | -0.30% | -0.10% | 94% | 96% |  |

Table 2. Disable chroma residual scaling of LMCS for pictures for GDR periods.

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proposal 2 over VTM 8.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.10% | 1.93% | 2.28% | 98% | 99% |  |
| Class C | -0.30% | 4.32% | 3.73% | 98% | 98% |  |
| Class E | -0.07% | 2.67% | 3.71% | 100% | 101% |  |
| Overall | -0.16% | 2.91% | 3.12% | 98% | 99% |  |
| Class D | -0.20% | 2.40% | 3.80% | 98% | 102% |  |
| Class F | -0.32% | 1.71% | 1.86% | 99% | 98% |  |

Table 3. Disable chroma residual scaling for CUs in clean area, when necessary, for pictures within GDR periods.

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CU-level Proposal over VTM8.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.03% | 0.00% | 0.28% | 99% | 99% |  |
| Class C | 0.02% | -0.09% | -0.05% | 100% | 98% |  |
| Class E | -0.16% | 0.01% | 0.92% | 101% | 100% |  |
| Overall | -0.02% | -0.03% | 0.33% | 100% | 99% |  |
| Class D | 0.05% | 0.25% | 0.97% | 100% | 99% |  |
| Class F | -0.04% | 0.02% | 0.04% | 100% | 99% |  |

Conclusion

This contribution reported a crosscheck verification of JVET-R0393, (AHG9: On LMCS for GDR). The experimental results match with those provided by proponent. It was confirmed that the CU-level proposal has almost no impact on overall coding performance (0.01%).

References

[1] L. Wang, S. Hong, K. Panusopone, M. M. Hannuksela, “AHG9: On LMCS for GDR”, JVET-R0393, April 2020,

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).