| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0042 |
| --- | --- |

| Title: | AHG9: On the parameter set extension mechanism |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kashyap Kammachi-Sreedhar Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1:

- VVC Draft 9 syntax allows no extension_data_flag syntax elements be present.

- The semantics in VVC Draft 9 state that extension_data_flag syntax elements are present.

This contribution requests JVET to decide if any clarifications on this asserted contradiction between the syntax and semantics would be needed. The contribution suggests the following options:

- Option 1: When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, enforce the presence of at least one extension_data_flag syntax element in the syntax.

- Option 2: Relax the semantics so that an extension_flag in DCI, VPS, SPS, PPS, or APS equal to 1 specifies that specifies extension_data_flag syntax elements may be present.

- Option 3: Do nothing.

# Introduction

## Background: parameter set extension mechanism

DCI, VPS, SPS, PPS, and APS RBSPs have the same extension mechanism, exemplified below in red font color with the DCI syntax:

| decoding_capability_information_rbsp( ) { | Descriptor |
| --- | --- |
| dci_reserved_zero_4bits | u(4) |
| dci_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dci_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0 ) |  |
| dci_extension_flag | u(1) |
| if( dci_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantics is specified as follows:

dci_extension_flag equal to 0 specifies that no dci_extension_data_flag syntax elements are present in the DCI RBSP syntax structure. dci_extension_flag equal to 1 specifies that there are dci_extension_data_flag syntax elements present in the DCI RBSP syntax structure.

dci_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to this version of this Specification shall ignore all dci_extension_data_flag syntax elements.

## Problem statement

When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1:

- The syntax allows no extension_data_flag syntax elements be present.

- The semantics state that extension_data_flag syntax elements are present.

This contribution requests JVET to decide if any clarifications on this asserted contradiction between the syntax and semantics would be needed.

# Proposal

Option 1: When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, enforce the presence of at least one extension_data_flag syntax element in the syntax as exemplified with the following DCI syntax:

| decoding_capability_information_rbsp( ) { | Descriptor |
| --- | --- |
| dci_reserved_zero_4bits | u(4) |
| dci_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dci_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0 ) |  |
| dci_extension_flag | u(1) |
| if( dci_extension_flag ) { |  |
| do |  |
| dci_extension_data_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Option 2: Relax the semantics so that an extension_flag in DCI, VPS, SPS, PPS, or APS equal to 1 specifies that specifies extension_data_flag syntax elements may be present.

Option 3: Do nothing.

Note that the same extension mechanism, both when it comes to the syntax and the phrasing of the semantics, is present in HEVC and has not caused issues as far as the proponents are aware.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).