| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0048-v1 |
| --- | --- |

| Title: | AHG9/AHG8: On reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 Kai Zhang1, Li Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to reference picture resampling:

- Change the value ranges of the scaling window offsets such that the scaling window width and height can be up to 16 times the picture width and height, respectively.

- Change the semantics of the sps_ref_pic_resampling_enabled_flag to be aligned with the derivation of the RprConstraintsActive variable, which would be set equal to 1 when one or more of the following 6 parameters differ between the current picture and the reference picture: 1) picture width, 2) picture height, 3) scaling window left offset, 4) scaling window right offset, 5) scaling window top offset, and 6) scaling window bottom offset.

# On scaling window offsets

## Introduction

In the latest VVC text, the variables CurrPicScalWinWidthL and CurrPicScalWinHeightL are derived as follows:

CurrPicScalWinWidthL = pps_pic_width_in_luma_samples −	(79)
		SubWidthC * ( pps_scaling_win_right_offset + pps_scaling_win_left_offset )

CurrPicScalWinHeightL = pps_pic_height_in_luma_samples −	(80)
		SubHeightC * ( pps_scaling_win_bottom_offset + pps_scaling_win_top_offset )

And the value ranges of the scaling window offset syntax elements are specified as follows:

The value of SubWidthC * ( Abs( pps_scaling_win_left_offset ) + Abs( pps_scaling_win_right_offset ) ) shall be less than pps_pic_width_in_luma_samples, and the value of SubHeightC * ( Abs( pps_scaling_win_top_offset ) + Abs( pps_scaling_win_bottom_offset ) ) shall be less than pps_pic_height_in_luma_samples.

Consequently, the value of the scaling window width CurrPicScalWinWidthL is always less than twice the picture width, i.e., pps_pic_width_in_luma_samples * 2, and the scaling window height CurrPicScalWinHeightL is always less than twice the picture height, i.e., pps_pic_height_in_luma_samples * 2.

However, such value ranges of the scaling window width and height are too restrictive for some use cases, as described below.

Figure 1. A viewport-dependent 360o video coding scheme based on subpictures and spatial scalability

Figure 1 shows a 360o video coding scheme, wherein a higher-resolution representation of the full video consists of multiple subpictures per picture, while a lower-resolution representation of the full video consists of one subpicture per picture and can be coded with less frequent random access points than the higher-resolution representation. The client receives the full video in the lower-resolution and for the higher-resolution video, it only receives and decode the subpictures that cover the current viewport.

Assuming that each EL picture (covering the full sphere of 360o × 180o) is split into 12 × 8 = 96 subpictures, and the user's viewport is 120o × 90o, i.e., about 4 × 4 subpictures of each EL picture are received and decoded by the decoder and rendered to the user. In the sub-bitstream extraction to generate the bitstream to send to the decoder (i.e., as in the low part of Figure 1), the PPSs (which contain the scaling window offset parameters) of the BL pictures remain unchanged, while the PPSs of the EL pictures would be rewritten. Assuming that in the original bitstream (before sub-bitstream extraction, as in the upper part of Figure 1) the scaling window offsets for EL pictures are all equal to 0, then the values of CurrPicScalWinWidthL and CurrPicScalWinHeightL for an EL picture in the original bitstream would be the same as the picture width and height, respectively. However, in the bitstream received by the decoder (after sub-bitstream extraction, like in the low part of Figure 1), the values of CurrPicScalWinWidthL and CurrPicScalWinHeightL for an EL picture would need to be equal to three times the picture width, pps_pic_width_in_luma_samples * 3, and twice the picture height, pps_pic_height_in_luma_samples * 2, respectively.

However, this would violate the existing value ranges of the scaling window offset syntax elements.

If the example is updated such that only one subpicture of each EL picture is received and decoded, then the values of CurrPicScalWinWidthL and CurrPicScalWinHeightL for an EL picture would need to be equal to 12 times the picture width, i.e. pps_pic_width_in_luma_samples * 12, and 8 times the picture height, i.e. pps_pic_height_in_luma_samples * 8.

Therefore, it is proposed to change the value ranges of the scaling window offsets such that the scaling window width and height can be up to 16 times the picture width and height, respectively.

## Proposed text changes

The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### 7.4.3.4	Picture parameter set RBSP semantics

...

pps_scaling_window_explicit_signalling_flag equal to 1 specifies that the scaling window offset parameters are present in the PPS. pps_scaling_window_explicit_signalling_flag equal to 0 specifies that the scaling window offset parameters are not present in the PPS. When sps_ref_pic_resampling_enabled_flag is equal to 0, the value of pps_scaling_window_explicit_signalling_flag shall be equal to 0.

pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset specify the offsets that are applied to the picture size for scaling ratio calculation. When not present, the values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset are inferred to be equal to pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, respectively.

The value of SubWidthC * ( Abs( pps_scaling_win_left_offset ) + Abs( pps_scaling_win_right_offset ) ) shall be greater than or equal to −pps_pic_width_in_luma_samples * 15 and less than pps_pic_width_in_luma_samples, and the value of SubHeightC * ( Abs( pps_scaling_win_top_offset ) + Abs( pps_scaling_win_bottom_offset ) ) shall be greater than or equal to −pps_pic_height_in_luma_samples * 15 and less than pps_pic_height_in_luma_samples.

The variables CurrPicScalWinWidthL and CurrPicScalWinHeightL are derived as follows:

CurrPicScalWinWidthL = pps_pic_width_in_luma_samples −	(79)
		SubWidthC * ( pps_scaling_win_right_offset + pps_scaling_win_left_offset )

CurrPicScalWinHeightL = pps_pic_height_in_luma_samples −	(80)
		SubHeightC * ( pps_scaling_win_bottom_offset + pps_scaling_win_top_offset )

Let refPicScalWinWidthL and refPicScalWinHeightL be the CurrPicScalWinWidthL and CurrPicScalWinHeightL, respectively, of a reference picture of a current picture referring to this PPS. It is a requirement of bitstream conformance that all of the following conditions shall be satisfied:

–	CurrPicScalWinWidthL * 2 is greater than or equal to refPicScalWinWidthL.

–	CurrPicScalWinHeightL * 2 is greater than or equal to refPicScalWinHeightL.

–	CurrPicScalWinWidthL is less than or equal to refPicScalWinWidthL * 8.

–	CurrPicScalWinHeightL is less than or equal to refPicScalWinHeightL * 8.

- CurrPicScalWinWidthL * sps_pic_width_max_in_luma_samples is greater than or equal to refPicScalWinWidthL * ( pps_pic_width_in_luma_samples − Max( 8, MinCbSizeY ) ).

- CurrPicScalWinHeightL * sps_pic_height_max_in_luma_samples is greater than or equal to refPicScalWinHeightL * ( pps_pic_height_in_luma_samples − Max( 8, MinCbSizeY ) ).

...

# On semantics of sps_ref_pic_resampling_enabled_flag

The current semantics of sps_ref_pic_resampling_enabled_flag is as follows:

sps_ref_pic_resampling_enabled_flag equal to 1 specifies that reference picture resampling is enabled and one or more slices of pictures in the CLVS may refer to a reference picture with a different spatial resolution in an active entry of a reference picture list. sps_ref_pic_resampling_enabled_flag equal to 0 specifies that reference picture resampling is disabled and no slice of pictures in the CLVS refers to a reference picture with a different spatial resolution in an active entry of a reference picture list. [Ed. (YK): Check whether the phrase "refer to a reference picture with a different spatial resolution" should be changed to be "refer to a reference picture with a different scaling window".]

NOTE 2 – When sps_ref_pic_resampling_enabled_flag is equal to 1, for a current picture the reference picture with a different spatial resolution may either belong to the same layer or a different layer than the layer containing the current picture.

On the other hand, whether RPR is on really depends on the value of the variable RprConstraintsActive[ i ][ j ], which is derived as follows:

RprConstraintsActive[ i ][ j ] = ( pps_pic_width_in_luma_samples  !=  refPicWidth  | |
				pps_pic_height_in_luma_samples  !=  refPicHeight  | |
				pps_scaling_win_left_offset  !=  refScalingWinLeftOffset  | |
				pps_scaling_win_right_offset  !=  refScalingWinRightOffset  | |
				pps_scaling_win_top_offset  !=  refScalingWinTopOffset  | |
				pps_scaling_win_bottom_offset  !=  refScalingWinBottomOffset )

Basically, as long as one of the six parameters of the current picture is different from that of the reference picture, the value of this variable is equal to 1, and RPR is on: 1) picture width, 2) picture height, 3) scaling window left offset, 4) scaling window right offset, 5) scaling window top offset, and 6) scaling window bottom offset.

Therefore, to make the semantics to be aligned with how RPR is determined to be on or off, i.e., the derivation of the variable RprConstraintsActive[ i ][ j ] (which BTW should be renamed to be RprConstraintsActiveFlag[ i ][ j ]), it is proposed to change the semantics as follows:

sps_ref_pic_resampling_enabled_flag equal to 1 specifies that reference picture resampling is enabled and one or more slices of pictures in the CLVS may refer to a reference picture with a different spatial resolution in an active entry of a reference picture list that has one or more of the following 6 parameters different than that of the current picture: 1) picture width, 2) picture height, 3) scaling window left offset, 4) scaling window right offset, 5) scaling window top offset, and 6) scaling window bottom offset. sps_ref_pic_resampling_enabled_flag equal to 0 specifies that reference picture resampling is disabled and no slice of pictures in the CLVS refers to a reference picture with a different spatial resolution in an active entry of a reference picture list that has one or more of the above 6 parameters different than that of the current picture.

NOTE 2 – When sps_ref_pic_resampling_enabled_flag is equal to 1, for a current picture the reference picture with a different spatial resolution that has one or more of the above 6 parameters different than that of the current picture may either belong to the same layer or a different layer than the layer containing the current picture.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).