| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0063-v1 |
| --- | --- |

| Title: | AHG9: On VPS Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed related to VPS signalling.

Following is proposed:

It is proposed to conditionally signal the syntax element vps_num_ptls_minus1, only when vps_max_layers_minus1 is greater than 0. When not signalled, the value for  vps_num_ptls_minus1 is inferred.

# Introduction

VVC WD [1] includes video parameter set which provides among other things, information about profile-tier-level for output layer sets.

It is asserted that when vps_max_layers_minus1 is equal to 0, it is a single layer CVS. In this case only one profile tier level syntax structure (i.e. profile_tier_level( )) is necessary to be signalled for the single layer. Thus the number of signalled profile tier level syntax structure is known to be equal to 1 and does not need to be signalled.

Thus it is proposed to conditionally signal the syntax element vps_num_ptls_minus1, only when vps_max_layers_minus1 is greater than 0. When not signalled, the value for  vps_num_ptls_minus1 is inferred.

# Proposal

It is proposed to conditionally signal the syntax element vps_num_ptls_minus1, which specifies the number of profile tier level syntax structures (i.e. profile_tier_level( ))  in VPS minus 1, only when vps_max_layers_minus1, which specifies the maximum allowed number of layers in each CVS referring to the VPS minus 1, is greater than 0. It is asserted that this saves 1 byte for the single layer case, which is asserted to be a common case.

When not signalled, the value for vps_num_ptls_minus1 is inferred.

The proposed changes are shown highlighted below compared to VVC Draft text (JVET-R2001-v8).

#### 7.3.2.2 Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) { |  |  |
| vps_max_tid_ref_present_flag[ i ] | u(1) |  |
| for( j = 0; j < i; j++ ) { |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| if( vps_max_tid_ref_present_flag[ i ] && vps_direct_ref_layer_flag[ i ][ j ] ) |  |  |
| vps_max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| vps_each_layer_is_an_ols_flag | u(1) |  |
| if( !vps_each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_ols_mode_idc | u(2) |  |
| if( vps_ols_mode_idc = = 2 ) { |  |  |
| vps_num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= vps_num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| vps_ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| vps_num_ptls_minus1 | u(8) |
| } |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| vps_pt_present_flag[ i ] | u(1) |  |
| if( !vps_all_layers_same_num_sublayers_flag ) |  |  |
| vps_ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( vps_pt_present_flag[ i ], vps_ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 && vps_num_ptls_minus1 + 1 != TotalNumOlss ) |  |  |
| vps_ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_each_layer_is_an_ols_flag ) { |  |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < VpsNumDpbParams; i++ ) { |  |  |
| if( !vps_all_layers_same_num_sublayers_flag ) |  |  |
| vps_dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( vps_dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < NumMultiLayerOlss; i++ ) { |  |  |
| vps_ols_dpb_pic_width[ i ] | ue(v) |  |
| vps_ols_dpb_pic_height[ i ] | ue(v) |  |
| vps_ols_dpb_chroma_format[ i ] | u(2) |  |
| vps_ols_dpb_bitdepth_minus8[ i ] | ue(v) |  |
| if( VpsNumDpbParams > 1 && vps_num_dpb_params != NumMultiLayerOlss ) |  |  |
| vps_ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| } |  |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| vps_num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= vps_num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : vps_hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, vps_hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( vps_num_ols_hrd_params_minus1 > 0 && vps_num_ols_hrd_params_minus1 + 1 != NumMultiLayerOlss ) |  |  |
| for( i = 0; i < NumMultiLayerOlss; i++ ) |  |  |
| vps_ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

vps_num_ptls_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the VPS. The value of vps_num_ptls_minus1 shall be less than TotalNumOlss. When not present vps_num_ptls_minus1 is inferred to be equal to 0.

# Conclusion

This document proposes modifications to VPS signalling in VVC. It is proposed to adopt the proposal in the next version of VVC Draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-R2001, Versatile Video Coding (Draft 9), Online, April 2020