| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0065-v1 |
| --- | --- |

| Title: | AHG9: On Virtual Boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed related to virtual boundary.

Following is proposed:

- Proposal 1: A missing inference rule is specified for sps_virtual_boundaries_present_flag when not present.

- Proposal 2: An asserted simplification of text is proposed for virtual boundary derivation.

# Introduction

VVC Draft text [1] includes signalling of virtual boundaries. Virtual boundaries can be signalled  either in SPS or picture header (or not signalled).

Following is proposed:

- Proposal 1: A missing inference rule is specified for sps_virtual_boundaries_present_flag when not present.

- Proposal 2: An asserted simplification of text is proposed for virtual boundary derivation.

# Proposals

## Proposal 1

A missing inference rule is specified for sps_virtual_boundaries_present_flag when not present.

It is noted that for certain parsers checking that sps_virtual_boundaries_enabled_flag is equal to 0 may be sufficient in the picture header, however it is implementation dependent.

It is asserted that the flag sps_virtual_boundaries_present_flag is used in the conditioning of syntax elements in picture header as shown below where having an inference rule for this flag when it is not present is useful.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_virtual_boundaries_enabled_flag && !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_x[ i ] | ue(v) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_y[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

sps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the SPS. sps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the SPS. When there is one or more than one virtual boundaries signalled in the SPS, the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present sps_virtual_boundaries_present_flag is inferred to be equal to 0. It is a requirement of bitstream conformance that when the value of sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_virtual_boundaries_present_flag shall be equal to 0.

## Proposal 2

It is asserted that in VVC Draft 9, text which specifies the derivation of virtual boundaries can be simplified by inferring the picture header virtual boundary related syntax elements to be equal to corresponding SPS virtual boundary related syntax elements when not present and when virtual boundaries are enabled. It is also asserted that this is the typical way of specifying the specification text in VVC draft for other syntax elements of a tool which may signal information at different places (e.g. in parameter set or picture header or slice header).

The proposed text for this is shown highlighted below compared to VVC draft text (JVET-R2001-v9) .

ph_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PH. ph_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PH. When there is one or more than one virtual boundaries signalled in the PH, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when sps_subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag shall be equal to 0.

The variable VirtualBoundariesPresentFlag is derived as follows:

VirtualBoundariesPresentFlag = 0
if( sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesPresentFlag = sps_virtual_boundaries_present_flag  | |
			ph_virtual_boundaries_present_flag		(83)

ph_num_ver_virtual_boundaries specifies the number of ph_virtual_boundary_pos_x[ i ] syntax elements that are present in the PH. When ph_num_ver_virtual_boundaries is not present, it is inferred to be equal to sps_virtual_boundaries_enabled_flag ? sps_num_ver_virtual_boundaries : 0.

The variable NumVerVirtualBoundaries is derived as follows: set equal to sps_virtual_boundaries_present_flag ? ph_num_ver_virtual_boundaries : 0

NumVerVirtualBoundaries = 0
if( sps_virtual_boundaries_enabled_flag )
	NumVerVirtualBoundaries = sps_virtual_boundaries_present_flag ?
			sps_num_ver_virtual_boundaries : ph_num_ver_virtual_boundaries	(84)

ph_virtual_boundary_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundary_pos_x[ i ] shall be in the range of 1 to Ceil( pps_pic_width_in_luma_samples ÷ 8 ) − 1, inclusive. When sps_virtual_boundaries_present_flag is equal to 1 and when not present ph_virtual_boundary_pos_x[ i ] is inferred to be equal to sps_virtual_boundary_pos_x[ i ].

The list VirtualBoundaryPosX[ i ] for i ranging from 0 to NumVerVirtualBoundaries − 1, inclusive, in units of luma samples, specifying the locations of the vertical virtual boundaries, is derived as follows set equal to ph_virtual_boundary_pos_x[ i ]*8:

for( i = 0; i < NumVerVirtualBoundaries; i++)
	VirtualBoundaryPosX[ i ] = ( sps_virtual_boundaries_present_flag ?
			sps_virtual_boundary_pos_x[ i ] : ph_virtual_boundary_pos_x[ i ] ) * 8	(85)

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

ph_num_hor_virtual_boundaries specifies the number of ph_virtual_boundary_pos_y[ i ] syntax elements that are present in the PH. When ph_num_hor_virtual_boundaries is not present, it is inferred to be equal to sps_virtual_boundaries_enabled_flag ? sps_num_hor_virtual_boundaries : 0.

The parameter NumHorVirtualBoundaries is derived as follows: NumHorVirtualBoundaries = sps_virtual_boundaries_present_flag ? ph_num_hor_virtual_boundaries : 0

NumHorVirtualBoundaries = 0
if( sps_virtual_boundaries_enabled_flag )
	NumHorVirtualBoundaries = sps_virtual_boundaries_present_flag ?
			sps_num_hor_virtual_boundaries : ph_num_hor_virtual_boundaries	(86)

When sps_virtual_boundaries_enabled_flag is equal to 1 and ph_virtual_boundaries_present_flag is equal to 1, the sum of ph_num_ver_virtual_boundaries and ph_num_hor_virtual_boundaries shall be greater than 0.

ph_virtual_boundary_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundary_pos_y[ i ] shall be in the range of 1 to Ceil( pps_pic_height_in_luma_samples ÷ 8 ) − 1, inclusive. When sps_virtual_boundaries_present_flag is equal to 1 and when not present ph_virtual_boundary_pos_y[ i ] is inferred to be equal to sps_virtual_boundary_pos_y[ i ].

The list VirtualBoundaryPosY[ i ] for i ranging from 0 to NumHorVirtualBoundaries − 1, inclusive, in units of luma samples, specifying the locations of the horizontal virtual boundaries, is derived as follow set equal to ph_virtual_boundary_pos_y[ i ]*8:

for( i = 0; i < NumHorVirtualBoundaries; i++)
	VirtualBoundaryPosY[ i ] = ( sps_virtual_boundaries_present_flag ?
			sps_virtual_boundary_pos_y[ i ] : ph_virtual_boundary_pos_y[ i ] ) * 8	(87)

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

# Conclusion

This document proposes modifications to related to virtual boundary. It is proposed to adopt the proposals in the next version of VVC Draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-R2001, Versatile Video Coding (Draft 9), April 2020