| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0071-v5 |
| --- | --- |

| Title: | AHG12: Cleanup of subpicture layout signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mitsuru Katsumata Mitsuhiro Hirabayashi Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-7393 Mitsuru.Katsumata@sony.com Mitsuhiro.Hirabayashi@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

The followings are proposed for a shortcut method of signalling subpicture layout for bit-saving in SPS when all subpictures have same spatial resolution.

- To specify sps_subpic_same_size_flag in SPS whether all subpictures have same spatial resolution.

- When all subpictures have same spatial resolution,

  - to specify common width/height of subpicture indicated by width/height of 0-th subpicture (sps_subpic_width_minus1[0] and sps_subpic_height_minus1[0]),

  - to omit width and height of other than 0-th subpicture, and

  - to omit positions of top left of i-th subpicture (sps_subpic_ctu_top_left_x[i] and sps_subpic_ctu_top_left_y[i]).

- To prohibit the use of the shortcut unless the picture is an exact multiple of common width/height of subpicture.

The impact of this proposal on the software is only deriving the omitted value of subpicture layout.

In v2,

- To change the flag name from sps_subpic_same_res_flag to sps_subpic_same_size_flag.

- To change the semantics that sps_subpic_same_size_flag equal to 0.

- To change the inferred value of sps_subpic_same_size_flag.

- To cleanup the syntax table in Proposal 1

In v3,

- To add Proposal 2

- To fix the formula that specified requirement of common width and height of subpicture when sps_subpic_same_size_flag is equal to 1.

- Editorial update

In v4,

- To remove Proposal 2.  In proposal 2, sps_num_subpics_minus1 could be omitted in the syntax because it is small effect for bit saving and needs some additional syntax.

- To includes implementation, configuration files for simulation, results of simulation.

- Editorial update

In v5

- To add a requirement of bitstream conformance related to number of subpictures when sps_subpic_same_size_flag is equal to 1

Introduction

VVC specification draft in JVET-R2001-vA includes support for subpicture layout signalling.  The signalling specifies from four syntax elements, below. And these syntax elements are signalling per subpicture.

sps_subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

sps_subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

sps_subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_width_minus1[ i ] is inferred to be equal to ( ( sps_pic_width_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − sps_subpic_ctu_top_left_x[ i ] − 1.

sps_subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_height_minus1[ i ] is inferred to be equal to ( ( sps_pic_height_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − sps_subpic_ctu_top_left_y[ i ] − 1.

There are use cases that picture consists of subpictures with only one spatial resolution.  For example, each subpicture with same spatial resolution represents a cubemap face for a 360 cubemap projection.  In other use case, several methods for viewport-dependent streaming for 360 video are enabled by the OMAF video media profiles that used MCTS tile of HEVC[1].  Considering the merging any tiles simply with these methods, the picture assumed to be encoded with same spatial tiles resolution. (yellow, green, and gray picture in Figure 1 a).  If these methods that used MCTS tile of HEVC adopt to the VVC, each picture would consist of independent subpictures with same spatial resolution.

Figure 1 example that picture consist of regions with same spatial resolution (Figure D.12 in OMAF 2nd edition)

In these use cases, all subpicture spatial resolution in picture can be expressed with common width and height.  In addtion, positions of top left of all subpicures can be derrived width/height of subpicture and width/height of picture.

Proposal

The followings are proposed for a shortcut method of signalling subpicture layout for bit-saving in SPS when all subpictures have same spatial resolution.

- To specify sps_subpic_same_size_flag in SPS whether all subpictures have same spatial resolution.

- When all subpictures have same spatial resolution,

  - to specify common width/height of subpicture indicated by width/height of 0-th subpicture (sps_subpic_width_minus1[0] and sps_subpic_height_minus1[0]),

  - to omit width and height of other than 0-th subpicture, and

  - to omit position of top left of i-th subpicture (sps_subpic_ctu_top_left_x[i] and sps_subpic_ctu_top_left_y[i]).

- To prohibit the use of the shortcut unless the picture is an exact multiple of common width/height of subpicture.

The impact of this proposal on the software is only deriving the omitted value of subpicture layout.

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-R2001-vA)

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) { |  |
| sps_independent_subpics_flag | u(1) |
| sps_subpic_same_size_flag | u(1) |
| } |  |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if(!sps_subpic_same_size_flag ¦¦ i == 0) { |  |
| if( i > 0 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_height_minus1[ i ] | u(v) |
| } |  |
| if( !sps_independent_subpics_flag) { |  |
| sps_subpic_treated_as_pic_flag[ i ] | u(1) |
| sps_loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Ceil( sps_pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( sps_pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive.

It is a requirement of bitstream conformance that when sps_subpic_same_size_flag is equal to 1, the value of sps_num_subpics_minus1 shall be equal to numSubpicCols *( ( sps_pic_height_max_in_luma_samples+ CtbSizeY − 1 ) / CtbSizeY ) / ( sps_subpic_height_minus1[ 0 ] + 1 ) - 1.

sps_subpic_same_size_flag equal to 1 specifies that all subpictures in the CLVS have the same width specified by sps_subpic_width_minus1[ 0 ] and the same height specified by sps_subpic_height_minus1[ 0 ]. sps_subpic_same_size_flag equal to 0 does not impose such a constraint.. When not present, the value of sps_subpic_same_size_flag is inferred to be equal to 0.

sps_subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) bits.

When sps_subpic_same_size_flag is equal to 1, the variable numSubpicCols, specifying the number of subpicture columns, is derived as follows:

numSubpicCols = ( ( sps_pic_width_max_in_luma_samples +
CtbSizeY − 1 ) / CtbSizeY )  / (sps_subpic_width_minus1[ 0 ] + 1  )	(xx)

When sps_subpic_ctu_top_left_x[ i ] is not present, it is inferred as follows:

–	If sps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of sps_subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

–	Otherwise, the value of sps_subpic_ctu_top_left_x[ i ] is inferred to be equal to ( i % numSubpicCols ) * (sps_subpic_width_minus1[ 0 ] + 1 ).

sps_subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits.

When sps_subpic_ctu_top_left_y[ i ] is not present, it is inferred as follows:

–	If sps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of sps_subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

–	Otherwise, the value of sps_subpic_ctu_top_left_y[ i ] is inferred to be equal to ( i / numSubpicCols )* (sps_subpic_height_minus1[ 0 ] + 1 ).

sps_subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits.

When sps_subpic_width_minus1[ i ] is not present, it is inferred as follows:

–	If sps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of sps_subpic_width_minus1[ i ] is inferred to be equal to ( ( sps_pic_width_max_in_luma_samples + CtbSizeY− 1 )  >> CtbLog2SizeY ) − sps_subpic_ctu_top_left_x[ i ] − 1.

–	Otherwise, the value of sps_subpic_width_minus1[i] is inferred to be equal to sps_subpic_width_minus1[ 0 ].

It is a requirement of bitstream conformance that when sps_subpic_same_size_flag is equal to 1, the value of ( ( sps_pic_width_max_in_luma_samples + CtbSizeY − 1 ) / CtbSizeY ) % ( sps_subpic_width_minus1[ 0 ] + 1 ) shall be equal to 0.

sps_subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits.

When sps_subpic_height_minus1[ i ] is not present, it is inferred as follows:

–	If sps_subpic_same_size_flag is equal to 0 or i is equal to 0, the value of sps_subpic_width_minus1[ i ] is inferred to be equal to ( ( sps_pic_height_max_in_luma_samples + CtbSizeY− 1 )  >> CtbLog2SizeY ) − sps_subpic_ctu_top_left_y[ i ] − 1.

–	Otherwise, the value of sps_subpic_height_minus1[ i ] is inferred to be equal to sps_subpic_height_minus1[ 0 ].

It is a requirement of bitstream conformance that when sps_subpic_same_size_flag is equal to 1, the value of ( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 ) / CtbSizeY ) % ( sps_subpic_height_minus1[ 0 ] + 1 ) shall be equal to 0.

Example

Example 1

Using the subpicture layout of that Figure 2 which each subpicture represents a cubemap face for a 360 cubemap projection, it compare the bit length of subpicture layout signalling between JVET-R2001-vA and proposal.  As a precondition, CtbSizeY is 128, picture spatial resolution is 3072x2048, each subpicture spatial resolution is 1024x1024.

Figure 2 each subpicture represents a cubemap face for a 360 cubemap projection,

Table 1 shows the bit length of syntax elements. “-“ indicates the syntax element is not present.

Table 1  bit length of the syntax element (example1)

| Syntax element | JVET-R2001-vA | Proposal |
| --- | --- | --- |
| sps_subpic_same_size_flag | - | 1 |
| subpic_ctu_top_left_x[ i ] | 5 (i=0 not signalled) | - |
| subpic_ctu_top_left_y[ i ] | 4 (i=0 not signalled) | - |
| subpic_width_minus1[ i ] | 5 (i=5 not signalled) | 5 (only i=0) |
| subpic_height_minus1[ i ] | 4 (i=5 not signalled) | 4 (only i=0) |
| Subpicture layout bit length | 90 | 10 |

Example 2

Using the subpicture layout of that Figure 3 which is example of yellow picture in Figure D.12 in OMAF 2nd edition specification it compare the bit length of subpicture layout signalling between JVET-R2001-vA and proposal. As a precondition, CtbSizeY is 128, picture spatial resolution is 6144x3072, each subpicture spatial resolution is 512x512 and number of subpictures is 12x6.

Figure 3  Figure D.12 in OMAF 2nd edition specification

Table 2 shows the bit length of syntax elements. “-“ indicates the syntax element is not present.

Table 2   bit length of the syntax element (example2)

| Syntax element | JVET-R2001-vA | Proposal |
| --- | --- | --- |
| sps_subpic_same_size_flag | - | 1 |
| subpic_ctu_top_left_x[ i ] | 6 (i=0 not signalled) | - |
| subpic_ctu_top_left_y[ i ] | 5 (i=0 not signalled) | - |
| subpic_width_minus1[ i ] | 6 (i=71 not signalled) | 6 (only i=0) |
| subpic_height_minus1[ i ] | 5 (i=71 not signalled) | 5 (only i=0) |
| Subpicture layout bit length | 1562 | 12 |

Simulation

The subpicture layout of Fig 3 that is used in simulation. The picture size is 1920x1080 with CTU size = 128x128.

Fig. 3 the subpicture layout that is used in simulation

bit savings estimate based on calculation

Table 2 shows the bit length of syntax elements. “-“ indicates the syntax element is not present.

Table. 2  bit length of the syntax element

| Syntax element | JVET-R2001-vA | Proposal |
| --- | --- | --- |
| sps_subpic_same_size_flag | - | 1 |
| subpic_ctu_top_left_x[ i ] | 4 (i=0 not signalled) | - |
| subpic_ctu_top_left_y[ i ] | 4 (i=0 not signalled) | - |
| subpic_width_minus1[ i ] | 4 (i=14 not signalled) | 4 (only i=0) |
| subpic_height_minus1[ i ] | 4 (i=14 not signalled) | 4 (only i=0) |
| Subpicture layout bit length | 224 | 9 |

Simulation results

The proposed methods were implemented on top of VTM-9.0, and simulations were conducted for ClassB/Random Access under the common test condition with subpicture layout in Figure 3.

- Test 1: VTM-9.0 vs JVET-S0071 Proposal

Configurations for subpicture layout in Figure 3 are:

- subpicture_15Slices-5x3rSubPic.cfg  (anchor)

- subpicture_15Slices-5x3rSubPic_S0071.cfg

Table 3 shows summary results of each test. It is observed that proposed methods provide a bit bdrate impacts.

Table 3 Results of each Test

|  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-8.0 |  |  |  |  |
| Test# | Class | Y | U | V | EncT | DecT |
| Test 1 | Class B | -0.0077% | -0.0084% | -0.0080% | 101% | 99% |

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- "Text of ISO/IEC DIS 23090-2 2nd edition OMAF", N19042, MPEG#129, Brussels, Belgium, March 2020