| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0072-v3 |
| --- | --- |

| Title: | AHG9: On PDPC and reference sample filtering of non-420 sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1-408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

PDPC and reference sampling filtering process are very efficient coding tool for capturing the characteristic of the natural camera capture content. However, it is observed that PDPC and reference filter may produce some visual artifacts for screen content sequences. This contribution proposes to add an SPS level flag to allow encoder flexibility to disable PDPC and reference filtering process for content (such as text, graphics, computer screen) where filtering and PDPC do not help.

It is asserted that disabling PDPC and reference filtering can reduce some visual artifacts especially for 444 content. It is also reported that as compared to VTM-8.0, disabling PDPC and reference filter can achieve -0.09% (Y), -0.07% (U), -0.04% (V) coding gain of YUV-444 TGM sequences.

# Problem Statement

In VVC, the neighboring reference samples are filtered in certain intra-prediction modes. In addition, the results of intra prediction of DC, planar and several angular modes are further modified by a position dependent intra prediction combination (PDPC) method.

Both the intra reference sample filtering process and PDPC are efficient for natural camera capture content where the correlation between the neighboring regions in an image is high. However, in case of screen content sequences rich in high frequency information, such as text and graphics content, it is observed that PDPC and reference filter produces some visual artifacts. Figure 1 shows an example of the visual artifact ( black dot around the diagonal line ) that appeared in VTM-8.0. The decoded image shown in Figure 1 is generated by VTM-8.0 with AI configuration, QP = 37.

Figure 1: Visual artifact near the diagonal yellow line of VTM-8.0 ( Console, YUV444, AI, QP=37, PLT = 0)

In HEVC, both reference sample filtering and boundary filtering (the concept of PDPC can be considered as an extension to the boundary filtering process of HEVC) process can be controlled by SPS flags, whereas in VVC, there is no high level flag to control the PDPC and reference sample filtering. That means, if an encoder wants to disable PDPC and reference filtering process for content (such as text, graphics, computer screen) where filtering and PDPC do not help, the current VVC specification does not allow it. It is further noted  that VVC has high level control of almost all of the major coding tools except PDPC and reference filtering.

# Proposed methods

This contribution proposes to allow the flexibility of enable/disable PDPC and reference filtering process in SPS level.  Following three options are proposed.

## Method 1: SPS flag to control PDPC

In method 1, it is proposed to add an SPS level flag to control PDPC. Following is the semantic of the proposed SPS flag:

sps_pdpc_disabled_flag equal to 1 specifies that the position-dependent intra prediction process is disabled for the CLVS. sps_pdpc_disabled_flag equal to 0 specifies that the position-dependent intra prediction process may be enabled for the CLVS. When not present, the value of sps_pdpc_disabled_flag is inferred to be 0.

Table 1 shows the SPS syntax table of the proposed method.

Table 1: SPS Syntax table of method 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| sps_pdpc_disabled_flag | u(1) |
| … |  |
| } |  |

## Method 2: SPS flag to control reference filter

In the method 2 of the proposed method, another SPS flag is introduced to control the intra reference sample filtering process. Following is the semantics of the proposed SPS flag:

sps_intra_reference_filter_disabled_flag equal to 1 specifies that the intra reference sample filtering process is disabled for the CLVS. sps_intra_reference_filter_disabled_flag is equal to 0 specifies that the intra reference sample filtering process may be enabled for the CLVS. When not present, the value of sps_intra_reference_filter_disabled_flag is inferred to be 0.

Table 2 shows the SPS syntax table of the proposed method.

Table 2: SPS Syntax table of method 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| sps_intra_reference_filter_disabled_flag | u(1) |
| } |  |

## Method 3: SPS flag for non-420 content

It is observed that disabling PDPC of 444 content has more coding gain and visual benefits as compared to 420 content. Therefore, as an alternative option, sps_pdpc_disabled_flag or sps_intra_reference_filter_disabled_flag may be signalled for both 444 content and 422 content, but not for 420 content.

## Constraint flags

This contribution also proposes the constraint flags for both PDPC and intra reference filter. The semantics of the proposed constraint flags are defined as follows:

no_pdpc_constraint_flag equal to 1 specifies that sps_pdpc_disabled_flag shall be equal to 1. no_pdpc_constraint_flag equal to 0 does not impose such a constraint.

no_ intra_reference_filter_constraint_flag equal to 1 specifies that sps_intra_reference_filter_disabled_flag  shall be equal to 1. no_intra_reference_filter_constraint_flag equal to 0 does not impose such a constraint.

Table 3: constraint flag for PDPC

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_pdpc_constraint_flag | u(1) |
| } |  |

Table 4: constraint flag for intra-reference filters

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_ intra_reference_filter_constraint_flag | u(1) |
| } |  |

# Simulation results

In order to verify the performance of the proposed method, following tests are conducted under CTC conditions. VTM-8.0 is used as anchor.

- Test 1: In this test, PDPC is disabled on top of VTM-8.0. Table 5 shows the simulation results of test 1 for YUV-420 TGM sequences. The results of test 1 of YUV444 TGM sequences are shown in Table 6. Table 7 shows the simulation results of test 1 of RGB sequences.

- Test 2: In this test, both PDPC and reference sample filtering process are disabled on top of VTM-8.0. Table 8 shows the simulation results of test 2 for YUV-420 TGM sequences. The results of test 2 of YUV444, and RGB TGM sequences are shown in Table 9 and Table 10, respectively.

Table 5: simulation results of YUV-420 sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC OFF)

|  | YUV-420 | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | 0.21% | -0.07% | -0.11% |
|  | Desktop | -0.15% | -0.30% | -0.26% |
|  | Console | -0.02% | 0.16% | 0.05% |
|  | ChineseEditing | -0.08% | -0.18% | -0.13% |
|  | Overall | -0.01% | -0.09% | -0.11% |
| RA | FlyingGraphics | 0.12% | -0.12% | -0.04% |
|  | Desktop | 0.01% | -0.20% | -0.18% |
|  | Console | -0.06% | 0.05% | 0.00% |
|  | ChineseEditing | -0.04% | -0.14% | -0.18% |
|  | Overall | 0.01% | -0.10% | -0.10% |
| LB | FlyingGraphics | 0.11% | -0.12% | -0.03% |
|  | Desktop | -0.10% | -0.27% | -0.12% |
|  | Console | -0.16% | 0.02% | -0.24% |
|  | ChineseEditing | -0.18% | -0.04% | -0.13% |
|  | Overall | -0.08% | -0.10% | -0.13% |

Table 6: simulation results of YUV-444 sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC OFF)

|  | YUV-444 | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | -0.1% | -0.1% | -0.1% |
|  | Desktop | -0.1% | 0.0% | 0.1% |
|  | Console | -0.2% | -0.1% | -0.1% |
|  | ChineseEditing | 0.1% | 0.1% | 0.1% |
|  | Overall | -0.08% | -0.04% | -0.02% |
| RA | FlyingGraphics | 0.1% | -0.1% | -0.1% |
|  | Desktop | 0.0% | 0.1% | 0.2% |
|  | Console | -0.1% | 0.0% | -0.1% |
|  | ChineseEditing | 0.0% | 0.1% | 0.1% |
|  | Overall | 0.02% | 0.03% | 0.02% |
| LB | FlyingGraphics |  |  |  |
|  | Desktop | -0.1% | -0.2% | 0.0% |
|  | Console |  |  |  |
|  | ChineseEditing | 0.0% | 0.0% | -0.1% |
|  | Overall |  |  |  |

Table 7: simulation results of RGB sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC OFF)

|  | RGB | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | -0.3% | -0.2% | -0.2% |
|  | Desktop |  |  |  |
|  | Console | -0.2% | -0.1% | -0.1% |
|  | ChineseEditing |  |  |  |
|  | Overall |  |  |  |
| RA | FlyingGraphics |  |  |  |
|  | Desktop | -0.2% | 0.0% | 0.0% |
|  | Console | -0.1% | 0.0% | -0.1% |
|  | ChineseEditing |  |  |  |
|  | Overall |  |  |  |
| LB | FlyingGraphics |  |  |  |
|  | Desktop | -0.3% | -0.1% | -0.1% |
|  | Console |  |  |  |
|  | ChineseEditing |  |  |  |
|  | Overall |  |  |  |

Table 8: simulation results of YUV-420 sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC off + filter OFF)

|  | YUV-420 | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | 0.21% | -0.19% | -0.16% |
|  | Desktop | -0.16% | -0.27% | -0.28% |
|  | Console | -0.08% | 0.15% | -0.03% |
|  | ChineseEditing | -0.09% | -0.20% | -0.17% |
|  | Overall | -0.03% | -0.13% | -0.16% |
| RA | FlyingGraphics | 0.02% | -0.02% | -0.08% |
|  | Desktop | -0.14% | -0.30% | -0.20% |
|  | Console | -0.02% | 0.01% | -0.04% |
|  | ChineseEditing | 0.00% | -0.03% | -0.21% |
|  | Overall | -0.03% | -0.09% | -0.13% |
| LB | FlyingGraphics | 0.09% | 0.04% | 0.09% |
|  | Desktop | 0.18% | -0.14% | -0.01% |
|  | Console | -0.12% | 0.13% | -0.17% |
|  | ChineseEditing | 0.09% | -0.26% | -0.04% |
|  | Overall | 0.06% | -0.06% | -0.03% |

Table 9: simulation results of YUV-444 sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC off + filter OFF)

|  | YUV-444 | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | -0.1% | -0.2% | -0.2% |
|  | Desktop | -0.1% | 0.0% | 0.1% |
|  | Console | -0.2% | -0.1% | -0.1% |
|  | ChineseEditing | 0.0% | 0.1% | 0.1% |
|  | Overall | -0.09% | -0.07% | -0.04% |
| RA | FlyingGraphics | 0.2% | 0.0% | 0.0% |
|  | Desktop | 0.1% | 0.1% | 0.1% |
|  | Console | -0.1% | -0.1% | 0.0% |
|  | ChineseEditing | 0.0% | 0.0% | 0.0% |
|  | Overall | 0.04% | 0.01% | 0.02% |
| LB | FlyingGraphics |  |  |  |
|  | Desktop | -0.1% | -0.1% | -0.1% |
|  | Console |  |  |  |
|  | ChineseEditing | 0.1% | 0.1% | 0.1% |
|  | Overall |  |  |  |

Table 10: simulation results of RGB sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC off + filter OFF)

|  | RGB | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | -0.4% | -0.2% | -0.2% |
|  | Desktop |  |  |  |
|  | Console | -0.3% | -0.1% | -0.2% |
|  | ChineseEditing |  |  |  |
|  | Overall |  |  |  |
| RA | FlyingGraphics |  |  |  |
|  | Desktop | -0.3% | 0.0% | 0.0% |
|  | Console | -0.1% | 0.0% | -0.1% |
|  | ChineseEditing |  |  |  |
|  | Overall |  |  |  |
| LB | FlyingGraphics |  |  |  |
|  | Desktop | -0.1% | -0.1% | 0.0% |
|  | Console |  |  |  |
|  | ChineseEditing |  |  |  |
|  | Overall |  |  |  |

Table 11 and Table 12 shows the additional results of YUV-444 sequences when palette mode and dual tree mode is disabled for both anchor and test.

Table 11: simulation results of YUV-444 sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC OFF; PLT and dual tree off for both anchor and test)

|  | YUV-444 | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | 0.2% | -0.1% | -0.1% |
|  | Desktop | -0.3% | -0.4% | -0.4% |
|  | Console | -0.3% | -0.3% | -0.3% |
|  | ChineseEditing | -0.3% | -0.6% | -0.6% |
|  | Overall | -0.17% | -0.33% | -0.35% |
| RA | FlyingGraphics |  |  |  |
|  | Desktop |  |  |  |
|  | Console | -0.1% | -0.2% | -0.2% |
|  | ChineseEditing | -0.3% | -0.4% | -0.6% |
|  | Overall |  |  |  |
| LB | FlyingGraphics | 0.1% | 0.0% | 0.0% |
|  | Desktop | -0.1% | -0.2% | -0.2% |
|  | Console |  |  |  |
|  | ChineseEditing | 0.0% | -0.3% | -0.2% |
|  | Overall |  |  |  |

Table 12: simulation results of YUV-444 sequences ( Anchor: VTM-8.0; Test : VTM-8.0 + PDPC off + filter OFF; PLT and dual tree off for both anchor and test)

|  | YUV-444 | Y | U | V |
| --- | --- | --- | --- | --- |
| AI | FlyingGraphics | 0.2% | -0.1% | -0.1% |
|  | Desktop | -0.3% | -0.5% | -0.4% |
|  | Console | -0.3% | -0.3% | -0.4% |
|  | ChineseEditing | -0.4% | -0.6% | -0.6% |
|  | Overall | -0.20% | -0.35% | -0.38% |
| RA | FlyingGraphics | 0.1% | -0.2% | -0.1% |
|  | Desktop | -0.2% | -0.3% | -0.4% |
|  | Console | -0.1% | -0.2% | -0.2% |
|  | ChineseEditing | -0.3% | -0.3% | -0.5% |
|  | Overall | -0.14% | -0.26% | -0.30% |
| LB | FlyingGraphics | 0.0% | -0.1% | -0.1% |
|  | Desktop | -0.1% | -0.4% | -0.4% |
|  | Console | -0.1% | -0.3% | -0.2% |
|  | ChineseEditing | 0.0% | -0.3% | -0.3% |
|  | Overall | -0.05% | -0.28% | -0.24% |

Figure 2: decoded image of VTM-8.0

Figure 3: decoded image ( VTM-8.0 + PDPC OFF)

Figure 4: decoded image ( VTM-8.0 + PDPC OFF + Filter OFF)

# Visual quality analysis

It is asserted that disabling PDPC and reference filtering can reduce some of the visual artifact of screen content sequences.  Figure 2, Figure 3 and Figure 4 shows the decoded image of console sequence under CTC test conditions.

Figure 5, Figure 6, and Figure 7 shows the part of the decoded image of the Desktop video sequence.

Figure 5: decoded image of VTM-8.0

Figure 6: decoded image of VTM-8.0 + PDPC-OFF

Figure 7: Decoded image VTM-8.0 + PDPC-off + Filter OFF

Figure 8, Figure 9, and Figure 10 shows the part of the decoded image of the Console_YUV_444 video sequence. In this test, the palette mode and single tree is disabled for both anchor and test. The images are encoded with AI configuration with QP = 37. As shown in Figure 8, it is shown that VTM-8.0 anchor introduces some black dot around the diagonal yellow line, whereas, in Figure 9 andFigure 10, there is no such black dots.

Figure 8: decoded image of VTM-8.0 anchor

Figure 9: decoded image of test 1 (VTM-8.0 + PDPC OFF)

Figure 10: decoded image of test 2 (VTM-8.0 + PDPC OFF + reference filter OFF)

# Conclusion

This contribution proposes an SPS flag to control PDPC and reference filtering process. Method 1 proposes the SPS flag for PDPC and method 2 proposes an SPS flag for reference filtering process. Method 3 proposes to signal that SPS flag only for non-420 sequences. It is observed that disabling PDPC and reference filtering can improve the visual quality of some sequences especially for YUV 444 sequences.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).