| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0082 |
| --- | --- |

| Title: | AHG8/AHG9: On signaling of inter-layer reference picture layer index |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Hendry Seethal Paluri Junghak Nam Seung-hwan Kim Jaehyun Lim | Email: | hm.jang@lge.com dr.hendry@lge.com seethal.paluri@lge.com junghak.nam@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution asserted that the syntax element ilrp_idx[ listIdx ][ rplsIdx ][ i ] is not needed to be present when it is known that there is only one direct reference layer for the current layer.

In this contribution, it is proposed that when the value of sps_inter_layer_ref_pics_present_flag is equal to 1, a new syntax element called sps_ref_layer_idx_plus1 is signalled to specify the layer index of the direct reference layer of the inter-layer reference pictures of the pictures in the CLVS. The new syntax element is used as follows:

- If the CLVS has only one direct reference layer, the value of sps_ref_layer_idx_plus1 is greater than 0. Syntax element ilrp_idx[ listIdx ][ rplsIdx ][ i ] is not present when sps_ref_layer_idx_plus1 is not equal to 0 and its value is derived to be equal to sps_ref_layer_idx_plus1 − 1.

- Otherwise (i.e., the CLVS has multiple reference layers), the value of sps_ref_layer_idx_plus1 is equal to 0.

# Introduction

It is observed that one of common use cases of scalable bitstream is the case where each layer has only one direct reference layer (e.g., use cases of spatial scalability). For such use case, there is no need to signal index of direct inter-layer reference picture list (i.e., ilrp_idx[ listIdx ][ rplsIdx ][ i ]) for every picture or for every RPL structure since all of them would have the same value.

In this contribution, we propose to exploit the above observed fact which allow us to avoid signalling ilrp_idx[ listIdx ][ rplsIdx ][ i ]) for every picture or for every RPL structure.

# Proposal and spec text changes

The following changes are proposed:

- When the value of sps_long_term_ref_pics_flag is equal to 1, a new syntax element called sps_ref_layer_idx_plus1.

- If the CLVS has only one direct reference layer, the value of sps_ref_layer_idx_plus1 is greater than 0. Syntax element ilrp_idx[ listIdx ][ rplsIdx ][ i ] is not present when sps_ref_layer_idx_plus1 is not equal to 0 and its value is derived to be equal to sps_ref_layer_idx_plus1 − 1.

- Otherwise (i.e., the CLVS has multiple reference layers), the value of sps_ref_layer_idx_plus1 is equal to 0.

The spec text changes for implementing the above proposed changes is as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_video_parameter_set_id > 0 ) |  |
| sps_inter_layer_ref_pics_present_flag | u(1) |
| if( sps_inter_layer_ref_pics_present_flag ) |  |
| sps_ref_layer_idx_plus1 | ue(v) |
| ... |  |
| } |  |

sps_ref_layer_idx_plus1 minus 1 specifies the layer index of the direct reference layer referred to by pictures in the CLVS. sps_ref_layer_idx_plus1 equal to 0 specifies that there are more than one direct reference layer referred to by pictures in the CLVS.

It is a constraint of conformance bitstream that the following applies:

- If the value of NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] is greater than 1, the value of sps_ref_layer_idx_plus1 shall be equal to 0.

- Otherwise (i.e., NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1), the value of sps_ref_layer_idx_plus1 shall be equal to DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ 0 ] + 1

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( sps_long_term_ref_pics_flag && rplsIdx < sps_num_ref_pic_lists[ listIdx ] ) |  |
| ltrp_in_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( sps_inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| ... |  |
| } else if( sps_ref_layer_idx_plus1 = = 0) |  |
| ilrp_idx[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

ilrp_idx[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of the direct reference layers, of the ILRP of the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. When not present, the value of ilrp_idx[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to sps_ref_layer_idx_plus1 − 1. The value of ilrp_idx[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).