| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0086 |
| --- | --- |

| Title: | AHG9: On AUD NAL units in the sub-bitstream extraction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes the following asserted bug fixes of the sub-bitstream extraction process (clause C.6) for determining which AUD NAL units of the input bitstream are kept in the output sub-bitstream:

- Add AUD_NUT among the NAL unit types that are kept in the output sub-bitstream regardless of its nuh_layer_id value.

- Add removal of an AUD NAL unit for which all VCL NAL units of an AU are removed by the sub-bitstream extraction.

# Problem statement

The sub-bitstream extraction process specified in clause C.6 starts with the following:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	…

The third bullet (indicated with red font color) removes AUD NAL units when they happen to have a nuh_layer_id value not among the layers in the target OLS. However, an AUD NAL unit is required to be present, when vps_max_layers_minus1 is greater than 0, as specified in clause 7.4.2.4.3.

# Summary of the proposal

It is proposed to modify the sub-bitstream extraction process of clause C.6 as follows:

- Add AUD_NUT among the NAL unit types that are kept in the output sub-bitstream regardless of its nuh_layer_id value.

- Add removal of an AUD NAL unit for which all VCL NAL units of an AU are removed by the sub-bitstream extraction.

# Proposed specification text

The additions indicated by red are proposed:

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, AUD_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	Remove from outBitstream all VCL NAL units for which all of the following conditions are true, and their associated non-VCL NAL units with nal_unit_type equal to PH_NUT, FD_NUT, SUFFIX_SEI_NUT, and PREFIX_SEI_NUT with PayloadType not equal to 0, 1, or 130:

–	nal_unit_type is equal to TRAIL_NUT, STSA_NUT, RADL_NUT, or RASL_NUT, or nal_unit_type is equal to GDR_NUT and the associated ph_recovery_poc_cnt is not equal to 0.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

–	When all VCL NAL units of an AU are removed by any derivation step above and an AUD NAL unit is present in the AU, remove the AUD NAL unit from outBitstream.

–	…

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).