| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0089 |
| --- | --- |

| Title: | AHG8: On TMVP derivation using inter-layer reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the current VVC specification, inter layer reference picture (ILRP) which is marked as a long-term reference is allowed to be a reference picture of current picture or collocated picture for temporal motion vector prediction (TMVP). When ILRP and LTRP of which nuh_layer_id is equal to nuh_layer_id of the current picture are used together, undesirable behavior is observed. In addition, the combination of LTRP and ILRP is not commonly used compared to the STRP and ILRP case. In this contribution, three solutions are proposed to address TMVP behavior more clearly when ILRP is used as a reference picture for TMVP.

Solution 1: Disabling TMVP when either RefPic or ColRefPic is ILRP, but not both.

Solution 2: Enabling TMVP with combination of STRP and ILRP on RefPic or ColRefPic, and disabling TMVP with combination of LTRP and ILRP.

Solution 3: Enabling TMVP when either RefPic or ColRefPic is ILRP.

Problem Statement

In current VVC, when inter layer prediction is available, ILRP (inter-layer reference picture) can be used for TMVP as a reference picture of the current picture (RefPic) or a reference picture of the collocated picture (ColRefPic). In current design, if reference picture type of the RefPic and ColRefPic is the same, colMv for TMVP is derived, but if the reference picture type is different, TMVP candidate becomes not available. Case 1 and Case 2 show the simple example of the current behavior for TMVP when reference picture type of RefPic and ColRefPic is same.

Case 1                                              Case2

However, unexpected behavior is performed in uncommon use cases like as RefPic or ColRefPic is ILRP. Case 3 ~ Case 6 show the example of RefPic is ILRP. Case 7 ~ Case 10 show the example of ColRefPic is ILRP. Actually, the ILRP is marked as “used for long-term reference” but does not mean having long distance between reference picture and current picture. But in current design, TMVP is available only when reference picture type of RefPic-ColRefPic is mixed like of LTRP-ILRP, ILRP-LTRP or ILRP-ILRP except for STRP-ILRP and ILRP-STRP.

Case 3                                              Case 4

Case 5                                              Case 6

Case 7                                              Case 8

Case 9                                              Case 10

Proposed Solutions

Three solutions are proposed to address TMVP behavior more clearly.

- The colMv derivation process for TMVP is performed only when both reference picture type of RefPic and ColRefPic are LTRP (here, ILRP is marked as LTRP). The combination case of LTRP – ILRP (or ILRP-LTRP) is not commonly used as the case of STRP-ILRP (or ILRP-STRP). Therefore, Solution1 is proposed not to consider TMVP for unusual combination cases.

Solution1: mixed cases including the ILRP are not considered for TMVP because the only rare cases, mixed LTRP are considered in current design.

- With another point from Solution1, it may not seem to be desirable to derive the TMVP when RefPic or ColRefPic is LTRP with nuh_layer_id less than nuh_layer_id of the current picture. Therefore, Solution2 is proposed as follow.

Solution 2: contrary to the current design, STRP-ILRP (or ILRP-STRP) is considered to ensure more common cases.

- At last point, considering that the ILRP is marked as “used for long-term reference”, it may not mean to have long distance between reference picture and current picture. Hence, Solution3 is proposed to remove restriction on TMVP when ILRP is used.

Solution 3: mixed cases including the ILRP are always considered for TMVP.

The below table shows the behavior of the current design and proposed solutions.

| RefPic | ColRefPic | Current | Solution 1 | Solution 2 | Solution 3 |
| --- | --- | --- | --- | --- | --- |
| STRP | ILRP | X | X | O | O |
| ILRP | STRP | X | X | O | O |
| LTRP | ILRP | O | X | X | O |
| ILRP | LTRP | O | X | X | O |

Proposed draft changes

Solution 1:

In 8.5.2.12 derivation process for collocated motion vectors, the LongTermRefPic() function is modified as follows. If the nuh_layer_id of the reference picture and the current, the return value of LongTermRefPic() function is set to 2, otherwise, the current rules are retained. The modified texts are marked as yellow highlight.

The function LongTermRefPic( aPic, aPb, refIdx, LX ), with X being 0 or 1, is defined as follows: - If nuh_layer_id of the picture with index refIdx from reference picture list LX of the slice containing prediction block aPb in the picture aPic was not equal to nuh_layer_id of the picture aPic at the time when aPic was the current picture, LongTermRefPic(aPic, aPb, refIdx, LX) is equal to 2. - Otherwise, if the picture with index refIdx from reference picture list LX of the slice containing prediction block aPb in the picture aPic was marked as "used for long-term reference" at the time when aPic was the current picture, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 1. - Otherwise, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 0. … - When availableFlagLXCol is equal to TRUE, mvLXCol and availableFlagLXCol are derived as follows: - If LongTermRefPic( currPic, currCb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colCb, refIdxCol, listCol ), both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0. - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicList[ listCol ][ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing coding block colCb in the collocated picture specified by ColPic, and the following applies: colPocDiff = DiffPicOrderCnt( ColPic, refPicList[ listCol ][ refIdxCol ] ) (607) currPocDiff = DiffPicOrderCnt( currPic, RefPicList[ X ][ refIdxLX ] ) (608) - The temporal motion buffer compression process for collocated motion vectors as specified in clause 8.5.2.15 is invoked with mvCol as input, and the modified mvCol as output. - If RefPicList[ X ][ refIdxLX ] is an LTRP, or colPocDiff is equal to currPocDiff, mvLXCol is derived as follows: mvLXCol = Clip3( −131072, 131071, mvCol ) (609) - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol as follows: tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (610) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (611) mvLXCol = Clip3( −131072, 131071, (distScaleFactor * mvCol + 128 − ( distScaleFactor * mvCol >= 0 ) ) >> 8 ) (612) where td and tb are derived as follows: td = Clip3( −128, 127, colPocDiff ) (613) tb = Clip3( −128, 127, currPocDiff ) (614)

Solution 2:

In 8.5.2.12 derivation process for collocated motion vectors, the LongTermRefPic() function is modified as follows. The modified texts are marked as yellow highlight.

The function LongTermRefPic( aPic, aPb, refIdx, LX ), with X being 0 or 1, is defined as follows: - If the picture with index refIdx from reference picture list LX of the slice containing prediction block aPb in the picture aPic was marked as "used for long-term reference" and had the nuh_layer_id same with nuh_layer_id of the picture aPic at the time when aPic was the current picture, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 1. - Otherwise, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 0. … - When availableFlagLXCol is equal to TRUE, mvLXCol and availableFlagLXCol are derived as follows: - If LongTermRefPic( currPic, currCb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colCb, refIdxCol, listCol ), both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0. - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicList[ listCol ][ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing coding block colCb in the collocated picture specified by ColPic, and the following applies: colPocDiff = DiffPicOrderCnt( ColPic, refPicList[ listCol ][ refIdxCol ] ) (607) currPocDiff = DiffPicOrderCnt( currPic, RefPicList[ X ][ refIdxLX ] ) (608) - The temporal motion buffer compression process for collocated motion vectors as specified in clause 8.5.2.15 is invoked with mvCol as input, and the modified mvCol as output. - If RefPicList[ X ][ refIdxLX ] is an LTRP, colPocDiff is equal to 0, or colPocDiff is equal to currPocDiff, mvLXCol is derived as follows: mvLXCol = Clip3( −131072, 131071, mvCol ) (609) - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol as follows: tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (610) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (611) mvLXCol = Clip3( −131072, 131071, (distScaleFactor * mvCol + 128 − ( distScaleFactor * mvCol >= 0 ) ) >> 8 ) (612) where td and tb are derived as follows: td = Clip3( −128, 127, colPocDiff ) (613) tb = Clip3( −128, 127, currPocDiff ) (614)

Solution 3:

In 8.5.2.12 derivation process for collocated motion vectors, followings are modified. The modified texts are marked as yellow highlight.

The function LongTermRefPic( aPic, aPb, refIdx, LX ), with X being 0 or 1, is defined as follows: - If the picture with index refIdx from reference picture list LX of the slice containing prediction block aPb in the picture aPic was marked as "used for long-term reference" at the time when aPic was the current picture, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 1. - Otherwise, LongTermRefPic( aPic, aPb, refIdx, LX ) is equal to 0. … - When availableFlagLXCol is equal to TRUE, mvLXCol and availableFlagLXCol are derived as follows: - If LongTermRefPic( currPic, currCb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colCb, refIdxCol, listCol ) and nuh_layer_id of the picture currPic and ColPic is equal to nuh_layer_id of the reference picture of its, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0. - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicList[ listCol ][ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing coding block colCb in the collocated picture specified by ColPic, and the following applies: colPocDiff = DiffPicOrderCnt( ColPic, refPicList[ listCol ][ refIdxCol ] ) (607) currPocDiff = DiffPicOrderCnt( currPic, RefPicList[ X ][ refIdxLX ] ) (608) - The temporal motion buffer compression process for collocated motion vectors as specified in clause 8.5.2.15 is invoked with mvCol as input, and the modified mvCol as output. - If RefPicList[ X ][ refIdxLX ] is an LTRP, colPocDiff is equal to 0, or colPocDiff is equal to currPocDiff, mvLXCol is derived as follows: mvLXCol = Clip3( −131072, 131071, mvCol ) (609) - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol as follows: tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (610) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (611) mvLXCol = Clip3( −131072, 131071, (distScaleFactor * mvCol + 128 − ( distScaleFactor * mvCol >= 0 ) ) >> 8 ) (612) where td and tb are derived as follows: td = Clip3( −128, 127, colPocDiff ) (613) tb = Clip3( −128, 127, currPocDiff ) (614)

Conclusion

If inter-layer prediction is available, and RefPic or ColRefPic is inter layer reference picture (ILRP), undesirable behavior is observed in TMVP decoding process. To address the TMVP behavior more clearly, three solutions are proposed. In solution 1, mixed cases including the ILRP are not considered for TMVP and in solution 2, contrary to the current design, STRP-ILRP cases are considered for TMVP. And finally, in solution 3, mixed cases including the ILRP are always considered for TMVP.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 9)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-R2001(JVET-Q2001-vA), 18th Meeting: teleconference, 15–24 April 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).