| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0094 |
| --- | --- |

| Title: | AHG9: Profile-level constraints on chroma format and bit depth constraint flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karl Sharman Adrian Browne Steve Keating | Email: | karl.sharman@sony.com adrian.browne@sony.com steve.keating@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

In the current draft of VVC, the bit depth (“sps_bit_depth_minus8”) and the chroma format (“sps_chroma_format_idc”) are constrained by the constraint flags “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc”. In addition, they are constrained in the specification of each profile.

However, there is no constraint of specification of what “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc” should be for each profile. This means that it is possible for a stream to indicate maximums of 16 bit 4:4:4, but only be a 4:2:0 10-bit profile.

The constraint flags are present in the VPS, describing each layer, and optionally present in the SPS.

Five options are explored, and it is proposed that a change is made to remove this ambiguity. The fifth option proposes modifying the coding scheme, which allows the value of 0 to be used to mean no additional constraint is present, aligning its use with that of other Boolean constraint flags.

# Problem statement

In the current draft of VVC, the bit depth (“sps_bit_depth_minus8”) and the chroma format (“sps_chroma_format_idc”) are constrained by the constraint flags “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc”. In addition, they are constrained in the specification of each profile.

However, there is no constraint of specification of what “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc” should be for each profile. This means that it is possible for a stream to indicate maximums of 16 bit 4:4:4, but only be a 4:2:0 10-bit profile.

The constraint flags are present in the VPS, describing each layer, and optionally present in the SPS.

It is proposed that the constraint flags are constrained by the profile to prevent such unusual conditions.

The options are as follows:

- The constraint flags may be set to any value less than or equal to that implied by the profile; and add these constraints to the profile definition.

- The constraint flags are set exactly to maximum value implied by the profile; and add these constraints to the profile definition.

- The constraint flags may be set to any value less than or equal to that implied by the profile; replace the bit_depth_minus8 and chroma_format_idc constraints with these constraints to the profile definition. The original constraints are not necessary, as the is already a constraint on chroma_format_idc and bit_depth_minus8.

- The constraint flags are set exactly to maximum value implied by the profile; replace the bit_depth_minus8 and chroma_format_idc with these constraints to the profile definition. The original constraints are not necessary, as the is already a constraint on chroma_format_idc and bit_depth_minus8.

- Use of a different coding scheme for max_chroma_format_constraint_idc and max_bitdepth_constraint_idc, where the value of 0 indicated that there is no additional constraint (over than defined by the profile) is placed on the respective values. This aligns these syntax elements to Boolean constraint flags, where 0 means that there is no constraint.

With this modified scheme, potentially the constraint section of the profile-tier-level structure could then be just 0 for the case of no constraints required on top of those defined in the profile, and therefore could potentially be skipped from the code-stream in those cases, as the default values of 0 would be sufficient.

In this modified coding scheme, effectively max_chroma_format_constraint_idc_plus1 is used. The value of 0 would be used to indicate that no additional constraint is placed. The element remains 2 bits, since the 4:4:4 format within a 4:4:4 profile would be equivalent to no additional constraint being added, and so have the value 0.

In addition, the max_bitdepth_constraint_idc (which is actually currently max_bitdepth_constraint_minus8), would be coded as max_bitdepth_constraint_minus7_idc, with the value 0 being reserved for unconstrained values.

# Draft text.

The draft text of each of these is shown in the following sections, relative to JVET-R2001-vA.docx.

They illustrate the text for one profile, “Main 10”. However, corresponding changes would be needed for the other profiles described.

## Option 1: Addition of non-specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have sps_chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 0 or 1.

- Referenced SPSs shall have sps_bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present in the range of 0 to 2, inclusive.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

## Option 2: Addition of specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have sps_chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 1.

- Referenced SPSs shall have sps_bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present equal to 2.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

## Option 3: Replacement of non-specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have sps_chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 0 or 1.

- Referenced SPSs shall have sps_bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present in the range of 0 to 2, inclusive.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

## Option 4: Replacement of specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have sps_chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 1.

- Referenced SPSs shall have sps_bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present equal to 2.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

## Option 5: Modified Coding scheme

## This modified coding scheme can be used with option 1 (though not required); potentially the syntax elements should be renamed (e.g. max_bitdepth_constraint_minus7_idc, and max_chroma_format_constraint_plus1_idc). In addition, if it becomes possible for the constraints to not be present, text for their inference is also given in [italics].

#### General constraint information semantics

…

max_bitdepth_constraint_idc greater than 0 specifies that sps_bit_depth_minus8 shall be in the range of 0 to max_bitdepth_constraint_idc-1, inclusive. [When not present, the value of max_bitdepth_constraint_idc is inferred to be equal to 0.]

max_chroma_format_constraint_idc greater than 0 specifies that sps_chroma_format_idc shall be in the range of 0 to max_chroma_format_constraint_idc-1, inclusive. [When not present, the value of max_chroma_format_constraint_idc is inferred to be equal to 0.]

# Conclusion

It is proposed that one of these changes is adopted to remove the ambiguity of these constraint flags.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).