| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0098 |
| --- | --- |

| Title: | AHG9/12: On subpicture conformance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Introduction

At the last meeting, it was agreed to allow higher layers of an OLS to contain multiple subpictures while the lower layers of the OLS do not use subpicture partitioning. Extensions to the conformance testing related sections of the VVC draft specification are required to handle such bitstreams, especially when individual subpictures are extracted.

This contribution proposes fixes for the following items:

- Extraction of subpictures with non-subpicture reference layers and scaling windows

- RprConstraintsActive[ i ][ j ] derivation mismatches

- OLS-specific and layer-specific constraints

- SubPicSet accumulation

# Proposals

Item 1) Extraction of subpictures with non-subpicture reference layers

When a higher layer of an OLS contains more than one subpicture and a lower layer contains no subpicture (inferred subpicture index equal to 0), the current subpicture sub-bitstream extraction process in C.7 would carry out many incorrect SPS/PPS adjustments and even remove the VCL NAL units belonging to the no-subpicture layer if different subpicture index than 0 is extracted. It is therefore proposed to add a condition on the layer-specific changes in C.7 based on the sps_num_subpics_minus1 in the SPS, which is inferred equal to 0 for non-subpicture layers as highlighted in green in the text below.

However, in the scenario described above, subpicture and non-subpicture layers require adjustment of the PPSs of non-subpicture layers with respect to scaling window offsets. Otherwise, the extracted subpicture sub-bitstream would carry incorrect scaling window offsets after extraction of a subpicture.

A simple example with scaling factor equal to 1 in the original bitstream is given in Figure 1, where the left-hand side depicts an original bitstream with three subpictures (index 0 to 2) in L1 and no subpictures in L0. When, for instance, the right subpicture with index 2 is extracted according to the current VVC draft specification as illustrated in the middle of Figure 1 (in this case assuming scaling window offsets are set to zero in the original bitstream), the L1 picture would be reconstructed incorrectly from the extracted sub-bitstream as an incorrect RefPicScale and fRefLeftOffset and fRefTopOffset would be derived in the decoding process compared to the original bitstream.

Figure 1: Scaling windows offset adjustments in case of extraction subpicture extraction.

Adjusting the scaling window of non-subpicture layers in the OLS as illustrated on the right-hand side of Figure 1 would allow correct reconstruction, i.e. setting scaling window offset in subpicture layers equal to 0 and adjusting the non-subpicture layer scaling window offset accordingly.

The alternative of adjusting only the subpicture layer scaling window offsets to accommodate the changing picture area might quickly violate the constraint that the sum of the absolute scaling window offsets (vertical and horizontal) shall not be bigger than the coded picture height or width, respectively.

Therefore, it is proposed to add further steps to the subpicture sub-bitstream extraction process in C.7 to adjust the scaling window signalling in subpicture and non-subpicture layers so that they correspond to the correct picture subsection that aligns with the extracted subpicture. For this purpose, the loop over layers is reversed, starting with the highest layer to store the subpicture coordinates which are used to set the scaling window offsets in non-subpicture layers later on as follows with related changes highlighted in yellow:

The output sub-bitstream outBitstream is derived as follows:

–	The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

–	If some external means not specified in this Specification is available to provide replacement parameter sets for the sub-bitstream outBitstream, replace all parameter sets with the replacement parameter sets.

–	Otherwise, when subpicture level information SEI messages are present in inBitstream, the following applies:

[…]

For the i-th layer with i in from NumLayersInOls[ targetOlsIdx ] – 1 to 0, the following applies.

–	When sps_num_subpics_minus1 is greater than 0, the following applies.

[…] // level_idc, picture size, conformance window rewriting

–	The variables subpicLeftOffsetToScalWin, subpicRightOffsetToScalWin, subpicTopOffsetToScalWin, subpicBottomOffsetToScalWin, subpicLayerScalWinWidth, and subpicLayerScalWinHeight are derived as follows:

subpicLeftOffsetToScalWin = sps_subpic_ctu_top_left_x[ subpicIdx ] * CtbSizeY −
		pps_scaling_win_left_offset		(C.XX)

subpicRightOffsetToScalWin = min ( ( sps_subpic_ctu_top_left_x[ subpicIdx ] +
	sps_subpic_width_minus1[ subpicIdx ] + 1 ) * CtbSizeY, 		pps_pic_width_in_luma_samples ) − pps_pic_width_in_luma_samples + 
	pps_scaling_win_right_offset		(C.XX)

subpicTopOffsetToScalWin = 	sps_subpic_ctu_top_left_y[ subpicIdx ] * CtbSizeY −
		pps_scaling_win_top_offset		(C.XX)

subPicBottomOffsetToScalWin = min( (sps_subpic_ctu_top_left_y[ subpicIdx ] +
	sps_subpic_height_minus1[ subpicIdx ] + 1 ) * CtbSizeY, 	pps_pic_height_in_luma_samples) − pps_pic_height_in_luma_samples + 
	pps_scaling_win_bottom_offset	(C.XX)

subpicLayerScalWinWidth = pps_pic_width_in_luma_samples −
	pps_scaling_win_left_offset − pps_scaling_win_right_offset	(C.XX)

subpicLayerScalWinHeight = pps_pic_height_in_luma_samples −
	pps_scaling_win_top_offset − pps_scaling_win_bottom_offset	(C.XX)

–Rewrite the values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset in all the referenced PPS NAL units to be equal to 0.

[…]

–	Otherwise (sps_num_subpics_minus1 is equal to 0, the following applies.

–	The variables curLayerScalWinWidthcurLayerScalWinHeight, subpicTopOffsetToScalWin, scalFactorH, and  scalFactorV are derived as follows:

curLayerScalWinWidth = pps_pic_width_in_luma_samples −
	pps_scaling_win_left_offset −  pps_scaling_win_right_offset			(C.XX)

curLayerScalWinHeight = pps_pic_height_in_luma_samples −
	pps_scaling_win_top_offset − pps_scaling_win_bottom_offset			(C.XX)

scalFactorH = curLayerScalWinWidth ÷ subpicLayerScalWinWidth		(C.XX)

scalFactorV = curLayerScalWinWidth ÷ subpicLayerScalWinHeight		(C.XX)

–	Rewrite the values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset in all the referenced PPS NAL units to be equal to pps_scaling_win_left_offset + subPicLeftOffsetToScalWin * scalFactorH, pps_scaling_win_right_offset + subPicRightOffsetToScalWin * scalFactorH, pps_scaling_win_top_offset + subPicTopOffsetToScalWin * scalFactorV, and pps_scaling_win_bottom_offset + subPicBottomOffsetToScalWin * scalFactorV, respectively.

Item 2) RprConstraintsActive[ i ][ j ] derivation mismatches

The variable RprConstraintsActive[ i ][ j ] is derived in the VVC draft specification to control constraints on TMVP (the collocated index pictures shall not use RPR) as well as to derive enable flags for PROF and motion vector wraparound on decoder-side.

An issue arises, when the extraction of subpictures changes the result of RprConstraintsActive[ i ][ j ] derivation because picture sizes or scaling window offsets change so that

- the signalled collocated reference picture index for temporal motion vector prediction sh_collocated_ref_idx becomes invalid.

- cbProfFlagLX derivation result changes

- refWraparoundEnabledFlag derivation results change

In order to avoid the above issues to occur after extraction, it is proposed to change the derivation of RprConstraintsActive[ i ][ j ] to incorporate the number of subpictures in reference picture and current picture and set RprConstraintsActive[ i ][ j ] to one if both values differ (e.g., the reference layer has sps_num_subpics_minus1 is equal to 0 but the current layer has a value larger than 0). Thereby, TMVP, PROF and MvWraparound would already be prohibited in the original bitstream and no mismatch occurs after extraction.

Otherwise, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ], the reference picture scaling ratios RefPicScale[ i ][ j ][ 0 ] and RefPicScale[ i ][ j ][ 1 ], and the reference picture scaled flags RprConstraintsActive[ 0 ][ j ] and RprConstraintsActive[ 1 ][ j ] are derived as follows.

for( i = 0; i < 2; i++ ) {
	for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
		[…]
		fRefWidth is set equal to PicOutputWidthL of the reference picture RefPicList[ i ][ j ]
		fRefHeight is set equal to PicOutputHeightL of the reference picture RefPicList[ i ][ j ]
		refPicWidth, refPicHeight, refScalingWinLeftOffset, refScalingWinRightOffset, refScalingWinTopOffset,
			and refScalingWinBottomOffset, are set equal to the values of pps_pic_width_in_luma_samples,
			pps_pic_height_in_luma_samples, pps_scaling_win_left_offset, pps_scaling_win_right_offset,
			pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset, respectively, of the reference picture
				RefPicList[ i ][ j ],
			fRefNumSubpics is set equal to sps_num_subpics_minus1 of the reference picture RefPicList[ i ][ j ]
		RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL  >>  1 ) ) / PicOutputWidthL
		RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL  >>  1 ) ) / PicOutputHeightL
		RprConstraintsActive[ i ][ j ] = (	 pps_pic_width_in_luma_samples  !=  refPicWidth  | |
				pps_pic_height_in_luma_samples  !=  refPicHeight  | |
				pps_scaling_win_left_offset  !=  refScalingWinLeftOffset  | |
				pps_scaling_win_right_offset  !=  refScalingWinRightOffset  | |
				pps_scaling_win_top_offset  !=  refScalingWinTopOffset  | |
				pps_scaling_win_bottom_offset  !=  refScalingWinBottomOffset | |
				sps_num_subpics_minus1 !=  fRefNumSubpics 
	}
}

Item 3) OLS-specific and layer-specific constraints

The subpicture level information SEI message carries the syntax element sli_ref_level_fraction_minus1[ i ][ j ] which is used to express the fraction of a reference level (e.g. general level idc of the bitstream) that is associated to a subpicture. This value is used in the extraction process to set the level_idc of the extracted bitstream correctly but it also is used to enforce the level constraints such as number of tiles, CPB size, bitrate and so on. Instead of explicit signaling, sli_ref_level_fraction_minus1[ i ][ j ] can also be inferred from the area that a subpicture occupies in the original coded picture before extraction.

It is asserted that for bitstreams that contain subpictures and non-subpicture layers, applying such an inferred fraction to all layers and for all level constraints would lead to undesirable results, as

- bitstream-specific level limits such as luma samples per second, bitrate or CPB size are over-constrained by disregarding the non-subpicture layer contribution to inferred sli_ref_level_fraction_minus1[ i ][ j ].

- layer-specific limits such as for the number of tiles or coded picture aspect ratio are tightened for unchanged non-subpicture layers.

Taking, for instance, the example from Figure 1, it hard to reason why tighter layer-specific constraints should apply to L0 after extraction, e.g. regarding the number of tiles. Also, it is clear that the portion that L0 contributes to the bitstream with respect to OLS-specific constraints such as bitrate remains unchanged after extraction.

Therefore, in order to solve the issues, it is proposed to indicate in the subpicture level information SEI message an additional fraction non_subpic_layers_fraction[ i ] that indicates the fraction of the OLS-specific limits such as bitrate that the non-subpicture layers within the OLS occupy.

In this proposal, layer-specific limits apply only to layers with subpictures based on the signalled or inferred sli_ref_level_fraction_minus1[ i ][ j ] and OLS-specific limits are imposed on the bitstream containing all layers based on a derived fraction OlsRefLevelFraction based on sli_ref_level_fraction_minus1[ i ][ j ] and non_subpic_layers_fraction[ i ] as follows.

| subpic_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| num_ref_levels_minus1 | u(3) |
| sli_cbr_constraint_flag | u(1) |
| explicit_fraction_present_flag | u(1) |
| if( explicit_fraction_present_flag ) |  |
| sli_num_subpics_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| sli_alignment_zero_bit | f(1) |
| for( i = 0; i <= num_ref_levels_minus1; i++ ) { |  |
| non_subpic_layers_fraction[ i ] | u(8) |
| ref_level_idc[ i ] | u(8) |
| if( explicit_fraction_present_flag ) |  |
| for( j = 0; j <= sli_num_subpics_minus1; j++ ) |  |
| ref_level_fraction_minus1[ i ][ j ] | u(8) |
| } |  |
| } |  |

non_subpic_layers_fraction[ i ] specifies the fraction of the bitstream level limits associated with layers in the bitstream that have sps_num_subpics_minus1 equal to 0. When vps_max_layers_minus1 is equal to 0 or when no layer in the bitstream has sps_num_subpics_minus1 equal to 0, non_subpic_layers_fraction[ i ] shall be equal to 0.

The variable OlsRefLevelFraction[ i ][ j ] of the j-th subpicture for the i-th ref_level_idc is set equal to non_subpic_layers_fraction[ i ]+ (256 − non_subpic_layers_fraction[ i ]) ÷ 256 * (ref_level_fraction_minus1[ i ][ j ] + 1).

The variables SubpicCpbSizeVcl[ i ][ j ] and SubpicCpbSizeNal[ i ][ j ] are derived as follows:

SubpicCpbSizeVcl[ i ][ j ] = Floor( CpbVclFactor * MaxCPB * OlsRefLevelFraction[ i ][ j ] ÷ 256)	(D.6)

SubpicCpbSizeNal[ i ][ j ] = Floor( CpbNalFactor * MaxCPB * OlsRefLevelFraction[ i ][ j ] ÷ 256)	(D.7)

with MaxCPB derived from sli_ref_level_idc[ i ] as specified in clause A.4.2.

The variables SubpicBitRateVcl[ i ][ j ] and SubpicBitRateNal[ i ][ j ] are derived as follows:

SubpicBitRateVcl[ i ][ j ] = Floor( CpbVclFactor * ValBR * OlsRefLevelFraction[ i ][ j ] ÷ 256)	(D.8)

SubpicBitRateNal[ i ][ j ] = Floor( CpbNalFactor * ValBR * OlsRefLevelFraction[ i ][ j ] ÷ 256)	(D.9)

[…]

It is a requirement of bitstream conformance that each layer in the bitstream resulting from extracting the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1 and sps_num_subpics_minus1 greater than 0, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to ref_level_idc[ i ] for i in the range of 0 to num_ref_level_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	Ceil( 256 * SubpicSizeY[ j ] ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1 for level ref_level_idc[ i ].

–	The value of Ceil( 256 * ( sps_subpic_width_minus1[ j ] + 1 ) * CtbSizeY ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of Ceil( 256 * ( sps_subpic_height_minus1[ j ] + 1 ) * CtbSizeY ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of SubpicWidthInTiles[ j ] shall be less than or equal to MaxTileCols and of SubpicHeightInTiles[ j ] shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1 for level ref_level_idc[ i ].

–	The value of SubpicWidthInTiles[ j ] * SubpicHeightInTiles[ j ] shall be less than or equal to MaxTileCols * MaxTileRows * RefLevelFraction[ i ][ j ], where MaxTileCols and MaxTileRows are specified in Table A.1 for level ref_level_idc[ i ].

It is a requirement of bitstream conformance that the bitstream resulting from extracting the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to ref_level_idc[ i ] for i in the range of 0 to num_ref_level_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	The sum of the NumBytesInNalUnit variables for AU 0 corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * ( Max(SubpicSizeY[ j ], fR * MaxLumaSr * OlsRefLevelFraction[ i ][ j ] ÷ 256 ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * RefLevelFraction[ i ][ j ] ) ÷ ( 256 * MinCr ) for the value of SubpicSizeInSamplesY of AU 0, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0, at level sli_ref_level_idc[ i ], and MinCr is derived as indicated in A.4.2.

–	The sum of the NumBytesInNalUnit variables for AU n (with n greater than 0) corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) * OlsRefLevelFraction[ i ][ j ] ÷ ( 256 * MinCr ), where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to AU n, at level sli_ref_level_idc[ i ], and MinCr is derived as indicated in A.4.2.

Item 4) accumulated level in subpicture coded

Equation D.10 and associated text in the SLI SEI message semantics derives accumulated variables for level fraction, CPB size, bitrate and number of tiles etc. for a set of subpictures. This section was introduced when the SLI message was initially integrated and subpicture conformance was not yet decided upon. Ultimately, the groups decision was to provide conformance information only for individual subpictures and it seems that there is not use in the specification of a subpicture set. Further, the accumulation is not appropriately designed for the current status of subpictures and scalability in the specification.

Therefore, it is proposed to do either:

Option 1) Remove the accumulation and adjust D.11 to use a single subpicture.

Option 2) Update D.10 to properly work with the above proposed change

Item 4 Option 1) Remove the accumulation and adjust D.11 to use a single subpicture.

The value of the subpicture set sequence level indicator, SubpicSetLevelIdc, is derived as follows:

SubpicSetLevelIdc = general_level_idc 
SubpicSetLevelIdx = 0
for (i = num_ref_level_minus1; i  >=  1; i− − )
	if( SubpicSetNumTiles[ i ]  <=  ( MaxTileCols * MaxTileRows )  &&	(D.11)
			SubpicSetAccOlsLevelFraction[ i ]  <=  256 ) {
		SubpicSetLevelIdc = sli_ref_level_idc[ i ] 
		SubpicSetLevelIdx = i
	}

Item 4 Option 2) Update D.10 to properly work with the above proposed change

For any subpicture set containing one or more subpictures and consisting of a list of subpicture indices SubpicSetIndices and a number of subpictures in the subpicture set NumSubpicsInSet, the level information of the subpicture set is derived as follows.

The variable SubpicSetAccLevelFraction[ i ] for the total level fraction with respect to the reference level sli_ref_level_idc[ i ] derived as follows.

for (i = 0; i  <=  num_ref_level_minus1; i ++) {
	SubpicSetAccLevelFraction[ i ] = 0
	for (j = 0; j < NumSubpicsInSet; j ++) {
		CurrSubpicIdx = SubpicSetIndices[ j ]
		SubpicSetAccLevelFraction[ i ]  +=  RefLevelFraction[ i ][ CurrSubpicIdx ]	(D.10)
	}
}

The variable OlsSubpicSetRefLevelFraction[ i ] of the subpicture set for the i-th ref_level_idc is set equal to non_subpic_layers_fraction[ i ]+ (256 − non_subpic_layers_fraction[ i ]) ÷ 256 * (SubpicSetAccLevelFraction[ i ]).

The variables SubpicSetCpbSizeVcl[ i ], SubpicSetCpbSizeNal[ i ], SubpicSetBitRateVcl[ i ], and SubpicSetBitRateNal[ i ] of the subpicture set, are derived according to D.6, D.7, D.8 and D.8, respectively with OlsSubpicSetRefLevelFraction[ i ] instead of OlsRefLevelFraction[ i ][ j ].

The value of the subpicture set sequence level indicator, SubpicSetLevelIdc, is derived as follows:

SubpicSetLevelIdc = general_level_idc 
SubpicSetLevelIdx = 0
for (i = num_ref_level_minus1; i  >=  1; i− − ) {
	if( SubpicSetNumTiles[ i ]  <=  ( MaxTileCols * MaxTileRows )  &&	(D.11)
			OlsSubpicSetRefLevelFraction[ i ]  <=  256 ) {
		SubpicSetLevelIdc = sli_ref_level_idc[ i ] 
		SubpicSetLevelIdx = i
	}
}

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)