| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0101 |
| --- | --- |

| Title: | AHG9: On HRD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution discusses two aspects related to HRD. This first one corresponds to an asserted bug in equation C-10 (also applicable to HEVC). The second aspect discusses about making the sum of InitialCpbRemovalDelay and InitialCpbRemovalOffset constant across CVSs unless signalled otherwise.

# Introduction

This contribution discusses two aspects related to HRD:

- An asserted bug in equation C-10 (also applicable to HEVC).

- An aspect related to the sum of InitialCpbRemovalDelay and InitialCpbRemovalOffset being constant across CVSs.

Item 1) Equation C-10

The CPB removal time for AUs that are the first AU in a buffering period SEI that are not AU0 are computed as follows.

Without splicing, when concatenationFlag is equal to 0, the removal time is computed as for any other access unit, i.e. the AuCpbRemovalDelayVal in the PT SEI message is added to the CPB removal time of firstPicInPrevBuffPeriod, i.e. the removal time of the previous AU with a BP SEI message that started the buffering period.

Otherwise, when concatenationFlag is equal to 1 after a splicing event, two values are compared. The first value is computed adding a delta (auCpbRemovalDelayDeltaMinus1 + 1) as signalled in the BP SEI message to the removal time of the previous non discardable picture (prevNonDiscardablePic). The second value is computed adding InitialCpbRemovalDelay to the final arrival time of the previous picture (which is the same as the earliest arrival time of the AU with the new BP SEI message). If the second value is larger than the first one, the AU with the new BP SEI message cannot be removed from the CPB with the removal time indicated by the first value (as it is not complete by then) but the second value needs to be used as removal time.

The different cases are illustrated in the following figure:

Figure 1: Removal time derivation for a first AU in BP that is not AU0.

The asserted bug in both HEVC and VVC is that a delta (CpbDelayOffset) is incorrectly subtracted from the removal times after derivation of the removal times as illustrated above in the case that alternative timing is used, e.g. removal of RASL pictures.

AuNominalRemovalTime[ n ] = baseTime + ( ClockTick * tmpCpbRemovalDelay − CpbDelayOffset )

Note that when concatenationFlag is equal to 1, the previous non-discardable flag has already accounted for the CpbDelayOffset in the derivation of its removal time and that in case 2 the removal time cannot be earlier than computed as this corresponds to the initialCpbRemovalDelay necessary for the spliced bitstream.

Figure 1: Removal time derivation for a first AU in BP that is not AU0 with CpbDelayOffset.

Therefore, the proposal is to only use the “correction delta” CpbDelayOffset when concatenationFlag is equal to 0.

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
	tmpCpbDelayOffset = CpbDelayOffset
} else {
	baseTime1 = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 =				(C.10)
			Ceil( ( InitCpbRemovalDelay[ Htid ][ ScIdx ] ÷ 90000 +
				AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick )
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 <
				baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
	tmpCpbDelayOffset = 0
}
AuNominalRemovalTime[ n ] = baseTime + ( ClockTick * tmpCpbRemovalDelay − tmpCpbDelayOffset

Item 2) sum of InitialCpbRemovalDelay and InitialCpbRemovalOffset

The current specification mandates the following.

Over the entire CVS, for each value pair of i and j the sum of bp_vcl_initial_cpb_removal_delay[ i ][ j ] and bp_vcl_initial_cpb_removal_offset[ i ][ j ] shall be constant, and the sum of bp_vcl_initial_alt_cpb_removal_delay[ i ][ j ] and bp_vcl_initial_alt_cpb_removal_offset[ i ][ j ] shall be constant.

It is asserted that in a non-spliced bitstream, IDRs should not allow changing the sum of bp_vcl_initial_cpb_removal_delay[ i ][ j ] and bp_vcl_initial_cpb_removal_offset[ i ][ j ] for each value pair of i and j similar as CRAs do not, as it is a property of the frame distance in the source video. Furthermore, the initial removal delay and initial removal offset for one buffering period could correspond to alternative removal times and be the “default” timings for a following buffering period.

Therefore, it is proposed to add a constraint in Annex C in section C.2.3 as follows:

When bp_concatenation_flag is equal to 0, the sum of InitCpbRemovalDelay[ Htid ][ ScIdx ] and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] shall be the same as in the previous buffering period.

Note also that C.2.3 has an editor note on why InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] is mentioned in C.2.3. As this value needs to be updated with each buffering period SEI that does not correspond to AU0 the derivation should be kept as this updated value needs to be used for the derivation of earliest arrival time of following AUs. Especially, when through splicing the value of the sum is changed for a bitstream.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)